___________________________________________________________________________

Power Glove Interface for Parallel Port on IBM

___________________________________________________________________________

Nintendo connector layout:   *1      1:Ground           2:Clock

7*  *2      3:Latch            4:Data

6*  *3      5:N/C (lightgun)   6:N/C (lightgun)

5*  *4      7:+5V

Connecting to Parallel Port

Glove        Printer Port

1            18 GND

2            2 DO

3            3 D1

4            13 SLCT (input)

___________________________________________________________________________

POWER

___________________________________________________________________________

Connect pin 7 on the glove to any location with +5V on the computer.

Pin 1 on any gameport is a good place.  You can also use the keyboard

connector.  On an XT pinout, the +5V connector is pin 5 as follows:

Looking at the back of the computer:

3    1

5    4

2

Be sure to check the voltage for any other configuration!

___________________________________________________________________________

The CABLE

___________________________________________________________________________

You can buy Nintendo extender cable to cut up for the connector... Get the

longest possible for easy location of the control box...

___________________________________________________________________________

The CODE

___________________________________________________________________________

If you don't get steady sampling on the glove, try adjusting the timing on

your machine by changing the N & D defines...

The Code is currently set for LPT2. (Inport=0x279, Outport=0x278).  For

LPT1, change Inport to 0x379 and Outport to 0x378...

The code is written for Turbo C/C++/Borland C++.  It uses the BGI library

for the graphics cursor, so it is easily ported to Microsoft C.

The code is named THEGLOVE.C

___________________________________________________________________________

NOTES

___________________________________________________________________________

This is reprinted from Waite Group's "Virtual Reality Playhouse", but they

swiped it from the VR boys on USENET so I don't feel bad about porting it.

Please distriute this freely...

___________________________________________________________________________