----==================================----

----====< DOS/WINDOWS VIRTUAL SB >====----

----====< BETA RELEASE SOFTWARE  >====----

----====<     Version 0.87	 >====----

----==================================----

What is the Virtual Sound Blaster?

The Virtual Sound Blaster is a 386 driver for

Windows 3.0. It allows SB specific code to run

on the Pro Audio Spectrum from either Windows

or 386 DOS shells.

How does it work?

By using the 386 hardware virtualization, this

code traps all low-level reads/writes to the SB

and makes the appropriate translation to the

PAS card. Both DOS and Windows SB applications

will be able to use the Virtual SB.

Are there any special conditions needed to run it?

This code is currently incompatible with the

Media Vision Multi Media drivers. Basically,

a device contention occurs when the MV drivers

try to use the hardware at the same time a

virtual SB session is running.

How Do I install the Virtual Sound Blaster?

PLEASE follow these instructions. You may be

sorry later if things dont work out.

1. As explained in "READ.ME", unpack the

the contents of SB2MV.zip

2. Copy SB2MV.386 to your Windows system

DOS>copy SB2MV.386 c:\windows\system

3. Change directories to the Windows

subdirectory. Typically, like this:

DOS>cd c:\windows\system

4. Make a backup copy of your SYSTEM.INI!!!

Typically, like this:

DOS>copy system.ini sysbk

This is critical in case the Windows

configuration is ever corrupted. You will

be able to restore everything back to

the original configuration just by

replacing the current SYSTEM.INI with the

backup copy.

5. Edit the SYSTEM.INI file using a text

editor. Add the next two lines:

DEVICE=SB2MV.386

DMABUFFERSIZE=064

to the end of following area:

[386Enh]

These two entries must be the last two

entries in this section.

At the Bottom of the SYSTEM.INI file,

insert the following lines:

[mvwave.drv]

dma=1

int=5

port=220h

mpu401=0

If you have already installed the Multi-

Media Extensions to Windows, the above

section may already exist. Just add the

"port=" and "mpu401=" lines. The MPU-401

emulation is not yet complete, so let it

equal zero. This will disable virtualizing

the MPU-401 I/O addresses.

6. Start Windows, then remove any PAS

Multi-Media drivers.

How do I run Wing Commander II?

You must have the following drivers loaded in

your config.sys:

HIMEM.SYS

EMM386.SYS

Checkout the example CONFIG.SYS later in this

file.

There must be 1.5 to 2 megabytes of expanded

memory to run Wing Commander using the speech

pack. Once installed, run Windows and shell

out to a DOS VM. Check to see how much memory

is installed. There must be at least 584k

available. I had to update to DOS 5.0, unload

most drivers and TSRs from config.sys and

autoexec.bat before I had enough memory.

Here is a very critical performance requirement.

Use the example GAMES.PIF included in this ZIP

to run Wing Commander II. This PIF specifies the

memory requirements, but more inportant, tells

Windows not to monitor the video I/O. Most of

the performance loss will occur due to wrong

PIF settings. GAMES.PIF gives requests all the

system resources to be used exclusively to this

DOS shell.

What other software works with the Virtual Sound Blaster?

Disney's Mickey Mouse Jigsaw.

Gary Maddox's Blaster Master.

Dave Young's Silly String.

Creative Labs' VPLAY.EXE.

Creative Labs' VREC.EXE .

Creative Labs' Dr. Sbaitso.

Creative Labs' Talking Parrot.

Sierra On-Line Leisure Suite Larry, I.

PG Music's Band In a Box.

and more...

What doesn'nt work?

Broderbund's Carmen San Diego, Tree House, and

KidPix. To use these games, run the DOS

Virtual Sound Blaster TSR.

Creative Labs' VOXKIT.EXE

What else is needed?

As mentioned above, you may need DOS 5.0 to

make enough memory available in the DOS shell.

Also, you MUST use the example PIF to get the

most performance out of the DOS shell.

How do I restore the Windows setup to it's original

configuration?

You can easily restore the configuration by

copying the backed up copy SYSTEM.INI into the

Windows subdirectory; thereby, overwriting

the current file.

The best approach would be to have two backup

copies of SYSTEM.INI. One that is the original

and the other having the Virtual SB installed.

To run either configuration, just copy the

correct file into the Windows subdirectory, then

fire up Windows.

My CONFIG.SYS file:

|

|device=c:\dos\setver.exe

|device=c:\dos\himem.sys

|device=c:\mwindows\emm386.sys 2048 frame=e000

|lastdrive=h

|files=20

|buffers=20

|shell=c:\dos\command.com c:\dos\ /e:512 /p

|DOS=HIGH

|

My SYSTEM.INI file:

|

|[boot]

|display.drv=mmv7vga.drv

|shell=progman.exe

|sound.drv=SOUND.DRV

|;;;;sound.drv=mmsound.drv

|sound.drv=sound.drv

|386grabber=v7vga.gr3

|286grabber=vgacolor.gr2

|network.drv=

|language.dll=

|fixedfon.fon=vgafix.fon

|comm.drv=comm.drv

|oemfonts.fon=vgaoem.fon

|fonts.fon=vgasys.fon

|mouse.drv=mouse.drv

|keyboard.drv=keyboard.drv

|system.drv=system.drv

|drivers=mmsystem.dll

|

|

|[drivers]

|;;;wave=tbwave.drv

|

|;;;[mci]

|;;;WaveAudio=mciwave.drv

|

|[keyboard]

|subtype=

|type=4

|oemansi.bin=

|keyboard.dll=

|

|[boot.description]

|display.drv=Video 7 VGA VRAM I 640x480 with 256 colors

|network.drv=Network not installed

|language.dll=English (American)

|keyboard.typ=Enhanced 101 or 102 key US and Non US keyboards

|mouse.drv=Microsoft, or IBM PS/2

|system.drv=MS-DOS or PC-DOS System

|

|[386Enh]

|ebios=*ebios

|display=*vddvga

|keyboard=*vkd

|mouse=*vmd

|network=*vnetbios, *dosnet

|device=*vpicd

|device=*vtd

|device=*reboot

|device=*vdmad

|device=*vsd

|device=*v86mmgr

|device=*pageswap

|device=*dosmgr

|device=*vmpoll

|device=*wshell

|device=*vhd

|device=*vfd

|device=*vpd

|device=*parity

|device=*biosxlat

|device=*vcd

|device=*vmcpd

|device=*combuff

|device=*cdpscsi

|local=CON

|FileSysChange=off

|DMABUFFERSIZE=064

|device=sb2mv.386

|

|CGA40WOA.FON=CGA40WOA.FON

|CGA80WOA.FON=CGA80WOA.FON

|EGA40WOA.FON=EGA40WOA.FON

|EGA80WOA.FON=EGA80WOA.FON

|

|[standard]

|

|[NonWindowsApp]

|

|[Multimedia.Setup]

|;;;audio=mvwave.drv,0,0,0

|display=v7vga

|

|[mvwave.drv]

|dma=1

|int=5

|port=220h

|mpu401=0

|

|[mvmidi.drv]

|int=5

|

|[tbwave.drv]

|port=220

|int=2

|