PrintGL  1.56

(c) Copyright Ravitz Software Inc. 1990,1995

PrintGL/D - pen plotter emulator for DOS

PrintGL/W - pen plotter emulator for Windows

PrintCAD  - printer driver for DOS CAD software

Ravitz Software Inc.                BBS/fax               606-268-0577

PO Box 25068

Lexington, KY  40524-5068           Compuserve  Cary Ravitz [70431,32]

USA                                 Internet   70431.32@compuserve.com

------------------------------ License -------------------------------

PrintGL (including PrintGL/D, PrintGL/W, and PrintCAD) is provided  as

is.  There  are no  warranties expressed or  implied.  PrintCAD  is  a

swapping   resident  program  that  may  have  problems  with  unusual

environments.

PrintGL is copyrighted by Ravitz Software Inc.  You  may copy  PrintGL

only for use under this license agreement.

PrintGL is distributed  as  shareware.  You  may use  PrintGL  without

charge on a trial basis to  determine its suitability for you.  If you

continue  to  use  it after  your  evaluation,  you  MUST  purchase  a

registered copy for  $50 (US$).  Registration  covers use by a  single

person  (on  multiple computers) or installation on a  single computer

(may  be  used by a group of people).  There is no cost to use a later

1.** version after registering any 1.** version.

You  may distribute PrintGL as a stand alone product if you  keep  the

entire  package  together,  unchanged,  clearly  label   the  disk  as

shareware,  and do not charge more than  $10.  You  may not distribute

PrintGL as  part  of another  product or  on  the  same disk with  any

commercial software.

See Order Form in PRINTGL.TXT for ordering information.

------------------------------ Contents ------------------------------

PrintGL Initial Information File ......................... PRINTGL.TXT

PrintGL Supplemental Documentation File                   PRINTGL2.DOC

Introduction ....................................................... 3

License and Registration                                             5

Installation ....................................................... 6

Operation                                                            7

How To Get HP-GL Plotfiles From Your Graphics Software ............. 9

Messages                                                             9

User Interface .................................................... 10

Example Setup Instructions For PrintCAD                             11

Overview of Options ............................................... 12

/1, /2, /3, /4, /5 - PrintGL/D and PrintGL/W Default Options        12

/1, /2, /3, /4, /5 - PrintCAD Alternate Configurations ............ 12

/A - Position Option                                                13

/B - Tiling Option ................................................ 13

/C - Pen Color Option                                               13

/D - Output Destination Option .................................... 15

/F - Output Format Option                                           15

/I - Scaling Point Location Option ................................ 18

/J, /K - Printer Code Prefix, Suffix Options                        18

/L - Page Layout Option ........................................... 19

/M - Magnification Option (Scale, Size)                             20

/N - Page Number Option ........................................... 20

/O - Origin and Orientation Option                                  20

/P - PrintGL/D and PrintGL/W Plotfile Option ...................... 21

/P - PrintCAD Intercept Mask Option                                 21

/Q - Number of Copies Option ...................................... 21

/R - Internal Resolution Option                                     21

/S - Pen Shading Option ........................................... 22

/T - Temporary File Option                                          22

/W - Pen Width Option ............................................. 22

/X - PrintGL/D and PrintGL/W User Interface Option                  23

/X - PrintCAD Control Option ...................................... 23

/Y - Plotter Options                                                24

/Z - Processing Options ........................................... 24

PrintGL/D and PrintGL/W Menu                                        25

Menu Specific Functions ........................................... 25

Menu Input and Output Choices                                       26

Menu Processing Options ........................................... 26

Menu Pen Options                                                    26

Menu Page Layout Options .......................................... 27

Menu Other Options                                                  27

Menu Auto Run Mode ................................................ 27

Large Paper                                                         28

Tips On AutoCAD ................................................... 29

Color Processing                                                    29

Compatibility and Technical Information ........................... 30

HP-GL, HP-GL/2, and ADI Commands                                    30

Output Devices (Displays, Printers, Plotters, Bit Maps, Fax) ...... 31

Useful Prefix and Suffix Codes                                      38

Answers ........................................................... 39

PrintCAD Answers                                                    40

User Support ...................................................... 40

p-3

Introduction ---------------------------------------------------------

PrintGL is a  shareware  pen plotter  emulator package  for  DOS and

Windows.  It includes a DOS program that can be run from the command

line  and BAT  files or via menu  (PrintGL/D), an equivalent Windows

program (PrintGL/W),  and a  resident DOS  program  that  intercepts

plotter data as it is being written to file (PrintCAD).

PrintGL/D  is  a  command line or menu driven program  for  DOS.  It

prints  or displays HP-GL (Hewlett-Packard Graphics Language,  7475,

17440 subsets), HP-GL/2 (limited subset), or  ADI  (Autodesk  Device

Interface 4.0  vector plotter  binary format)  plotfiles  on most PC

graphics  devices.  HP-GL  is  widely  supported  by  CAD,  drawing,

plotting,   and   other   graphics   programs   including   AutoCAD,

Microstation,  FastCAD, Generic CADD, MathCAD,  Schema,  OrCAD,  and

SAS.  PrintGL will  print on  Epson and IBM 9 pin  printers,  Epson,

Fujitsu,  IBM, NEC, and Toshiba 24 pin printers, Canon BJ,  BJC, and

LBP  printers,  Epson  Stylus,  HI  JetPro,  HP  Laserjet,  Deskjet,

PaintJet, and HP-GL,  HP-GL/2,  and HP-RTL devices,  IBM ExecJet and

LaserPrinter, PDP ProTracer, Star  SJ-144, PostScript printers,  and

others.  It will  also display  plots with a CGA, EGA, HGC, VGA,  or

VESA SVGA, and  output a PCX or DCX bit map file or an  Encapsulated

PostScript file with preview bit map.

PrintGL/W is a Windows version of PrintGL/D.

PrintCAD  is a resident pen plotter  emulator that acts as a printer

driver for most DOS based CAD packages.  It lets  you print drawings

directly from CAD programs, via  the CAD program's  HP-GL or HP-GL/2

driver or AutoCAD's ADI plotter driver. PrintCAD works with AutoCAD,

AutoSketch,  Microstation,  Cadkey,  FastCAD,  Generic  CADD,  Micro

CADAM, Draft Choice,  PC-Draft-CAD, and other CAD programs.  It will

print on the same printers noted above but does not do screen output

or  generate  PostScript preview bit maps.  PrintCAD  is set  up via

command line (or BAT file).  PrintCAD is a swapping resident program

that uses 5K of base memory and 400K  of EMS or XMS  memory.  Either

400K  of  EMS  or XMS  memory  is required and  a  VCPI  handler  is

recommended.

Even if  your  graphics  program  supports  your  printer,  you will

probably  find  that PrintGL is faster, gives better  print quality,

and/or  gives more  formatting  flexibility.  PrintGL uses  the best

graphics modes available for  each printer that it supports and uses

transfer data  compression to improve print speed  on many printers.

And it gives options such  as orientation,  magnification, position,

and pen  width, color, and  shading.  If  you need  printed graphics

output from personal software, you can output  HP-GL and use PrintGL

to do the printing.  This gives immediate support to a wide range of

printers.  And  it lets  you use advanced graphics  functions  while

supporting low function plotters.

continued

p-4

PrintGL  interprets all of the HP 7475 and 7440/17440 (ColorPro with

GEC)  command  set  that  is  applicable  to  plotfiles  except  for

character set 8 (Katakana).  None of the digitize, output, or device

control commands (except plotter  on and off) are supported as these

are only used with  direct control of a plotter.  And the error mask

command is not supported.  In addition to the HP 7475 and 7440/17440

commands, PrintGL handles paper feed, label origin, extra space, and

enhanced clipping window  commands and  proportional fonts  and line

types -8..-1, 7, and 8.

PrintGL's HP-GL/2 subset  includes pen color, shading, transparency,

and width control, compressed data, a few other useful functions. It

does not include line attributes, user defined line types and fills,

three point  arcs,  Bezier  curves,  and  HP-PCL and  HP-RTL related

functions.

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to  make  sure  that  the  shareware

principle  works   for  you.   If  you   are  unable  to  resolve  a

shareware-related  problem  with  an ASP  member  by  contacting the

member directly, ASP may be able to help. The ASP Ombudsman can help

you resolve a  dispute or problem  with an  ASP member, but does not

provide technical support for members' products. Please write to the

ASP  Ombudsman at  545  Grover Road,  Muskegon, MI 49442  or send  a

CompuServe message via CompuServe Mail to ASP Ombudsman 70007,3536.

p-5

License and Registration ---------------------------------------------

PrintGL (including PrintGL/D, PrintGL/W, and PrintCAD)  is  provided

as is.  There are no warranties expressed or implied.  PrintCAD is a

swapping  resident  program  that  may  have  problems with  unusual

environments.

PrintGL is copyrighted by Ravitz Software Inc.  You may copy PrintGL

only for use under this license agreement.

PrintGL  is  distributed as shareware.  You may use PrintGL  without

charge on a trial basis to determine its suitability for you. If you

continue to  use  it after  your  evaluation,  you  MUST purchase  a

registered copy for $50 (US$).  Registration covers  use by a single

person (on  multiple computers) or installation on a single computer

(may be used by a group of people).  There is no cost to use a later

1.** version after registering any 1.** version.

You may distribute PrintGL as a stand alone product if you  keep the

entire  package  together,  unchanged,  clearly label  the  disk  as

shareware,  and do not charge more than $10.  You may not distribute

PrintGL  as  part of another product or  on the same  disk with  any

commercial software.

See Order Form in PRINTGL.TXT for ordering information.

PrintGL  Printer  Driver  is  the  graphics  engine  used  in  these

programs. It may be licensed for use in commercial programs.  Send a

note to request more information on this.

p-6

Installation ---------------------------------------------------------

The PrintGL package includes these files:

FILE_ID.DIZ   standard description of package

PRINTCAD.EXE  resident DOS program

PRINTGL.DOC   documentation

PRINTGL.PLT   sample plotfile

PRINTGL.TXT   initial information

PRINTGL2.CHR  HP-GL character set plotfile

PRINTGL2.DOC  supplemental documentation

PRINTGLD.EXE  DOS program

PRINTGLW.EXE  Windows program

README.BAT    displays the initial information file

To install PrintGL create a directory  called  PRINTGL, and  copy or

unpack the distribution  files into this directory.  It is preferred

that the PRINTGL directory be in your DOS PATH.

To install PrintGL/W as an  icon in  the  Windows  Program  Manager,

select  File, then New, then program  item  to bring up the  Program

Item Properties menu.  Enter the item  name - PrintGL/W, the command

line - C:\PRINTGL\PRINTGLW, and the working directory - wherever you

keep your plotfiles.  You can choose  from  three icons designed for

two, sixteen, and 32K color setups.

PRINTGL.TXT  is  initial  information.  You can view it from DOS  by

running README.BAT or from PrintGL by pressing F1. While viewing it,

you can print pages or the entire file. Press T repeatedly until the

correct setup is  displayed and  then press  P.  Or from  DOS, "COPY

PRINTGL.TXT PRN" to print the entire file.

PRINTGL.DOC  is the documentation.  This is viewed  and printed just

like PRINTGL.TXT.

PRINTGL2.DOC  is supplemental  documentation  for  supported  HP-GL,

HP-GL/2, and ADI commands and the program interface. PRINTGL2.CHR is

a plotfile that documents the supported HP-GL characters sets.

PRINTGL.PLT is a sample plotfile that you can  print or display with

PrintGL.

p-7

Operation ------------------------------------------------------------

PrintGL reads a plotfile, does arc,  character, fill, etc.  to  line

conversion,  stores the data in a  coordinate  list, and  if needed,

puts the list on disk. It then reorients the data and determines the

plot size. Next the data is resized and relocated. For vector output

formats (HP-GL,  HP-GL/2, PostScript) data is output from here.  For

bit   mapped  devices  (most  printers,  displays,   bit  maps)  the

coordinate  list is rasterized and output in swaths.  You are likely

to  see  the  printer  print  a  swath  and  stop  while PrintGL  is

processing the next swath.

Esc  stops PrintGL at the next good break point, leaving the printer

ready for new data. Ctrl-B stops PrintGL as quickly as possible. You

generally need to do a printer reset after a Ctrl-B.

When you  display a  plot, press Enter or  Esc  to return to the DOS

text screen,  or  any  of  the four  cursor movement  keys to scroll

across the plot.  Press S to cycle between full, half (the default),

quarter, and eighth page scrolling.

PrintGL  works within a  printer's current margins and starts at the

current vertical print position. If you decline the form feed option

(/L or "form feed") then PrintGL  leaves the printer at the start of

the line directly below the plot.

PrintGL will not automatically change a printer's operating mode. On

multimode printers,  the  right operating  mode must be  set  before

running  PrintGL or with  the  prefix  code  option (/J  or  "modify

output" "prefix codes: ").

Internally PrintGL  uses two byte  coordinates  on  a 1016 dots/inch

grid. This allows for plots with coordinates of over 30 inches - far

larger than an HP 7475 can handle. However, if you are using PrintGL

to emulate a large plotter, you may overflow the internal coordinate

system.  This condition  is  not checked, and  it will cause garbage

output.  You can change the internal resolution to 508 dots per inch

(giving 60 inch maximum plot sizes) with the /R option (/R508). This

only helps  store the  original plot  internally.  The  next step in

PrintGL converts  these  coordinates  to the  requested size  in the

resolution of the output device, and these coordinates must also fit

in the two byte coordinate list.

PrintCAD is a resident program that intercepts plot data output from

CAD software, processes the data, and prints the resulting  picture.

When PrintCAD is first run it installs itself.  Subsequent runs look

for the resident copy  and  send control information to it, updating

the parameters.

PrintCAD must  be installed before running the CAD software.  Do not

install it from a DOS shell.  PrintCAD automatically loads high if a

5K DOS or XMS upper memory block is available.  Do not use LOADHI or

LOADHIGH.

continued

p-8

PrintCAD  can  intercept data from  two sources, DOS file writes and

AutoCAD/Sketch ADI  4.0  interrupts.  With  the  option  /PADI,  ADI

interrupts are intercepted.  With /Pmask,  for example  /P*.PLT, DOS

file writes are intercepted.

PrintCAD can interpret two kinds of data,  HP-GL and AutoCAD ADI 4.0

binary plotter data.  HP-GL  data is always interpreted.  If you use

the /PADI or /YA options then ADI plotter data is  also interpreted.

PrintCAD  can only interpret plotter  data,  not printer/plotter  or

rendering data.

To  use PrintCAD as an HP-GL interpreter, use /Pmask to tell it what

files to intercept. Use /P*.PLT, /P*.DPF, /P*.HPG, /P*.000, etc.  to

match the names that the CAD software uses when writing HP-GL files.

Configure the CAD software  for  HP-GL  (HP 7550 or 7585) output  to

file.  Then plot (do  NOT print).  PrintCAD  intercepts the data and

prints it.

To use  PrintCAD  with AutoCAD/Sketch as  an  ADI  interpreter  that

intercepts files use /P*.PLT /YA.  Configure AutoCAD  for an ADI 4.0

plotter,  binary output to file, up to forty  pens, 1016 steps/inch,

and eight non-solid line types. Then plot.

To use PrintCAD  with AutoCAD/Sketch as an installed ADI 4.0 plotter

driver use /PADI. After installing PrintCAD configure AutoCAD for an

ADI 4.0 plotter,  installed at  interrupt hex 78, up to  forty pens,

1016 steps/inch, and eight non-solid line types. Then plot.

After  starting  PrintCAD, you  can  verify  the  installation  with

"printcad  /xq".  You can  check  its function from DOS  with  "COPY

PRINTGL.PLT PCAD.PLT".

PrintCAD  causes a  high/low beep  the  first time that data is sent

after a file open. If you turn on sound (/XS), it causes a high beep

after swapping in its code and a low beep before swapping it out, to

let you monitor its processing.

p-9

How To Get HP-GL Plotfiles From Your Graphics Software ---------------

To  use PrintGL you must be  able to  output HP-GL plotfiles or  ADI

binary plotfiles from your graphics application.  Most CAD, drawing,

and graphing software lets you do this. Just tell the application to

output  your  picture  to  a  plotter.  In  either the configuration

process or the output  process, you  will need  to  specify that you

have  an HP  7475 or  HP 7440/17440 plotter  and  that you want  the

output to go to a file instead of a COM port (some programs name the

file automatically and some ask  you for a name).  You might need to

set up a pen assignment table that specifies what color each pen is.

Once you have the file, you can print it with PrintGL/D, either from

a DOS shell or after exiting the application. With PrintGL/W you can

run  without exiting the  application.  With  PrintCAD  the  file is

intercepted and printed instead of being written to disk.

Messages -------------------------------------------------------------

PrintGL/D and  PrintGL/W display  a number of messages to  keep  you

informed of what they are doing. The output looks like this.

Reading PRINTGL.PLT

Plotfile has 1 page(s)    Processing page 1    2283 data records

Overall magnification    0.9964

Print window   Horizontal   0.00   7.54    Vertical   0.00  10.14

Plot area      Horizontal   0.42   7.13    Vertical   0.61   9.54

Output format /FT   HxV dpi 180x180 corrected to 180x180  color

Writing to device LPT3

The first  line  is the plotfile  that is currently being processed.

The second line tells how many pages are in the plotfile, which page

is being processed, and the number of data points in that page.  The

number of  data points  is roughly  equivalent  to the number of pen

moves and  is  shown  to  give  you  an  indication  of  the  plot's

complexity.

The  overall magnification is  either  the  magnification  that  you

specified  with the /M option or the  effective  magnification after

fitting to the print  window with /MF.  The  print window line gives

the horizontal  and vertical coordinates of the print window, always

starting  at 0,0 and in inches.  The plot area  gives the horizontal

and vertical coordinates of a box that just covers the plotted data.

This uses the same coordinate system as the print window, so you can

tell where the plot is relative to the print window.

The output format and  resolution are  given  and  then  the  output

destination is noted  (except for screen  plots).  If three  or four

plane color  is being used, the word  "color" is  displayed.  If you

have a black only printer, but the output format allows color  (this

is  common  with 9 and  24 pin  printers),  using color will greatly

increase the PrintGL run time with no improvement in print quality.

During processing,  PrintGL/D and PrintGL/W inform you when they are

processing and writing data, and you can watch  the progress from 0%

to 100%, along with the elapsed time.

After initial startup PrintCAD runs without screen output.

p-10

User Interface -------------------------------------------------------

PrintGL can be run from  the command  line  (DOS or  Windows)  using

various  options to tell it  what to  do  or run  with no options to

bring up a menu system.  The menu options have corresponding command

line  options,  so the  two  forms  of  options  will  be  described

together.  The menu options  are noted by  the  menu name in quotes,

sometimes followed by the menu choice in quotes.

From the command line, the calling syntax is:

PRINTGLD plotfile /option1 /option2 ...

PRINTGLW plotfile /option1 /option2 ...

PRINTCAD /option1 /option2 ...

Spaces between parameters  are  not required.  The  plotfile  may be

specified with  the /P option if it  is inconvenient to make it  the

first parameter.

Except for the  /X and /P options in PrintCAD,  if a given option is

entered more than  once, only the  last  entry is  used.  And  if an

option is  invalid (but not  incorrect), it is ignored.  Only one /X

and one /P option is allowed in PrintCAD.

When run  from the  command line,  PrintGL/D  and  PrintGL/W use the

current menu options as the default, so if  you have the menu set up

as desired, you can  run PRINTGLD plotfile from the command line and

avoid going through the menu to select the plotfile.

The DOS environment variables  PRINTCAD,  PRINTCAD2,  PRINTCAD3, and

PRINTCAD4 are read by PrintCAD and put  in front of the command line

parameters.  When you need to exceed the  127 character DOS  command

line limit, put some of the parameters in the PRINTCAD variable, for

example

SET PRINTCAD=/CRGB... /S... /W...

PRINTCAD /P*.PLT ...

SET PRINTCAD=

Below,  [ and ] enclose optional suboptions and | separates mutually

exclusive suboptions. The options and suboptions may be in any order

except  that  multiple numeric  suboptions are  ordered  and numeric

suboptions must follow character suboptions where the characters may

be digits. "c" refers to a suboption character, "x", "y", and "z" to

suboption numbers,  "i"  and  "j"  to  suboption integers, "b"  to a

suboption byte, and "f" to a DOS file or device name.

For  numbers (x, y, z), a  decimal  point  is  allowed  but  is  not

required, and scientific notation is not allowed. "-" is allowed but

not "+".  For  integer input  (i, j), only  base ten  integers  from

-32768 to 32767 are allowed.  "+" is not allowed. A byte (b) must be

a base ten integer  from 0 to 255  or a  hexadecimal number prefixed

with $  from  $00  to $FF.  Further range checking  is done for most

options to flag  unreasonable  input.  If  an  option  has  multiple

numeric suboptions, these may be separated with commas or blanks.

p-11

Example Setup Instructions For PrintCAD ------------------------------

Below  are instructions for simple setups  to  help  you get started

with PrintCAD.  After you have  PrintCAD  running,  there  are  many

additional options that might be useful.

To use  AutoCAD (or  AutoSketch)  with  an HP  DeskJet 500C or 550C,

install PrintCAD with "printcad /padi /fa! /L7.9,10,0,.3 /crygcbmk".

Configure AutoCAD  for  an  ADI  4.0  vector  plotter  installed  on

interrupt  78h,  7  pens,  1016  steps/inch,  plot area  10x7.9,  no

calibration.  Set  up the AutoCAD pen assignment  table so that  the

seven pens are assigned red, yellow, green, cyan, blue, magenta, and

black. Set skip perforation off on the printer by setting DIP switch

1-8 up. Then plot.

To use  a Canon BJC800 or 820 with 11x17 paper with AutoCAD, install

PrintCAD with "printcad /p*.plt  /ya  /f& /L10.5,16,0,0  /crygcbmk".

Configure AutoCAD for  an ADI 4.0 vector plotter with  binary output

to  a  file,  7  pens,   1016  steps/inch,  plot  area  16x10.5,  no

calibration.  Set up the AutoCAD  pen  assignment table so  that the

seven pens are assigned red, yellow, green, cyan, blue, magenta, and

black. Then plot.

To  use  a Canon BJ330 with cut sheet  paper  with  AutoCAD, install

PrintCAD with:

printcad /f7 1,1.01 /L8,10,0,.4 /j27,67,0,11

printcad /2/f7 1,1.01 /L10.5,16,0,.4 /j27,67,0,17

printcad /3/f7 1,1.01 /L13.5,21,0,.4 /j27,67,0,22

Configure AutoCAD for an HP  7585 plotter with output to a file.  To

print on 8.5x11 paper set the plot area to 10x8 and output  the plot

to file PCAD1.  For 11x17  paper set  the  plot area to 16x10.5  and

output to  PCAD2.  For 17x22  paper set the plot area to 21x13.5 and

output to PCAD3.

To  use the BJ330 with another CAD  package, replace the ".plt" with

the  default   plotfile   extension  for  that   package.   AutoCAD,

AutoSketch,  and  Micro CADAM use  ".plt", Microstation uses ".000",

GenCADD  uses ".dpf", and EasyCAD and FastCAD use ".hpg".  DesignCAD

has no default extension - use ".plt" and enter the entire file name

when plotting (PCAD1.PLT for example).

To use a LaserJet with Microstation, install PrintCAD with "printcad

/p*.000  /fL /L7.9,10,0,.1".  Configure Microstation for an  HP 7585

plotter, set  the  plot  area  to 10x7.9, and plot.  Instead of /fL,

choose the best LaserJet driver from this list:

/FL  any LJ

/FL! any LJ2 and any LJ3 with less than 1.5 Meg

/FI  any LJ3 with 1.5 Meg or more and the LJ4L

/FI+ any 600 dpi LJ4

For   manual  feed  legal  paper  use  "/p*.000  /fL   /L7.9,13,0,.1

/J27,38,108,50,72,27,38,108,51,65" and set the plot area to 13x7.9.

p-12

Overview of Options --------------------------------------------------

PrintGL's options fall into several categories.  The  first is input

and output choices.  The  input is chosen with /P, the plotfile, and

/N,  the  page number.  Output options  are  /F,  the output  format

(printer model), and /D, the destination port, device, or file.

The second category is the  page  layout.  /L controls the size  and

location  of the print window (hard clip limits), and the option  to

box the window, not send a form feed,  and reverse the image  color.

Tiling is set with /B and copies with /Q.

The third category  defines  how  the  plot  looks  within the print

window.  /M,  the  magnification, sets the  size  of  the  plot.  /A

determines the location of the plot within the print window. /O sets

the  origin and orientation, and  is  used to  rotate  or mirror the

plot. /I sets the plotter's initial scaling points.

The  fourth category defines the pens, /W for the width,  /C for the

color, and /S for the shade.

The  fifth category determines how PrintGL processes the plot.  This

includes the temporary file /T, the default and minimum chord angle,

enhanced  font,  and color to shade options  /Z, the plotter Y/D and

S/E  switch positions  and HP-GL/2 and ADI handling /Y, the internal

resolution /R, user interface  options /X, and /J and /K let you set

up and reset the printer for special situations.

PrintCAD  has separate setup options, /P sets the intercept mask, /X

sets up the  initial configuration, /1,  /2,  /3, /4, and /5  set up

multiple configurations.

And PrintGL/D and PrintGL/W use /1, /2, /3, /4, and /5 to choose one

of the menu setups as the command line default.

/1, /2, /3, /4, /5 - PrintGL/D and PrintGL/W Default Options ---------

These  option select a configuration from the menu to be used as the

default.  This option must match the first character of the title of

one of the five menu configurations.

/1, /2, /3, /4, /5 - PrintCAD Alternate Configurations ---------------

These options select up to 5 alternate configurations, labeled 1..5.

/1  is the  default.  The configuration used  for printing is chosen

based on the file name and mask (/P option).

p-13

/A - Position Option -------------------------------------------------

/A[O][x,y] - position - default /A

"position"

The A option determines the location of the plot in the print window

via  a point on  the plot that  is aligned with a point in the print

window.  You can specify the plot alignment point with x,y in inches

from the plot's  origin  (before  PrintGL applies magnification), or

let  it default to the plot  center.  For the print window alignment

point, you can default to the center or specify the origin with O.

/A      puts the center of the plot at the center of the print

/AO0,0  puts the plot 0,0 at the print window origin

/B - Tiling Option ---------------------------------------------------

/B[i,j,x] - tiling parameters - default /B1,1,0

"tiling"

/B  lets you specify  tiling  options that  break a  large plot into

tiles. Each tile is defined by the page layout option (/L).  I and j

specify the number of tiles horizontally and vertically. X specifies

the overlap  in inches.  Displayed  plots are  sized  based  on  the

tiling, but only the first tile is displayed. Use the cursor keys to

scroll the plot.

/B2,2,.5  breaks the plot into 4 pages with .5 inch overlap

/B        is the same as /B1,1,0 and produces no tiling

/C - Pen Color Option ------------------------------------------------

/C[O|T]c.. - color - default /COK

"pen color"

"pen mode"

"pen all"

/C specifies  the  color of each of 40  pens.  The colors are  B for

blue, C for  cyan, G  for green, K  for black, M for magenta,  R for

red, W for white,  and Y  for yellow.  Unspecified pens use the last

selected  color (/CRGB is the same  as /CRGBBBBBB).  For black/white

output  formats,  color  are translated into either  black  or shade

levels, based on the /Z option.

continued

p-14

Additional colors can be created with shade mixing. These colors use

shading  patterns to mix  the  eight pure colors.  The format for  a

mixed color is .??  where  each ?  represents any of  the eight pure

colors.  The first  color is printed  with the pen's shading pattern

(/S option)  and the  second  is printed  with  the  inverse shading

pattern.  If  the shade value is 0 (solid) it is replaced by 2 (50%)

which  produces the most  uniform  mixed colors.  On many  printers,

using  a shade of 1 (75%) or 3 (25%) gives a better color.  Here are

some useful mixed colors for inkjet printers.

color shade   description      color shade   description

.bc   3 (25%) azure            .ry   3 (25%) orange

.bm   3 (25%) violet           .rk   1 (75%) brown

.gc   3 (25%) jade             .ck   1 (75%) teal

.gy   3 (25%) lime             .rk   3 (25%) black on DeskJet 500C

Mixed  colors  need line widths  of at least 2 (4 for coarse shading

patterns) to  look good, and they may have visual artifacts on edges

near a 45 degree angle.  Multipass dot matrix formats may not handle

color mixing well.

The  following  single  letters may be  used  in place of the  color

mixing notation.

equivalent                         equivalent

color  mixed color  description    color  mixed color  description

a      .bc          Azure          i      .bk          dark blue

v      .bm          Violet         e      .ck          tEal

j      .gc          Jade           f      .gk          Forest

l      .gy          Lime           p      .mk          Purple

s      .rm          roSe           d      .rk          dark red

n      .ry          oraNge         h      .yk          dark yellow

By default, colors are overwritten into the  print, so  they  appear

opaque.  You can  switch to translucent colors (ored into the print)

with  the  T suboption, and then back  to opaque  with O.  Make  the

switch in  front of the pen that you want to change.  For PostScript

and  plotters, the color  processing is  done  by  the  device,  not

PrintGL.  PostScript uses opaque colors and plotters use translucent

colors.

" means repeat the previous color.

/COKR.MB""T.KY  sets pen 1     to opaque black

pen 2     to opaque red

pen 3..5  to opaque magenta on blue

pen 6..40 to translucent black on yellow

p-15

/D - Output Destination Option ---------------------------------------

/D[+][*]f - destination - default /D1

"destination"

The D option specifies the output printer port, device, or file. The

+ suboption causes files to be appended rather than overwritten.  1,

2,  and 3 select the  BIOS printer  routines.  P1..P3  select direct

parallel port drive - add extension 001  to 999 (for example P1.010)

to slow the  function  of  the  port  for  compatibility  with older

printers.  X1..X4  select xon/xoff serial port drive.  H1..H4 select

hardware (RTS/CTS) serial port drive.  W  selects the Windows  print

manager (PrintGL/W only).  You can also  use the  DOS  devices LPT1,

COM1, etc. /D is ignored for display output.

The * suboption causes PrintGL to output a continuous stream of data

with pauses of less than one second. This is useful for networks and

multiple  PC printer  buffers, where  print  jobs  are separated  by

pauses in the data stream.

PrintGL cannot set up the serial ports.  This is usually done in the

AUTOEXEC.BAT with MODE (for example MODE COM1:9600,N,8,1,P).

If you choose  a three  digit number  for  the  file  extension (for

example .000), and multiple pages are output via the multiple copies

option, tiling, or multiple HP-GL pages then,  instead  of appending

the destination file, additional files are  created  with sequential

extensions.  If you  specify *, then the extension is incremented to

avoid overwriting existing files. This is useful for PCX output.

/D2           sends output to the second parallel port via BIOS

/D+TEMP.PRN   appends output to file TEMP.PRN

/DOUTPUT.000  sends output to OUTPUT.000, OUTPUT.001, ...

/F - Output Format Option --------------------------------------------

/Fc[%][-|+|*|^][!|][~|`][x,y] - output format - default /FV

"output format"

"modify output" "h,v size multiplier: "

"modify output" "compression mode: "

The  F  option  specifies the  output format.  -, +, *,  and  ^  are

resolution modifiers, ! and  select compression levels, ~ turns off

color processing, and ` switches four  plane color processing  on or

off.  These suboptions are allowed  even if they have  no effect.  %

selects a different driver.

For  HP-GL  (/F8),  HP-GL/2  (/F0-, /F0), HP-RTL (/F0+),  PostScript

(/FS), PCX/DCX (/FZ),  and  PS  preview  (/FY)  you  can append  the

nominal device resolution.  Matching the  nominal  resolution to the

printer  is critical  for  HP-RTL and  is  often useful  for  HP-GL,

HP-GL/2, and  PostScript.  For PCX/DCX and  PS preview  the  nominal

device resolution may be one or two numbers.

continued

p-16

For output  formats other  than bit  maps,  you  can append the true

horizontal  and  vertical   resolution  of   the  output  device  in

dots/inch.  This overrides the normal output  device resolution, but

does not affect the structure of the  output.  These numbers  should

follow all other suboptions. This lets you compensate for print size

error. For example, a 360x360 dpi printer consistently under indexes

so  that a 6  inch high box comes out  5.94 inches.  Using 360,363.6

(6/5.94*360 =  363.6) removes the error.  For print size correction,

you also specify a number  from .5 to 2 which  is interpreted as the

nominal  resolution  times this  number.  For  example on  a 360 dpi

printer, 1.0,1.01 is the same as 360,363.6.

Each printer driver has a default level of  data compression, chosen

to give maximum compatibility among the different  printers that the

driver supports.  Many  drivers have  an optional enhanced  level of

compression, chosen with  !.  This  will be incompatible  with  some

printers.  And  some drivers let you turn all compression off with

(ASCII 173).  This is rarely of any use.  Where these suboptions are

effective, they are marked in the table below.

PrintCAD cannot use any  of the  display drivers or the EPS  preview

bit map driver.

Where effective, the !, , `, and ~ modifiers are listed below.

/F0-[i]      ~ HP-GL/2 printers ....................... 600x600, ixi

/F0[i]       ~ HP-GL/2 raster plotters                  600x600, ixi

/F0+[i]      ~ HP-RTL raster plotters ................. 300x300, ixi

/F1         `~ IBM 9 pin                                      120x72

/F1+        `~ IBM 9 pin (2 pass) .......................... 120x144

/F1*        `~ IBM 9 pin (4 pass)                            240x144

/F2            IBM Quietwriter 2 ........................... 240x240

/F3            IBM Quietwriter 3                             240x240

/F4-        ! IBM LaserPrinter (PPDS mode) ................ 150x150

/F4         ! IBM LaserPrinter (PPDS mode)                  300x300

/F4+        ! IBM LaserPrinter (PPDS mode) ................ 600x600

/F5        !`~ IBM 24 pin alternate 1                        180x180

/F5+       !`~ IBM 24 pin alternate 1 (2 pass) ............. 360x180

/F5*       !`~ IBM 24 pin alternate 1 (4 pass)               360x360

/F6          ! IBM Proprinter X24 .......................... 180x182

/F6+         ! IBM Proprinter X24 (2 pass)                   360x182

/F6*         ! IBM Proprinter X24 (4 pass) ................. 360x364

/F7           Canon BJ IBM mode, IBM ExecJet                360x360

/F7+          Canon BJ IBM mode, IBM ExecJet (2 pass) ..... 360x360

/F8[i]         HP-GL                                  1016x1016, ixi

/F9        !`~ NEC 24 pin .................................. 180x180

/F9+       !`~ NEC 24 pin (2 pass)                           360x180

/F9*       !`~ NEC 24 pin (4 pass) ......................... 360x360

/F#           JRL J bubblejet Epson mode                    360x360

/F$         `~ Epson Esc/P2 ................................ 360x360

/F$+        `~ Epson Esc/P2                                  720x720

/F&         `~ Canon BJ/BJC native mode .................... 360x360

/F:         `~ Star Micronics SJ-144                         360x360

/F@          ~ Canon BJC CaPSL mode ........................ 360x360

continued

p-17

/FA-       !`~ HP DeskJet 500C, color PCL .................. 150x150

/FA        !`~ HP DeskJet 500C, color PCL                    300x300

/FB        `~ Canon BJ/BJC Epson mode, Epson Stylus ....... 360x360

/FB+       `~ Canon BJ/BJC Epson mode (line overlap)        360x360

/FC            CGA display ................................... 62x25

/FD-        ! Canon LBP (ISO/CaPSL mode)                    150x150

/FD         ! Canon LBP (ISO/CaPSL mode) .................. 300x300

/FD+        ! Canon LBP (ISO/CaPSL mode)                    600x600

/FE         `~ 128K EGA color display ........................ 62x45

/FF        !`~ Fujitsu 24 pin                                180x180

/FF+       !`~ Fujitsu 24 pin (2 pass) ..................... 360x180

/FF*       !`~ Fujitsu 24 pin (4 pass)                       360x360

/FG           PDP Protracer (IBM mode) .................... 360x360

/FH            Hercules graphics card display                  70x45

/FI-           HP LaserJet 3, DeskJet ...................... 150x150

/FI            HP LaserJet 3, DeskJet                        300x300

/FI+           HP LaserJet 4 ............................... 600x600

/FJ            Canon BJ BJ130 mode                           360x360

/FK          ~ Kodak Diconix Color 4 ....................... 192x192

/FL-        ! HP LaserJet                                   150x150

/FL         ! HP LaserJet ................................. 300x300

/FM            128K EGA monochrome display                     62x45

/FN         `~ Epson 9 pin .................................. 120x72

/FN+        `~ Epson 9 pin (3 pass)                          120x216

/FN*        `~ Epson 9 pin (6 pass) ........................ 240x216

/FO        !`~ Toshiba 24 pin                                180x180

/FO+       !`~ Toshiba 24 pin (2 pass) ..................... 360x180

/FO*       !`~ Toshiba 24 pin (4 pass)                       360x360

/FP         !~ HP PaintJet ................................. 180x180

/FQ            HP QuietJet (PCL mode)                        192x192

/FR         `~ Tektronix ColorQuick ........................ 216x216

/FS[i]       ~ PostScript                               600x600, ixi

/FT        !`~ Epson 24 pin ................................ 180x180

/FT+       !`~ Epson 24 pin (2 pass)                         360x180

/FT*       !`~ Epson 24 pin (4 pass) ....................... 360x360

/FV         `~ VGA display                                     62x62

/FV+        `~ VESA 800x600 VGA display ...................... 78x78

/FV+b       `~ 800x600 VGA display - mode b                    78x78

/FV*        `~ VESA 1024x768 VGA display ................... 100x100

/FV^        `~ VESA 1280x1024 VGA display                    125x134

/FV%        `~ Windows display (PrintGL/W only) ....................

/FW         `~ Epson 9 pin alternate                          120x72

/FW+        `~ Epson 9 pin alternate (3 pass) .............. 120x216

/FW*        `~ Epson 9 pin alternate (6 pass)                240x216

/FW%        !~ Windows default printer (PrintGL/W only) ............

/FX        !`~ IBM 24 pin alternate 2                        180x180

/FX+       !`~ IBM 24 pin alternate 2 (2 pass) ............. 360x180

/FX*       !`~ IBM 24 pin alternate 2 (4 pass)               360x360

/FY[i[,j]]     append EPS preview bit map .......... 72x72, ixi, ixj

/FZ[i[,j]]  `~ ZSoft PCX                           100x100, ixi, ixj

/FZ![i[,j]] `~ DCX ............................... 100x100, ixi, ixj

See Output Devices (Displays, Printers, Plotters, Bit  Maps, Fax) to

match your printer with one of these drivers.

p-18

/I - Scaling Point Location Option -----------------------------------

/IA|R|W[N] - IP point location - default /IA

"scaling pnts"

This  option is not useful with AutoCAD or any software that outputs

HP-GL that  does not use  scaling commands  or that initializes  the

plotter or sets the scaling points.

The  I option  sets the initial scaling points (P1, P2 - the scaling

points  may  be set  in  the HP-GL  file  with  the IP  command).  A

(absolute) sets the points  to the default  for  the paper size (see

below).  R fits P1 and P2 to the print window with at least .25 inch

margins  and an x/y ratio of 10/7.2 (the same ratio as  the HP  7475

with small paper).  W fits P1 and P2  to the print window with a .25

inch margin. N makes the margin 0 for R or W and is ignored with A.

The   default  scaling   points  are  plotter   specific.   If   the

magnification is  1.0  then  PrintGL looks at  the print window  and

orientation to decide if they match one of the plotters/papers noted

below.  If so  the default scaling points match that plotter.  If no

match  is  found or the magnification is not 1 then the defaults are

taken from the 7440.

7440                     7475                     7550

paper  window            paper  window            paper  window

A       7.54,10.14       A       7.84,10.20       A       7.72, 9.92

A4      7.54,10.74       A4      7.60,10.88       A4      7.48,10.70

B      10.20,16.38       B      10.01,16.19

A3     10.88,15.90       A3     10.70,15.72

/IWN  sets the scaling points to fit the print window

/J, /K - Printer Code Prefix, Suffix Options -------------------------

/J[b,b,..] - printer code prefix - default /J

/K[b,b,..] - printer code suffix - default /K

"modify output" "prefix codes: "

"modify output" "suffix codes: "

/J lets you send codes to the printer before PrintGL's usual data is

sent and /K  lets you send codes after PrintGL's usual data is sent.

This lets you set up and reset the  printer for special  situations.

For  HP-GL  and   PostScript  output,   the  /J  codes   follow  the

initialization commands.

The codes  are  specified  with  byte  values  (0..255  or  $0..$FF)

separated with blanks or commas.

/J27 69  does a printer reset before printing on HP-PCL printers

p-19

/L - Page Layout Option ----------------------------------------------

/L[B][F|N][R][x,y[,x,y]] - page layout - default depends on printer

"window/margins"

"form feed"

"box"

"reverse image"

The L option defines the page layout.

B boxes the plot at the print window edge using pen 40.

F, the default,  turns  form feeds on and  N  turns  form feeds off.

These are ignored for bit maps and display output.

R  reverses the  colors on displays, bit maps, and output formats 4,

D, I, and L  (Canon,  HP, and IBM  page  printers) and is  otherwise

ignored. This gives the equivalent of a photographic negative.

The first optional x and y are the print window width  and height in

inches.  These define  the hard  clip  limits.  These values are not

checked against the printer's capabilities.  If you specify  a print

window  that  exceeds  the  output  device limits, the  results  are

unknown.  You may need to reset the printer's margins or page length

for large plots (see Large Paper). The size needed for a given width

and height exceeds x and y by 8 dots to allow for line widths.

The  second  optional x and y are the left and top margins, measured

from the printer's  left margin and current vertical position to the

print window. The margins for HP-GL and PostScript are measured from

0,0 (left, bottom).  Margins are ignored for display output  and bit

maps.

THE PRINT WINDOW PLUS THE MARGINS MUST FIT WITHIN THE PRINTABLE AREA

OF  THE PRINTER.  To find the largest printable area turn on the box

and form feed  options  and set a  window and margins of 7.5,10,0,0.

Any plotfile will do - the  only  concern  is the box.  Increase the

window until you have the largest  box that fits on the paper.  This

is the best that you can do. To improve centering you can reduce the

print window and increase the corresponding margin.

The default print window is 7.54x10.14 for printers and bit maps and

10.14x7.54 for HP-GL and displays, corresponding to HP 7440 A paper.

The  default  margins are printer  specific to compensate for  paper

handling.

/L7.5,3,.5,0   print window 7.5x3, left margin .5, top margin 0

/L7.54,10.14   sets window equivalent to HP 7440 with A  paper

/L7.54,10.74   sets window equivalent to HP 7440 with A4 paper

/L7.84,10.20   sets window equivalent to HP 7475 with A  paper

/L7.60,10.88   sets window equivalent to HP 7475 with A4 paper

/L10.20,16.38  sets window equivalent to HP 7475 with B  paper

/L10.88,15.90  sets window equivalent to HP 7475 with A3 paper

p-20

/M - Magnification Option (Scale, Size) ------------------------------

/M[F]x - magnification - default /MF.9

"magnification"

The M option  sets the magnification.  F fits  the plot to the print

window.  The default, /MF.9, prints at  90% of the largest size that

fits in the print window.

/MF.9  prints at 90% the largest size that fits in the print window

/M1    plots at the same size as an HP 7475

/N - Page Number Option ----------------------------------------------

/N[i] - page number - default /N

"plot page"

/Ni says to print only page i of the plotfile. If you do not specify

i then all pages in the plot will be printed (this is the default).

This option is not allowed with PrintCAD.

/N   prints all pages in the plotfile

/N3  prints only page 3

/O - Origin and Orientation Option -----------------------------------

/O[1|2|3|4[L]] - origin, orientation - default /O

"origin/rotate"

The O  option  sets  the print  window origin  and orientation.  The

origin  is  one of four corners numbered 1..4  for upper left, lower

left, lower right, and upper right, and implies a plot rotation. The

L suboption specifies a left handed coordinate system, which mirrors

the plot.  If no origin is specified then it is chosen so that the x

axis is the longer dimension.

Here are  pictures of PrintGL's view of the output media compared to

an HP 7475's view.

HP 7475                               PrintGL

display or

A size           B size               printer       plotter

y --------     0,0 -------- y       ul  ----- ur   ul  -------- ur

|        |       |        |         1 |     |4     1 |        |4

|        |       |        |           |     |        |        |

0,0 -------- x      |        |           |     |     ll  -------- lr

|        |           |     |      2           3

|        |        ll  ----- lr

|        |         2        3

x --------

/O2  puts the print window's origin in the lower left corner

p-21

/P - PrintGL/D and PrintGL/W Plotfile Option -------------------------

/Pf - plotfile - no default

"plotfile"

The plotfile may be specified as the first parameter with no prefix,

or  it may be specified anywhere in the option list with /P.  If you

use a  file name  mask with "*"  and/or  "?",  each matching file is

processed, but the first failure or user break stops all processing.

/P - PrintCAD Intercept Mask Option ----------------------------------

/PADI|mask - intercept mask

/PADI sets up PrintCAD as an AutoCAD ADI plotter driver. /Pmask lets

you  choose  a DOS file mask as PrintCAD's intercept mask.  The mask

should not include disk  or directory information - PrintCAD ignores

this information when looking at  file names.  The mask may  contain

the character #.  This is the same as ?, but when a file matches the

mask   the  character  that  matches  the  #  selects  the  PrintCAD

configuration - 1..5  selects configurations 1..5  and anything else

selects configuration 1.

AutoCAD 10 will work with PrintCAD only via /PADI.

Regardless of the /P option, PrintCAD always responds to PCAD#.PLT.

/Padi     sets up PrintCAD to take data from AutoCAD interrupt 78

/P*.plt   intercepts data written to *.plt

/Px#.plt  intercepts data written to x1.plt, x2.plt, etc.

/Q - Number of Copies Option -----------------------------------------

/Q[H]i - number of copies - default /Q1

"copies"

/Q sets the number of copies for each printed  page.  The default is

one. The H suboption specifies to send the codes to have the printer

produce  multiple  copies, which is much faster than having  PrintGL

send the data for each page. This only works with page printers.

/R - Internal Resolution Option --------------------------------------

/Ri - internal resolution - default /R1016

"plotter" "internal dpi: "

/R sets the internal grid resolution  in dots/inch.  It is  normally

1016 and rarely needs  to be changed.  To fit very large plots  into

the   -32768..32767  coordinate  limit,  use   /R508.   Setting  the

resolution to a multiple  of the output device  resolution times the

magnification may yield a small improvement in print quality.

p-22

/S - Pen Shading Option ----------------------------------------------

/Sc.. - pen shading patterns - default /S0

"pen shade"

"pen all"

/S  assigns a shading pattern to each pen.  Unspecified pens use the

last specified  shade.  The  shade values  are  0..4  and A..M.  The

percent coverage for each shade is listed below.  Narrow line widths

may not work well with some shade patterns.

0 100%     1 75%       A 6.2%      D 87.5%     G 93.7%     K 43.7%

2 50%       B 3.1%      E 62.5%     H 81.2%     L 31.2%

3 25%       C 1.6%      F 37.5%     I 68.7%     M 18.7%

4 12.5%                             J 56.2%

Some  printers,  especially dot matrix printers in  multipass modes,

wash out high percentage shade patterns because they have a dot size

that is large relative to the dot spacing.

/S024  sets pen 1 to solid, pen 2 to 50%, pen 3..40 to 12.5%

/T - Temporary File Option -------------------------------------------

/T[+]f - temporary file - default /TPLOT.TMP

"temp file"

PrintGL uses a temporary  file  when there is  not enough memory  to

handle a plotfile.  You can use /T to put it on a RAM disk for extra

speed. You can force the use of a temporary file with + (if the plot

fills the buffer). Using a RAM disk may speed printing.

/W - Pen Width Option ------------------------------------------------

/Wc.. - pen widths - default /WA

"pen width"

"pen all"

The W option assigns pen line widths. Each width may be 0..9, .10 ..

.24 dots, A..I for  2..10 units of 4/1016 inch (.1 mm), M for 4/1016

inch  (.1  mm), or N to  turn off  the  pen.  A..I and  M widths are

approximations, based on the device  technology.  Odd widths of 9 or

above are modified to the next higher (even) width.  Unassigned pens

use the last assigned width.

For nonsquare matrix printers, PrintGL uses pen points optimized for

an h/v ratio of .67, 1, or 1.5.  Widths are figured in the direction

of higher resolution, and are not accurate in the other direction.

Line widths  of over eight dots are  distorted at  the print  window

edge to fit within the four dot border.

/WA      sets pens 1..40 to .2 mm

/WB2.16  sets pen 1 to .3 mm, pen 2 to 2 dots, pens 3..40 to 16 dots

p-23

/X - PrintGL/D and PrintGL/W User Interface Option -------------------

/X[B][C][O][P][S][F][U][M][Z][V] - interface options - default /X

"other"

The X option sets various interface options. C causes a break when a

plot is clipped.  P causes  a pause  before  each  printed  page.  O

suppresses file overwrite queries.

S suppresses  location  and  size messages.  F  suppresses location,

size,  file  name, and page  number messages.  B suppresses progress

messages.  U  suppresses unsupported  command and  incorrect  number

messages. V starts PrintGL/W as an icon (command line only).

M prevents PrintGL from allocating DOS upper memory blocks.

Z turns off the shareware notice panel (menu only).

/XS  gives only the most important messages

/X - PrintCAD Control Option -----------------------------------------

/XQ - queries PrintCAD installation

/XR - removes PrintCAD if allowed

/X[E|N|X][A][M][L][W][P][S][Z][-|+] - controls PrintCAD's setup

/XQ  checks to see  if  PrintCAD is installed.  /XR removes PrintCAD

from memory, if allowed. /XQ and /XR disallow all other options.

The other  suboptions  control setup,  and  are  effective only  for

initial setup. X specifies XMS for swapping - this is the default. E

forces the use  of  EMS  for  swapping instead  of  XMS.  N inhibits

swapping.  This forces the  entire program to load in low memory and

it will  take a large chunk of  DOS  memory.  In return,  all of the

compatibility problems of  swapping TSRs  are removed.  This  can be

very useful with DOS extender based CAD programs.

A sets  an  alternate  handle  (use  with PC-Draft-CAD,  Drafix, and

DesignCAD).  M sets an alternate mouse handler. These suboptions may

improve compatibility.

L forces PrintCAD to load its resident  part low.  W forces PrintCAD

to load its swapping work area low.  P causes program prefix segment

checking.  These  suboptions may  improve compatibility  in  unusual

situations, but are generally not needed and may make things worse.

+ or  -  increases  or  decreases  the PrintCAD  memory  space,  for

improved speed or reduced resource usage.

S adds progress beeps. This is not advised with the N suboption.

Z turns off the shareware notice panel.

/XM  sets up PrintCAD with its alternate mouse handler

p-24

/Y - Plotter Options -------------------------------------------------

/Y[D|Y][S|E][A][1|2][N][B] - plotter options - default /YDS

"plotter" "D/Y switch position: "

"plotter" "S/E switch position: "

"plotter" "read ADI: "

"plotter" "read HP-GL/2 (Y|except Pens|N): "

"plotter" "start on for Y, HP-GL/2: "

"plotter" "large polygon fill buffer: "

The  Y option sets the plotter's D/Y switch  to D  or Y  and the S/E

switch to S or E.  Y  starts with  the plotter off  and responds  to

esc.(, esc.Y, esc.), and esc.Z commands.  D starts with the  plotter

on and ignores these commands. S sets the standard interpretation of

IW (clipping  window) commands.  E sets 7550 enhanced interpretation

of IW commands, so that they use scaled coordinates.

The A suboption turns on  PrintGL's ADI handler.

The 2 suboption turns on PrintGL's HP-GL/2 subset interpretation and

sets the Y and E suboptions.  The 1 suboption is the same but causes

PrintGL   to  ignore   HP-GL/2  pen   width,   color,  shading,  and

transparency control.

The N  suboption starts  with the  plotter  on  if the  Y  switch or

HP-GL/2 processing is on.  This is useful  for HP-GL/2  that assumes

the plotter is already in HP-GL/2 mode rather than HP-RTL mode.

B increases the polygon fill buffer by 4000 line segments.

/Z - Processing Options ----------------------------------------------

/Z[x,y[,z]][E][C] - processing options - default /Z10,5,180

"plotter" "def, min, max chord angle: "

"plotter" "enhanced font: "

"plotter" "color to shade on b/w formats: "

Except  for the  C suboption, this  option is not  useful with HP-GL

(including AutoCAD's) or ADI that  draws circles, arcs, and text  as

line segments rather than using circle, arc, and label commands.

The Z option  specifies  the  default,  minimum, and  maximum  chord

angles for circles and arcs.  The HP 7475 defaults to  a chord angle

of 5 with  no  minimum.  Using chord  angles of less than 10 returns

little print quality  improvement and adds  to processing time.  The

default is /Z10,5,180. For better emulation use /Z5,2,180.

The E suboption says to  use  the enhanced  font, which doubles  the

number of chords on curves in text,  improving the curve smoothness.

This is only effective on character heights above .5 cm.

C says  to  convert colors to  shade  levels  for monochrome  output

formats. Otherwise all colors except white are converted to black.

/Z5,2  sets the default chord angle to 5 degrees, the minimum to 2

p-25

PrintGL/D and PrintGL/W Menu -----------------------------------------

PrintGL/D  and PrintGL/W include  a menu interface to let you select

options, choose a  list  of  plotfiles, and print.  Run PRINTGLD  or

PRINTGLW with no command line parameters to bring up the menu.

The main menu is displayed at the top of the  screen, and the bottom

of the screen is used differently for each main menu item.  The main

menu is divided  by column  into  six  groups, described below.  The

functions of the menu items are described in detail above.

When the  cursor is  on a main menu item, the current  state of  the

submenu is displayed below.  To change the submenu, press Enter. The

items are  modified with  cursor movement and  typing numbers/names.

Enter returns you to the main menu with the new values.  Esc returns

you to the main menu with the values unchanged.

The PrintGL documentation can be viewed by pressing F1 from any menu

that displays F1 on its key line.

If one of the first two letters of  a main menu item is capitalized,

that item may be accessed immediately by pressing the letter. When a

cursor bar has a notch, the value in the notch may be edited.

Menu Specific Functions ----------------------------------------------

The first column  of  main  menu items is  dedicated to menu related

functions  These  include   run  PrintGL,  switch  to  an  alternate

configuration, exit, save the configurations,  set menu options, and

create a BAT file corresponding to the current configuration.

The "Alt config" item rotates between five menu configurations.  The

current  configuration name is in the upper left corner.  Menu items

that begin with * are the same for every configuration.

The "save configs" item saves all of the current menu configurations

by writing them into the EXE file.  Only the first nine lines in the

plotfile list are saved.  You  can copy the EXE  file to a different

name and it will have its own configurations.

The "menu config" item lets you  set the  configuration name, screen

colors,  and the name of  the BAT  file created by  the "create BAT"

option.  For PrintGL/W  you  can  choose  the  screen  font  height,

selected from  current  "fixedsys" fonts,  VGAFIX (15), and  8514FIX

(20). Screen colors are specified with the mnemonics listed here:

k black    r red             k+ gray            r+ bright red

b blue     m magenta         b+ bright blue     m+ bright magenta

g green    y yellow/brown    g+ bright green    y+ bright yellow

c cyan     w white           c+ bright cyan     w+ bright white

For  PrintGL/W you can use  - and * as  color  modifiers.  These are

effective with 15, 16, or  24 bit color setups.  To use  the default

Windows colors, blank out the desired line of colors.

"create BAT" creates a single configuration BAT  file for  PrintCAD.

You will have to add any /X or /P options as desired.

p-26

Menu Input and Output Choices ----------------------------------------

Column two  of the main menu lets  you choose the plotfiles to  plot

(the /P  command line option), the plot page number (/N), the output

destination  (/D),  output format  (/F),  and modifications  to  the

output  -  compression  mode  (/F),  horizontal  and  vertical  size

multiplier (/F) and prefix and suffix printer codes (/J, /K).

You  may enter up to 99  plotfiles  in the "plotfile" submenu.  Each

file may  be preceded  by a repetition count from 2 to 5.  To choose

from a list of files, enter a mask name and press  Enter (the cursor

must  be  on  the  mask).  The menu  will  then  display  a list  of

corresponding  files.  Select any number of the  files by moving the

cursor  to  the desired file and pressing Space (or the  right mouse

button).  Consecutive presses increment the repetition count.  If no

files have been selected, then pressing Enter selects the file under

the cursor.  The selected files are added to the file list below the

cursor.  Files that would overflow the 99th position on the plotfile

list are dropped.

Directories are shown at the top of file lists and are designated by

a trailing \.  If you select  a directory (repetition counts are not

allowed), then you can use that to display a new file list.  This is

handy for navigating a disk to search for image files.

The first nine plotfiles are saved when you do a "save configs".

When you "Run PrintGL", PrintGL is run for each file on the plotfile

list. Masks are ignored. For any plotfiles that are not successfully

processed, an arrow is inserted in front of  the plotfile name.  You

can  view the results from the "plotfile" main menu item.  The arrow

is  not  considered  part  of the file  name, so you do not need  to

remove it to retry PrintGL.

Printers that  support  color  have  a  color  option ("clr:" on the

menu).  This lets you specify a  black/white printer even though you

may have set  up colors with the  "pen color"  menu.  Output formats

that do not support color do not have this option and always convert

the selected colors to black and white.

Menu Processing Options ----------------------------------------------

The  third  column  of  main  menu  items  define  how  the  plot is

processed.  The  options  include  magnification  (/M), origin (/O),

position (/A), plotter (/Y,  /Z,  and  /R options  such  as ADI  and

HP-GL/2   interpretation,   default  chord   angles,   and  internal

resolution), and initial scaling points (/I).

Menu Pen Options -----------------------------------------------------

The fourth column of main menu items covers the pen characteristics:

width (/W), color and opaque/transparent mode (/C),  and shade (/S).

Each of these submenus use the up  and  down cursor keys to select a

pen, and the left and right cursor keys to choose an attribute.  The

"pen all" submenu combines the four other submenus to let you see or

choose all of a pen's attributes at once.

continued

p-27

For  color  selection,  the  one character  colors (b,c,g,k,m,r,w,y)

represent  pure  colors,  and  the  two character  colors are  shade

pattern mixes.  The  shade pattern  mixed  colors  use  the  current

shading  option  for  that  pen,  unless the  shade  pattern is 100%

(solid), in which case 50% is used.

Menu Page Layout Options ---------------------------------------------

The fifth  column  of  main menu items  covers the page layout.  The

first three items are suboptions of the /L option - print window and

margins, form feed, and box. Tiling options (/B) and multiple copies

(/Q) are also here.

Menu Other Options ---------------------------------------------------

Other options are  in column six.  These  include the temporary file

name (/T),  and  under  "other", the pause  to load  paper, suppress

unsupported command  messages and other  /X suboptions and one  line

that lets you enter command line options to override anything on the

menu.

At the bottom of column six are the auto run options.

Menu Auto Run Mode ---------------------------------------------------

The lower right corner of the menu  includes  "auto  opts" and "auto

run".  In auto run mode,  the disk is  continually searched  for new

files that match the mask or file specified in "auto opts". Whenever

one  is  found, PrintGL is run on  that file.  This  is useful  with

multitasking systems such  as DESQview and  Windows.  Run  PrintGL's

auto run mode and switch it to the background.  Then you can  output

files and they will be printed automatically.

The auto run mask may  include the # character.  This is the same as

?, but when a file matches the mask the character that matches the #

selects the PrintGL configuration - 1..5 selects configurations 1..5

and anything  else  selects  the  current configuration.  The  first

character of the  configuration  title must  be  1..5 to  match  the

selected configuration.

Auto run mode uses  one of two ways of choosing new files (specified

under "auto opts"). The first is by time/date stamp. In this mode it

will only catch files that have a time/date stamp that is later than

when  auto  run  mode was started  and  is later than the  last file

processed  in auto run mode.  The file  with the  earliest time/date

stamp  is  processed first.  Copying  a  file does  not  change  its

time/date  stamp, so  copying a  file  to the auto run mask will not

cause it to be seen as a new file.

The second way is by archive bit. When auto run mode is started, any

file  that  matches the  mask and has its archive bit  set  will  be

processed, regardless  of  when it  was created.  The file  with the

earliest time/date stamp is processed first. After it is chosen, its

archive bit is unset so that it will not be chosen again.  This lets

you copy files to the auto run mask to print them.

p-28

Large Paper ----------------------------------------------------------

PrintGL can handle printing on paper  larger than the  usual 8.5x11,

but this takes some extra work. First you need to increase PrintGL's

print window  to cover  the large  paper.  This  is done with the /L

option  or "window/margins".  The print window plus the margins must

fit within the printer's printable area.

And then you must make  sure that your printer recognizes the larger

paper height.  For  continuous  forms paper  this  is  best  done by

setting skip perforation to off,  either by  printer switch or  with

PrintGL prefix codes.  For Canon, Epson, Fujitsu, IBM, and NEC  line

printers (dot  matrix,  ink jet, thermal) use  /J27  79,  or "modify

output" "prefix codes: 27 79".

For  cut  sheet  paper,  on Canon  (except BJ  native  mode), Epson,

Fujitsu, IBM, and  NEC line printers use /J27 67 0 n, where n is the

page length in inches.  For the PDP ProTracer use /J27 67 n, where n

is the page length in lines, at six lines per inch.

And for some printers you need to increase the right margin. For the

Canon BJC  in  Epson  mode  use  /J27 81 114.  For the HI V50/100 in

printer mode use /J27 91 88 2 0 1 255.

For page  printers, legal size  paper  may  be specified  with these

printer code prefixes: HP-PCL  printers  /J27 38 108 51 65  (/K27 38

108 50 65 to  reset to letter), Canon  LBP  /J27 91  51 50 59 59 112

(/K27 91 51 48  59 59 112 to reset), IBM LaserPrinter /J27 91 70 5 0

3 1 2 1 (/J27 91 70 5 0 3 1 1 1 to reset).

To  get  a 10x16  print window on  11x17 paper with  a Canon, Epson,

Fujitsu,  IBM, or NEC line printer, use these options: /L10 16 0 .25

/J27 67 0 17.  You  will probably need to adjust the margins (0 .25)

for best centering.

The  maximum  width  of  the print  window on  most  narrow carriage

printers is 8  - 8/(horizontal dpi).  For 300 dpi printers,  this is

7.97.  At 120 dpi it is 7.93.  The maximum width of the print window

on most wide carriage printers is 13.6 - 8/(horizontal dpi).  At 180

dpi this is 13.55.  At 360 dpi it is 13.57.  The  horizontal  margin

should be set to 0 to get this width.

p-29

Tips On AutoCAD ------------------------------------------------------

There are many ways to use PrintGL with AutoCAD.  Here is a good way

to start.

Configure  AutoCAD for an HP 7585 plotter.  Select output to a file.

Do not do any plotter calibration. Set a specific scale, do not fit.

Do not rotate.  Set  a plotter  window that is larger than  you will

ever  need,  30x30  for  example  (this  will  prevent  AutoCAD from

clipping the  plot).  Set  up your plotter  pen table so  that  each

screen color is assigned a different pen.

With  PrintGL  you  can  rotate  (/O  or "origin"), magnify  (/M  or

"magnification"), assign pens (/C,/S,/W or "pen color", "pen shade",

"pen  width"),  and  calibrate  (/F or  "modify  output"  "h,v  size

multiplier").  Use /A or  "position"  "plot  center  at print window

center" to automatically center your plot in the print window.

To use  the AutoCAD  12 plot preview  function  set the AutoCAD plot

area  to match the PrintGL print window with horizontal and vertical

numbers reversed (for an 8x10 print window set a 10x8 plot area).

Color Processing -----------------------------------------------------

Color graphics devices use a variety of color technologies.  PrintGL

supports these basic types.

CMY or cyan/magenta/yellow is used by inkjet printers with  no black

ink. This uses three color planes and gives eight pure colors.

CMYK or  cyan/magenta/yellow/black  is used by inkjet and dot matrix

printers.  This uses  four color planes and gives eight pure colors.

Using black ink instead of  a  cyan/magenta/yellow  mix gives a more

neutral black color.

RGB  or red/green/blue is used  by displays and bit maps.  This uses

three color planes and gives eight pure colors.

RGBI  or red/green/blue/intensity is used  by displays and bit maps.

This  uses  four  color planes.  All simple colors except black  are

intense.  When you specify any  color  mixed with black at 50%, then

instead  of  mixing the colors  at 50%, the intensity is turned off.

This gives fifteen pure colors.

CMYK and RGBI are used by default when they are appropriate. You can

switch to CMY or RGB with the ` suboption of /F.  For displays  this

gives a better picture of how a printer will handle the colors.  For

dot matrix printers it may speed printing by  eliminating the use of

the black ribbon.

/FA!  (300  dpi  color HP-PCL)  uses  CMY  color by  default.  /FA!`

switches to CMYK color to access true black on the DeskJet 550C.

p-30

Compatibility and Technical Information ------------------------------

PrintGL/D  will run on any DOS  computer  with MS-DOS  3.0 or above.

MS-DOS 3.3 or later is recommended.  It requires up to 256K of  free

memory.  More memory may speed printing.  The PrintGL/D menu can use

any  text display  mode  with  at  least 80  columns and  25  lines.

PRINTGLD.EXE should  not  be  compressed  with  EXE file compressors

because it rewrites the EXE file when saving its configurations.

PrintGL/W is an MS Windows 3.1 program  and requires a 286 computer.

It uses its own printer drivers, not Windows printer drivers.

PrintCAD will  run on any DOS 286 computer with  MS-DOS 3.0 or above

with 400K  of  EMS or XMS memory and  a base memory region of  512K.

Because it is a swapping resident program, it  may  have problems in

some environments. PrintCAD can handle VCPI (Virtual Control Program

Interface) based  DOS  extenders, but DPMI based  programs and stand

alone DOS extenders may cause problems.

When  using a DOS extender  based program  such  as  AutoCAD  386 or

Microstation   4,  use  an  extended/expanded  memory  manager  that

provides VCPI, such as MS-DOS 5 EMM386/HIMEM, QEMM 386, or 386MAX.

PrintGL was written  by Cary Ravitz and compiled with Borland Pascal

7.0.

HP-GL, HP-GL/2, and ADI Commands -------------------------------------

PrintGL supports  the  7475  and 7440/17440 command  sets except for

character  set 8 (Katakana), digitize commands, output commands, the

error mask command, and some device control commands.  In  addition,

PG, AF,  ES,  LO, character set 5, line types -8..-1, 7,  and 8, and

proportional spaced  fonts  are  supported.  Forty  pens are allowed

instead of eight.

With /YA or "plotter" "read ADI: Y", PrintGL supports ADI 4.0 vector

plotter binary commands created at 1016  steps/inch with up to forty

pens and up to eight non-solid line types.

With  /Y2  or "plotter" "read HP-GL/2 (Y|except Pens|N): Y", PrintGL

adds support for a subset of HP-GL/2, switches to enhanced IW and RO

interpretation,  and  switches to  plotter  on/off mode  (/YY)  with

esc%..B switching  the  plotter  on and esc%..A  switching  it  off.

HP-GL/2 support includes pen color, shading, transparency, and width

control, compressed data, shaded fills, 256 pens, and  the AC and IR

commands.  PrintGL  acts  like  an HP-GL/2  plotter, not an  HP-GL/2

printer, so form feeds are done with  the  PG  command not the  form

feed character.  If you  are using a  monochrome printer, you should

turn PrintGL's color off  with ~ or "clr:  N" to prevent color being

turned  on from the  plotfile.  Many  HP-GL  files  will not  run if

HP-GL/2 interpretation is on.

With  /Y1 or "plotter" "read HP-GL/2 (Y|except Pens|N): P",  PrintGL

interprets HP-GL as  with /Y2 except that the HP-GL/2 pen control is

ignored and the PrintGL pen attributes are used.

PRINTGL2.DOC and PRINTGL2.CHR include details on this subject.

p-31

Output Devices (Displays, Printers, Plotters, Bit Maps, Fax) ---------

Canon BJ and BJC Printers ------------------------------------------

PrintGL has five basic drivers for Canon BJ and BJC printers.  /F7

is  the  IBM  mode  driver.   It   works  with  the  BJ130e,  BJ5,

BJ10/10e/10ex, BJ20, BJ200/230, BJ300/330, and BJC4000. /F7+ gives

dark print with line to line overlap.

/FJ is the BJ130 mode driver.  It works with all Canon BJ printers

in IBM or native mode. The printer is set to 10 pitch text.

/FB is the Epson mode driver. It works with the BJ200/230, BJC600,

BJC800/820,  and BJC4000.  Color is supported.  /FB+  gives sparse

line to line overlap. For the BJC800/820 the prefix code 27 81 114

sets the print width to 11.4 inches.

/F&  is  the native mode  driver.  It  works  with  the BJ200/230,

BJC600, BJC800/820, and BJC4000 in  any mode.  Color is supported.

Epson/IBM mode prefix codes are not compatible and are not needed.

Always  use form feed with the  BJ native  driver unless  you have

previously set BJ native mode.

/F@ is the CaPSL mode driver for the BJC880. Color is supported.

Some  of these printers have  an image density option (DIP  switch

and/or  front panel switch).  Low density  eliminates every second

dot. To get the full 360 dpi resolution, use high density.

Canon LBP Printers -------------------------------------------------

/FD works with any Canon LBP 4 and 8 printers that do not have the

SX or 30 suffix.  If the printer is in Diablo mode, it is switched

to ISO  mode.  At 300 dpi it takes up to one meg of printer memory

to print a full page of graphics.

The ! suboption switches to CaPSL 4 data compression. /FD+! is the

best driver for the LBP 8 Mk4 at 600 dpi.

You might need to set up the printer with /J27 59 27 91 50 38 122.

To switch back to Diablo mode on completion, use /K27 58.

Any Canon  LBP printer with  the SX or  30  suffix  is  an  HP-PCL

printer and should use the /FI drivers.

Display ------------------------------------------------------------

When you display a plot, the cursor keys scroll the plot, S cycles

between  full,  half  (the  default),  quarter,  and  eighth  page

scrolling, and Enter or Esc returns to DOS.

PrintCAD cannot drive displays.

/FC drives a CGA, but does not give color.

continued

p-32

The EGA drivers, /FE and /FM, require a 128K or 256K EGA. /FC will

work with a 64K EGA.  /FM is for an EGA  card with a digital (TTL)

monochrome monitor.

/FH supports the Hercules monochrome graphics card.

/FV works with any VGA card.

/FV+ works with VESA 800x600 mode, /FV* works  with VESA  1024x768

mode, and /FV^ works with VESA 1280x1024 mode.

For MCGAs, use the VGA driver without color - /FV~.

The Windows  display  driver (/FV%) does not  use  the  resolution

modifier,  and instead sets the resolution to the screen  width in

dots divided by 10.8.

Display output cannot be redirected to a file.

Epson, IBM 9 Pin Printers ------------------------------------------

/FN, /FW, and /F1 cover a wide range of Epson/IBM compatible 9 pin

dot matrix printers.  You need a  cyan/magenta/yellow/black ribbon

to get color prints. The * drivers use 1/240 graphics. /FN+, /FN*,

/FW+,  and  /FW*  use  1/216  indexing  and  do  three  vertically

interleaved passes.  /F1+  and /F1* use  1/144 indexing and do two

vertically interleaved passes. /FW sets line feeds to 1/6 inch.

For  Epson  printers  and  other  printers  with  1/216   indexing

hardware,  /FN  is  preferred.   For  some  not  quite  compatible

printers,  /FW works.  For IBM Proprinters and other printers with

1/144 indexing hardware, /F1 gives better results.

These drivers use the following escape codes:

esc 3  set n/216 line feeds for all /FW

esc J  index n/216 (n=2 for 1/144 index) for all /FN and /F1

esc L  graphics command for /FN, /FN+, /FW, /FW+, /F1, and /F1+

esc Z  graphics command for /FN*, /FW*, and /F1*

esc r  set ribbon color (only if color is specified)

Epson, Fujitsu, NEC, Toshiba 24 Pin Printers -----------------------

/FT  covers  all  Epson LQ printers and  many  compatible printers

(Panasonic in LQ mode and others).  /FF works with  Fujitsu 24 pin

printers in Fujitsu mode.  /F9 works with NEC 24 pin printers. /FO

works  with Toshiba 24  pin printers.  /FO  resets the  line  feed

distance to 1/6 inch on completion.

For all of these  drivers,  you need  a  cyan/magenta/yellow/black

ribbon to get color prints. The + and * drivers use 1/360 graphics

mode, and the *  drivers use 1/360 (1/120  for /FO*)  indexing and

reset the line feed distance  to  1/6 inch on completion.  A clean

paper  path  with equal tension on  each side  is  needed for good

360x360 graphics.

continued

p-33

Many 24 pin printers are not capable of 360x360 graphics and a few

cannot handle 360x180 graphics.

The compression mode  drivers  (!  suboption  or  "modify  output"

"compression mode: Y") may or may not be an improvement.

The /FT drivers use the following printer escape codes:

esc $    absolute tab

esc * '  graphics command for /FT

esc * (  graphics command for /FT+, /FT*

esc +    set n/360 indexing for /FT*

esc 2    set 1/6 indexing for /FT*

esc J    index n/180 for /FT, /FT+

esc r    set ribbon color (only if color is specified)

Epson Stylus and Esc/P2 printers -----------------------------------

/F$  drives  Epson Stylus  and  other  Esc/P2 printers.  Note that

Esc/P2 requires a different prefix code for paper size  than other

line  printers.  Always use the form feed option with  this driver

unless you have previously set compressed graphics mode.

/F$+, the 720 dpi Esc/P2 driver, is only for the Stylus Color with

special paper.  The  prints  are  screened  by  50%  to  eliminate

excessive ink  flow  (which  would  result  in  ink  bleeding  and

pooling).

/FB~  drives  Epson  Stylus  and  older  Epson  48  nozzle  inkjet

printers,  using  compressed  Esc/P  control codes.  It  does  not

require a form feed or interfere with previous printer settings.

Fax/Modems ---------------------------------------------------------

PrintGL will not create data for direct use by  fax/modem devices.

Most fax software will read PCX or DCX (for multiple images) files

and PrintGL can generate these files.

Use /FZ~203,196 or "ZSoft PCX b/w h,v dpi: 203,196" to  generate a

PCX at 203x196 dpi which is the native fax high resolution.

HI Jetpro V50/100 --------------------------------------------------

You can drive the HI V50/100 with  HP-GL/2.  Set the resolution to

360  (/F0  360)  and  reverse the  width and  height  in the print

window, for example 10x8  instead of  8x10 for  8.5x11 paper.  The

maximum window height is 15.25.

In printer mode the HI V50/100 acts as a Canon BJ IBM mode printer

(/F7).  The  maximum  print width is 15.25 and you need the prefix

code 27,91,88,2,0,1,255 to get beyond 13.6 inches.

p-34

HP DeskJet ---------------------------------------------------------

/FI works with any black/white DeskJet in portrait mode.

/FA has three modes for different levels of color HP-PCL. /FA uses

RGB color and works with the DeskJet 500C. /FA! uses CMY color and

works with all color DeskJets.  /FA!` uses CMYK color to give give

better  blacks  only on the  DeskJet 550C and 560C  (the DJ  1200C

converts from CMY to CMYK color internally).

HP LaserJet --------------------------------------------------------

/FL works with any LaserJet compatible printer. /FL! works with LJ

2 (PCL 4) printers.  /FI works with LJ 3 (PCL 5) printers. /FI+ is

a 600 dpi driver for the LaserJet 4.

All  of these drivers work in portrait mode, which may be set with

/J 27 38 108 48 79 or you can reset the printer with /J 27 69.

LaserJets with under one meg of memory may not be able to handle a

full page of graphics. /FL!  uses less printer memory than /FI, so

it may be preferred on LaserJet 3s with limited memory.

HP PaintJet --------------------------------------------------------

/FP drives 180 dpi PaintJet printers. /FP! uses data transfer mode

2 and works only with a PaintJet XL.

Use /FA! (the DeskJet CMY driver) with the PaintJet XL300.

HP QuietJet --------------------------------------------------------

/FQ works with QuietJets in HP-PCL mode.

HP-GL, HP-GL/2, and HP-RTL -----------------------------------------

/F8 drives HP-GL and HP-GL/2 pen plotters. The output is low level

HP-GL, converting  text, arcs,  fill, and  clips to  vectors.  The

output is  HP  7220 compatible, containing IN, IP, SC, SP, IW, LT,

PA, PU, and PD commands. PG is used for form feeds. If you specify

no form feed then multiple plots may be appended to the same page.

/F0 drives HP-GL/2 raster plotters (HP-GL/2 / HP-RTL devices).

/F0- drives HP-GL/2 printers (HP-PCL 5 devices).

/F0+ drives HP-RTL raster plotters.

For any of these drivers,  you can append  the  nominal resolution

(for example /F8 300).  When you  set  the nominal  resolution  of

HP-GL to other than 1016,  enhanced  IW command interpretation (HP

7550 enhanced mode) is needed to handle dashed lines correctly.

For  HP-GL  output  the  width  parameter (/W) is  used  as  a pen

selector, so for each pen in the original HP-GL, you can specify a

new  pen  in  the  output.  The  default  /W  option  for  /F8  is

/W12345678. Shading and color are ignored.

p-35

IBM Inkjets - ExecJet 4072, ExecJet II, Color Jetprinter 4079 ------

These are three completely different printers. The ExecJet 4072 is

Canon BJ330 compatible. Put the printer in IBM mode and use /F7.

The ExecJet  II and IIc  are DeskJet and  DeskJet 500C  compatible

printers and should be used with /FI and /FA!. Use the prefix code

27,38,108,48,69 to start printing at the top of the paper.

The  4079 is basically  a PostScript printer and it can be  driven

with /FS360.  Newer versions of  the printer can be  set  to ASCII

text mode and driven with /F& (Canon BJ  native mode),  which will

likely be faster than PostScript.  Older printers can be turned on

with  the two leftmost buttons held down until startup is complete

and then put in ASCII dump mode to use  /F&.  Always use form feed

with the BJ native driver unless you have previously set BJ native

mode.

IBM LaserPrinter 4019, 4029, 4037, 4039 ----------------------------

/F4  drives  any LaserPrinter 4019, 4029, or 4037 in PPDS (native)

mode.  /FL will work for HP LaserJet mode.  At 300 dpi it takes up

to one meg of printer memory to print a full page of graphics.

The !  suboption or "modify output" "compression mode: Y" switches

to 4029 data compression.  /F4+!  works with the 4029 with 600 dpi

PPDS upgrade.

All 4039s are HP-PCL printers and  should  be  driven with /FI  or

/FI+ (the printer menu must be set to 600 dpi in this case). These

printers do not support PPDS mode (/F4).

IBM 24 Pin Printers ------------------------------------------------

PrintGL has three drivers  for Proprinter X24 compatible printers.

Use  native  mode, not AGM.  /F6 is for all IBM X24s and  the 24P.

These  printers  do 1/144 inch indexing and using  them at 180 dpi

gives horizontal  white streaks  every 2/3 inch.  The  /F6 drivers

compensate  for this by indexing 19/144  per line, giving 182 dpi.

To work properly the  printer must start on  a 1/72 boundary so it

is best to keep line feeds at a multiple of 1/72 (1/6, 1/8, 1/9).

/F5 is referred to as X24 alternate 1 and is for printers that are

X24 compatible  except for  1/180 inch indexing.  This is the case

with  most X24 compatible printers, including the IBM  Quickwriter

and IBM 2390 and 2391. /F5* works only with printers that do 1/360

indexing, including the 2390 and 2391 but not the Quickwriter.

/FX  is referred to  as X24  alternate 2  and is  the same  as /F5

except for the  vertical units command.  /FX  works with Panasonic

printers in X24 mode.

The compression mode  drivers  (!  suboption  or  "modify  output"

"compression mode: Y") may or may not be an improvement.

p-36

IBM Quietwriter 2 and 3 --------------------------------------------

/F2  and /F3 drive the Quietwriter 2 and 3.  Line feeds are set to

1/6 inch on completion.

JRL J bubblejet ----------------------------------------------------

/F# drives the JRL J bubblejet in Epson LQ mode.

Kodak Diconix Color 4 ----------------------------------------------

/FK drives the Color 4.

Mannesmann Tally MT92C ---------------------------------------------

/FA (the DeskJet 500C RGB driver) drives the MT92C.

Pacific Data Products ProTracer ------------------------------------

/FG drives the ProTracer (base model) in IBM Proprinter mode.  The

maximum print window width is 14.97.

PostScript ---------------------------------------------------------

/FS outputs printable Encapsulated PostScript with no  preview bit

map.  Text, arcs,  fill,  and clips are  converted to vectors,  so

PrintGL is not a general purpose HP-GL to PostScript converter. If

no form feed is specified then "showpage"  is not output, and more

PostScript  may be appended to the page.  PostScript overwrites as

it draws, so overlapping lines appear opaque.

You  can  specify  the  nominal  resolution  of the  PostScript by

appending it to the /F option (for example /FS 300).

You  can specify  the paper tray with  a prefix code that includes

the tray number followed by " XP". For example /J 49 32 88 80 uses

tray 1.

/FY (not included in PrintCAD) appends  a black/white TIFF preview

bit map to a PostScript file.  To use it, create a PostScript file

with PrintGL  and then  run  PrintGL  again, specifying  the  same

options  except /FY  instead  of  /FS.  The  destination  file  is

appended by  default.  /FY assumes a PostScript file, generated by

PrintGL, is  being  appended.  You  may set  the resolution  - the

default is 72x72 dpi. Here is an example:

PRINTGLD printgl.plt /Dsample.eps/LB6,6/O2/FS

PRINTGLD printgl.plt /Dsample.eps/LB6,6/O2/FY

Star Micronics SJ-144 ----------------------------------------------

/F: drives  the SJ-144.  Always use the form feed option with this

driver unless you have previously set CDM mode.

When  using /F: with PrintCAD and color you might need to use /XM+

to give PrintCAD enough memory for processing.

p-37

Tektronix ColorQuick -----------------------------------------------

/FR drives the ColorQuick.

Windows Printer ----------------------------------------------------

/FW%  drives the default Windows printer.  The data is sent at the

resolution of the  printer and is always sent to the Print Manager

regardless of  the PrintGL destination.  !  causes PrintGL to band

the  output to the driver.  This can  improve print speed or cause

Windows to abort the print.

ZSoft PCX, DCX Bit Map ---------------------------------------------

/FZ and /FZ~ output 1 bit/plane color and monochrome ZSoft PCX bit

maps.  The horizontal  and vertical dots/inch  may be appended  to

override the default 100x100, for example /FZ80,90.

PCX  output should generally  be  directed to  a  file with the /D

option.  For multiple page plots and tiled plots use a three digit

number for the output file (/D option) so that a new (sequentially

numbered) file will be created for each page.

Since  most fax  software can  take  PCX  files  as input, this is

useful  for  faxing images.  Use  /FZ~203,196  to  match  the  fax

resolution.

DCX is an extension to PCX that allows multiple images  in a file.

/FZ! and /FZ!~ are analogous to the PCX drivers but you can append

these images  to other DCX images by setting the destination to an

appended file.

p-38

Useful Prefix and Suffix Codes ---------------------------------------

Below, n represents  an integer,  nA represents an  integer in ASCII

format (1 becomes 49, 2 becomes 50, 43 becomes 52 51, etc.).

Canon,      Reset                    27 64

Epson,      N inch paper             27 67 0 n

Fujitsu,    17 inch paper            27 67 0 17

IBM, NEC    Right margin n columns   27 81 n           (not for IBM)

dot matrix, Skip perforation off     27 79

inkjet      Unidirectional printing  27 85 1

Canon BJ    Page length, print width x 10   27 40 103 3 0 n 1 n

native

Esc/P2      N/dpi inch paper         27 40 67 2 0 lo(n) hi(n)

Unidirectional printing  27 85 1

HP PCL,     Reset                    27 69

DeskJet,    Portrait mode            27 38 108 48 79

LaserJet,   Top margin 0             27 38 108 48 69

PaintJet    Skip perforation off     27 38 108 48 76

11 inch paper            27 38 108 50 65

14 inch paper            27 38 108 51 65

11x17 inch paper         27 38 108 54 65

Manual feed              27 38 108 50 72

HP DJ500C   High quality, slow       27 42 111 49 81

No ink depletion         27 42 111 49 68

HP PJXL300  Low quality, fast        27 42 111 45 49 81

HP QJ       HP-PCL mode              27 37 65

DIP switch mode          27 37 64

IBM LP      11 inch paper, tray 1    27 91 70 4 0 3 1 1

14 inch paper, tray 1    27 91 70 4 0 3 1 2

14 inch paper, manual    27 91 70 4 0 1 1 2

Canon       Reset                    27 99

CaPSL,      11 inch paper            27 91 51 48 59 59 112

LBP,        14 inch paper            27 91 51 50 59 59 112

BJC880      Manual feed              27 91 49 113

Automatic feed           27 91 48 113

Full page memory mode    27 59 27 91 50 38 122

Cursor to top of page    27 91 48 100

Diablo mode              27 58

ISO mode                 27 59

PostScript  Paper tray n             nA 32 88 80

ProTracer   N line paper             27 67 n

HI V50/100  Max margins              27 91 88 2 0 1 255

p-39

Answers --------------------------------------------------------------

If  your plot is messed up, with wraparound lines covering the page,

you have probably exceeded PrintGL's internal coordinate limit. This

happens when the  HP-GL  coordinates exceed  32 inches.  Reduce  the

internal resolution with "plotter" "internal  dpi: 762" or /R762 for

coordinates up to 42 inches or /R508 for up to 64 inches.

If  your  printer ejects the paper before the  plot is finished, you

probably need to specify a  paper length prefix code.  For most line

printers use /J27 67 0 n where n is the page length in inches. Canon

BJ  native  mode uses /J27 40 103 3 0 n 1  114  where  n is the page

length in inches times 10  (the  default is 17).  Epson Esc/P2  uses

/J27 40 67 2 0 lo(n) hi(n) where n is  a two byte integer specifying

the page length in dots.  The PDP ProTracer in IBM mode uses /J27 67

n where n is the page length in lines (6 lines/inch usually).

If  an  extra sheet of paper feeds  after a print, your print window

probably  exceeds the print area of the printer.  Turn form feed and

box  on and reduce the vertical window  or margin until then  entire

box prints and just one sheet feeds.

If  you  are having trouble getting the print size  or  location  as

desired, check your /L option or "window/margins".  The print window

plus  the  print  margins  must define  an area  that  fits  in  the

printable area of  the paper.  Printers cannot print over the entire

area of the paper. They usually require .25 to .5 inch margins.

To  get  color output from PrintGL set  up the pens in your graphics

package so that each pen is assigned  a  color  and assign the  same

colors to PrintGL's pens with /C or "pen color".

If  your prints are  coming  out not  quite to  scale, check that no

calibration is being  done by the  graphics package and then use the

"modify  output" "h,v size multiplier:" option  or  the /F option to

adjust the print size. See /F - Output Format Option for information

on print size correction.

If  your print is  being clipped unexpectedly, and  PrintGL  is  not

giving  a "Plot will clipped" message, the clipping may be caused by

the  program  that   generated   the   plotfile,   the  printer,  or

misinterpretation of the plotfile. To ensure that the printer is not

doing  any clipping,  turn box  on and reduce the  window or margins

until then entire box prints.  If the plotfile is using the enhanced

clipping  window  command,  you  must   set  "plotter"  "S/E  switch

position: Y" to interpret it correctly.

For the Canon BJC-800 in Epson mode, the right margin  defaults to 8

inches. Use /J27 64 27 81 114 to set it to 11.4 inches.

For the  Canon BJ-330 and most wide  carriage printers,  the maximum

print window  width is 13.57  inches.  This is a printer  limitation

that you cannot get around.

p-40

PrintCAD Answers -----------------------------------------------------

Do not use PrintCAD with Windows. Use PrintGL/W instead.

To access PrintCAD from a CAD package you must PLOT. Do NOT print.

AutoCAD 10 will work with PrintCAD only via /PADI.

If you are getting system  crashes or nothing from PrintCAD, try the

options /XAM and /XAEM.

To get COLOR output from PrintCAD, you must:

- Define a multipen plotter in your CAD package.

- Set up the  pens in the CAD package so that each color is assigned

to a pen.

- Use the /C option to assign the same colors to PrintCAD's pens.

Plot spoolers  are not generally compatible with PrintCAD, and would

be of little value if they were.  Print buffers/caches are generally

compatible and useful. You must install the buffer first.

If your prints are coming out not quite to scale:

- For ADI plotters check that the plotter is set to 1016 steps/inch.

- Check that no calibration is being done by the CAD package.

- See /F - Output Format Option for print size correction.

User Support ---------------------------------------------------------

Correspondence  about  this  program  may be  sent  via the  support

bulletin board, fax, Compuserve, Internet, or mail.

BBS:        606-268-0577  1200..14400,N,8,1  24 hours/day

To ask a  question prepare an  ASCII  text file with the  details.

Call  the bulletin  board and choose the  upload question  option.

XMODEM,  1K  XMODEM, or ZMODEM file transfer protocol is  required

for this. Your question will be assigned a number (such as 1015) -

remember  this number.  An answer  file (with the assigned number)

will be posted, usually  within 24 hours on  weekdays.  To get the

answer, call  the bulletin board, choose the download option, then

the answer option, and then enter the answer number.

Compuserve: Cary Ravitz [70431,32]

Internet:   70431.32@compuserve.com

Fax:        606-268-0577

If  your fax machine does not send the fax calling signal, the BBS

will  answer and  hang  up (about  30  seconds), then the  call is

routed to the fax.

Mail:       Ravitz Software Inc.

PO Box 25068

Lexington, KY  40524-5068

USA