(C) 1994  T.C. Green, PEng (t.green@ieee.org)

EXCON -- direct control of X10 events via CP-290

Syntax:

excon [c=config] [+colour|+mono] [-mouse] [p=comport]

[h=house_code] [u=unit_code] [i=intensity] [all]

[lights] [house_code|unit_code|both_code]

[on|off|dim|fade]

Options:

c=      specify the name of the configuration file

(default is EX10.CFG in the current directory, or

if this fails, EX10.CFG in the directory from

which EX10.EXE was executed).  This configuration

file is the same used by the EX10 program.

+c      override the screen colours specified in the

configuration file with the default set for a

colour screen

h=      house code used in direct command mode (default

is "h=A"), in range A to P

i=      intensity used in direct command mode,

percentage in range 0 (full off) to 100 (full on)

+m      override the screen colours specified in the

configuration file with a default set visible on a

monochrome screen

-m      assume there is no mouse (default is to assume

there is a mouse and complain if it can't be

found).  This means that the mouse cannot be used

to control the program.

p=      override the comport specified in the

configuration file (if any).  If there is no

configuration file, and the "p=" option is not

used, p=2 is assumed (interface on COM2).  COM1

through COM4 (p=1 through p=4) are supported.

u=      unit code used in direct command mode, in range

1 to 16

lights  direct mode command applies to lights only

all     direct mode command applies to all units (or

possibly all lights if "lights" is given)

on      turn specified unit(s) on; direct mode

off     turn specfied units(s) off; direct mode

dim     turn specified units down to specfied intensity;

direct mode (only works for lamp modules that can

be dimmed)

fade    dim units down to zero intensity if they are lamp

modules, or turn them off if they are not; direct

mode

house_code  house code will apply to all unit codes that

follow it, for specifying direct mode command

(same has "h=")

unit_code  unit code to specify which unit will respond

to direct mode command (same as "u=")

both_code  house code and unit code (e.g. A12) of units to

be controlled by direct mode command

Examples:

excon

excon A10 off

excon A10 A12 i=50 dim

excon D all off

Description:

EXCON is a companion program to EX10 and shares a common

configuration file.  It requires a CP-290 to work.  It has an

interactive mode (invoke without direct command mode arguments)

arguments as in the first example) for which the interface is

much like that of EX10.  Read the documentation for EX10 first.

EXCON can also issue X10 commands throught the CP-290 to

specified units.  Units to be controlled can be specified with

explicit house and unit code arguments (e.g. "h=", "u=") or with

the combined form (e.g. A12 or P2).  Units can also be specified

as "all" meaning all units for the specified house code, or as

"lights" (usually "all lights").  Multiple units for a single

house code can be controlled in a single command.

EXCON actions include "off" (turn specified units off), "on"

(turn specified units on), "dim" (dim dimmable units to the

intensity previously specified with the "i=" argument) and "fade"

(uses an undocumented feature in the CP-290, and may not work in

all situations).  Using "all lights" may not work properly in all

situations either since it also uses an undocumented feature.

Limitations:

There can only be one action for each command line in direct

command mode; only the last command found as the command line is

parsed left to right will be executed.  For example, the following

will not work properly:

excon A10 on A12 off

Units for only a single house code can be controlled in a single

direct mode command.  For example the following will not turn

both A10 and B12 off:

excon A10 B12 off