BBS Weather (C)   Version 3.0   1-8-94

Copyright 1994 OctagonSoft, Inc.   All Rights Reserved

Written by Ben Shoval

_________________________________________________________________

Table of Contents

I..... Intro

II.... Outside Information

III... BBSW.OPT

IV.... BBSW.CFG

V..... Bugs

VI.... Fossil Drivers

VII... Disclaimer

VIII.. Credits

IX.... Upgrades

_________________________________________________________________

I.

To many people's  surprise, you, a BBS SysOp, can provide

your users  with local  and state  weather  information,  updated

every 3 to 4 hours.  And the best part is, it's absolutely free!

_________________________________________________________________

II.

For the program to work you find out the following:

1) Your local airport's 3 letter identification.  This can be

easily  obtained by  calling your local  airport.  Keep in

mind that  weather for extremely  small  airports  is  not

available.  Ask if they have a  FAA people  working there.

If they don't, you  can't get weather information.  Try to

find a larger airport in your area.

2) Your  state's two letter  abbreviation.  It's the one used

on letters that you certainly know by heart!

_________________________________________________________________

III.

Once you've gotten  this information, use  your  favorite

text editor  to modify (or  create if it does not exist) BBSW.OPT

The file must be in the same directory as BBSW.EXE

Follow these guidelines exactly:

Line 1: Airport ID

Line 2: State Abbreviation

Line 3: Local Forecast File

Line 4: Extended Forecast File

Line 5: Zone Forecast File

Line 6: Recreational Forecast File

Line 7: Redials

_________________________________________________________________

IV.

Next  modify (or create) BBSW.CFG in the same  directory.

Follow these guidelines exactly:

Line 1: $3F8 for COM1

$2F8 for COM2

$3E8 for COM3

$2E8 for COM4

Line 2: 4 for COM1 or COM3

3 for COM2 or COM4

Line 3: Modem Speed (300, 1200, 2400, etc.)

Line 4: Parity

Line 5: Bits

Line 6: Stop Bits

Line 7..14: Init Strings 1 through 8

LIne 15: Dial String

Line 16: Alternate Dial String

Line 17: Termination String

_________________________________________________________________

V.

If you  spot a bug, please  report it to  OctagonSoft  at

(717) 288-3414 or write to 46 E. Dorrance St, Kingston, PA 18704.

You may also reach the  author, Ben Shoval, on The  Tower of High

Sorcery BBS by calling (717) 696-2236.

_________________________________________________________________

VI.

SysOps  running fossil  drivers must  first turn  off the

fossil  driver before running  BBS Weather.  To accomplish  this,

you can create a  batch file  called BBSW.BAT which turns off the

fossil driver,  runs BBSW.EXE, and  then turns  the fossil driver

back on.

_________________________________________________________________

VII.

OCTAGONSOFT  HEREBY DISCLAIMS  ALL  WARRANTIES  RELATING TO  THIS

SOFTWARE,   WHETHER   EXPRESS  OR   IMPLIED,  INCLUDING   WITHOUT

LIMITATION ANY  IMPLIED WARRANTIES OF  MERCHANTABILITY OF FITNESS

FOR A PARTICULAR PURPOSE.  OCTAGONSOFT WILL NOT BE LIABLE FOR ANY

SPECIAL,  INCIDENTAL, CONSEQUENTIAL, INDIRECT OR  SIMILAR DAMAGES

DUE TO LOSS OF DATA  OR ANY OTHER REASON, EVEN  IF OCTAGONSOFT OR

ANY AGENT OF OCTAGONSOFT  HAS BEEN ADVISED  OF THE POSSIBILITY OF

SUCH DAMAGES.  IN NO EVENT  SHALL OCTAGONSOFT'S LIABILITY FOR ANY

DAMAGES  EVER  EXCEED  THE  PRICE  PAID  FOR  THE  LICENSE TO USE

SOFTWARE, REGARDLESS OF THE  FORM OF THE CLAIM.  THE PERSON USING

THE SOFTWARE BEARS  ALL RISK AS TO THE QUALITY AND PERFORMANCE OF

THE SOFTWARE.

_________________________________________________________________

VIII.

Special thanks to the following for their help:

Raistlin Majere

Ken (from the Docksider BBS)

Wayne E. Conrad

Connecticut State BBS

Jeffery Weiss

_________________________________________________________________

IX.

You can always get the newest version of  BBS Weather and

all   OctagonSoft  software   on  The  Tower   of  High   Sorcery

(717) 696-2236 in file  directory #19.  Messages  to the  author,

Ben Shoval, may also be left in [F]eedback to Co-SysOp #5.