Voice              B.J. Guillot            Data/Fax

713.893.9320      2611 Rushwood Circle      713.893.9124

Houston Texas 77067-1941

Copyright (C) 1993-1994 B.J. Guillot.  All Rights Reserved.

=============================================================

BGFAX 1.21                                      FRI 25 MAR 94

EXPIRES 30 JUN 94

=============================================================

Dedicated to Gene Roddenberry

1921-1991

-------------------------------------------------------------

ABSTRACT                                  FidoNet 1:106/400.0

-------------------------------------------------------------

* Allows sysops to receives faxes on their BBS telephone line

without needing a fax switch box and real fax machine

* Everything runs in the foreground, no TSR programs needed

* If you are already running a Fido mailer, you can configure

your mailer to exit and run BGFAX when the line is hot, thus

BGFAX does not necessarily have to answer the phone itself

* Comes with a small and quick fax viewing program that supports

CGA, EGA, and VGA display modes as well as FAX->PCX conversion

* Works with modems that offer a feature known as adaptive

answering on Class 1, Class 2, Class 2.0 and ZyXEL fax modems

* Can send faxes

-------------------------------------------------------------

INCLUDED FILES

-------------------------------------------------------------

1.     BGFAX.DOC -- general documentation

2.     WHATS.NEW -- release notes

3.     BGFAX.EXE -- the main executable

4.    HELPME.ARJ -- specialized help files for various setups

5.      VIEW.EXE -- fax viewing and PCX conversion utility

6.      NFAX.COM --\  optional TSR utility to notify the user

7.    FAXOFF.COM --/  if a fax has been received

8.   MAKEFAX.EXE --\ converts PCX and ASCII files to ZFAX format

9.     BGFAX.FNT --/ font file for MAKEFAX

10.   SOURCE.ARJ -- TP 7.0 source to VIEW.EXE and MAKEFAX.EXE

11. REGISTER.FRM -- registration form

-------------------------------------------------------------

DISCLAIMER

-------------------------------------------------------------

BGFAX and accompanying files are provided AS IS without any

warranty, expressed or implied.  This includes without limitation

the fitfulness for a particular purpose or application and any

warranties of merchantability.

While I tried to be as through as possible in preparing BGFAX,

B.J. Guillot shall not be liable for any damages, whether direct,

indirect, special, incidental or consequential arising from a

failure of this program or accompanying files to operate in a

manner desired by the user.  B.J. Guillot shall not be liable for

any damage to data or property which may by caused directly or

indirectly by use of this program.

In no event will B.J. Guillot be liable to you for any damages,

including any lost profits, lost savings or other incidental or

consequential damages arising out of your use or inability to use

the program, or for any claim by any other party.  Individuals using

the software bear all risk as to its quality and performance.

-------------------------------------------------------------

CONTACTING ME

-------------------------------------------------------------

FidoNet:  1:106/400                   Voice: +1 713.893.9320

InterNet:  st1r8@jetson.uh.edu      Data/Fax: +1 713.893.9124

The latest versions of BGFAX can be obtained by Fido FREQ at the

above Fido node number with the magic name "BGFAX" and can be

obtained by anonymous FTP for InterNet users at the site

"CSN.ORG" in the "Computech" directory.

BGFAX can also be obtained from Fido 2:512/39.  This system is

called The Owl's Nest and is the Dutch Supra Distributor.  They

have three nodes:

+31 0.2155.10921      +31 0.2155.12571      +31 0.2155.28096

An alternate distribution site is Fido 3:635/552.  Cloud Nine.

+61-3-803-6954

-------------------------------------------------------------

MISCELLANEOUS NOTES

-------------------------------------------------------------

Currently, BGFAX does not come with a utility to print faxes.  If

you wish to be able to print faxes, you will need to have a copy

of ZyXEL's ZFAX software (version 2.23 perferred) or Smith Micro

Software's QuickLink II, a commerical program included with many

fax modems.

Support for Fax Talk Plus-style fax images will be added in a

later version.  WinFax support may or may not be offered.  It

depends on whether or not Delrina will release the information to

me I need.

BGFAX supports Desqview's alternate video buffer zone, if used,

and will return ticks to other tasks.  BGFAX also issues an

Interrupt 28h call every now and then for OS/2 time slices.

This program was compiled under Borland's Turbo Pascal 7.0.

I routinely test BGFAX with a Supra Fax Modem (v.32bis external

with V1.80-02 firmware), a USR 21.6K terbo (with 12/08/93

firmware), a Hayes Optima 288 (V3.1 firmware), and a generic

SIERRA based 2400 data/9600 fax modem from Computer City.

If you wish to put BGFAX in DEBUG mode (where it logs most of the

strings sent out and in from the modem in a DEBUG.LOG file), you

must set the following environment variable:

SET FAX=DEBUG

-------------------------------------------------------------

TALKING WITH THE COM PORT

-------------------------------------------------------------

BGFAX does not require a FOSSIL driver.  It will normally use its

own internal communications routines to talk to your UART chip.

BGFAX supports the 8250, 16450, and 16550A chips directly.

BGFAX assumes...

COM1=3F8x4     COM2=2F8x3     COM3=3E8x4     COM4=2E8x3

NONSTANDARD PORTS

=================

If you are using a nonstandard port configuration, whenever BGFAX

requires a port number to be entered (such as po=2), instead of

entering the port number, enter the base address and IRQ seperated

by an 'x' character (such as po=2E8x5).  BGFAX requires the base

address be given in HEXIDECIMAL notation, and the IRQ to be given

in DECIMAL notation, since all IO boards seem to use this method.

If your COM3 was at 2F8h and IRQ 10, you would say (in the .CNF):

po=2f8x10

THE 19200 SHIFT

===============

Many Class 2 fax modems require a DTE shift to 19200 bps when

performing fax operations.  Yes, even if you have your port locked

at 57600 bps, BGFAX will temporarily shift the speed to 19200.  It

should be noted, however, that there are some exceptions to this

rule.  Some modem manufactuers did not follow the 19200 shift

method, and thus, you will have to tell BGFAX if you have one of

these special modems.  The ZyXEL modem is a prime example.  Also,

it has been brought to my attention that some Zoom modems also do

not need to be shifted to 19200 bps.  (Which ones?  It seems very

random.  You'll have to discover for yourself with trial and

error.)  So, how do you tell BGFAX _not_ to shift to 19200?

Putting an exclamation mark (bang) after the port will tell BGFAX

not to shift the DTE speed to 19200 when receiving a fax.  (i.e.,

whenever you want to specify COM2, do as as "2!" instead of "2").

Class 1 and Class 2.0 fax modems do not require the 19200 DTE

shift and BGFAX already knows this.

IF YOU ARE NUTS AND WANT TO USE BGFAX WITH A FOSSIL DRIVER

==========================================================

If for some reason, you NEED to use a FOSSIL driver (this is not

recommended), you can put the letter 'F' in front of the COM port

designator and BGFAX will use your FOSSIL driver.  BGFAX treats

COM1 as F1, COM2 as F2, etc.  So, to use FOSSIL-COM2:

po=F2

If you are using a Class 2 fax modem that requires a 19200 shift

when operating in fax mode, if you "lock" your FOSSIL driver,

BGFAX will not be able to shift the port speed to 19200!  You will

have to tell your FOSSIL driver to "unlock" the port, i.e.:

BNU /L0=0

BGFAX /FAX C:\BGFAX F1 Z

BNU /L0=38400

Note the "F1" means for BGFAX to use the FOSSIL on COM1.  The

"/L0=0" parameter on BNU means for BNU to "unlock the port".

Why?  Because, the FOSSIL driver will *ignore* a DTE shift request

when its in "locked" mode.  BGFAX will be unable to relock your

port with the FOSSIL driver, so that is the purpose of the

second BNU line.  It re-locks COM1 at 38400 (or whatever) rate

you want to use.

Rememeber that BGFAX has its own communications routines, and

doesn't need to use a FOSSIL driver, but it can if you absolutely

have to use one.

OPTIONS WHEN NOT USED IN THE BGFAX.CNF FILE

===========================================

If you are running BGFAX in rear end mode (using one of the /Fxxxx

parameters on the command line), BGFAX does NOT look for the

BGFAX.CNF file.  Everything is passed to it on the command line,

so if you needed to specify an alternate address and IRQ:

bgfax /fax c:\bgfax 3f8x9 z

...or use a FOSSIL driver on COM4...

bgfax /fax c:\bgfax F4 z

-------------------------------------------------------------

RECOMMENDED INITIALIZATION AND ANSWER STRINGS

-------------------------------------------------------------

This section tells you the recommended initialization and answer

strings for your modem.  It will not tell you what the commands

do.  Look in the "Quick tutorial" second to find that out.

Class 2 modems (Supra v.32bis, PPI, Zoom, etc.)

Init-1 ... ATZ|

Init-2 ... AT+FCR=1;+FDCC=1,5,0,2,0,0,0,0|

Init-3 ... AT+FLID="713 555 1212"|

Answer ... AT+FAA=1;A|

Dial ..... ATDT

Class 2.0 modems (USR 21.6K)

Init-1 ... ATZ|

Init-2 ... AT+FCLASS=2.0|

Init-3 ... AT+FLI="713 555 1212"|

Answer ... AT+FCLASS=2.0|~AT+FAA=1+FNR=1,1,1;A|

Dial ..... AT+FCLASS=0DT

Class 1 modems (Hayes Optima 288, Supra 144LC)

Init-1 ... ATHZ|

Answer ... AT+FCLASS=1|~AT+FAE=1;A|

Dial ..... AT+FCLASS=0DT

ZyXEL modems (ZyXEL E, S, E+, S+, etc.)

Init-1 ... ATZ|

Init-2 ... AT#P713 893 9124

Init-3 ... AT#B1+FCLASS=6|

Answer ... ATA|

Dial ..... ATDT|

-------------------------------------------------------------

QUICK TUTORIAL ON FAX COMMANDS AND ODDITIES

-------------------------------------------------------------

AT+FCLASS=?

===========

This command asks the modem to tell you what fax classes it

is compatible with.  It will respond with something like:

0,1,2  or  0,2  or  0,1,2.0  or  0,2,6  or  0,1,2,6  or  ERROR

0 = Data

1 = Fax Class 1 capable

2 = Draft Fax Class 2 "compatible"

2.0 = Fax Class 2.0 capable

6 = ZyXEL or Rockwell voice mode compatible

ERROR = no fax capabilities

GROUP and CLASS

===============

Many people are often confused between the terminology "GROUP" and

"CLASS" with regards fax modems.

ALL FAX EQUIPMENT is "GROUP 3" compatible.  This includes fax

machines, fax modems, standalone PC fax cards, etc.  Group 3 just

defines the fax specifications, e.g., black-and-white, the

resolutions, etc.

"CLASS" refers only to fax modems.  CLASS has NOTHING AT ALL to do

with fax machines.  Class 1, Class 2 and Class 2.0 are different

methods of sending COMMANDS to a fax modem from a computer serial

port.  Many fax modems offer the Class 2 command set.

GROUP III

=========

Most Group III fax images are 1728 bits across.  A normal

resolution page is approximately 1100 scan lines long, while

a high resolution page is approximately 2200 scan lines long.

When the fax devices are handshaking, this occurs at 300 bps,

regardless of what the negoiated fax DCE rate is (i.e., 9600).

Computer fax modems use v.17 modulation at 12000 and 14400,

v.29 at 7200 and 9600, and v.27ter at 2400 and 4800 bps.

Fax images can use one or two-dimensional modified huffman

encoding.  One-dimensional is known as MH, and two-dimensional is

known as MR (Modified Read).  MR encoding sends one scan line,

followed by special bits that spell out differences between the

above line and the current line.  Some higher tech machines also

use MMR encoding (Modified, Modified Read).  MMR encoding sends

the first scan line of the page, followed by x-number of

difference lines, rather than just 1 difference line.  BGFAX

currently supports only MH encoding.  The remote fax machines will

always default to MH encoding since MR and MMR is not available.

Fax images have things known as EOLs and RTC.  EOL stands for

End-Of-Line and is represented as 000000000001.  Fax viewing

programs read through the fax image file, scan line by scan line.

They know where each scan line ends by searching for all the EOLs

in the file.  RTC, Return-To-Control, is a series of six EOLs.

The RTC sequence usually indicates an "end of page".  No huffman

codes start with 11 zeros like the EOL character.  Why am I

telling you all this?  Because many older fax machines send "fill"

to your fax modem when receiving.  "Fill" is just extraneous zeros

that are stored on your hard disk that take up space.  For

example, you might have an scan line that ends with...

0000000000000000000000000000000000000000000000000000000000001

...but, all that is actually needed is 000000000001.  As you can

see, many extraneous zeros could be filtered from this line to

reduce the size of fax image files a few bytes per scan line.

Another interesting Group III fact is that a "training check" is

performed by fax machines shortly after they connect.  The

training check consists of the sender sending zeros for

approximately 1.5 seconds (at the negoiated rate, i.e., 9600

bps).  The receiving end will count all the zeros received, and

any non-zeros received (bad bits that got corrupted by noise over

the phone line), and if 10% of the bytes received were bad

(non-zero), the receiving end will request a RETRAIN.  This

usually involves a downshift of 2400 bps, i.e., 9600 -> 7200 bps.

After the shift, another training check will be performed.

AT+FAA=1

========

This command will put a Class 2/Class 2.0 modem in ADAPTIVE

ANSWERING mode.

Your modem will properly mate with either a standard modem

data call or a fax machine/fax modem.  Some modems, such as the

Supra, will report a "FAX" response the instant a fax CNG tone is

heard from the remote end.

All modems, on true fax calls, will report a "+FCON" response code

when the fax connection is truly established.  (This is similar to

a "CONNECT 9600" or "CONNECT 14400" on a data call.)

Many modems are slightly brain dead and will forget the +FAA

setting when another command is issued to the modem.  To ensure

you always answer in adaptive mode, rather than using "ATA" to

answer the phone, use the following: "AT+FAA=1;A".

The ";", semicolon, is used as a seperator for fax commands.  Some

modems, such as the Supra, do not require it.  Others, such as the

PPI, will "ERROR" out if the semicolons are not used.

Please note that many modem manufactuers did not test adaptive

answering as much as they should have before they released their

modems.  Some modems won't allow low speed callers into your BBS

when adaptive answering is enabled.  Others won't allow high

speed callers into your BBS when adaptive answering is enabled.

If this is happening, you will need to call your modem manufactuer

and request new ROMs if any are available.  For example, the 14.4K

PC Logic modem's adaptive answering doesn't work at all.  Even

well known companies sometimes overlook adaptive answering.  A key

example is that early version of the USR 21.6K modem would lock up

whenever an HST call would come into their BBS when adaptive answer

mode was enabled.

The Supra Fax Modem, for example, is pretty good concerning

adaptive answering.  (Note that very earlier versions of the

Supra, did not offer adaptive answering until the 1.2-H and 1.2-J

ROMs appeared on the scene.)  However, many older fax machines do

not send out a CNG tone when they are engaged.  Many fax modems,

including the Supra, won't know what to do, and will assume the

old fax machine is really a data caller.  Again, this is not good,

but its not that much of a problem.  Tell the operator of the old

fax machine to push their "START" button *IMMEDIATELY* after dialing

the last digit of the telephone number instead of waiting to hear

the fax tones on our end like many office people do.

AT+FAE=1

========

This is the Class 1 command to enable adaptive answering.

AT+FCR=1

========

This is a Class 2/2.0 command to give your modem PERMISSION to

take faxes.  Note that Class 2.0 modems default +FCR=1, so you

won't need to use this command.

AT+FLID="713 893 9124"

======================

( The Class 2.0 command is AT+FLI="713 893 9124" )

The above command sets your modems fax ID string.  This is the

string that is send to the remote end on fax connects.  (It's the

deal that's printed on the little 'confirmation reports' that many

fax machines make.)  The fax ID can ONLY BE A MAXIMUM OF *20*

CHARACTERS.  If you try to stuff more than 20 characters in a fax

ID string, the modem will respond with an "ERROR".  DO NOT USE

HIGH-BIT ASCII CHARACTERS IN THIS STRING.  Use only simple

letters, numbers, and punctuation marks.  (High-bit ASCII

characters include foreign accented letters, inverted question

marks, etc.)

I've been told that some older fax machines will go nuts if you

use a fax ID with ALPHABETIC letters and that the only true valid

values, as specified by the CCITT (PLUS, SPACE, DASH, and the

numbers ZERO through NINE).  I've never seen this happen

personally, but am informing you of it nevertheless.

Class 1 fax modems have their ID set by the fax software.

AT+FDCC=1,5,0,2,0,0,0,0

=======================

The above command is what the power on defaults to a 14400 fax

modem are, with one exception.  The first number, 1, allows your

fax modem to accept high resolution fax documents.  The power on

default to most modems are to accept only low resolution

documents.  The second number, 5, allows your fax modem to use fax

speeds up to 14400 bps.

If you issue the above command on a fax modem with an upper limit

of 9600 on fax tranmissions, you will receive an "ERROR" or cause

the modem to behave odd.  Use this string on a 9600 fax modem:

AT+FDCC=1,3,0,2,0,0,0,0

The remaining numbers are not that important.  Changing them

around, though, may cause BGFAX to fail.  The command can also be

abbreviated to "AT+FDCC=1,5" or "AT+FDCC=1,3".

You do not need to use this command on Class 1 and 2.0 modems.

STACKING

========

A single command string can be made using what we need ...

AT+FAA=1;+FCR=1;+FLID="713 893 9124";+FDCC=1,5,0,2,0,0,0,0

However, if you are using BGFAX in rear end mode with a Fido

mailer, you may have a hard time of making an initialization

string that long, so you may be required to split the line into a

few pieces.  Remeber to make your ATA command "AT+FAA=1;A" if it

is possible to change in your mailer.

The +FDCC string must come at the end of the stack of commands!

AT+FMFR?

========

( The Class 2.0 command is AT+FMI? )

If you want to find out who makes the fax datapump for your modem,

this command should tell you if you enter it in while in terminal

mode.  My Supra (and many others) report "ROCKWELL".  My Cheap

2400 data, 9600 S/R fax modem reports "SIERRA".  Others report

"EXAR" and other things as well.

AT+FNR=1,1,1

============

This is a Class 2.0 command that tells the USR modem to let BGFAX

know many details about the current fax reception, such as the

resolution of the fax, what the fax DCE speed is currently

negotiated at, etc.  Class 2 modems always report this.  However,

for some strange reason, the Class 2.0 spec writers thought it's

be "nice" to have the fax program NOT knowing this critical

information.  I can think of no logically reason for this command

existing, but if you forget to set it, your faxes on your Class

2.0 modems may look elongated or smashed together.

AT#B1+FCLASS=6

==============

This is a ZyXEL fax command.  This is of use only to ZyXEL owners,

and the ZyXEL user manual explains it very nicely.

FAX MODE

========

Probably 95% of you will be using BGFAX with a high speed modem

(v.32 or v.32bis).  Many of you may have started off with slower

300, 1200, or 2400 modems from long ago.  If you can remember back

that far, when an incoming data call came in, you would see a

string like "CONNECT 1200".  If your port was set at 2400, you

would garbage on the screen unless you shifted your bps rate down

to 1200 to match that of the caller.  Many newer terminal programs

refer to this speed shift as "Autobaud".

Modern modems require your serial port be locked at a specific

speed (such as 19200, 38400, 57600, etc.) so that you can achieve

higher thruput via compression engines such as MNP and v.42bis.

The modem-to-modem speed may only be 14400, but the modem-to-

computer speed stays "locked" at 38400.  Autobaud is not required,

and if used, would result is garbage on the screen.

Why am I telling you all this?  Well, I don't know who's silly

idea it was, but the powers that be decided that fax modems should

use the ancient method of changing the port rate when a fax is

incoming or outgoing.

After your modem reports the result code "+FCON" all further

communications *MUST* be done at 19200 bps, even if you are locked

at 38400 or 57600.  If the port speed is not relocked at 19200,

nothing but garbage will come on the screen.

I'm telling you this because I frequently get mail asking me why

I'm changing the port speed to 19200.  It's not my fault!  That's

the way is has to be done!  The only modem that I know that does

not require this idiotic speed shift is the ZyXEL modem.  In fact,

a special option must be used to tell BGFAX you are using a ZyXEL

or BGFAX will fail as it will try talking to the ZyXEL modem at

19200 instead of 57600 or whatever rate it's locked at.

Note that the 19200 shift is not needed on Class 1 and 2.0 modems.

BIT ORDER

=========

Class 2 fax modems receive in reverse bit order and transmit in

direct bit order.  Actually, this is not in the Class 2 spec, but

a bug in Rockwell's Class 2 implementation.  This reverse bit

order bug became so wide spread, that other fax modem manufactuers

now have to receive in reverse bit order to make their modems

compatible with existing fax software.  :-)

BGFAX will save all incoming fax files in direct bit order, so it

effectively flips all the bytes on the fly when receiving.

Class 1, Class 2.0, and ZyXEL modems receive and transmit in

direct bit order.

-------------------------------------------------------------

FRONTEND/HOST MODE

-------------------------------------------------------------

If you want BGFAX to answer the phone, BGFAX is started up using

the /HOST command line parameter.

If you plan on using BGFAX in Frontend mode, you need to do a

couple of things first.

1. An evironment variable called BGFAX needs to be inserted in

your AUTOEXEC.BAT or someother batch file.  Example:

SET BGFAX=C:\BGFAX

This environment variable points to the location of your BGFAX

configuration file and the directory where faxes will be

received into.

(Note, some people don't like cluttering their environment with

variables.  You can start up /HOST mode using a different config

file other than BGFAX.CNF, by using the "$" modifier, e.g.:

BGFAX /HOST /50 $C:\BAT\ROBIN.CNF

BGFAX /SEND C:\BGFAX\FAX0001.FAX 893-9124 $C:\BAT\BATGIRL.CNF

The "$" modifer must be the last thing on the command line.

Instead of looking for a BGFAX.CNF file as specified by the

BGFAX environment variable, this will cause BGFAX to look for

a "ROBIN.CNF" (in host mode) or "BATGIRL.CNF" if sending.

The "$" modifer has NO EFFECT on rear end mode operation.)

2. Make sure the BGFAX.CNF configuration file is in the directory

pointed to by the BGFAX environment variable and make any

changes you think necessary.  The included configuration file

is self explantory.  I won't repeat the format here.

3. BGFAX can exit with 7 possible internal errorlevels:

6 - host mode idle for more than x minutes

4 - fax was received

3 - local logon

2 - data call

1 - restart batch file

If no errorlevel is encountered, it means the sysop aborted

BGFAX by pressing the [Esc] key or a serious error occurred.

4. When BGFAX is waiting for a call, pressing Ctrl-R will reset

the on screen counters.  Ctrl-A will force BGFAX to send the

answer string to the modem.

5. If you some reason, you want to use the USR Class 2.0 modem in

Class 1 mode with BGFAX's /HOST mode, you will need to use the

/ATO switch which will cause BGFAX to send an "ATO" to the

modem after the "DATA" response is received.

6. When a data call comes in, BGFAX creates a DOBBS.BAT file that

looks like so:

C:\BGFAX\EXEBBS.BAT 9600 1 528 /ARQ

The first parameter is the DCE data speed, then the com port,

number of minutes until the next event, and then the error

correcting string, if any.  The EXEBBS.BAT file must be created

by you so that your bulletin board software is loaded up in

frontend mode (ie., the data connection is already hot.)

%1 -> the DCE speed, modem-to-modem real connect speed

%2 -> the COM port

%3 -> number of minutes until the next event

%4 -> error correction/reliable mode indicators

DOBBS.BAT theory

================

If you have never used a frontend program uses DOBBS.BAT theory,

there are several things you need to know.  You have to be pretty

confident in DOS and batch files, or you will get lost very

easily.  First, you need a main batch file that runs BGFAX.  Let's

call this FBBS.BAT:

@echo off

c:

cd\bgfax

bgfax /host

if errorlevel 4 goto faxrcvd

if errorlevel 3 goto datacall

if errorlevel 2 goto datacall

if errorlevel 1 fbbs.bat

goto end

:faxrcvd

rem [you can put a printing batch here if you want]

%bgfax%\doprint.bat

del %bgfax%\doprint.bat

:datacall

rem [errorlevel 3 is local login, 2 is data caller]

%bgfax%\dobbs.bat

:end

Note that DOS requires all if-errorlevel statements to go in

DESCENDING ORDER!  (255,254,253,...,4,3,2,1).  This is a fact that

many people forget.

This FBBS.BAT file will run BGFAX and when a data caller calls

your system, BGFAX will exit with an errorlevel of 3.  This will

cause the batch file to jump to the ":datacall" label.  Here, the

line "%bgfax%\dobbs.bat" is executed, causing DOS to shift control

of the computer to the DOBBS.BAT file.  Rememeber, this is a file

that BGFAX creates!  You do not need to make a DOBBS.BAT file.

The DOBBS.BAT then executes _another_ batch file called EXEBBS.BAT

which will look something like:

@echo off

bbsprog /f:%1:%2

fbbs

The EXEBBS.BAT file will look different for EVERY different type

of BBS software!  PC Board, Wildcat, GT, etc. all use different

formats.  You will have to look up the info for the EXEBBS.BAT

file in your BBS user's manual.  I can't help you with that.

Notice that the last line "fbbs" restarts the original FBBS.BAT

file that runs BGFAX in /HOST mode.

In review: (1) Modify FBBS.BAT to meet your needs, (2) BGFAX will

MAKE a DOBBS.BAT file by itself, (3) YOU will need to MAKE an

EXEBBS.BAT file, (4) the EXEBBS.BAT file must restart FBBS.BAT.

Do not use the CALL statement anywhere unless you really know what

you are doing.  "FBBS.BAT" and "CALL FBBS.BAT" causes two

completely different things to happen.  Only experienced DOS/batch

file writers need to experiment with that.

-------------------------------------------------------------

INITIALIZATION MODE

-------------------------------------------------------------

If you want BGFAX just to initialize the modem, you can have it do

so with the BGFAX /INIT parameter.  BGFAX will get the port,

speed, and initialization strings from the BGFAX.CNF file in the

directory specified by the BGFAX environment variable.

This mode is provided for people who need to use BGFAX in rear end

mode, but their Fido mailers (or BBS software) do not allow them

to use the long initialization strings required for fax usage.

-------------------------------------------------------------

REAR END MODE (FIDO MAILERS, ETC.)

-------------------------------------------------------------

Rear end mode is useful for those of us who are FidoNet members

where a Fido mailer is required to be running 24 hours a day.  It

will be your job to figure out how to get your mailer to exit to

BGFAX when your modem informs the mailer a fax call is coming

through.  (See the HELPME.ARJ file for hints on specific mailers.)

You will need to know the result code your modem sends when an

incoming fax call is detected.  Some modems send TWO fax connect

strings.  For example, the Supra will send "FAX" followed by an

"+FCON" response about 2 seconds later.  The ZyXEL will send

"CONNECT FAX" followed immediately by "ZyXEL".  Most fax modems

report only one string though.  Use this chart for reference:

Result   Modems

======   ====================================================

FAX      Supra, Zoom, Hayes, most Rockwell based chipsets

+FCON    PPI, other Class 2 modems

+FCO     USR 21.6K and other Class 2.0 modems

You need to configure your mailer (or BBS software) to exit

IMMEDIATELY upon receipt of the proper fax result code.  You

need to call BGFAX in the following manner:

BGFAX /how [faxpath port faxtype [pid]]

FAXPATH will be the directory where incoming faxes are to be

placed.  PORT is a valid communications port from 1 to 4, or,

optionally, a hexidecial base address and IRQ formatted as

followed "3F8x5".  Also, optionally, if the port has a bang, !,

following it, it will force the port to stay locked at the current

speed rather than shifting to 19200 on fax connects.  FAXTYPE is a

single character 'Z' or 'Q'.  'Z' for ZyXEL ZFAX format, 'Q' for

Smith Micro Software's Quick Link II fax format.

The "/how" deal is the most critical switch.  There are three

possible settings.

/FAX   ... Modem reports "FAX" or "CONNECT FAX".

BGFAX will automatically detect whether it should

use Class 2 (Supra, Zoom) or the ZyXEL fax mode

based on the next message (ie, "+FCON" or "ZyXEL").

/FAX:713_893_9124     If your modem in Class 1, try one of

/FHAY:713_893_9124    these two options.

/FAST  ... Modem reports "+FCON" (PPI, other Class 2 modems)

/FCON  ... Try this option if /FAST doens't work.

/FCO   ... Modem reports "+FCO" (USR 21.6K)

/FCOS  ... Try this option if /FCO doesn't work.

/FZYX  ... ZyXEL owners use this if /FAX doesn't work.

Here is a schematic of how each option behaves:

/FAX [ Supra, ZyXEL, i.e. all modems that report "FAX" ]

1. keeps port at locked DTE

2. waits for "+FCON"     _or_  2. waits for "ZyXEL"

3. drops port to 19200         3. waits for high CTS

4. waits for "OK"              4. sends <DC2>

5. sends "AT+FDR"

/FAX:713_893_9124 [ Supra LC modem, Class 1 modem ]

1. keeps port at locked DTE

2. waits for "CONNECT"

3. begins sending Class 1 HDLC frames

/FHAY:713_893_9124 [ Hayes 288, other Class 1 modems ]

1. keeps port at locked DTE

2. blindly starts sending Class 1 HDLC frames

/FAST [ PPI, i.e. all modems that report "+FCON" ]

1. drops port to 19200

2. waits for "OK"

3. sends "AT+FDR"

/FCON [ PPI, use this if above options don't work ]

1. drops port to 19200

2. blindly sends "AT+FDR"

/FCO  [ USR, i.e. all Class 2.0 modems that report "+FCO" ]

1. keeps port at locked DTE

2. waits for "OK"

3. drops port to 19200 _only if_ DTE rate is 38400

(leaves port alone if set at 19200, 57600 or 115200)

4. sends "AT+FDR"

/FCOS [ USR, i.e. all Class 2.0 modems that report "+FCO" ]

1. drops port to 19200 _only if_ DTE rate is 38400

(leaves port alone if set at 19200, 57600 or 115200)

2. blindly sends "AT+FDR"

/FZYX  [ ZyXEL, use this only if /FAX doesn't work ]

1. waits for high CTS

2. sends <DC2>

When a fax is inbound, I call BGFAX in rearend mode with the

following command line for my Supra:

BGFAX /FAX C:\BGFAX 1 Z

bgfax /fax c:\bgfax f3 z

bgfax /fax c:\bgfax 3f8x7 z

Using my Hayes 288, example command line options might be:

bgfax /fhay:713_893_9124 c:\bgfax 1 z

...note that the ID string MUST be presented on the command line

when you are receiving with a Class 1 modem in rear end mode.

In Class 1, the fax software is responsible for sending the ID

string to the remote machine.  (In contrast to Class 2/2.0 modems,

which automatically send the ID to the remote machine.)

The /FAX means that I used the modem response "FAX" as a trigger

for FD (see BGFAX.HLP for more details), C:\BGFAX is the path

where I want all faxes to be stored, "1" is the communications

port, and the "Z" represents I want faxes saved in ZyXEL's ZFAX

image format.

If you want to run BGFAX on a multinode system, an optional fifth

parameter can be used.  This fifth parameter can be up to three

characters and it is called the PID.  Example:

BGFAX /FAX C:\BGFAX 1 Z 99

The only difference between the command above and the previous one

is that rather than making a BGFAX.LOG, all log entries will be

written to a file called BGFAXpid.LOG, in this case, BGFAX99.LOG.

-------------------------------------------------------------

MISCELLANEOUS /HOST AND REAREND (/Fxxx) OPTIONS

-------------------------------------------------------------

Stop that hopping blue box

==========================

In /HOST mode, the BGFAX status box hops around the screen every

10 seconds to prevent screen burn-in.  However, some people got

dizzy watching the box, and others complained that the screen move

was taking up too many time slices in multitasking environment.

The /NH parameter will force "No Hop" mode which has considerably

less screen writes when sitting idle.

bgfax /host /nh

50 Line Mode

============

BGFAX can use VGA's 50 line mode if you wish by using the /50

command line parameter:

bgfax /host /50

bgfax /send c:\bgfax\fax0004.fax 893-9124 /50

bgfax /fco c:\bgfax 1 z /50

BGFAX and the USR 21.6K modem with September 93 chips

=====================================================

If you are using BGFAX with the USR 21.6K modem, BGFAX will

default to use the 12/08/93 firmware revision.  If you wish to use

BGFAX with the 09/20/93 firmware, you will need to use the /OU

switch on the BGFAX command line.  Example:

bgfax /host /ou

bgfax /fco c:\bgfax 4 q /ou

...DO NOT USE THE /OU SWITCH WITH ANY OTHER MODEM *EXCEPT* THE USR

21.6K WITH SEPTEMBER 1993 FIRMWARE!

Turning off the alert sound BGFAX makes when it receives a fax

==============================================================

BGFAX will automatically sound an "alarm" to let you know when a

succesful fax reception occurs.  This "alarm" is annoying to some

people.  To disable the "alarm", use the /NS switch for No Sound

bgfax /host /ns

bgfax /fast c:\bgfax 1 z /ns

Telling Class 1 modems to receive at a maximum of 9600 bps

==========================================================

If your Class 1 modem can receive at a maximum of 9600, strange

things will happen if you forgot to tell BGFAX this, since it will

default to tell the other side you will be using 14400 bps for fax

operations.  To force BGFAX to receive at a maximum of 9600 bps

with Class 1 faxmodems, use the /DIS:79 parameter, i.e.:

bgfax /fhay:713_893_9124 c:\bgfax 1 z /dis:79 /ns

Don't try using any other values on the /DIS: parameter.  It is

mainly available as a debuging option for me.

TSR fax notification

====================

BGFAX comes with an optional TSR program (NFAX.COM) that will

place the word "FAX" on your screen whenever a successful incoming

fax occurs.  However, you must tell BGFAX that you wish to alert

NFAX.COM to display the message.  You can do this by using the /NF

switch.  Example:

bgfax /fcon c:\bgfax 2 q /nf

This will activate the NFAX.COM interrupt and cause the message to

get displayed.  To turn the message off, you can run the FAXOFF.COM

program contained in the BGFAX archive.  Remember, NFAX.COM is a

TSR and must be loaded in your AUTOEXEC.BAT properly if you wish

to use it with BGFAX.  It is totally optional!

NFAX.COM/FAXOFF.COM techinical information:

===========================================

Here is a snip of the Turbo Pascal source code that tells NFAX.COM

to put the fax indicator on the screen:

var

regs : registers;

begin

writeln('Activating "NFAX" interrupt.');

regs.ah := 30; { nfax-function }

regs.al := 21; { nfax-sub function }

regs.bl := 95; { nfax-"FAX" attribute, white on purple }

intr($55, regs); { nfax-tsr }

end;

When you want to turn off the "FAX" alert message, you need to

run the included "FAXOFF.COM" file.  It basically issues the

same code above except that "regs.al := 22" instead of 21.

(Go into DEBUG and do an unassemble on the .COM file to see

exactly how to turn it off.)

-------------------------------------------------------------

AFTER YOU RECEIVE THE FAX

-------------------------------------------------------------

NAMING CONVENTION

=================

The name of the fax file will be FAXnnnn.xFX where "nnnn" is a

number that is padded with zeros and "x" is the letter "Z" or "Q"

depending upon which fax format you save the file in.  If you ever

see a BGFAX.$$$ file in your directory, it should *NOT* be there,

as it should have been renamed in the FAXnnnn.xFX format.  If it

is there, it means some kind of problem happened during the fax

reception.

FAXIN.LOG

=========

BGFAX creates a BGFAX.LOG file that contains a bunch of

information that is mainly only for debug purposes.  A much

cleaner log file is called FAXIN.LOG and looks like this:

Date  Time  ET    Bytes   Rate  Filename     Remote Fax ID        Pgs  Notes

-------------------------------------------------------------------------------

12-21 11:52 00:42   18183  9600*FAX0001.FAX  Unknown                2  Finished

12-21 18:04 00:38   18182  9600 FAX0002.FAX  Unknown                2  Not Done

12-22 21:30 00:29   17793 14400*FAX0003.FAX  7138939124             1  Finished

12-22 21:42 00:29   17541 14400 FAX0004.FAX  TRANQUILITY BASE       1  Finished

12-23 06:44 00:32   16941  9600*FAX0005.FAX  Null ID Received       1  Finished

12-25 17:21 01:22  107392 14400*FAX0006.FAX  7132424708             1  Finished

The asterick between the fax DCE rate and the filename indicates

the fax was marked as high resolution.  "Not Done" under Notes

means that BGFAX believed more pages were to have followed, but

were not actually received.  "ET" is estimated time.  Everything

else is self explanatory.

VIEWING/PCX CONVERSION

======================

You can use the included VIEW.EXE file to view the faxes you have

received.  It's use is quite simple:

VIEW FAX0002.FAX

...will cause VIEW.EXE to start viewing operations on FAX0002.FAX.

Note that VIEW will default to VGA graphics mode.  If you need to

use EGA or CGA modes, do so like this:

VIEW FAX0002.FAX /CGA   or  VIEW FAX0002.FAX /EGA

When you are viewing the fax, you have a few options:

Alt-F ... Flip the image (makes upside down and mirrors)

Alt-I ... Inverse the image (white-black -> black-white)

Alt-S ... Shrink current on-screen image to 1/3 width

PgDn .... go to next page

PgUp .... go to previous page

...and the Up/Down/Right/Left arrows do their normal job.

Note that viewing the fax, with my VIEW.EXE program, takes up

quite a bit of memory.  (It uses 216 bytes of memory for each scan

line, so a single high resolution fax page of 2200 lines uses

a minimum of 475K of memory!)

If you want to convert a fax to PCX ...

VIEW FAX0002.FAX /PCX

...would create a FAX0002.P01 and FAX0002.P02 file.  (Two PCX

files since there are two pages in this fax image.)

A special option, /BO, is also available with VIEW.EXE.  /BO

tells VIEW that you wish to flip all the bytes in the fax data

stream before processing it.  This should NEVER need to be used,

as it is for my debugging purposes only.  (Basically, using the

/BO parameter will let VIEW display/convert reverse bit order

faxes.  Normally, BGFAX will save all faxes in direct bit order,

no matter what bit order the modem actually receives in.)

Another special option, /DB, forces VIEW to display the bad scan

lines of a fax image.  Normally, if VIEW encounters a bad scan

line, it will display it is a dotted line going all the way across

the width of the fax image.  If /DB is active, VIEW will put up to

1728 bits of the incorrectly decoded scan line on the page.

DOPRINT.BAT

===========

After you successfully receive a fax, BGFAX will exit with an

errorlevel of 4 and append (or create) a DOPRINT.BAT file.

CALL C:\BGFAX\EXEPRINT.BAT C:\BGFAX\FAX0085.FAX

CALL C:\BGFAX\EXEPRINT.BAT C:\BGFAX\FAX0086.FAX

You do not have to do anything with this at all, but it is

provided for those of you who wish to print faxes as they are

received.  It will call the EXEPRINT.BAT file and pass it the

parameter with the path and name of the fax file that was just

received.  If you are using Smith Micro Software's Quick Link II,

for example, and are one of those people that leave your printer

on 24 hours a day, your EXEPRINT.BAT file might contain this:

@echo off

printfax %bgfax%\%1 24pin.qlp

Remeber to delete the DOPRINT.BAT file after you process it.

-------------------------------------------------------------

SEND FAX MODE

-------------------------------------------------------------

Currently, if you want to send a fax to another person, the fax

file must already be in either ZFAX or Quick Link II fax image

format.

BGFAX /SEND FAX0001.FAX 893-9124

See the BGFAX.CNF file for more info, such as maximum amount of

busy signals and failures before exit, etc.

At this time, only previously converted fax files can be sent.

(i.e., ASCII files have to be translated to ZFAX .FAX format or

QLII .QFX format.)

BGFAX can exit with three possible errorlevels when sending...

errorlevel 5  -> fax sent successfully

errorlevel 1  -> fax send failure

no errorlevel -> file not found, keypress abort, etc.

If the number is BUSY, BGFAX will try a few more times (the number

of times specified in the MB= entry in the BGFAX.CNF file) and

will pause for SW= number of seconds between dials.  If a failure

occurs during handshaking or during the actual sending of the fax,

BGFAX will terminate with an failure errorlevel.

If you are a programmer, and need to use BGFAX to send faxes, but

need more information than success or failure, you need to use the

/ER switch on the BGFAX /SEND command line.  BGFAX will make only

ONE attempt to send the fax in /ER mode (regardless of BUSY, etc.)

bgfax /send fax0001.fax 893-9124 /er

The following "Extended Results" are available:

errorlevel 14 -> a timeout occurred

errorlevel 13 -> no +FCON ever occurred (NO CARRIER, etc.)

errorlevel 12 -> a failure after connect (+FHNG, no XON, etc.)

errorlevel 11 -> "NO DIALTONE"

errorlevel 10 -> "BUSY"

errorlevel 7  -> the number is BLACKLISTED by the modem

errorlevel 5  -> fax sent succesfully

errorlevel 1  -> generic fax send failure

no errorlevel -> file not found, keypress abort, etc.

MISCELLANEOUS /SEND OPTIONS

===========================

Some Class 2 modems have bugs and cannot send to all fax machines

reliably in Class 2 mode.  Because of this, the /C1 parameter is

available.  It causes BGFAX to force the use of Class 1 when

sending:

bgfax /send c:\bgfax\fax0001.fax 893-9124 /c1

Some fax machines and fax modems don't operate very well together

at 14400 speeds.  You can force BGFAX to use a lower maximum speed

by putting the BPS rate after the send command, e.g.:

bgfax /send:9600 fax0001.fax 893-9124

bgfax /send:7200 fax0001.fax 893-9124

bgfax /send:12000 fax0001.fax 1-713-893-9124 /c1

Some Class 1 fax modems, when sending, send the first few scan

lines on each page garbled unless there is a pause before the

actual page data is sent.  Using the /HW command will force a

small delay, which should clear up the pages on the receiving

end.  The Hayes modem seems to need this wait, while the Supra

will loose the connection if the delay is initiated.

bgfax /send fax0001.zfx 893-9124 /hw

A special option, /SMD, is available for my testing purposes only.

You should NEVER use this option.  Basically, between each page,

it will tell the remote fax machine another "document" is coming

next, rather than the normal response that another "page" is

coming next.  Example:

bgfax /send fax0001.fax 893-9124 /smd

REMEMBER, /SMD is only for testing purposes.  DON'T USE /SMD!

FAXOUT.LOG

==========

A FAXOUT.LOG file is created when sending out faxes to indicate

the success or failure.  Here is an example:

Date  Time  ET    Bytes   Rate  Filename     Phone Number         Pgs  Notes

-------------------------------------------------------------------------------

03-22 13:18 00:25    3882 14400 RESPOND.FAX  8939124                2  Finished

03-22 13:18 00:11    3884 NOFAX RESPOND.FAX  8939124                0  Busy

03-22 13:19 00:11    3884 NOFAX RESPOND.FAX  8939124                0  No Fcon

03-22 13:30 00:31    3208  7200 RESPOND.FAX  8939124                0  Bad Conn

03-22 13:31 00:28    3882  9600 RESPOND.FAX  8939124                2  Finished

03-23 14:42 00:26    3882 14400 RESPOND.FAX  8939124                1  Timeout

03-23 14:44 00:29    3882 NOFAX RESPOND.FAX  8939124                0  BlackLst

03-23 14:47 00:21    3882 NOFAX RESPOND.FAX  8939124                0  Max Busy

03-23 14:51 00:23    3882 14400 RESPOND.FAX  8939124                1  Gen Fail

03-23 14:54 00:05    3882 NOFAX RESPOND.FAX  8939124                0  No Dial

-------------------------------------------------------------

HANGUP STATUS CODES

-------------------------------------------------------------

When BGFAX reports "Problematic fax reception", it will indicate a

fax hangup code, such as [+FHNG: 73].  This chart, taken from the

Supra CLASS_2.TXT fax command set documentation (which, I believe

was taken from the Rockwell Class 2 documentation), allows you to

match up the +FHNG message with its true meaning.  For example,

+FHNG: 73 would indicate "T.30 T2 timeout, expected page not

received", which really does not say much, but it may give you

some clue.  I don't know what most of these mean myself, so I

don't know whether it will provide you any luck or not, but, many

people asked that it be included, so here it is.

/-----------------------------------------------------------------\

| 2.0   Class 2 |  Cause Description                              |

| --------------|-------------------------------------------------|

| 00-0F     0-9 |  CALL PLACEMENT AND TERMINATION                 |

|---------------|-------------------------------------------------|

| 00          0 |  Normal and proper end of connection            |

| 01          1 |  Ring Detect without successful handshake       |

| 02          2 |  Call aborted, from +FK/+FKS or <CAN>           |

| 03          3 |  No Loop Current                                |

| 04        n/a |  Ringback detected, no answer (timeout)         |

| 05        n/a |  Ringback detected, answer without CED          |

|---------------|-------------------------------------------------|

| 10-1F   10-19 |  TRANSMIT PHASE A & MISCELLANEOUS ERRORS        |

|---------------|-------------------------------------------------|

| 10         10 |  Unspecified Phase A error                      |

| 11         11 |  No Answer (T.30 T1 timeout)                    |

|---------------|-------------------------------------------------|

| 20-3F   20-39 |  TRANSMIT PHASE B HANGUP CODES                  |

|---------------|-------------------------------------------------|

| 20         20 |  Unspecified Transmit Phase B error             |

| 21         21 |  Remote cannot receive or send                  |

| 22         22 |  COMREC error in transmit Phase B               |

| 23         23 |  COMREC invalid command received                |

| 24         24 |  RSPEC error                                    |

| 25         25 |  DCS sent three times without response          |

| 26         26 |  DIS/DTC received 3 times; DCS not recognized   |

| 27         27 |  Failure to train at 2400 bps or +FMS/          |

|               |                                  +FMINSP value  |

| 28         28 |  RSPREC invalid response received               |

|---------------|-------------------------------------------------|

| 40-4F   40-49 |  TRANSMIT PHASE C HANGUP CODES                  |

|---------------|-------------------------------------------------|

| 40         40 |  Unspecified Transmit Phase C error             |

| 41        n/a |  Unspecified Image format error                 |

| 42        n/a |  Image conversion error                         |

| 43         43 |  DTE to DCE data underflow                      |

| 44        n/a |  Unrecognized Transparent data command          |

| 45        n/a |  Image error, line length wrong                 |

| 46        n/a |  Image error, page length wrong                 |

| 47        n/a |  Image error, wrong compression code            |

|---------------|-------------------------------------------------|

| 50-6F   50-69 |  TRANSMIT PHASE D HANGUP CODES                  |

|---------------|-------------------------------------------------|

| 50         50 |  Unspecified Transmit Phase D error             |

| 51         51 |  RSPREC error                                   |

| 52         52 |  No response to MPS repeated 3 times            |

| 53         53 |  Invalid response to MPS                        |

| 54         54 |  No response to EOP repeated 3 times            |

| 55         55 |  Invalid response to EOP                        |

| 56         56 |  No response to EOM repeated 3 times            |

| 57         57 |  Invalid response to EOM                        |

| 58         58 |  Unable to continue after PIN or PIP            |

|---------------|-------------------------------------------------|

| 70-8F   70-89 |  RECEIVE PHASE B HANGUP CODES                   |

|---------------|-------------------------------------------------|

| 70         70 |  Unspecified Receive Phase B error              |

| 71         71 |  RSPREC error   (before TCF??)                  |

| 72         72 |  COMREC error   (after TCF??)                   |

| 73         73 |  T.30 T2 timeout, expected page not received    |

| 74         74 |  T.30 T1 timeout after EOM received             |

|---------------|-------------------------------------------------|

| 90-9F   90-99 |  RECEIVE PHASE C HANGUP CODES                   |

|---------------|-------------------------------------------------|

| 90         90 |  Unspecified Receive Phase C error              |

| 91         91 |  Missing EOL after 5 seconds                    |

| n/a        92 |  < Not assigned >           /--- Rockwell only  |

| 92 -Note-> 94 |  Bad CRC or frame (ECM {or BFT} modes)          |

| 93         93 |  DCE to DTE buffer overflow                     |

|---------------|-------------------------------------------------|

| A0-BF 100-119 |  RECEIVE PHASE D HANGUP CODES                   |

|---------------|-------------------------------------------------|

| A0        100 |  Unspecified Receive Phase D errors             |

| A1        101 |  RSPREC invalid response received               |

| A2        102 |  COMREC invalid response received               |

| A3        103 |  Unable to continue after PIN or PIP            |

|---------------|-------------------------------------------------|

| C0-DF     n/a |  RESERVED FOR FUTURE STANDARDIZATION            |

| E0-FF     n/a |  REVERVED FOR MANUFACTURER SPECIFIC USE         |

| n/a   120-255 |  RESERVED CODES                                 |

\-----------------------------------------------------------------/

-------------------------------------------------------------

ACKNOWLEDGEMENTS

-------------------------------------------------------------

I would like to thank the following people for their assistance

during the beta test phase.

Bill Huther

Brian Wood

Russell Kroll

Ethan Brofman

Ed Lucas

...and many people from the Internet and FidoNet.

US Robotics and Hayes have also been very helpful in getting

BGFAX to work with their modems.

-------------------------------------------------------------

SHAREWARE

-------------------------------------------------------------

THIS PROGRAM IS SHAREWARE.

If you use this program for more than four weeks you are required

to register it for its low cost of only $25 US DOLLARS.  If you

cannot afford this we can work something out.

Please make sure BGFAX works before registering.

Registrations encourage frequent updates.

If you are paying by check or money order, please print out the

REGISTER.FRM file, fill it out, and mail it to my address.

If you are paying by credit card (Visa, Mastercard, Discover or

American Express) you can (1) mail the form to me, (2) fax the

form to me, (3) netmail the form to me, or (4) call my BBS and

open door #6 and instantly register.

All people will have to call my BBS and open door #6 to pick up

the BGFAX.KEY registration key file.

I will mail (or air mail) registration letters out as well.

-------------------------------------------------------------

Regards,

B.J. Guillot