POSTALBC

By: David Frankenbach

CIS:72147,2635 or FRNKNBCH on USSBBS (408) 464-0350

DF Software Development, Inc.

9101 Timberlake Rd. Suite D

Lynchburg, VA 24502-3327

1001010100101010110000010100110001100010110001001011

Postal Bar Code Printer for the Hewlett Packard LaserJet or other printer

that uses PCL4 and above. An original article by Andrew Binstock in C Gazette,

Volume 5.2 was the starting point for these routines.  I have adapted the

routines from C into dBASE IV 1.1. I have made several optimizations to the

PCL commands used by the original article.

I have included two versions of the dBASE IV code. POSTALBC.1 is the first

version of the optimized code.  POSTALBC.2 makes use of printer macros to

reduce the printer I/O by a factor of 30.  You should use POSTALBC.2 unless

your printing environment (shared printers) might cause macro interference

with other users.

Number of characters sent to printer:

Original   PostalBC.1   PostalBC.2

5 Digit Zip    988         742          37

9 Digit Zip   1616        1214          53

As you can see PCL can be very inefficient if not used to its fullest

capabilities.  Sending 988 characters to print the bar code for a 5 digit

zipcode is pretty excessive.  Even after optimizing the PCL in POSTALBC.1 it

still takes 742 characters.  Some other optimizations which aren't presented

here got the count down to 480.  But when you look at the numbers when using

macros there is simply no comparison.

When you are printing envelopes by the hundreds you gain a large reduction in

I/O time, not to mention the calculation time.

To use the programs copy either POSTALBC.1 or POSTALBC.2 to POSTALBC.PRG.

You will have to position the printer cursor prior to calling PostalBC.

The barcode should start 3.875 to 4 inches from the right edge of the

envelope and the bottom of the barcode should be 0.25 inches from the bottom

edge. You should contact your post office for more information because the

bar code can now be placed in other locations.

To print the barcode use:

do PostalBC with "24551"

or

do PostalBC with "12345-6789"

The routines do not error check the argument, so make sure you send them

a character string zip_code, with numeric digits and '-' only (sorry Canadians

send me a copy of your barcode specification and maybe I can modify the

routines to handle them.)

If you are using POSTALBC.2 you must initialize the printer macros by:

do PostalBC_Init

before you issue any PostalBC calls.

The code has been tested on a Hewlett Packard LaserJet III but should work

on any HP printer from the LaserJet+ and up.