DiskQwik v1.1  Shareware version Released August 20th, 1993

DiskQwik v1.2 Registered version Released August 20th, 1993

�����͵  DiskQwik v1.1  �

�                               ����������������Ŀ    ����������������Ŀ

�   Dustbowl Designs, Inc.      �      2 SPB     �    �����0  SPB�������

�   -=� NOT REGISTERED �=-      � ������������� �    ������������������

� "Dedicated to enhancing the   ������������������    ������������������

� world's personal computers."        Drive 0         DiskQwik v1.2 only

�    (405) 741-2721 BBS

���������������������������������������������������������������������������Ŀ

�     Copyright 1993 by Dustbowl Designs, Inc. All rights reserved.         �

�         12600 S.E. 38th Street, Choctaw, Oklahoma USA 73020-6107          �

�   See DiskQwik.doc for v1.2 registration information. (405) 741-4705      �

�����������������������������������������������������������������������������

The block device driver for AT/IDE interface hard disk drives that

enables MULTIPLE SECTOR Block Transfer Mode.

��������������������������������������������������������������������Ŀ

���� What's New: v1.1 ������������������������������������������������

����������������������������������������������������������������������

Initial public release of DiskQwik v1.1 Shareware.

��������������������������������������������������������������������Ŀ

���� Introduction ����������������������������������������������������

����������������������������������������������������������������������

DiskQwik is a block device driver that is designed exclusively for

AT/IDE Hard Disk Drives. Many newer IDE drives have the built in

capability to significantly increase their Data Transfer Rates by

activating the MULTIPLE SECTOR Block Transfer Mode. In a typical

scenario, the transfer rate can be increased by up to 45% over the

rate offered by the motherboard bios. Some of the newest motherboards

and high-end Host Adapters are beginning to offer this MULTIPLE Mode,

but this great feature of our new IDE drives has essentially remained

untapped.... until now, thanks to DiskQwik!

Included in this ZIP file is DISKQWIK.PCX, in a standard graphics file

format used by most paint programs and picture viewers, that shows an

example of one drive's performance results. It shows in clear detail

the remarkable increase in Data Transfer Rate that one can expect from

an IDE drive that has activated MULTIPLE SECTOR Block Transfers. With

the Sectors per Block (SPB) set to 2, we immediately see an increase

of 15% over the system motherboard bios int13 routines. This is the

capability of DiskQwik's Shareware release. With the SPB set to 32

(only Registered versions of DiskQwik can set values greater than 2),

the Data Transfer Rate of our test drive jumps to 43% higher than the

native system motherboard bios int13 routines.

This shareware release of DiskQwik is designed to give you the

opportunity to test for yourself if your IDE drive can be motivated

into MULTIPLE SECTOR Block Transfer Mode. It is VERY important to note

here that while a large percentage of IDE drives can use this mode,

some cannot (some say they can, but actually don't... more on this

later), therefore, shareware provides a perfect opportunity for "try

before you buy" software evaluation.

��������������������������������������������������������������������Ŀ

���� Upgrade Offer ���������������������������������������������������

����������������������������������������������������������������������

As noted in the screen representation and at the beginning of this

document, DiskQwik v1.1 is shareware and supports a SINGLE hard disk

drive (unit 0) in the MULTIPLE SECTOR Block Transfer Mode.

Additionally, The shareware version is "hard coded" to a 2 Sectors per

Block maximum in the READ Multiple mode.

In the event that you want to activate the higher Data Transfer Rates

gained by increasing the Sectors per Block value, add a SECOND drive

in the MULTIPLE SECTOR Block Transfer Mode or desire to have WRITE

Multiple, we offer the program: Registered DiskQwik v1.2 .

Registered version benefits and features:

* Custom control of the values used for setting Sector per Block

(SPB).

* WRITE Multiple companion to the READ Multiple routine.

* Ultra-fast program loading without delays or required user

intervention. (If you're getting tired of the random character

starter and prolonged registration reminder, then it is probably

time to register <grin>.)

* Postscript version of the documentation.

* Advanced technical support and continuous upgradability to all

future registered versions via our BBS.

��������������������������������������������������������������������Ŀ

���� System Requirements ���������������������������������������������

����������������������������������������������������������������������

A few simple conditions must be met in order to utilize the DiskQwik

device driver.

They are:

1. Computer system MUST be either a 286, 386sx, 386dx, 486sx or

486dx (and Pentium, probably...).

2. The hard disk drive must be of the AT/IDE interface type.

��������������������������������������������������������������������Ŀ

���� Preparing the Disk Drives ���������������������������������������

����������������������������������������������������������������������

No special preparation or formatting of the hard disk drives is

required.

If the two minimum hardware requirements are met, then proceed through

the following tests to determine if the drive's microcode is up to the

job:

1. Run 4DRVUTIL.EXE (included in this zip file) on the Primary port

address, unit 0.

2. Does 4DRVUTIL show that Multiple Sectors per Interrupt are

available?

NO: Stop. The DiskQwik utility exists to activate this option.

YES: Go to 3.

3. Does 4DRVUTIL show that the drive's buffer size is larger than

the Maximum number of Multiple Sectors per Interrupt?

NO: Stop ...maybe.  If they are the same size, then the chances

are that the drive's microcode is giving a false positive on

Multiple Sectors. Given that the drive has a finite "memory

pool" available to support both the Read/Write caching and

the drive memory requirement for the Multiple sectors, it

makes no sense that these two should be the same. If the

entire buffer were given to the task of Multiple Sector

support, the drive's performance will degrade to an all time

low! Nonetheless, you still might continue on as long as you

configure DiskQwik to a block size that is half the size, at

most, of the buffer. This happens automatically with the

shareware release, since it is hard coded to 2 Sectors Per

Block (SPB).

YES: Things are looking good. Go to 4.

4. Run 4_SPEED.EXE (included in this zip file) on drive C: and save

the test results to a Log file for comparison later and be sure

to attach an appropriate comment to the test.  (Note: 4_SPEED

will quickly be picked up by drive caching software like

SmartDrive etc. Therefore, if you are seeing a Data Transfer

Rate for the drive that is higher than 3 or 4 MBytes per second,

then you will need to REM out the drive cache software in order

to collect data on the drive's true unassisted transfer rate.)

5. Add to your config.sys file the line DEVICE=DQWIK110.HDD, save

and reboot your computer.

6. Did DiskQwik load OK without error?

NO: Stop.  One of the few error messages is simply "Error

setting Multiple Mode, DiskQwik NOT LOADED".  This means

that the drive's microcode is configured to reject attempts

to set the Multiple feature.  This is too bad and

unfortunately the end of the line.  It is important to note

that this DOES NOT mean that the drive is defective, poorly

designed, or any such implication that the drive

manufacturer has failed in any way. It merely means that the

feature was never available in the first place.  The

development of Multiple Sector per Interrupt features in

various drives amongst the drive manufacturers is still

maturing.

YES: Almost home. Go to 7.

7. Run 4_SPEED again on drive C: and save the test results to the

same Log file that was prepared during the first test run.

8. Using 4_SPEED, select the "Analyze" feature, and graph the two

test runs.  This is the moment of truth.

9. Does the graph show that there has been an increase in the

drive's Data Transfer Rate?

NO: Stop.  Oh well... @#$%!!. So close, and yet we've hit a

stopping point. At least you have the satisfaction of

knowing that through shareware, you are able to "try before

your buy," and that you have in your possession two very

fine and no-obligation-to-register Freeware utilities

(4DRVUTIL and 4_SPEED) as consolation prizes.  Thanks for

trying.  In a more serious vein, again it is very important

to recognize the points made in step number 6. In general,

the drives that seem to have the multiple capability are

usually larger than 150 MBytes and have drive buffers at

least 32 KBytes in size.

YES: How a_bout that! If everything has gone according to plan,

you should be looking at an increase in the Data Transfer

Rate of around 15%; with ONLY 2 Sectors Per Block. You can

expect to see a substantial increase in the DTR by

increasing the SPB value, which is the prime feature of the

Registered version of DiskQwik (as well as setting up unit 1

and activating WRITE MULTIPLE). Go to 10.

10. Next, read to the section below titled "App Notes" and pick out

the points that apply to your system setup. Please consider

registering this fine shareware product, upload the shareware

zip file to as many BBS as you can afford, and tell you friends!

Congratulations.

��������������������������������������������������������������������Ŀ

���� Command Line Switches �������������������������������������������

����������������������������������������������������������������������

No command line switches are needed in the Shareware version of

DiskQwik since it is hard coded to Unit 0, READ MULTIPLE and 2 Sectors

Per Block (SPB).

The Registered version switches can be set up in the CONFIG.SYS file

after the complete driver name and should be separated by spaces for

clarity.

The following command line switch have been put into place for the

Registered version:

No Command Line Switches - Driver will NOT load since it requires the

command line switches for instruction.

B(?)xx - Block Size of xx for Unit ?. Where (?) is replaced by either

0 or 1 and xx is replaced by a value (typically an even

number) indicating the number of Sectors Per Block (SPB).

The SPB value cannot exceed the maximum number of sectors per

interrupt defined in the drive's own microcode.

RM(?) - Activate READ MULTIPLE for unit ?= 0 or 1.

WM(?) - Activate WRITE MULTIPLE for unit ?= 0 or 1.

P - Pause the progress of the config.sys after loading DiskQwik.  Press

C to continue.  This is handy when confirming the status of the

driver.

Example: DISKQWIK B016 RM0 WM0 B132 RM1 P

means: block size of 16 for unit 0 with READ and WRITE

MULTIPLE, block size of 32 for unit 1 with READ

MULTIPLE only, with a "Press C to Continue" pause

after loading.

CONFIRMATION of Device options:

Visual confirmation of the I/O option combinations will be indicated

by the COLOR of the "led" indicator on the DiskQwik banner screen drive

icon....

����������������Ŀ

�     32 SPB     �

� �*������������ �

���^��������������

|

|

RM(?)  - will be denoted by a BLUE * (led) indicator for the drive

selected. (This is the default color you should see with the

shareware version of DiskQwik.)

WM(?)  - will be denoted by a YELLOW * (led) indicator for the drive

selected.

RM(?) & WM(?) - will be denoted by a RED * (led) indicator for the

drive selected.

P      - The "Press c to continue", message will be displayed in the

bottom left corner of the yellow text box and the 'c' key

must be pressed before the driver's init code is terminated.

��������������������������������������������������������������������Ŀ

���� Background ������������������������������������������������������

����������������������������������������������������������������������

MULTIPLE SECTOR Block Transfer Mode has its origins in the ESDI hard

disk drive interface.  Just prior to the development of the AT/IDE

interface, the ESDI controllers were about ready to institute this

very interesting ability.  Similar to the IDE inquiry command, ESDI

drives will report 512 bytes of information about themselves where

word 47 is a yes/no variable about Multiple Sector capability.  If the

byte is "yes" then the following bytes will tell how many maximum

sectors per interrupt may be used.  In fact, 4DRVUTIL, and other IDE

inquiry utilities, will easily receive this information from the ESDI

drive controllers.

The rapid pace of hard drive technology, however, has since made the

ESDI interface obsolete.  This is lamentable from the standpoint that

the interface has a sterling reputation for quality and the drives for

ruggedness.  ESDI drives were typically large capacity units (of the

time) that found homes in file servers and other environments that

demanded critical performance from their drives.  Most network

managers will speak highly of the interface.

Drive manufacturers soon found that the cost per megabyte could be

drastically reduced by building the controllers directly onto the

drive.  This concept holds true for the AT/IDE interface (as well as

SCSI, but that's a whole different ball game).  This integrated

controller also allowed the drive manufacturers to use Zone Bit

Recording methods (variable sectors per track) and drive geometry

translation schemes to exceed the DOS limitation of 1024 cylinders

max.  By building RAM buffers into the drives we finally begin to

reach the point in hard drive technology where MULTIPLE SECTOR Block

Transfer Mode begins to be a reality.

A discussion about the microcode which manages the drive RAM buffer is

worthwhile.  Just like the RAM memory in our systems, the RAM buffers

on an AT/IDE drive should be thought of as a "memory pool". Today's

modern drives have buffers that range from simple to sophisticated. The

simplest buffer schemes employ basic Read Look Ahead techniques that

operate on the assumption that the next data you will request will be

contiguously after the data you just got.

These Look Ahead buffers generally are FIFO types (first in, first

out) that either read a pre defined number of sectors, or read through

to the end of the physical cylinder.  It is easy to imagine that the

transfer rate and speed of the data delivery to the host system is

greatly increased when it is dumped from RAM to RAM (nanoseconds)

instead of physically picking it up off of the drive (milliseconds).

The early AT/IDE drives had buffers of only 2 to 8 KiloBytes.  In

terms of sectors, that is 4 to 16 (2 512 byte sectors equal 1

KiloByte). This is enough to Read Ahead the rest of a track of a 17

sector per track drive (typical of the day).  Reading Ahead to the end

of the cylinder requires a much larger amount of memory.  Also, basic

competition amongst the drive manufacturers to be faster "than the

other guy" has caused the buffer sizes to increase.  Other factors

like spindle speeds, recording densities, and access times combine

together to be part of the overall improvements that have increased

drive performance.

When the RAM buffer reaches a certain point in size, either the Read

Look Ahead must cross a physical cylinder boundary or something else

more desirable; this moves us into Segmented Buffers.  From here we

see Adaptive Segmented Buffers, and so on.  A typical modern drive may

describe its buffer as Read Look Ahead Multi-Segmented Adaptive,

combining all types.

Write caching is the current newcomer to drive buffer techniques.

These are interesting in that the drive reports that a write has

completed as soon as the data arrives in the buffer, thereby freeing

up the interrupt hold on the CPU, allowing it to move on to more

processing. Then the drive, under its own control, attends to laying

down the data on the spinning magnetic media.  This happens very

quickly and does not carry with it the same negative implications that

some report about write caching software.

The balance between RAM allotted to write cache and read cache is

usually preset to around 40/60 and may someday actually dynamically

adjust to true system usage.  You can begin to see that these models

employ sophisticated microcode and algorithms working with a memory

pool which is subdivided into various areas.  The size of this total

buffer memory is climbing continuously, with state-of-the-art models

offering 1 megabyte of RAM. (Why do I get the feeling that this will

be old news in a few months.... <grin>?)

So, what about MULTIPLE SECTOR Block Transfer?  Simple, really...

whatever Block Size is set, is deducted from the memory pool.  For

example, if a 32 sector block is set, then 16 KiloBytes of RAM are

removed from the Read/Write caching on the drive.  While this sounds

like a setback, an actual increase in the Data Transfer Rate results

from the way this can interact with DOS.

��������������������������������������������������������������������Ŀ

���� Outline ���������������������������������������������������������

����������������������������������������������������������������������

Fortunately for all of us, the convenience of personal computers is

due, in large part, to the simplification of the User Interface.  We

have the advantage today, over the early pioneers, of being able to

simply sit down and create within our applications.  We "suspect" that

low level hardware operations are taking place as we work, because the

equipment tends to respond to our commands.   We see the hard drive

activity light flicker when we open and close our files.  This is as

it should be for the popular acceptance of personal computers in our

society.  User Interfaces have become friendlier each day, and will

continue to do so.  We can all look forward to the near future when

systems interact with more than just our fingers.

In some respects, interface is synonymous with translator.  As we work

within our high level applications, layers of translations are taking

place to convert our commands into machine language.  We enjoy the

benefit of not needing to know how these clever manipulations are done

and can count ourselves lucky in the process.  A typical read

translation sequence might be as follows:

1.  Application Command Open File

This level has its own layers of simplification, but roughly

boils down to the fact that as a programmer's source code is

prepared for use, a compiler translates the file handling

components into the standard DOS level software interrupts,

notably Interrupt 21.

source code:   assign DataFile the name mydata.dat

open (DataFile)

read (DataFile, DataWeNeed)  : Int 21, Fn 3Fh

so many sectors...

close (DataFile)

use (DataWeNeed)

These DOS interrupts provide even the programmer with the ability

of being able to avoid low level interaction with the system and

allows an application to operate on many types of machines.  Once

an application issues a DOS file command, the programmer can

count on a trustworthy sequence of events and can just let it

happen while waiting for a confirmation of success from the

operating system. (Some say that a REAL programmer always begins

with COPY CON PROGRAM.EXE <groan>.)

2.  DOS File Allocation Table (FAT)

The resident portion of our operating system, that part which

always stays in memory, is triggered into action by the DOS read

file interrupt.  The first order of business is to determine if

the file already exists, and if it does... where?  The DOS file

directories are used to make this determination and give the

operating system a starting point.  DOS uses a method of ordering

our data into clusters, or groups of sectors, that begins with

zero and numbers on up to the end of a drive's partition. This

might be thought of as a kind of linear, two dimensional map and

is sometimes call logical block address.

Since most files are larger than a single cluster, the location

of the next cluster after the start is required.  This location

of the next portion of the file is determined by inspecting the

File Allocation Table.  This table tells DOS the logical

whereabouts of the next data; which does not necessarily

contiguously follow the location of the last data. With DOS

reading the data into memory as it goes along, these steps are

repeated until the end of the file is reached.  The link from

location to location create a virtual chain of connections that

insure that data is not lost.

3.  DOS Kernel Resident Block Device Driver

To this point the data is logically ordered in the two

dimensional manner described above. Yet, we need to translate

this into a specifically located sector on the drive, itself.

Disk drives order their data by Cylinders, Heads and Sectors

which is a kind of spacial three dimensional coordinate.  The

transition from the logical linear location to the physical

spacial location is the job of DOS's own Resident Block Device

Driver (Block a.k.a Disk).  This requires a straight forward

calculation whose result depends on the individual geometry of

the drive being accessed; this geometry is stored in the Boot

Records and things called Bios Parameter Blocks and is read into

memory when the operating system loads.

If an imaginary drive had 10 sectors per track, 10 heads, and 10

cylinders, the drive would have a total of 1000 sectors. If we

were to count up through the sectors, the Heads digit (10's)

would increment after the ninth sector and the Cylinders digit

(100's) would increment after the ninth head.  With this model,

it is easy to see the relationship between the logical and

physical locations.  For example, the 123rd logical sector might

physically be located at Cyl=1, Hd=2 and Sect=3.  Aside from the

fact that DOS doesn't recognize a zero in the sectors digit, this

is the oversimplified way things are.  Disk drives, however, come

in many different capacities and make calculating a physical

location more interesting.  A drive with 17 Sectors per track, 6

Heads and 820 Cylinders would find the 123rd sector at Cyl=1,

Hd=1 and Sect=3 (right?).

4.  Interrupt 13 Call

Fun and games aside, the DOS Block Device Driver then builds a

hardware interrupt command that says something like "unit 0, at

cylinder 1, head 1, sector 4... read it."  Things start to look

just like assembly language programming at this point:

mov ah, 02h   ; read function

mov al, 1     ; number of sectors

mov ch, 1     ; cylinder number

mov cl, 4     ; sector number

mov dh, 1     ; head number

mov dl, 0     ; unit number

int 13        ; disk interrupt

Believe it or not, the fact is we still are looking at a language

designed to provide a user friendly interface, really.  Many

programmers actually write their programs at this level because

the finished and compiled  code ends up being smaller and faster

than the code produced by higher level programming languages like

Basic, Pascal and C.

5.  AT BIOS Port Address Command

Interrupt 13 Function 02h is a program, too, in a way. Its

routines are provided on a chip we all have someplace in our

system, called a BIOS (Basic Input Output Services).  When we

power on the computer the contents of the BIOS are stored in

memory and everything we do flows through these routines.  All

the hardware components of the computer - video, disk, keyboard,

etc. - have complicated little routines which handle

communicating with the hardware device.

Repetition is the name of the game at this level.  In the case of

our read a file example, every involved sector is seeked to

(sought?), read from and checked out for success, individually.

What is simplified for convenience as Int 13 Fn 02h ends up being

a near endless stream of machine language Port Address commands.

Hard disk drives have a specific port address at 1F0h for the

Primary port address and 170h for the Secondary port address.

While Int 13 serves both hard and floppy disk drives, the port

addresses for these two different types of drives are split apart

and managed by separate BIOS routines.

6.  Enter DiskQwik

We've finally reached the level where it is time to consider how

DiskQwik figures into the scheme of things.  First, it is

important to look at the challenge faced by the BIOS programmer.

The hard disk drive to be used in the computer system will be one

of many hundreds of types across several interfaces that range

from old to new, all needing to be supported by the one BIOS

routine.  Given this obligation, the routines that are written

are understandably generic with the same code that runs our older

MFM drive also running our new AT/IDE drive.  The need for

general compatibility creates a situation where the special

enhancements of the modern AT/IDE disk drive are left

unsupported.

The phrase "Multiple Sectors Per Interrupt" correctly implies the

notion that normally we have only one sector per interrupt. This

is the case with the standard BIOS service and is the default

start up configuration of the drive.  The following diagram shows

that a large amount of time is spent in overhead checking the

interrupt after every sector read from the port:

Interrupt Confirmation (overhead)

�

�i�   �i�   �i�   �i�   �i�   �i�   �i�   �i�   �i�   �i�   �i�

s��� �s��� �s��� �s��� �s��� �s��� �s��� �s��� �s��� �s��� �s��� �s�

�

Sector Read

With Multiple Sector Block Transfer Mode enabled on the drive,

block size equals to 8, a flow of data like this results:

Interrupt Confirmation (overhead)

�

�i�                               �i�

s���s���s���s���s���s���s���s��� �s���s���s���s���s���s���s���s��� �

�

Sector Read

A new routine is required to pass this type of data flow back to

DOS; software of this type is called an Interrupt Service Routine

(ISR). DiskQwik is an ISR replacement for the native BIOS Int 13

read and/or write hard disk drive service routines.  DiskQwik

resides in memory, monitoring all Int 13 requests.  When a Read

and/or Write request comes along, it intercepts the command and

manages it directly, "hand carrying" it through to the

port addresses, and the drive.

The turn around time on the delivery of the data is significantly

improved because much of the overhead from the interrupt

confirmations has been eliminated. This causes the Data Transfer

Rate to increase significantly.  DiskQwik is loaded as a device

driver through the Configure System CONFIG.SYS file.  Since

DiskQwik operates at such a low level, it remains compatible with

virtually all applications.  A few noteworthy exceptions do exist

and are noted in the App Notes section, below.

��������������������������������������������������������������������Ŀ

���� App Notes �������������������������������������������������������

����������������������������������������������������������������������

Some Application Notes:

1.  We've seen a few programs which provide an interesting "public

safety" feature, namely that they block attempts to  write to

track 0 (i.e. cylinder 0, head 0).  The purpose is to provide a

layer of defense against boot sector viruses. Since this is such

a good idea, we decided to join in and provide the same

protection.  After all, our device driver is custom tailored to

reading and writing to the hard disk drive and this capability

only added a byte or two of additional code. DiskQwik provides

this general safety precaution because no legitimate applications

have any business, whatsoever, writing changes to sectors on this

track without your knowledge.

Reading data on track 0 is allowed, however writing to track 0

will produce a write protect error.  If you need to modify data

on these "hidden" sectors, then you will need to REM out the

device=dqwik110.hdd statement in the config.sys, and reboot.

2.  Drive compression software programs like DoubleSpace work

perfectly well with DiskQwik.

3.  Concerning drive caching software: SmartDrive works because it

creates phantom block devices for each of the drives being cached

and operates in the DOS interrupt arena.  HyperDisk doesn't work,

which is a shame since it is one of the finest shareware programs

available on the market today, because it is a true Int13 level

software cache. We are hoping to see this resolved soon, and look

forward to new versions of HyperDisk to start supporting the

multiple sector block transfer modes.

4.  Concerning Windows Swap Files: A Temporary swap file works

because the file is like any other typical file with FAT updates.

A Permanent swap file doesn't work because it is unlike any other

typical file.  Basically, a permanent swap file locks itself into

an area on the drive and never moves, and since it never moves,

DOS and FAT updates are no longer required. A permanent swap file

is read and written directly with Int13 and cannot handle

multiple sector block transfer mode.  Windows should refuse to

load if it sees an Int13 interrupt service routine like DiskQwik.

We'd like to point out that the net gain in data transfer rate,

while in Windows, from using multiple sector block transfer mode

access to a temporary swap file far exceeds the gain of using

native Int13 access to a permanent swap file.

5.  DOS version levels and OEM versions of DOS work because they all

follow the same standards accessing Int13.

6.  When determining the value for Sectors Per Block (SPB) in the

registered version, it is worth noting that the rate of change in

the Data Transfer Rate tends to level out around 32 sector per

block. Even if your drive says it can handle a higher amount,

you'll probably find the increase is fairly small and not really

worth it considering the RAM requirement is removed from the

drive's buffer memory pool (i.e. read/write caching on the

drive).

7.  The Sector Per Block value can be odd or even values, however

setting values to  2, 4, 8, 16, or 32 seem to make better sense

as they are more adapted to the math routines involved.

8.  File defragmentation and optimization utilities will generally

work well with DiskQwik, however it is a good practice to

simplify one's system before running this type of utility by

disabling programs like drive caching software and DiskQwik.

Always make sure you have a current backup before optimizing a

hard disk drive.

��������������������������������������������������������������������Ŀ

���� Error Messages ��������������������������������������������������

����������������������������������������������������������������������

The device driver may display a single of error message during the

loading process of the CONFIG.SYS file.  It is "Error Loading

DiskQwik" and results from the drive returning an aborted command when

the set multiple command is issued.

��������������������������������������������������������������������Ŀ

���� Legal Disclaimer ������������������������������������������������

����������������������������������������������������������������������

LICENSE AGREEMENT

DiskQwik, 4_Drive Utilities, 4_Speed All versions.

Dustbowl Designs, Inc. provides  the accompanying object code software

("Software")  and nonexclusively  licenses  its  use on  the following

terms and conditions. The Software is copyrighted by Dustbowl Designs,

Inc. with all rights reserved.  YOU ASSUME FULL RESPONSIBILITY FOR THE

SELECTION OF THE  SOFTWARE TO ACHIEVE YOUR INTENDED  PURPOSES, FOR THE

PROPER INSTALLATION  AND USE. Dustbowl Designs,  Inc. DOES NOT WARRANT

THAT THE  SOFTWARE WILL MEET  YOUR REQUIREMENTS, THAT  THE SOFTWARE IS

FIT FOR ANY PARTICULAR PURPOSE OR THAT THE USE OF THE SOFTWARE WILL BE

ERROR FREE. Dustbowl Designs, Inc. EXPRESSLY DISCLAIMS ALL WARRANTIES,

WHETHER  ORAL  OR  WRITTEN,  EXPRESSED  OR  IMPLIED, INCLUDING WITHOUT

LIMITATION WARRANTIES  OF MERCHANTABILITY OR FITNESS  FOR A PARTICULAR

PURPOSE. IN  NO EVENT WILL  Dustbowl Designs, Inc.  BE LIABLE TO  YOU,

YOUR  CUSTOMERS   OR  OTHER  USERS   FOR  ANY  INDIRECT,   INCIDENTAL,

CONSEQUENTIAL,  SPECIAL  OR  EXEMPLARY  DAMAGES  ARISING  OUT OF OR IN

CONNECTION WITH THE USE OR INABILITY TO USE THE SOFTWARE.

DiskQwik is a trademark of Dustbowl  Designs, Inc. The DiskQwik

software and documentation are Copyright 1993 by Dustbowl Designs with

all rights reserved. The Registered DiskQwik v1.2 software programs

ARE NOT shareware or freeware and unauthorized redistribution is

STRICTLY PROHIBITED.

End of License agreement.

*****************************************************************

��������������������������������������������������������������������Ŀ

���� Revision History ������������������������������������������������

����������������������������������������������������������������������

v1.1    08/93 - public release of DiskQwik SHAREWARE v1.1.

v1.2    08/93 - public release of DiskQwik REGISTERED v1.2.

Contents of the DiskQwik Distribution Set. DQWIK110.ZIP (DQWIK.ZIP on

CompuServe (tm) go IBMHW forum Lib 1)

DQWIK110.HDD    Device driver file. Compiled 08-20-93 01:10:00.

DISKQWIK.DOC    Documentation file with installation instructions.

4DRVUTIL.EXE    An AT/IDE Identify Drive inquiry utility that shows

which 3_Drive/4_Drive and DiskQwik command line

switches work best. Designed to work on both Primary

and Secondary Port Addresses. Freeware.

4DRVUTIL.DOC    Auxiliary documentation.

4_SPEED.EXE     Our Data Transfer Rate Test utility. Freeware.

4_SPEED.DOC     Auxiliary documentation.

DBOWLDEM.EXE    Simple demo that gives a look at registered

4_Drives v3.01 and 4_Park v1.1, and our new DiskQwik

programs.

DISKQWIK.PCX    Graphics picture file showing Data Transfer Rates with

various command line options. Test and graphics file

created by our new 4_Speed Data Transfer Rate Test

utility. Freeware.

ORDERDQ.FRM     Mail-in registration form

READMEDQ.TXT    User shareware policy

VENDORDQ.DOC    Shareware vendor's distribution policy

*****************************************************************

Dustbowl Designs, Inc.          |

12600 S.E. 38th Street          |

Choctaw, Oklahoma  73020-6107   |

U.S.A.                          | CIS: 71062,2542

phone:

(405) 741-4705 ordering, tech support, general information

(405) 741-2721 BBS (9600 HST/v.everything) Tech Support, Hard disk

drive shareware, drive specs, Online Shareware

Registration/Download for VISA or MasterCard orders!

NAME : ___________________________________Password:_________________

ADDRESS : _____________________________________________________________

___________________________________________ ZIP :____________

TELEPHONE: ___________________________________________ (Day & Night)

COMMENTS : _____________________________________________________________

_____________________________________________________________

___________________________________________________________

Qty    Description                            Each       Total

____   DiskQwik v1.2  Registration            $25.00     ________

Subtotal ________

(Oklahoma residents add 6.5% sales tax)                 Tax ________

U.S. diskette orders add $3 shipping & handling, Foreign diskette

orders add $5 U.S. shipping. This also applies to Special Upgrade for

Registered users (If you want to download from our BBS put $0.00

dollars for shipping and write the password you'll be using next to

your name above.  Allow 7 days for Postal transit.)

Shipping ________

DATE : ______________________________

TOTAL $________

PAYMENT:

[ ] Visa/MasterCard  [ ] Check (number ____________)

(see below)       (make checks payable to Dustbowl Designs, Inc.)

[ ] Cash US$         [ ] US$ Money Order (No foreign  currency)

Total amount enclosed or authorized: $________________

_______________________________________________________________________

If Payment by Visa or MasterCard:

We can only accept Visa or MasterCards (no American Express, please.)

We will need the following information:

Credit card number: ______________________________________________________

Expiration date: _________________________________________________________

Bank the card's drawn on: ________________________________________________

The name as it appears on the card: ______________________________________

The signature of its owner authorizing us

to debit it for the amount indicated above: ______________________________

_______________________________________________________________________

THE FOLLOWING INFORMATION IS REQUIRED FOR REGISTRATION:

HARD DISK  #1 MAKE ____________________MODEL_________________SIZE________

HARD DISK  #2 MAKE ____________________MODEL_________________SIZE________

CONTROLLER #1 MAKE_____________________MODEL_________________

COMPUTER   286 ( )   386 ( )  486 ( )      OTHER ( ) ______________________

SX ( )    DX ( )  DX2 ( )  OverDrive ( )

OPERATING SYSTEM / DOS VERSION ___________________

VIDEO CARD  Standard VGA ( )  Super VGA ( )  Coproccessed/Accelerator  ( )

MOUSE      YES ( )  NO ( )

JOYSTICK   YES ( )  NO ( )

MEMORY     640 ( )  1 meg ( )   2 meg ( )   4 meg ( )  more ( ) ______

MODEM      YES ( )  NO ( )  .... BAUD RATE ___________________________

HOW DID YOU ACQUIRE THIS PROGRAM: bbs name &#__________________________

Fold the pages at the break below with print facing out.  Fold letter style

with mailing address exposed.  Tape/staple edges, enclosing payment.

___________________________                                          .-----.

|Place|

___________________________                                          |Stamp|

|Here |

___________________________                                          `-----'

Dustbowl Designs, Inc.

12600 S.E. 38th Street

Choctaw, OK    73020-6107

U.S.A.

DiskQwik v1.1

-=EOF: DISKQWIK.DOC=-