Documentation for

PageMate

Shareware Version 1.01

=================================================================

PageMate (TM)

Copyright (C) 1991 by R.J. Matter & Associates

All Rights Reserved

The author makes no warranty of any kind, expressed or implied,

with regard to this program or documentation. The author shall  not

be liable in any event for incidental or consequential damages in

connection with, or arising out of, the furnishing, performance, or

use of this program.

-----------------------------------------------------------------

DESCRIPTION

PageMate is a program that monitors your telephone line for an

incoming ring and then dials a number you specify after a specific

delay.

PageMate is most commonly used in conjunction with display type

pagers (or "beepers") and answering machines to notify owners of

incoming messages, but it can call any telephone number and an

answering machine is not necessary. PageMate will work with rotary

pulse or tone telephone line service.

-----------------------------------------------------------------

SETUP

What you need: An IBM compatible computer with a 300 baud or

greater Hayes compatible modem configured as COM1 or COM2, a text

editor or word processor and a telephone line.

With a text editor or word processor, edit the comma delimited

PAGE.DAT file to include the com port number your modem is

installed on (1 or 2 ONLY), a "P" or "T" to indicate PULSE or TONE

line service, the number of seconds PageMate should delay before

calling the dialing string, and the dialing string surrounded by

quotation (" ") marks in the format:

COM PORT NUMBER,SERVICE TYPE,SECONDS TO DELAY,"DIALING STRING"

Note: If you use a word processor to do this, make sure you save

the file in ASCII or non-document form! Also make sure the

parameters are in the exact order shown and that each parameter is

separated by a comma (,)!

You can place commas in your dialing string to force the modem to

pause for 2 seconds per comma. Special characters like * # are

allowed also.

Example 1:     1,T,180,"555-5555,,,,999-9999#"

Example #1 translation: Modem is installed on COM PORT 1.

Telephone line service is TONE. After a delay of 180 seconds,

PageMate will call 555-5555, pause for 8 seconds, then send the

number 999-9999 to display on the pager followed by the common #

character indicating the end of number entry.

Note: You must have TONE service to send a number to a display

pager.

Example 2:     2,P,120,"9,555-5555"

Example #2 translation: Modem is configured as COM PORT 2.

Telephone line service is PULSE. After a delay of 120 seconds

PageMate will dial 9 (to get an outside line for example), pause

for 2 seconds, then dial pager number 555-5555.

If you are using PageMate with an answering machine, allow enough

delay time for the caller to leave a message and for the answering

machine to cycle for the next call. Usually 3 or 4 minutes will be

enough time (180 to 240 seconds).

-----------------------------------------------------------------

DIRECTIONS

The modem and telephone/answering machine must be on the same

line. The computer and modem must be turned on. Internal modems are

always on when the computer is on.

PAGE.EXE and PAGE.DAT must reside in the same directory. Type

"PAGE" and the PageMate status window will appear. The date and

time will be displayed and the seconds should be ticking.

If you specified the wrong communications port in the PAGE.DAT file

or your modem is not on, you will get a "Bad file name" or "Device

timeout" error. You will need to edit the PAGE.DAT file and enter

the correct COM port number and make sure your modem is on. Valid

COM port numbers are 1 or 2 ONLY (COM3 and COM4 are not supported

yet). The version 1.0 documentation incorrectly stated COM3 & COM4

were supported. Future updates will try to incorporate COM3 & COM4.

You can test PageMate by typing "T". The delay time for test mode

is fixed at 10 seconds. You can exit PageMate at any time by

pressing the Escape key.

If everything is installed and working correctly, you can turn off

your monitor and leave PageMate and your computer unattended. When

PageMate detects the first incoming ring on your telephone line,

it will begin counting down the delay seconds you specified in

PAGE.DAT and then call the dialing string you specified in

PAGE.DAT.

-----------------------------------------------------------------

SHAREWARE VERSION NOTES

The shareware version of PageMate allows only one incoming call per

session. You must register to receive version 2.02 which allows

unlimited calls. To register, print and fill out PAGE.FRM and mail

it with your check or money order for $25 to:

Robert J. Matter - PageMate

P.O. BOX 9042

Highland, IN  46322

To print PAGE.FRM from the DOS prompt type

A> PRINT PAGE.FRM

or

A> COPY PAGE.FRM LPT1

-----------------------------------------------------------------

DISTRIBUTION

This software may be distributed freely provided that it is

distributed as a complete set (PAGE.EXE, PAGE.DOC, PAGE.DAT,

PAGE.FRM) without modification and no fee is charged other than a

nominal handling or duplication fee of $5.00 or less.

-----------------------------------------------------------------

HISTORY

1.01 - Fixed "DEVICE I/O ERROR" bug for external modems.

- Set delay time for testing at 10 seconds.

by Robert J. Matter

P.O. BOX 9042

Highland, IN 46322

CompuServe ID: 71021,2654

GEnie: R.Matter