COM3.DOC

By: Matt Trask

13 Dec 87

CAVEAT EMPTOR: this a megahack!  There exists the possibility of

destroying your hardware if you do this, and the software is not

supported and probably will not work with future releases of OS/2. I

cannot accept any responsibility for your use of the techniques

described here.  If you are not confident in your abilities, PLEASE

get help from someone who knows how to make the modifications.  In

addition, the device driver that is provided here is trash - I won't

admit to having written it as it is terrible style.  However, it will

do the required work.

I have only tested this with an AT klone, a Kaypro AT-style

serial/parallel card, and the IBM beta OS/2 (Internal version 3.86) -

it works as documented on my system but I can't guarantee that it will

work anywhere else.  Use this stuff at your own risk!

Now that the formalities are done, I will try to support this

hack - if you try it and it doesn't work, leave bixmail to

'matt.trask', or better yet, come join my 'boating' conference

(and bring some friends), and I'll see if I can help.

Purpose

I found that the two comm ports supported by OS/2 on an AT

were not adequate for my needs (modem, mouse, and TNC for packet

radio) so thus this project was born.  A little poking around with

my favorite debugger (Pfix Plus - highly recommended) showed that

when the serial mouse driver is loaded for COM2, it gets the base

address for the COM2 card from 40:2 and stuffs a word of zeroes in

its place so that other drivers (such as the COM01.SYS serial

driver for AT machines) will not know that the port exists.  From

this I reasoned that if I could create a serial port at an address

other than 0x2f8 or 0x3f8 that used an IRQ other than 3 or 4, OS/2

should be happy to support it for me.  What follows is a description

of the cuts and jumpers that do this, and the necessary software

hacks to complete the job.  The software is pretty simple - just a

matter of patching the COM01.SYS driver to find the new port and

set its interrupt level.  The only other part is a simple device

driver that pokes the base port address of the modified comm card

into the BIOS data are at 40:2.  So, gentlemen, start your

soldering irons, E pluribus hackum!

Hardware Modification

This mod should work on most any AT style serial/parallel adapter.

In addition, it will probably work on XT type serial cards but I

can't guarantee that they will work at AT bus speeds.  The idea is to

change the address decoding logic to select base ports of 0x3f8/0x1f8

instead of 0x3f8/0x2f8, and change the IRQ level for COM2 from IRQ3

to IRQ5.  (You can use IRQ2 if you prefer - I used IRQ5 because my

LAN card wants IRQ2)  Although the AT Tech Ref claims that the fixed

disk controller uses 0x1f0-0x1f8, that is a typographic error.  It

really uses 0x1f0-0x1f7, as you can verify by reading the fixed disk

controller docs.  IRQ5 is reserved for use by a second parallel port,

if you are using two interrupt driven LPT ports in your system you

will have to use IRQ2 for this modification.

AT the card edge connector cut the traces for address lines A8 and

A9 (connector tabs A22 and A23 on the component side of the board).

Solder two jumpers to cross-connect these two pads to each others

traces.  This allows the jumper block to change the sense of A9

instead of A8 as it was designed.  Then turn the board over and cut

the trace to IRQ3 (pad B25).  Jumper this trace to IRQ5 (pad B23) or

to IRQ2 (pad B4) as required.

There, you're done.  Easy wasn't it?  Recheck your work - a

multimeter will be helpful to make sure that you didn't accidentally

short one of the lines to an adjacent pad or trace.

Software

There are two parts to the software used to support the

additional serial port - patching COM01.SYS and writing a port

initialization driver.  This example assumes that you have made the

hardware mods for base port 0x1f8 and IRQ 5 - make changes as needed

in the following instructions.

I've provided the driver to support a base address of 0x1f8 in

COMINIT.SYS and the source in COMINIT.ASM in case you need to change it.

The batch file 'MAKE.CMD' will run in protected mode to assemble and

link the driver if you make changes to it.

The patches to COM01.SYS are as follows:

C> debug com01.sys

- n com03.sys

- u 22a7

cmp dx, 2f8		/* you should see something like */

mov	cx, 045a	/* this, otherwise don't continue */

jz	22b3

.

.

- e 22aa	02.01

- e 22b6	03.05

- w

- q

C>

This will write out a copy of 'COM01.SYS' with the changes under the

new name 'COM03.SYS'.

Usage

The following is an excerpt from my CONFIG.SYS that shows the

correct order for installation.  This assumes that you are using

COM2 for your serial mouse, so the new COM port will be supported

by OS/2 as logical device 'COM2'.

DEVICE=C:\POINTDD.SYS

DEVICE=C:\MOUSEA02.SYS SERIAL=COM2

rem DEVICE=C:\COM01.SYS		/* original OS/2 serial driver */

device=c:\cominit.sys		/* port base setup driver */

device=c:\com03.sys			/* modified COM01.SYS */

Reboot OS/2, use MODE.COM to set up the proper parameters for the

device you have on the port, and off you go.  Happy hacking, let me

know if this works for you.