A typical net session:

a dos batch file:

slfp /s 2400 /d atdt764-4800

if errorlevel 1 goto end

net -d c:\net

termin 0x60

:end

a 4dos alias:

slfp /s 2400 /d atdt764-4800 ^

iff errorlevel 1 then ^

net -d c:\net ^

termin 0x60 ^

endiff

Try running STAT.EXE to get statistics on loaded

packet drivers. TERMIN unloads the packet driver from memory, (and resets

the COM port). The AUTOEXEC.NET file must be in the directory that the

NET command points at, or in the root directory.

SLFP packet driver info:

param	default 	description

-----	------- 	-----------

/p	0x60		Packet driver interrupt number

/i	4		IRQ for com port. Use 4 for COM1 and 3 for COM2.

/b	0x3F8		I/O base address for com port. Use 0x3F8 for

COM1 and 0x2F8 for COM2.

/s	2400		Line speed bit rate.

/d	-none-		Dial sequence. Use atdt764-4800 for local dial.

/t	60		Initial timeout. Amount of time to wait for

the dialup and initial handshake with Merit.

/h      1               Initial SLFP handshake. This can be turned off

with value 0. This allows one to make the

initial connection with a terminal emulator

like kermit, and enter PCIP at the TERMINAL=

prompt. This is useful if you have problems

with the SCP training up into SLFP protocol

mode.

/f     1               Initial FIFO check value. If set to one,

slfp checks for a 16550a UART for the COM

port. This allows the high performence FIFO

on chip to be used. If set to zero, slfp

assumes a normal 8250 chip set.

/q      0               Quit parameter. If set to one, a soft hangup

is performed when the terminate function is

called. This only works with hayes-compatable

modems, as this sends "+++" and "ATH0".

If set to zero, a hardware hangup is performed

by dropping DTR.

NET program command line:

net [-d directory] [-s #sockets] [-b] [startupfile]

-d   The root directory for NET files. This defaults to the root

directory of the logged in drive. This is where autoexec.net,

domain.txt, and the ftp and spool directories are rooted.

-s   Number of sockets. Defaults to 40. You might need to

increase this if you're running a bunch of sessions.

-b   Use the BIOS for all writes to video.

none The startup file. Defaults to autoexec.net. Found in the

net directory root if a relative file name, but you may

specify an absolute path.