MEGAlink

A File Transfer Protocol

Specifications

By: Paul Meiners

P&M Software Company

9350 Country Creek #30

Houston, Texas 77036

April 18, 1987

Revision history:

-----------------

August 9, 1987 ... Forsberg's CRC Variation

Acknowledgements

----------------

I would like to acknowledge the work of those who have done this before me.

First, Chuck Forsberg, for his never ending quest for speed and reliability.

Second, Ward Christensen, whose ideas are the genesis of this whole branch

of file transfer technology.  Third, Peter Boswell, for showing me the way

to a network friendly protocol.  Last, Tom Jennings, for the work he did

on Fido and the Telink protocol, which introduced the "link" style header

record, now in use by all "link" type protocols.

For those that are not aware, these pioneers are responsible for the following

work:

Ward Christensen ______________ Xmodem.

Tom Jennings __________________ Fido, Telink.

Chuck Forsberg ________________ Ymodem, Ymodem Batch, Zmodem.

Peter Boswell _________________ WXmodem.

Why Another Protocol?

---------------------

In many respects, the design of a protocol has become a discussion between

the above mentioned pioneers, each building on the work of the other.  Each

has perceived a weakness in the protocols that have gone before and sought

to improve the art.  And that is the most elegant reason for a new protocol,

to advance the art.

I won't bother with a discussion of the merits of Xmodem.  Others have done

that far better than I could.  Let me address the more modern variations.

These are my opinions, based on experience and reading the literature.

Zmodem  - The ideal protocol.  Highly reliable and fast.  Only failing

is that the cost of implementation is high.

Ymodem  - Very fast, but not network friendly.  Also leaves something

to be desired on the reliability front.  Error correction is

slow.

WXmodem - Fast and network friendly.  However, several questions exist

with regard to reliability.

So, it our goal to design and implement a protocol that meets or exceeds

the following criteria:

1.  Fast.  Efficency must exceed 95% on average basis.

2.  Reliable.  Known defects in prior protocols must be corrected.

3.  Inexpensive.  Implementation cost must be low.

How Do We Do It?

----------------

Let us address these issues in reverse sequence.

COST OF IMPLEMENTATION

----------------------

By closely modeling the protocol after Xmodem, we hope to reduce cost

of implementation.  This may be a vain effort, because implementation

of any high performance protocol requires a certain degree of complexity.

RELIABILITY

-----------

To improve the reliability of the protocol, we have choosen to use the

new CRC-32 technology.  The algorithm for this procedure will be given

later in this document.  Also, the Ack/Nak packets will be composed of

3 bytes, as was done in the SEAlink protocol.  Like WXmodem we will

honor all network XOFF signals at the transmitter end.

SPEED

-----

Several things are going to be done to improve speed.  First, the block

size will be increased from 128 to 512 bytes.  This reduces the number

of envelope characters.  Like Zmodem, MEGAlink will be a full streaming

protocol, thus eliminating the turn-around time involved with Xmodem

and Ymodem.

Some definitions

----------------

Description             Character           Decimal     Control Char

-----------             ---------           -------     ------------

XON ................... DC1 ............... 17 ............ ^Q

XOFF .................. DC3 ............... 19 ............ ^S

Acknowledge ........... ACK ............... 6 ............. ^F

Negative Acknowledge .. NAK ............... 21 ............ ^U

End of File ........... EOT ............... 4 ............. ^D

Start of Header ....... SOH ............... 1 ............. ^A

MegaLink Block Id ..... EM ................ 25 ............ ^Y

Request Status ........ RS ................ 30 ............ ^^

Cancel ................ CAN ............... 24 ............ ^X

Synchronize ........... SYN ............... 22 ............ ^V

Data Link Escape ...... DLE ............... 16 ............ ^P

CP/M EOF .............. SUB ............... 26 ............ ^Z

Receiver discipline

-------------------

The receive side of a MEGAlink transfer has three possible packets it can

use.  Each is three characters in length as follows:

Byte No.          Content

--------          -------

1              Ack/Nak/'C'.  No other character may appear.

2              Packet number.  0 thru 255.

3              Complement of packet number, i.e. (Pkt# XOR 255).

The 'C' packet is sent only at the start of each file transfer.  It is sent

by the receiver at 5 second intervals, until the transmitter begins.  After

that it is not used again until the next file is begun, or the next session

is begun, if only 1 file is transmitted.

The Nak packet is used to request the retransmission of the specified packet.

The Ack packet is used to indicate the highest packet received without error.

Normally, the receiver remains quiet.  The only time packets are required of

the receiver are at the beginning of the transfer, as outlined below, and upon

receipt of an RS character, the Request Status character.  The receiver must

respond to the RS character with an Ack packet, reflecting the highest block

received in without error.  Of course, the receiver should send a Nak packet

whenever a block is received with an error condition.

Note, that to maintain network friendliness, no packet may contain a XON or

XOFF character.  These characters must be sent as two characters, first a

DLE, followed by the folded XON or XOFF.  Characters are folded by XOR'ing

them with decimal 64.  This scheme requires that the DLE character is

escaped and folded in the same manner.

Transmitter discipline

----------------------

The transmitter can do four different things:

1)  Send a header block.  Contains file name and other information

about the file.

2)  Send a data block.  A data block contains 512 bytes of data.

3)  Send an RS character, forces receiver to Ack the highest packet

he has received without error.

4)  Send an EOT character, signals end-of-file to the receiver.

Data blocks are sent without pause.  The transmitter should have enough

buffers to cover the turn-around delay, so that any block the receiver

may Nak will still be available in memory for retransmission.

Note, that to maintain network friendliness, no packet may contain a XON or

XOFF character.  These characters must be sent as two characters, first a

DLE, followed by the folded XON or XOFF.  Characters are folded by XOR'ing

them with decimal 64.  This scheme requires that the DLE character is

escaped and folded in the same manner.

Typical transmit/receive dialog:

--------------------------------

Transmitter                           Receiver     Description

-----------                           --------     -----------

C   00 FF    opening Nak.

SOH 00 FF  header[128]  CRC CRC                    header block sent,

using CRC-16.

ACK 00 FF    Ack of header block.

EM  01 FE  data[512]  CRC CRC CRC CRC              file transmitted in one

EM  02 FD  data[512]  CRC CRC CRC CRC              or more data blocks,

EM  03 FC  data[512]  CRC CRC CRC CRC              using CRC-32.

EM  04 FB  data[512]  CRC CRC CRC CRC

RS                                                 transmit requests status.

ACK 04 FB    receive replies to RS.

EOT                                                end-of-file sent.

ACK 04 FB    Ack sent for end-of-file.

C   00 FF    opening Nak.

EOT                                                end-of-batch sent.

ACK 04 FB    Ack sent for end-of-batch.

If necessary, the last data block can be padded with CP/M EOF characters.

Note: all CRC bytes are transmitted from high to low order, NOT in the

more usual byte-reversed format.

At the end of the transaction, the "opening Nak" is repeated by the receiver.

This is to allow for batch transmission, if more than one file were to be sent

the transmitter would pick-up and start by sending the next header block and

the transaction would continue from that point as shown.

The format of the header block conforms to the standard "link" format.

Established by Tom Jennings when he designed the Telink protocol, also

used by the SEAlink protocol.

Byte Offset     Length       Content

-----------     ------       -------

0             4         Original file length. Integer in byte

reversed format.

4             4         Date and time file was last mofified, in

seconds since 1979.  Same format DOS uses

in the directory entry.

8            16         Original file name, as a null terminated

string.

24             1         Binary version number.

0 ... Original MegaLink.

1 ... Forsberg's CRC Variation.

25            15         Name of transmitting program, as a null

terminated string.

40            88         Null filler and expansion area.

CRC Calculator

--------------

The following routines demonstrate the technique used to calculate CRC's,

both 16 & 32 bit varieties, used in MEGAlink.  The code is written in

Turbo PASCAL.

Thanks to Mr. Chuck Forsberg, "Forsberg's CRC Variation" can be introduced.

It involves initializing the CRC Register to $FFFFFFFF instead of the normal

zero.  The use of this variation is signaled by a handshake between transmitter

and receiver.  The receiver sends an opening nak packet containing a 'C' and 2

bytes of 00 and FF hex.  If the receiver desires to use  "Forsberg's CRC

Variation", he should code the bytes following the 'C' as 01 and FE.  If the

transmitter is capable of using "Forsberg's CRC Variation" he will signal this

in the header block, see offset 24 above.  The transmitter will adjust to use

whichever CRC is requested by receiver, thus maintaining downward compatibility.

{  Global Variables  }

TYPE

ARRAY512       = RECORD

Len        : INTEGER;

LongString : ARRAY[1..512] OF CHAR;

END;

STRING128      = STRING[128];

VAR

crc_input        : INTEGER;      { 2 byte integer format }

crc_reg_lo       : INTEGER;

crc_reg_hi       : INTEGER;

forsberg_variant : BOOLEAN;

PROCEDURE

ccitt_crc16_calc;                {  CRC-16  }

BEGIN

inline( $8B/$1E/crc_reg_hi );    {        mov     bx,crc_reg_hi    }

inline( $B9/>$08 );              {        mov     cx,8             }

inline( $A1/crc_input );         {        mov     ax,crc_input     }

inline( $D0/$D0 );               {  u1:   rcl     al,1             }

inline( $D1/$D3 );               {        rcl     bx,1             }

inline( $73/$04 );               {        jnc     u2               }

inline( $81/$F3/$1021 );         {        xor     bx,1021h         }

inline( $E2/$F4 );               {  u2:   loop    u1               }

inline( $89/$1E/crc_reg_hi );    {        mov     crc_reg_hi,bx    }

END;

PROCEDURE

ccitt_crc32_calc;                {  CRC-32  }

BEGIN

inline( $8B/$1E/crc_reg_lo );    {       mov     bx,crc_reg_lo     }

inline( $8B/$16/crc_reg_hi );    {       mov     dx,crc_reg_hi     }

inline( $B9/>$08 );              {       mov     cx,8              }

inline( $A1/crc_input );         {       mov     ax,crc_input      }

inline( $D0/$D8 );               {  u1:  rcr     al,1              }

inline( $D1/$DA );               {       rcr     dx,1              }

inline( $D1/$DB );               {       rcr     bx,1              }

inline( $73/$08 );               {       jnc     u2                }

inline( $81/$F3/$8320 );         {       xor     bx,8320h          }

inline( $81/$F2/$EDB8 );         {       xor     dx,EDB8h          }

inline( $E2/$EE );               {  u2:  loop    u1                }

inline( $89/$1E/crc_reg_lo );    {       mov     crc_reg_lo,bx     }

inline( $89/$16/crc_reg_hi );    {       mov     crc_reg_hi,dx     }

END;

PROCEDURE

calc_crc32(VAR cs : ARRAY512);

VAR

i  : INTEGER;

BEGIN

(*

Note: this routine calculates a 32 bit CRC based on the CCITT

polynomial.  The result is stored in the crc register,

variables crc_reg_hi & crc_reg_lo.

*)

IF (forsberg_variant) THEN BEGIN

crc_reg_hi:=$FFFF;

crc_reg_lo:=$FFFF;

END

ELSE BEGIN

crc_reg_hi:=0;

crc_reg_lo:=0;

END;

WITH cs DO BEGIN

FOR i:=1 TO Len DO BEGIN

crc_input:=ORD(LongString[i]);

ccitt_crc32_calc;

END;

END;

crc_input:=0;

ccitt_crc32_calc;

ccitt_crc32_calc;

ccitt_crc32_calc;

ccitt_crc32_calc;

END;

PROCEDURE

calc_crc16(VAR cs : STRING128);

VAR

i  : INTEGER;

BEGIN

(*

Note: this routine calculates a 16 bit CRC based on the CCITT

polynomial.  The result is stored in the crc register,

variable crc_reg_hi.

*)

crc_reg_hi:=0;

crc_reg_lo:=0;

FOR i:=1 TO Length(cs) DO BEGIN

crc_input:=ORD(cs[i]);

ccitt_crc16_calc;

END;

crc_input:=0;

ccitt_crc16_calc;

ccitt_crc16_calc;

END;

Buffer Management

-----------------

It is the responsibility of the transmitter to have enough buffer space

to cover the Nak turn-around time at a particular baud rate.  Otherwise,

in full stream mode, the receiver may Nak for a block that is not available.

For example, at 2400 baud, assuming a turn-around delay of 6 seconds, the

transmitter should have at least room for 4 blocks in his buffer area.  This

in my opinion, would be cutting it TOO CLOSE, I would recommend a margin of

at least 2 times or more.  In GT PowerComm, the first program to implement

MEGAlink, we have a ring buffer of 32 blocks.  This is very easy to use,

because the the Nak'ed block numbers can be AND'ed with $1F to produce the

buffer number.  At 2400 baud, 32 blocks in the buffer amounts to more than

60 seconds.  This gives the receive side ample time to turn-around any Nak.

Notice that at higher baud rates, the time margin shrinks.  For example,

at 9600 baud, the time margin for 32 blocks shrinks to about 14 seconds.

This would probably be fine for a direct connect, but would not be good

over a network such as PC Pursuit.  (This is not very important now, PC

Pursuit is barely scratching the surface of 2400 baud at this time, 4/20/87.)

At these higher baud rates, the transmitter must use flow control techniques to

insure that an adequate time margin is maintained.  This can be done by issuing

an RS, Request Status, command to the receive side.  The receiver should reply

with an Ack packet indicating the last good packet received.  The transmitter

should wait for reception of this Ack prior to continuing the dialog.  In

effect this is a self-imposed flow control.  The transmitter must be smart

enough to recognize the need for such, based on the baud rate and available

buffer space.  Theoretically, if enough buffer space was available to the

transmitter, it could continue to stream at any baud rate.

Error Corrections Procedure

---------------------------

When the receiver detects an error, it must immediately send a Nak packet

with the offending packet # within.  Then the receiver dumps the contents

of the serial port input buffer and waits for the requested packet.  The

transmitter will not usually be able to respond immediately, so the receiver

must expect packets with higher than expected packet #'s, until the requested

packet arrives.  These extra packets should be disgarded without comment by

receiver.

The transmitter, upon receipt of a Nak, will immediately dump the contents of

the serial port output buffer and resend the requested packet.  At this point,

due to the probability of a loss-of-sync error, which cannot be recovered, the

send and receive side now drop to start-stop mode to correct the error.  In

other words, the receiver must Ack a packet that he receives in good shape

after a prior Nak has been sent for the packet.  The sender will not restart

the stream of packets until this Ack has been received.  This prevents any

chance for loss-of-sync at this juncture.  Also important to recognize is

that the receive side of the transfer must have good code to find the start

of a packet.  It will have a fairly unique signature, i.e. the character EM

followed by two characters that are the complements of each other.  Especially

considering the fact that the receiver is looking for a particular packet at

this point, the occurence of a false signature is unlikely.

The question of how to handle packets arriving from the receiver that are them-

selves in error has not been addressed.  It is true however, that the trans-

mitter will respond to any Nak.  If the transmitter fails to get a good Nak

the receiver will continue to Nak, until the requested block is received.

This procedure works in practice, however there may well be a more elegant

solution.  For example, the transmitter, upon receipt of a faulty Nak, could

simply wait for the receiver to resend the Nak.  Of course, the transmitter

could also send an RS, request status, command to the receiver, which could

be interpreted in a Nakking situation as a "nak" to the "nak".  This should

cause the receiver to resend the Nak.

GT implements the "resend-and-see" approach, this is the most direct and

usually the best.  Even if the Nak is faulty, GT will choose from the 32

buffers and send a block.  If the block is too high or too low, the receiver

will Nak again.  This type of dialog usually leads to a resolution of the

error condition, on all but the worst lines.  Naturally, even the best

protocols fail on the worst lines!