XMODEM File Transfer Protocol

By Larry Jordan

When transferring files between computers using the telephone

system, there is always the chance that electrical noise will

result in data transmission errors. To ensure proper transfer of

files it is necessary to detect data transmission errors and to

retransmit data that contains errors. Most people think that

asynchronous parity error detection provides that capability. It

does not. Parity error detection does tell you when a data

transfer error has occurred, but it is up to you to retransmit

the data to correct errors. The problem is that parity error

detection is not actually performed by most IBM PC communication

packages. If a package does perform the error detection, it may

not inform you of errors in such a way that you know to

immediately retransmit the data. ASCOM, for example, places an

asterisk in a file where parity errors are detected, but you

may not realize the errors occurred until long after the file is

transferred. To ensure "error-free" data transfer you need a

protocol file transfer technique. Andrew Fluegelman has added

such a technique to PC-TALK.III called the XMODEM protocol.

A protocol is a set of rules and conventions that apply to a

specific area of communications that allow participants to

properly communicate regardless of the hardware brand or software

package being used. The protocol file transfer is a set of rules

for transferring files which specifies a set of ASCII handshaking

characters and the sequence of handshaking required to perform

certain file transfer functions. Protocol handshaking signals

allow communication software to transfer text, data and machine

code files, and to perform sophisticated error-checking. The

handicap in using protocol file transfer techniques is that the

computers on both ends of the communications link must be using

compatible software; there is no standard that controls these

protocols and almost all communication packages that have a

protocol file transfer option use a protocol unique to that

package. This means that a business or group of people must

standardize its microcomputer communications software to take

advantage of protocol transfers.

The Ward Christensen XMODEM protocol is one specific file

transfer protocol that may become a default standard in personal

communications because of its widespread use on  bulletin

boards and because of its inclusion in low cost personal computer

communication packages such as PC-TALK. It has not gained

widespread acceptance in business communication packages partly

because the protocol is public domain; most business

communication package designers use unique protocols to force

businesses to use their software on both ends of communication

links. By providing you with this insight into protocol transfer

and explaining in detail the operation of the XMODEM protocol, I

hope to add momentum to the development of a "standard protocol"

whether it be the XMODEM model or some other model. Users of

communication software deserve a standard protocol that will

allow them to use the technique with any microcomputer regardles

of the software packages employed.

The XMODEM protocol is illustrated in Figure 1. As you can see

from that figure, XMODEM does not begin the transfer of data

until the receiving computer signals the transmitting computer

that it is ready to receive data. The Negative Acknowledge (NAK)

character is used for this signal and is sent to the transmitting

computer every 10 seconds until the file transfer begins. If the

file transfer does not begin after 9 NAK's are sent, the process

has to be manually restarted.

After a NAK is received, the transmitting computer uses a Start

of Header (SOH) character and two block numbers (a true block

number followed by a 1's complement of the number) to signal the

start of a 128-byte block of data to be transferred then sends

the block followed by an error-checking checksum.  The checksum

is calculated by adding the ASCII values of each character in the

128 character block; the sum is then divided by 255 and the

remainder is retained as the checksum.  After each block of data

is transferred, the receiving computer computes its own checksum

and compares the result to the checksum received from the

transmitting computer.  If the two values are the same, the

receiving computer sends an Acknowledge (ACK) character to tell

the receiver to send the next sequential block.  If the two

values are not the same, the receiving computer sends the

transmitter an NAK to request a retransmission of the last block

This retransmission process is repeated until the block of data

is properly received or until 9 attempts have been made to

transmit the block.  If the communications link is noisy,

resulting in improper block transmission after 9 attempts, the

file transfer is aborted.

XMODEM uses two block numbers at the start of each block to be

sure the same block is not transmitted twice because of a

handshake character loss during the transfer. The receiving

computer checks the transmitted block to be sure that it is the

one requested and blocks that are retransmitted by mistake are

thrown away. When all data has been successfully transmitted, the

transmitting computer sends the receiver an End of Transmission

(EOT) character to indicate the end of file.

The XMODEM protocol offers the IBM PC several advantages over other

protocols and file transfer methods. First, the protocol is in

the public domain which makes it readily available for software

designers to incorporate into a communications package. Second,

the protocol is easy to implement using high level languages such

as BASIC or Pascal. Third, the protocol only requires a 256-byte

communication receive buffer which makes it attractive for IBM PC

owners who only have 64K systems. Forth, the protocol allows a

user to transfer non-ASCII 8-bit data files (i.e., COM, EXE and

tokenized BASIC) between microcomputers because it calculates the

end of a file based on file size and uses handshake signals to

indicate the end of a file instead relying on an end of file

marker character (control-Z) to terminate a file transfer.

Fifth, XMODEM error-checking is superior to normal asynchronous

parity error checking.  The parity method of error-checking is

95% effective if the software on the receiving end checks for

parity errors.  XMODEM error-checking is 99.6% effective, and the

software on the receiving end must check for errors.  Parity

errors detected also do not result in automatic retransmission of

the bad data; XMODEM detected errors result in data

retransmission until no errors are detected or until 9

retransmissions have been attempted.  Finally, the protocol is

used by many CP/M bulletin boards and having the protocol in a

communications package allows the IBM PC user to receive

error-checked files from these bulletin boards.

Andrew Fluegelman has given the XMODEM protocol a real boost in

the IBM PC world by including it in his package. He has also

added significant power to the package by including the protocol

Rumor has it that Don Withrow will soon add to the XMODEM

momentum by adding it to his HOSTCOMM software package. Keep up

the good work guys -- we will get a standard one way or the

other!

[This article was derived from material contained in a book

written by Larry Jordan and Bruce Churchill to be published this

Summer by The Brady Company. The article will also be in the

5th issue of PC World magazine.]

XMODEM Protocol File Transfer

Receiving                                      Transmitting

Computer                                         Computer

Ready to                                         Ready to

Receive                                          Transmit

|                                                |

|                                                |

|---------------------\NAK\--------------------->|

|                                                |

|<------/SOH/Blk #1/Blk #1/Good Data/CkSum/------|

|                                                |

|---------------------\ACK\--------------------->|

|                                                |

|<------/SOH/Blk #2/Blk #2/Good Data/CkSum/------|

|                                                |

|---------------------\ACK\--------------------->|

|                                                |

|<------/SOH/Blk #3/Blk #3/Garbled Data/CkSum/---|

|                                                |

|---------------------\NAK\--------------------->|

|                                                |

|<------/SOH/Blk #3/Blk #3/Good Data/CkSum/------|

|                                                |

|---------------------\ACK\--------------------->|

|                                                |

|<--------------------/EOT/----------------------|

|                                                |

|---------------------\ACK\--------------------->|

|                                                |

V                                                V

File                                             File

Receipt                                          Transmit

Ends                                             Ends

Figure 1

#########################################################