October 25, 1985                               AF Section:   32.5

Doc #     :   0988 B-0

A terminal program called Vip.

This document describes the Vip terminal emulator.  Vip was

written to run on the IBM PC and compatible computers.  Several

of the features of Vip are oriented toward use on the CP-6

operating system.

This document is divided into several sections as follows:

Section 1.   System Requirements and general information

Section 2.   Configuration

Section 3.   Honeywell VIP 7800 series terminal emulation

Section 4.   ANSI X3.64 terminal emulation

Section 5.   Local commands

Section 6.   Text file transfers

Section 7.   MODEM file transfer protocol

Section 8.   Kermit file transfer protocol

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   2

1.0   System Requirements and general information

1.1   Hardware Requirements

o  IBMPC, PC/XT, PC/AT or "100% compatible" computer.

o  At least one serial port.  Two serial ports are

supported.

o  80 column display compatible with either IBM monochrome

display adapter or IBM color graphics adapter.

Acceptable display adapters include the IBM enhanced

graphics adapter and the Hercules monochrome graphics

card.

o  One floppy disk drive.

o  Minimum of 128K bytes of memory.

1.2   Software Requirements

Vip requires version 2.0 or later of MS-DOS or PC-DOS.  There are

no additional software requirements.

The ANSI.SYS driver supplied with some versions of MS-DOS will

sometimes cause certain conflicts with Vip.  The symptom will be

characters appearing at incorrect places on the screen when

inputting to Vip's local command prompts.  This conflict does not

occur with PC-DOS.

1.3   Program files

Vip makes use of several files on the PC.  Whenever Vip looks for

one of it's file it will use the search path established by the

DOS "PATH" command.  Thus, Vip and it's files need not reside in

the current directory.  The files used are:

VIP.EXE  This file contains the object code for Vip.  This is the

file placed into execution by DOS when the command Vip

is given.

VIP.MNU  This file contains various menus and messages displayed

by Vip.  This file is required, and Vip will not execute

if it cannot find the menu file.

VIP.CMD  The command file contains commands to Vip that are

executed when Vip starts up.  The allowable commands are

described in section 5.  This file is especially useful

for setting keyboard definitions.  It is optional.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   3

VIP.CNF  This file contains configuration settings for Vip.  It

is created by saving the configuration from the

configuration menu described in section 2.  If Vip

cannot find this file a special greeting message will be

displayed and a default configuration used.

VIP.PHD  This file contains the phone directory for dialing

capable modems.  It is created automatically the first

time an attempt is made to update the phone list.

1.4   Serial I/O considerations

Vip contains an interrupt driven handler for the serial I/O

ports.  This handler is very specific to the hardware.  The

supported serial ports must adhere to the serial I/O and

interrupt controller specifications contained in the IBM PC

Technical Reference Manual.  These requirements include:

o  INS8250 or NS16450 UART for PC and PC/AT respectively.

o  UART is addressed at X'3F8' for port 0 or X'2F8' for

port 1.

o  8259 interrupt controller addressed at X'20'.

o  Port 0 generates IRQ 4 interrupts.  Port 1 generates

IRQ 3 interrupts.

Although the serial ports may be set to speeds up to 19200 bits

per second the IBM asynchronous communication adapters are only

specified to run up to 9600 bits per second.  No damage should

occur if the 9600 bit per second limit is exceeded, however the

serial port may not function properly or the serial port handler

may not b able to keep up with the high incoming character rate,

especially on an 8088 base PC.  Additionally Vip is not capable

of displaying characters at 960 characters per second so XON/XOFF

(DC1DC3) flow control is required.  Still, the higher line speeds

may provide some benefit when performing file transfers.

1.5   The capture buffer

Vip will use a portion of the computer's memory as a capture

buffer to save information that has appeared on the screen.  The

size of the capture buffer is fixed when Vip is first started.

The buffer size may be changed from the configuration menu, but

the change will not become effective unless it is saved and Vip

is restarted.  The specified size is always in lines.  The

specified size will be reduced as necessary to leave

approximately 21K bytes of free memory.

Movement through the capture buffer is controlled by the cursor

control keys at the right of the keyboard.  The Home, End, PgUp,

PgDn, up arrow and down arrow keys control movement through the

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   4

capture buffer.  The Home and End keys move the display to the

beginning and end of the capture buffer.  The PgUp and PgDn keys

move up (toward the beginning) or down twenty-four lines.  The up

arrow and down arrow keys move up or down one line.

Visual attributes, such as reverse video and underscore, are not

saved in the capture buffer.  Any scrolling in the capture buffer

will cause visual attributes of the current screen to be lost.

When the screen is displaying a portion of the capture buffer

other than the end, incoming data will be saved in the capture

buffer and not displayed on the screen.  It may be displayed

later by scrolling down through the capture buffer.  If the

capture buffer fills up, that is, wraps around to the currently

displayed area, an XOFF will be sent to the host computer to

request it to stop sending data.

1.6   The Status Line

The status line contains information about the current state of

Vip.  From left to right, the items on the status line are baud

rate, current port, port status, keypad mode, printer status, log

file status, log file size, cap lock state, num lock state, time

of day and help message.

The current port is displayed as "P0" for the primary serial port

and "P1" for the secondary serial port.  The port status is

"ready" if the current port is enabled or "disc" if it is

disabled.

The keypad mode is "scroll" if the cursor control keys control

the capture buffer or "keypad" if the cursor control keys would

send cursor position sequences to the host.  This state is

toggled by the Alt-K sequence.

The printer status displays "prt" if the parallel printer port is

enabled for printing text as it scrolls off the bottom line.  The

log file status displays "log" if the log file is active and the

size of the log file in bytes if a file is currently open.

The characters "CapLk" are displayed if Caps Lock is active and

the characters "NumLk" are displayed if Num Lock is active.  The

time of day is displayed following these two items.  These items

are updated during clock interrupts, so the status line may

appear when running other programs from within Vip.  This may

disturb programs that think they have control of the entire

screen.  For more information read the descriptions of the

DOSCOMMAND and DOSWINDOW commands in section 5.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   5

1.7   Template strings

Template strings are generally used to specify text strings to be

transmitted from Vip to the host.  Uses of template strings

include LOGON strings, MODEM command templates and user defined

keys.

In addition to plain text the template strings may contain

tokens, each token preceeded by a dollar sign ($) that perform

special functions.  The characters in columns 4 and 5 of the

ASCII chart when preceeded by a dollar sign are transmitted as

the corresponding control character in column 0 or 1 of the ASCII

chart.  For instance "$M" transmits a carriage return (CR) and

"$X" transmits a control-X (CAN).

The dollar sign preceeding a single decimal digit will wait that

number of seconds before transmitting the next character.  For

instance "$4" will wait four seconds.

In most cases a carriage return (CR) will be transmitted

following the template string.  The CR may be inhibited by having

a dollar sign as the last character of the template string.  The

other tokens that may follow the dollar sign are listed in the

following table:

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   6

Char.                     Description

-----   -------------------------------------------------------

?     Sends a delete (DEL) character.

!     Reads one character from the keyboard and transmits it.

The key must be a standard ASCII code, not a function

key, or any other non-ASCII key.

"     Reads characters from the keyboard and transmits them

until the return key is hit.  The carriage return is

not transmitted.  The keys must be standard ASCII

codes, not function or cursor control keys.

c     Waits for a key to be pressed.  The key pressed is

thrown away.

$     Sends a dollar sign.

%     Sends CP-6 autobaud sequence.

&     Sends a break.

(     Waits for a character other than a control character

to come in through the serial port.

*     Parameter replacement for MODEM templates.

+     Sends an equal sign (=).  This is necessary because

environment strings set by the PC-DOS SET command

may not contain an equal sign.

.     Set character delay.  The . is followed by a single

decimal digit that will set the number of ticks (0 to

9) to delay after each character of the template.  A

tick is approximately 1/18 second.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   7

2.0   Configuration

This section describes the configuration parameters that are

saved in the file VIP.CNF and set from the configuration menu.

VIP.CNF also contains several parameters that cannot be changed

from the configuration menu.  The program VIPCNF is provided to

set these parameters.  VIPCNF must be run with the current

directory set to the directory containing the VIP.CNF file.  That

is, VIPCNF does not use the search path to find BUD.CNF.  BUDCNF

presents one or more pages of menus for selection of

configuration parameters.  When a parameter is selected, VIPCNF

will present a short description of the parameter and display

it's current value.  At the end of the VIPCNF session the changed

parameters may be either saved or discarded.

2.1   The Configuration Menu

The configuration menu is used to set and save various

operational parameters of Vip.  The options available from the

configuration menu are described in this section.

2.1.1  Save Configuration

The current setting of configuration options is saved to the file

VIP.CNF.  The saved configuration will be restored on subsequent

invocations of Vip.  If this item is not selected then any other

configuration changes made will only apply to the current session

of Vip.

2.1.2  Toggle Port

The current port is toggled between port 0 and port 1.  Separate

configuration items are maintained for baud rate and parity of

each of the serial ports.  Whichever port is currently selected

will be the active port upon exit from the configuration menu.

2.1.3  Baud Rate

The line speed for the currently selected port may be set by

selecting this item.  Line speeds of 50 to 19200 bits per second

are supported.

2.1.4  Parity

Set the parity for the currently selected port.  Even and odd

parity with 1 parity bit and seven data bits per character are

supported, along with no parity, eight data bits.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   8

2.1.5  XMODEM parameters

This selection calls up a sub-menu of parameters relating to the

MODEM file transfer protocol.

2.1.5.1  Upload template

The upload template is the template for a command that is sent to

the host to initiate a MODEM transfer of a file from the PC to

the host.  In addition to the tokens described in section 1.7 the

command template should contain a double quote (") character

which will be replaced by the name of the file to be transferred

to the host.

Suggested upload templates for CP-6 are:

XEQ $$MODEMUP FID='$*' or MODEM.X OVER $*

The XEQ file $MODEMUP is provided on the Vip distribution disk.

2.1.5.2  Download template

This template provides a function corresponding to the upload

template for file transfers from the host to the PC.  The

suggested download templates are:

XEQ $$MODEMDOWN FID='$*' or MODEM.X $*

The XEQ file $MODEMDOWN is provided on the Vip distribution disk.

2.1.5.3  Startup timeout

This timeout value is the number of seconds Vip will wait for the

first transaction to commence when starting a file transfer.  The

default value is 60 seconds.

2.1.5.4  Receive timeout

Receive timeout is the number of seconds Vip will wait for the

next character when receiving a data packet from the host.  The

default is 10 seconds.

2.1.5.5  Send timeout

This is the time interval Vip will wait for an acknowledgment of

a transmitted packet before re-transmitting the packet.  The

default is 20 seconds.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   9

2.1.5.6  Block size

This is the size of each data block transmitted.  Both the host

and Vip must use the same data block size.  The default (and

standard for the MODEM protocol) block size is 128 bytes.  Since

MODEM is a half duplex protocol, larger block sizes will reduce

the line inefficiency due to the turnaround time between blocks.

Smaller block sizes will reduce the probability of an error,

either detected or not, on a given block.  The maximum

recommended block size is 512 bytes.

2.1.5.7  Play fanfare

Vip plays a little tune at the successful completion of a file

transfer.  This option allows the user to turn it off.

2.1.6  Buffer size

This option allows setting of the capture buffer size.  The size

of the capture buffer is only adjusted when Vip is initializing

itself, so changes in the capture buffer size only take effect if

the configuration is saved and Vip is exited and re-entered.

2.1.7  Update phone list

This selection allows for updating of the phone list used to dial

Hayes and other compatible modems.  Each item in the phone list

contains a name for identification, a phone number, and optional

logon template and baud rate and parity settings for that host.

When this item is selected the phone list is displayed and an

entry may be updated by selecting it and then answering the

prompts that follow.  The last entry in the phone list may be

used to specify the dialing command sent to the modem.  For

instance, "ATDT " to tone dial a Hayes modem.

Updates to the phone list are always saved, even if the Save

Configuration option is not later selected.

2.1.8   Screen attributes

This item allows selection of default screen colors if a color

display is available.  A sub-menu appears that allows for

separate settings for the foreground a background colors of both

the main display and the status line.  Sixteen foreground and

eight background colors are available.  Changes made to the

status line colors are displayed immediately.  Changes made to

the main display colors will only be effected after exiting the

configuration menu.  Because there are some interactions between

the default screen attributes and the attribute settings that may

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   10

be made by host programs, it may be desirable to set black

background and either white or bright white foreground when using

certain host software.

2.1.9   Set Duplex or Talk Mode

The duplex setting determines whether or not characters typed at

the keyboard are echoed to the screen.  The possible settings are

Full duplex, Half duplex and Talk mode.  Full duplex means the

host computer will echo keyboard input and Vip is not to echo the

input locally.  Half duplex means that Vip will echo keyboard

input locally.  Full duplex is the normal setting for CP-6.

Talk mode is meant to be used when two PCs are connected

together, each running Vip.  It is similar to half duplex with

two additions.  First, the return key sends and echoes both the

CR and LF characters, instead of just the CR character it

normally sends.  Second, rather than using the MODEM command

templates, an upload or download command will send a special

escape sequence to the other computer to initiate the file

transfer on the other computer.  These escape sequences are

described in section 4.2.5.

2.1.10  Toggle CP-6 mode

The only current purpose of CP-6 mode relates to MODEM file

transfers.  If CP-6 mode is on the default host filenames will

have all periods (.) changed to dashes (-).  See section 7 for

more information on the default filenames.

2.1.11  Select Emulation

Vip will emulate either a Honeywell VIP7800 series terminal in

character mode, or and ANSI X3.64 terminal.  This item toggles

between the two emulation modes.  The terminal emulations are

described in Sections 3 and 4.

2.1.12  Set Delays

This selection brings up a submenu with options for setting delay

times for several different functions.  The delay times are all

set in timer ticks, which are approximately 1/18 second.

2.1.12.1   XMITFILE Character Delay

This is a delay after each character when sending a file to the

host using the XMITFILE command.  A delay of one tick will

effectively reduce the transfer speed to 18 characters per

second.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   11

2.1.12.2   XMITFILE Line Delay

This selection sets the delay after each line when sending a file

to the host using the XMITFILE command.  A line is defined as an

occurrence of a carriage return character.

2.1.12.3   Template Character Delay

This item sets the delay after each character of a template

string.  This is a default value, which may be overridden by a

"$.n" token in the template string.

2.1.12.4   Break Length

This selection sets the length of a break signal in ticks.  A

minimum value of two ticks is recommended.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   12

3.0   Honeywell VIP7800 series terminal emulation

In the VIP7800 emulation mode Vip recognizes a set of commands

that is compatible with the Honeywell 7800 series of terminals.

The VIP7800 emulation is not a complete emulation of any

Honeywell 7800 series terminal.  In particular, only the

character mode of the 7800 series terminals is supported.  Forms

and text mode along with non-visual attributes are not supported.

Visual attributes are supported within the limits of the PC

display adapters.

3.1   Host to PC command set

This section describes those commands that may be sent from the

host to Vip.  Each character of the command is represented by two

decimal digits separated by a slash (/).  The first digit

represents the column of the ASCII chart and the second digit is

the row.  For instance 1/11 is the character ESC.

Those ANSI X3.64 commands described in section 4.2 that do not

conflict with VIP7800 command sequences are also available in

VIP7800 emulation mode.

3.1.1    Control characters

Control characters are the characters in columns 0 and 1 of the

ASCII chart along with the DEL character (7/15).  The following

control characters are recognized by Vip.  All other control

characters are ignored.

Char    Name                    Action

----    ----   ------------------------------------------

0/7     BEL    Sounds speaker

0/8     BS     Moves cursor one character position to left

0/9     HT     Moves cursor right to next tab stop

0/10    LF     Moves cursor down one line, scrolling screen

if cursor is already on bottom line

0/13    CR     Moves cursor to beginning of current line

1/11    ESC    Marks beginning of command sequence

3.1.2    Auxilliary port disconnect (APD)

Coding:  1/11 6/2      (esc b)

Turns the printer connected as LPT1 off.

3.1.3    Attribute (ATR)

Coding:  1/11 7/3 char  (esc s char)

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   13

Sets a visual attribute starting at the current cursor position

and extending to the end of the line or the beginning of the next

field, whichever occurs first.  The current cursor position is

marked as the beginning of a field.  If the cursor already

resides at the beginning of a field then the attribute currently

at the cursor position is used as the basis for the new

attribute.  Otherwise the default screen attribute is used as the

basis.

The character following the 1/11 7/3 sequence specifies the

attribute to be applied.  The following characters are

recognized, alphabetic characters may be either upper or lower

case.

Char                     Attribute

----    -------------------------------------------------

R      Use default attribute

B      Blink

H      Hidden  (foreground color set to background color)

I      Inverse Video

L      Toggles intensity (Note 1)

_      Underline (Note 2)

X      Blue

G      Green

C      Cyan

S      Red

V      Violet

Y      Yellow

W      White

Note 1:  The L attribute is low intensity on a VIP7800 terminal.

The Vip emulation will set the intensity to the opposite of

the intensity of the default screen attribute.

Note 2:  The underline attribute is supported only on the

monochrome display adapter.  It is equivalent to the

blue attribute on a color display adapter.

3.1.4    Clear (CLR)

Coding:  1/11 6/0  (esc `)

Clears the screen and all attribute fields.  Moves cursor to home

position.

3.1.5    Cursor Position Binary  (CPB)

Coding:  1/11 6/6 col row   (esc f col row)

Positions the cursor to the specified screen coordinates.  The

upper left corner of the screen has the coordinates 0,0 and the

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   14

lower right corner the coordinates 79,23.  Col and row are single

characters representing the desired screen coordinates with 32

added to each coordinate.  Thus 2/0 represents a coordinate of

zero and 6/14 represents a coordinate of 79.

3.1.6    Cursor Request Binary (CRB)

Coding:  1/11 6/14   (esc n)

Causes Vip to return the current cursor position to the host as a

Cursor Position Binary (CPB) string.

3.1.7    Cursor Backward (CUB)

Coding:  1/11 4/4    (esc D)

Moves the cursor back one character position.  The cursor will

wrap from column 0 to column 79 of the same line.

3.1.8    Cursor Down (CUD)

Coding:  1/11 4/2    (esc B)

Moves the cursor down one line.  The cursor will wrap from the

bottom line of the screen (line 23) to the top (line 0).

3.1.9    Cursor Forward (CUF)

Coding:  1/11 4/3    (esc C)

Moves the cursor one character position to the right.  The cursor

will wrap from column 79 to column 0 of the same line.

3.1.10   Cursor Home (CUH)

Coding:  1/11 4/8    (esc H)

Moves the cursor to the upper left corner of the screen (position

0,0).

3.1.11   Cursor Up (CUU)

Coding:  1/11 4/1    (esc A)

Moves the cursor up one line.  The cursor will wrap from the top

line of the screen (line 0) to the bottom (line 23).

3.1.12   Delete Attribute (DAT)

Coding:  1/11 5/11 5/1  (esc [ Q)

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   15

Deletes the attribute and field mark at the current cursor

position.  If no field begins at the current cursor location, no

action is taken.

3.1.13   Delete Character (DCH)

Coding:  1/11 5/11 5/0  (esc [ P)

Delete the character at the cursor position.  All characters to

the right of the cursor are moved one character position left.

3.1.14   Delete Line (DL)

Coding:  1/11 5/11 4/13 (esc [ M)

Deletes the line in which the cursor resides.  All lines below

the cursor are moved up one line.

3.1.15   Erase to End of Line (EOF)

Coding:  1/11 4/11      (esc K)

Erases from the current cursor position to the end of line.

Attributes are not erased.

3.1.16   Erase to End of Page (EOP)

Coding:  1/11 4/10      (esc J)

Erases from the current cursor position to the end of screen.

Attributes are not erased.

3.1.17   Insert Line (IL)

Coding:  1/11 5/11 4/12 (esc [ L)

Inserts a blank line at the line where the cursor currently

resides.  All lines at and below the cursor line are moved down

one line.  The bottom line of the screen is deleted.

3.1.18   Insert Mode (IM)

Coding:  1/11 5/11 4/9  (esc [ I)

Places the terminal in insert mode.  Each subsequent printable

character that is received will be inserted at the cursor

position, moving the cursor and all characters to the right of

the cursor one character position right.  The rightmost character

on the line will be lost.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   16

3.1.19   Insert Mode Reset (IMR)

Coding:  1/11 5/11 4/10 (esc [ J)

Removes the terminal from insert mode.

3.1.20   Line Graphics Reset (LGR)

Coding:  1/11 4/6    (esc F)

Removes the terminal from line graphics mode.

3.1.21   Line Graphics Set (LGS)

Coding:  1/11 4/7    (esc G)

Places the terminal in line graphics mode.  The line graphics

mode is useful for drawing various forms of boxes and vertical

and horizontal lines.  While in line graphics mode the terminal

will substitute special line graphics characters for the

uppercase alpha characters A through K as shown in the following

chart:

Char               Description

----   ----------------------------------------

A     Upper left corner

B     Upper right corner

C     Lower left corner

D     Lower right corner

E     Top tee

F     Right tee

G     Left tee

H     Bottom tee

I     Horizontal line

J     Vertical line

K     Intersection

3.1.22   Reset (RES)

Coding:  1/11 6/5  (esc e)

Identical to CLR command.

3.1.23   Reset to Initial State (RIS)

Coding:  1/11 6/3  (esc c)

Identical to CLR command.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   17

3.1.24   Status Line Lock (SLL)

Coding:  1/11 4/15  (esc O)

Moves the cursor from the status line to the position it occupied

before the SLS command was used to move it to the status line.

The message in the status line remains displayed.

3.1.25   Status Line Reset (SLR)

Coding:  1/11 7/6   (esc v)

Moves the cursor from the status line to the position it occupied

before the SLS command was used to move it to the status line.

The message in the status line is cleared and the terminal status

is restored to the status line.

3.1.26   Status Line Set (SLS)

Coding:  1/11 7/7   (esc w)

Saves the current cursor position, clears the status line to

blanks and moves the cursor to the beginning of the status line.

The SLL or SLR command may be used to remove the cursor from the

status line.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   18

3.2   PC to host command set

In addition to the standard ASCII codes several of the keyboard

keys will transmit "escape" sequences of multiple characters to

the host.  This section documents the sequences transmitted by

these keys.  Note that any of the keys described in this section

may be re-defined via the DEFINE command described in section 5.

3.2.1 Function keys

The keys labeled F1 through F10 on the lefthand side of the

keyboard are defined to produce the same codes as the

corresponding function keys on the VIP7801 terminal.  In addition

the F1 through F4 keys are defined to provide the function of the

F11 and F12 keys on the VIP7801 terminal.  The sequence

transmitted is 1/11 (esc) followed by a single character to

identify the key.  The identifying characters are in the

following table:

Key        Normal      Shifted     Alt

---       --------     -------   -------

F1        3/0    0     3/1   1   5/12  \  (F11)

F2        3/2    2     3/5   5   5/14  ^  (F12)

F3        3/6    6     3/7   7   5/13  ]  (shifted F11)

F4        3/8    8     3/9   9   5/15  _  (shifted F12)

F5        3/10   :     3/11  ;

F6        3/12   <     3/13  =

F7        3/14   >     3/15  ?

F8        5/0    P     5/1   Q

F9        5/2    R     5/3   S

F10       5/4    T     5/6   V

3.2.2 Other specially defined keys

The following keys also have default definitions:

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   19

Key            Command Sequence

-------   -------------------------------------------------

Home      1/11 4/8            (esc H)             (note 1)

Up        1/11 4/1            (esc A)             (note 1)

Down      1/11 4/2            (esc B)             (note 1)

Right     1/11 4/3            (esc C)

Left      1/11 4/4            (esc D)

Ins       1/11 5/11 4/9       (esc [ I)

Backtab   1/11 5/11 5/10      (esc [ Z)           (note 2)

Note 1:  These keys do double duty, also controlling scrolling

within the capture buffer.  The TOGKEYPAD command is

used to toggle between controlling the capture buffer

and sending the cursor control sequences.

Note 2:  The backtab code is generated by shifting the tab key.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   20

4.0   ANSI X3.64 terminal emulation

The X3.64 mode of terminal emulation follows the ANSI X3.64-1979

standard for terminal control sequences.  Certain extensions have

been made to accommodate features of Vip that are not supported by

the standard.  The standard and extra-standard sequences are

documented in separate sub-sections.

In the command notation that follows the control sequence

introducer (CSI) consists of the characters 1/11 5/11 (esc [).

The eight bit environment is not supported and thus character

9/11 may not be substituted as CSI.

Numeric parameters, designated P1 through Pn, are strings of

decimal digits (that is, characters in the range 3/0 - 3/9) with

no intervening blanks or other characters.  Multiple numeric

parameters are separated by semicolons (3/11).  Missing numeric

parameters have a default value of zero which is often

equivalent to one.  An example coding is:

CSI P1;P2 H

If P1 and P2 had the values 3 and 27 respectively the actual

character string would be:

1/11 5/11 3/3 3/11 3/2 3/7 4/8

esc   [    3   ;    2   7   H

4.1   Standard X3.64 control sequences

4.1.1    Application Program Command (APC)

Coding:  1/11 5/15 command string 1/11 5/12

The command string delimited by APC and ST (1/11 5/12) is

executed by Vip.  The command string may be made up of any of the

commands documented in section 5.

4.1.2    Cursor Horizontal Absolute (CHA)

Coding:  CSI P1 4/7     (esc [ p1 G)

Moves the cursor to the specified column of the current line.

The columns are numbered 1 through 80.  If P1 is omitted then

column 1 is the default.

4.1.3    Cursor Next Line (CNL)

Coding:  CSI P1 4/5     (esc [ p1 E)

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   21

Moves the cursor to the first column and down P1 lines.

Equivalent to a carriage return followed by P1 linefeeds.  P1 of

zero is treated as one.

4.1.4    Cursor Preceeding Line (CPL)

Coding:  CSI P1 4/6     (esc [ p1 F)

Moves the cursor to the first column and up P1 lines.  Equivalent

to a carriage return followed by P1 reverse index commands.  P1

of zero is treated as one.

4.1.5    Cursor Position Report (CPR)

Coding:  CSI P1;P2 5/2  (esc [ p1;p2 Q)

This sequence is transmitted from Vip to the host in response to

a DSR (Device Status Report) request for the current cursor

position.  P1 is the current row and P2 is the current column.

4.1.6    Cursor Tabulation Control (CTC)

Coding:  CSI 5/7        (esc [ W)

Sets a horizontal tab stop at the current cursor column.  Note

that this is only a partial implementation of the standard.  Any

supplied parameter will be ignored.

4.1.7    Cursor Backward (CUB)

Coding:  CSI P1 4/4     (esc [ p1 D)

Moves the cursor left P1 character positions.  The cursor will

wrap from the first to the last column of the same line.  P1 of

zero is treated as one.

4.1.8    Cursor Down (CUD)

Coding:  CSI P1 4/2     (esc [ p1 B)

Moves the cursor down p1 lines.  The cursor will wrap from the

bottom of the screen to the top.  P1 of zero is treated as one.

4.1.9    Cursor Forward (CUF)

Coding:  CSI P1 4/3     (esc [ p1 C)

Moves the cursor right p1 character positions.  The cursor will

wrap from the last column to the first column of the same line.

P1 of zero is treated as one.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   22

4.1.10   Cursor Position (CUP)

Coding:  CSI P1;P2 4/8  (esc [ p1;p2 H)

Moves the cursor to the specified absolute position.  P1 is the

row (1 through 24) and p2 is the column (1 through 80).  A zero

parameter is treated as one.  If either coordinate is outside the

screen the entire positioning command is ignored.

4.1.11   Cursor Up (CUU)

Coding:  CSI P1 4/1     (esc [ p1 A)

Moves the cursor up p1 lines.  The cursor will wrap from the top

of the screen to the bottom.  P1 of zero is treated as one.

4.1.12   Device Attributes (DA)

Coding:  CSI P1 6/3     (esc [ p1 c)

This sequence may be sent by the host with p1 of zero to request

Vip to return it's device attributes.  The response from Vip is

contained in the configuration file, VIP.CNF.  The default

response is:

1/11 5/11 3/15 3/1 3/11 3/0 6/3

esc   [    ?    1   ;    0   c

4.1.13   Delete Character (DCH)

Coding:  CSI P1 5/0     (esc [ p1 P)

Deletes P1 characters starting at the current cursor position.

All characters to the right of the deleted string are shifted

left and the line is filled on the end with blanks.  P1 of zero

is treated as one.  If P1 exceeds the number of character

positions from the cursor to the end of line then only the number

of characters to the end of line will be deleted.

4.1.14   Device Control String (DCS)

Coding:  1/11 5/1 string 1/11 5/12

The delimited string is received as a device control string.  No

action is taken on the string.

4.1.15   Delete Line (DL)

Coding:  CSI P1 4/13    (esc [ p1 M)

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   23

Deletes P1 lines starting from the line on which the cursor

currently resides.  All lines below the deleted region are

scrolled up.  The vacated lines of the screen are filled with

blanks.  P1 of zero is treated as one.

4.1.16   Device Status Report (DSR)

Coding:  CSI P1 6/14    (esc [ p1 n)

This command may be sent to Vip with a parameter of either five

or six.  If any other parameter is sent the command will be

ignored.  A parameter of five will cause Vip to return a DSR

sequence with a parameter of zero, which means device ready.  A

parameter of six will cause Vip to report the current cursor

position using a CPR sequence.

4.1.17   Erase in Display (ED)

Coding:  CSI P1 4/10    (esc [ p1 J)

Erases some or all of the characters on the screen according to

the parameter value.  The allowed parameter values are shown

below, all other parameter values cause the command to be

ignored.

0 - Erase from cursor position to end of screen.

1 - Erase from beginning of screen to cursor position.

2 - Erase entire screen and home cursor.

4.1.18   Erase in Line (EL)

Coding:  CSI P1 4/11    (esc [ p1 K)

Erases some or all of the characters on the current line

according to the parameter value.  The allowed parameter values

are:

0 - Erase from cursor position to end of line.

1 - Erase from beginning of line to cursor position.

2 - Erase entire line.

4.1.19   Horizontal Position Absolute (HPA)

Coding:  CSI P1 6/0     (esc [ p1 `)

This sequence provides exactly the same function as CHA.

4.1.20   Horizontal Tabulation Set (HTS)

Coding:  1/11 4/8       (esc H)

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   24

Sets a tab stop at the current cursor column.

4.1.21   Insert Character (ICH)

Coding:  CSI P1 4/0     (esc [ p1 @)

Inserts P1 erased character positions at the current cursor

position.  Characters to the right of the inserted characters are

shifted right.  Characters at the end of the line are lost.  P1

of zero is treated as one.

4.1.22   Insert Line (IL)

Coding:  CSI P1 4/12    (esc [ p1 L)

Inserts P1 blank lines starting at the line in which the cursor

currently resides.  Lines below the cursor are scrolled downward.

Lines at the bottom of the screen are lost.  P1 of zero is

treated as one.  The cursor position is unchanged.

4.1.23   Index (IND)

Coding:  1/11 4/4       (esc D)

Moves the cursor down one line, scrolling the screen if

necessary.  Equivalent to linefeed.

4.1.24   Next Line (NEL)

Coding:  1/11 4/5       (esc E)

Moves the cursor to the first column of the next line, scrolling

the screen if necessary.  Equivalent to carriage return/linefeed.

4.1.25   Next Page (NP)

Coding:  CSI P1 5/5     (esc [ p1 U)

Moves the currently displayed page P1 pages forward in the

capture buffer.  Equivalent to hitting the PgDn key P1 times.

This command has no effect on the active cursor position.

4.1.26   Operating System Command (OSC)

Coding:  1/11 5/13 command string 1/11 5/12

The delimited command string is processed by the DOS command

interpreter, COMMAND.COM.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   25

4.1.27   Privacy Message (PM)

Coding:  1/11 5/14 privacy message 1/11 5/12

The delimited privacy message is tossed in the garbage.

4.1.28   Preceeding Page (PP)

Coding:  CSI P1 5/6     (esc [ p1 V)

Moves the currently display page P1 pages back in the capture

buffer.  Equivalent to hitting the PgUp key P1 times.  This

command has no effect on the current cursor position.

4.1.29   Reverse Index (RI)

Coding:  1/11 4/13      (esc M)

Moves the cursor up one line, scrolling the screen downward if

necessary.  A reverse linefeed.

4.1.30   Select Graphic Rendition (SGR)

Coding:  CSI P1 6/13    (esc [ p1 m)

Sets the display attribute for subsequent characters.  Parameter

values not in the list below cause the command to be ignored.

Certain attribute combinations may not be supported on all (or

any) display adapters.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   26

P1          Attribute

--   ------------------------

0   Restore normal rendition

1   High intensity

2   Low intensity

4   Underscore

5   Blink

6   Blink

7   Inverse video

8   Hidden

30   Black foreground

31   Red foreground

32   Green foreground

33   Yellow foreground

34   Blue foreground

35   Magenta foreground

36   Cyan foreground

37   White foreground

40   Black background

41   Red background

42   Green background

43   Yellow background

44   Blue background

45   Magenta background

46   Cyan background

47   White background

4.1.31   String Terminator (ST)

Coding:  1/11 5/12      (esc \)

This sequence is used to terminate the delimited strings

Application Command, Operating System Command, Privacy Message

and Device Control String.

4.1.32   Tabulation Clear (TBC)

Coding:  CSI 6/7        (esc [ g)

Clears horizontal tab stop at the current cursor column.  The

parameters specified in the X3.64 standard are not supported, and

any supplied parameters will be ignored.

4.2   Extra-standard X3.64 Control Sequences

This section documents those control sequences available in X3.64

emulation mode that are not part of the X3.64 standard.  These

control sequences are syntactically compatible with the standard

and use character combinations that are reserved for private use

by the standard.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   27

4.2.1    Set Tab Stop

Coding:  CSI 7/0        (esc [ p)

Sets a tab stop at the current cursor column.

4.2.2    Set Scrolling Region

Coding:  CSI P1;P2 7/2  (esc [ p1;p2 r)

Sets the scrolling region of the screen to the area between lines

p1 and p2.  The defaults for p1 and p2 are 1 and 24 respectively.

A linefeed at the bottom of the scrolling region will cause the

scrolling region to scroll up one line.  A reverse index at the

top of the scrolling region will cause the scrolling region to

scroll down one line.  If the cursor is at the top of the screen

and the scrolling region starts at other than the top of the

screen, reverse index will be ignored.  Similarly for linefeeds

at the bottom of the screen.  Setting the scrolling region cause

the cursor to move to the home position on the screen.

4.2.3    Set Time of Day

Coding:  CSI P1;P2;P3;P4 7/4   (esc [ p1;p2;p3;p4 t)

This command sets the PC's time of day clock.  The parameters

are:

P1 - Hours * 100 + minutes

P2 - Seconds * 100 + hundredths of seconds

P3 - month * 100 + day of month

P4 - Year

4.2.4    Message Line

Coding:  CSI P1 7/5     (esc [ p1 u)

This command performs a function regarding the message line

according to the supplied parameter.  The values for P1 are:

0 - Clear message line a move cursor to the first column

of the message line.

1 - Restore cursor from message line to previous position

on screen.  Leave message intact.

2 - Restore cursor from message line to previous position

on screen.  Restore terminal status to message line.

4.2.5    Initiate MODEM File Transfer

Coding:  CSI P1;P2 7/9   (esc [ p1;p2 y)

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   28

This command may be used to initiate the transfer of a file using

the MODEM file transfer protocol.  P1 indicates the direction of

the transfer.  A value of 1 indicates the receiver of the command

will be receiving the file.  A value of 2 indicates the receiver

of the command should send the file.  P2 indicates the length of

the filename.  The filename immediately follows the "y" character

for p2 characters.

4.3   PC to host command set

In addition to the standard ASCII codes several of the keyboard

keys will transmit "escape" sequences of multiple characters to

the host.  This section documents the sequences transmitted by

these keys.  Note that any of the keys described in this section

may be re-defined via the DEFINE command described in section 5.

4.3.1    Function keys

The keys labeled F1 through F10 on the lefthand side of the

keyboard are defined to produce the same codes as the

corresponding function keys on the VIP7801 terminal.  In addition

the F1 through F4 keys are defined to provide the function of the

F11 and F12 keys on the VIP7801 terminal.  The sequence

transmitted is 1/11 (esc) followed by a single character to

identify the key.  The identifying characters are in the

following table:

Key        Normal      Shifted     Alt

---       --------     -------   -------

F1        3/0    0     3/1   1   5/12  \  (F11)

F2        3/2    2     3/5   5   5/14  ^  (F12)

F3        3/6    6     3/7   7   5/13  ]  (shifted F11)

F4        3/8    8     3/9   9   5/15  _  (shifted F12)

F5        3/10   :     3/11  ;

F6        3/12   <     3/13  =

F7        3/14   >     3/15  ?

F8        5/0    P     5/1   Q

F9        5/2    R     5/3   S

F10       5/4    T     5/6   V

4.3.2 Other specially defined keys

The following keys also have default definitions:

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   29

Key            Command Sequence

-------   -------------------------------------------------

Home      1/11 5/11 4/8       (esc [ H)           (note 1)

Up        1/11 5/11 4/1       (esc [ A)           (note 1)

Down      1/11 5/11 4/2       (esc [ B)           (note 1)

Right     1/11 5/11 4/3       (esc [ C)

Left      1/11 5/11 4/4       (esc [ D)

Ins       1/11 5/11 4/0       (esc [ @)

Backtab   1/11 5/11 5/10      (esc [ Z)           (note 2)

Note 1:  These keys do double duty, also controlling scrolling

within the capture buffer.  The TOGKEYPAD command is

used to toggle between controlling the capture buffer

and sending the cursor control sequences.

Note 2:  The backtab code is generated by shifting the tab key.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   30

5.0   Local Commands

The commands described in this section are used to perform local

operations from within Vip.  Many of these commands are, by

default, assigned to various keys on the keyboard.  These key

assignments may be overridden via the DEFINE command.  Commands

may also be entered in the command mode, which by default is

entered by hitting Alt-Z.

Vip uses a token oriented command parser with keyword lookup

tables.  The keyword lookup routine will accept the minimum

unique abbreviation for a keyword.  Commands requiring parameters

will call the get token routine with a set of delimiters for the

token and a prompt to be issued if a token is not already

available in the input buffer.  Blank is a usual delimiter.

Semicolon is a special delimiter that will cause the get token

routine to return null parameters until the beginning of the next

command.  This allows multiple commands, separated by semicolons,

to be entered at the same time.

In the command descriptions that follow the key to which the

command is assigned by default will be indicated in parenthesis

along with the command name.

5.1   AUTOBAUD (Alt-8)

Sends a CP-6 autobaud sequence.  The character "8" is sent,

followed by an approximate 1/4 second delay until the "is" in

"please type a left parenthesis" is detected.  After every ten

8's a break is sent.  Once the "is" has been detected the

character "(" is sent.  The command may be aborted by hitting

Ctrl-Break.

5.2   BREAK (Ctrl-Break)

This command sends a break signal lasting approximately 1/2

second.  The duration of the break signal may be set in the

configuration file.

5.3   BUFEND (End)

This command causes the end of the capture buffer (the last 24

lines) to be displayed.

5.4   BUFHOME (Home)

Causes the beginning (the first 24 lines) of the capture buffer

to be displayed.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   31

5.5   COMMAND (Alt-Z)

Enters Vip command mode.  The prompt "Vip: " will be issued and

commands may be entered.  Command mode is exited by entering an

empty line.

5.6   CONFIGURE (Alt-C)

Calls up the Vip configuration menu.  Menu choices may be

included on the command line, with the dollar sign ($) character

substituting for escape.  For instance, the command:

CONFIG CF$

would call up the configuration menu, change the baud rate to 300

baud and exit from the configuration menu.

5.7   CP6LOGON (Alt-6)

This is a combination of the AUTOBAUD and SENDLOGON commands.

5.8   DEFINE

Syntax:  DEFINE keyname string

This command allows a combination of command or text strings to

be attached to certain keys.  The keyname is from the following

list:

Function Keys:  F1, F2, F3, F4, F5, F6, F7, F8, F9, F10

Shifted Function Keys:  S-F1, S-F2, S-F3, S-F4, S-F5, S-F6,

S-F7, S-F8, S-F9, S-F10

Control Function Keys:  C-F1, C-F2, C-F3, C-F4, C-F5, C-F6,

C-F7, C-F8, C-F9, C-F10

Alt Function Keys: A-F1, A-F2, A-F3, A-F4, A-F5, A-F6, A-F7

A-F8, A-F9, A-F10

Alt Alphanumeric Keys:  A-0, A-1, A-2, A-3, A-4, A-5, A-6,

A-7, A-8, A-9, A--, A-=, A-A, A-B,

A-C, A-D, A-E, A-F, A-G, A-H, A-I,

A-J, A-K, A-L, A-M, A-N, A-O, A-P,

A-Q, A-R, A-S, A-T, A-U, A-V, A-W,

A-X, A-Y, A-Z

Miscellaneous Keys:  BACKTAB, BREAK, DEL, DOWN, END, HOME,

INS, LEFT, NUL, PGDN, PGUP, RIGHT, SCRLLOCK,

UP

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   32

Miscellaneous Control Keys:  C-2, C-END, C-LEFT, C-PGDN, C-PGUP,

C-RIGHT

The key named BREAK is the Ctrl-Break combination.  The key named

NUL may be any key that has been set to produce a NUL (zero) code

by using a keyboard translation table.  (See the TRANSLATION

command.)  A key name of "?" will list the valid key names.

Definitions applied to the Home, Up arrow, PgUp, End, Down arrow

and PgDn keys are only effective in keypad mode.

The "string" may contain a combination of commands and template

strings (template strings are described in section 1.7) separated

by semicolons.  Note that since the definition string may contain

semicolons, multiple DEFINE commands may not be placed on the

same line.  Template strings begin with a double quote, commands

do not.  There is no closing double quote on template strings.

Some example key definitions are:

DEFINE A-X "XEQ MYXEQFILE

DEFINE HOME AUTOBAUD;"ACCOUNT,NAME,PASSWORD

DEFINE PGUP UPPAGE;UPPAGE;UPPAGE

5.9   DIAL (Alt-A)

This command displays the phone directory and allows a dial

command to be sent to the modem.  The command "DIAL B" will dial

the second entry in the phone directory.

5.10  DISCEXIT (Alt-E)

This command drops the DTR control signal on the current serial

port, effectively hanging up or disconnecting the line, and then

exits from Vip.

5.11  DISCONNECT (Alt-Q)

This command drops the DTR control signal as does the DISCEXIT

command, but does not exit from Vip.

5.12  DOSCMD (Alt-Y)

Syntax:  DOSCMD command string

This command allows DOS commands, or even other programs to be

executed from within Vip.  The command string is passed on to the

DOS command processor.  Vip remains resident in memory and will

return when the DOS function completes.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   33

5.13  DOSWINDOW (Alt-W)

This command is similar to DOSCMD, except that a full screen

window is opened up for the DOS application program.  This is

necessary for those programs that require full control of the

screen or full use of the cursor control keys, such as text

editors, word processors and BASIC.

5.14  DOWNLINE (Down arrow)

Scrolls down one line in the capture buffer.

5.15  DOWNLOAD (Alt-D)

Syntax:  DOWNLOAD fname1 fname2

This command initiates a MODEM file transfer from the host to

Vip.  Fname1 is the name the file will have on Vip.  Fname2 is

the name of the file on the host.  More information on this

command is available in section 7.

5.16  DOWNPAGE (PgDn)

Scrolls down one page in the capture buffer.

5.17  DUMPBUFFER

This command allows data in the capture buffer to be dumped to a

file or device on the PC.  The command causes a prompt to be

issued for a filename.  If the last character of the filename is

a slash (/) then an existing file will be extended, otherwise a

new file will be created.  Once the filename has been entered,

scroll the capture buffer so that the top line of the screen

contains the first line to be dumped and hit the enter key.  Then

scroll the capture buffer so that the bottom line of the screen

contains the last line to be dumped and hit enter again.  The

specified contents of the capture buffer will be dumped to the

file.  Ctrl-Break may be used to abort the dump.

5.18  EXIT (Alt-X)

Exits from Vip back to DOS.

5.19  FINDMARK (Alt-9)

Used in conjunction with the MARKBUF command, this command will

return the display to a previously marked point in the capture

buffer.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   34

5.20  HELP (Alt-M)

Syntax:  HELP topic

This command displays the first page of a help menu.  If no topic

is supplied the key definitions menu is displayed.  The topic is

the name of a menu that has been defined in VIP.MNU.

5.21  KERMIT

Invokes the Kermit file transfer protocol.  Kermit is described

in section 8.

5.22  MARKBUF (Alt-0)

This command marks the currently displayed location in the

capture buffer so it may later be returned to using the FINDMARK

command.  Only one position is marked at a time.

5.23  OPENLOG (Alt-O)

Syntax:  OPENLOG filename

Opens a file that will log all characters received through the

serial communications port.  The actual logging of data may be

toggled on and off using the TOGLOG command.  The log file is

closed by issuing an OPENLOG command with no file name.

5.24  READ

Syntax:  READ filename

Reads commands from the specified file until the end of file is

hit.  The commands may be either commands described in this

section, or template strings beginning with a double quote.

5.25  READY (Alt-R)

Initializes the current serial port.  After a DISCONNECT command

has been issued, a ready command will re-activate the port.

5.26  SENDLOGON (Alt-L)

Sends the logon string defined by the LOGON environment string.

This environment string must be set from the DOS command

processor prior to entering Vip.  The command to set the string

is:

SET LOGON=whatever you like

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   35

5.27  TOGKEYPAD (Alt-K)

This command toggles the function of the keypad keys between

being used to send cursor control sequences to the host and

controlling scrolling within the capture buffer.  When the word

"scroll" appears in the status line the keypad controls the

capture buffer.  The word "keypad" indicates cursor control

sequences will be sent to the host.

5.28  TOGLOG (Alt-F)

Toggles the logging of incoming data to the log file.  The log

file remains open.  The word "log" will appear on the status line

when the log file is active.

5.29  TOGPORT (Alt-P)

Toggles between serial port 0 and serial port 1 being active.

The currently active port is displayed on the status line as P0

or P1.

5.30  TOGPRT (Alt-H)

This command toggles output to the printer on and off.  When the

printer is on, the word "prt" appears on the status line.  The

printer must be a parallel printer attached as LPT1.  The printer

output is actually an image from the bottom line of the screen.

A line is printed each time the screen scrolls up a line.  This

has the advantage of honoring cursor control sequences (for

horizontal positioning only) in the printed output.  The

disadvantage is it limits output to 80 columns with no

overstriking.  If wider output or overstriking is necessary a

CP-6 terminal profile that describes the printer attributes

should be used, and the OPENLOG function of Vip should be used to

send output to the printer.  For example, if a profile EPSFX100

was created for Epson FX-100 printers:

Issue to CP-6:   !PROFILE EPSFX100

Hit Alt-O:       Log File: LPT1

5.31  TOGSTATUS (Alt-S)

This command toggles the status line on and off.  When on, the

terminal status as described in section 1.6 is displayed.  When

off the 25th line of the display will be blank.  This command

will also restore the terminal status display after the host has

displayed a message on the status line.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   36

5.32  TOGXLATE

Translation tables may be provided for the keyboard and screen

via the TRANSLATION command.  This command toggles use of the

translation tables on and off.  See the description of the

TRANSLATION command for more information.

5.33  TRANSLATION

Syntax:  TRANSLATION filename

This command loads translation tables for the keyboard and

screen.  The translate table is a 512 byte file.  The first 256

bytes are the screen translation table, the second 256 bytes are

the keyboard translation table.  The screen translation table is

used whenever a character is to be displayed on the screen.  The

character code will be used as in index into the screen

translation table to arrive at the character actually displayed.

The keyboard translation table is similarly used to translate

keyboard input before it is sent out through the serial port.

The translation tables may be bypassed using the TOGXLATE

command.

5.34  TYPE (Alt-T)

Syntax:  TYPE filename

This command will display a text file from local storage on the

screen.

5.35  UPLINE (Up arrow)

The UPLINE command scrolls up one line in the capture buffer.

5.36  UPLOAD (Alt-U)

Syntax:  UPLOAD fname1 fname2

This command sends the file specified by fname1 from Vip to the

host with the name fname2 on the host.  The MODEM file transfer

protocol is used.  For more information see section 7.

5.37  UPPAGE (PgUp)

The UPPAGE command scrolls up one page in the capture buffer.

5.38  XMITFILE (Alt-I)

Syntax:  XMITFILE filename

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   37

This command sends the specified text file from Vip to the host.

If a log file is open the logging operation will be temporarily

suspended while the text file is being transmitted.  For more

information see section 6.

5.39  ?

The ? command lists all command keywords.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   38

6.0   Text file transfers

Vip provides functions for transfer of text files with no error

checking protocol.  These functions are recommended only when the

MODEM and Kermit file transfer protocols are not available, or to

maintain a record of an online session.

6.1   Host to Vip Text File Transfers

The OPENLOG command is used for host to Vip text file transfers.

All characters incoming through the serial port will be sent to

the log file.  Use of this form of file transfer will probably

require that the host honor XON/XOFF (DC1DC3) flow control.

6.2   Vip to Host Text File Transfers

The XMITFILE command is used for Vip to host text file transfers.

The PC file is transmitted byte for byte except that, unless

"talk mode" is active, a linefeed immediately following a

carriage return will be omitted.

If files are transmitted at line speed it may be necessary that

the host flow control Vip to avoid losing data.  Vip will honor

either XON/XOFF flow control or the Clear to Send (CTS) RS232

signal.

Delays may be introduced following each character or line of text

via parameters in the configuration file.  The delay periods are

in clock ticks, which are approximately 1/18 second.  These

delays may be set from the delays submenu of the configuration

menu.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   39

7.0   MODEM File Transfer Protocol

The MODEM file transfer protocol provides for the transmission of

files with error checking and retransmission of data where errors

occur.  MODEM requires a fully transparent eight bit data path

between Vip and the host.  MODEM may also be somewhat sensitive

to delays introduced by communication networks, and thus may not

function properly even when a transparent data path is available.

(Transparent data path means that all 256 possible combinations

of eight data bits may be transmitted and received.)

7.1   Data Block Size

MODEM uses a fixed size data block for transmission.  The

standard size of this data block is 128 bytes.  Both Vip and the

CP-6 implementations of MODEM allow for other size data blocks.

Both sides of the transfer must agree on the size of the data

blocks.  The maximum recommended data block size is 512 bytes.

Because MODEM is a half-duplex protocol significant increases in

throughput may be achieved by increasing the block size beyond

the standard 128 bytes.  On the other hand, if line noise is a

problem, decreasing the size of the data blocks may allow more

blocks to get through without errors.

7.2   Binary and Text Files

The Vip side of MODEM does not care whether a file contains text

or binary data, such as object code.  The CP-6 side, however,

does care and has a command line option for specifying TEXT or

BINARY mode.  Provided with Vip are two CP-6 XEQ files for use

with the MODEM file transfer protocol.  These XEQ files establish

a convention for dealing with binary and text files.

The XEQ file $MODEMUP is used to upload a file from the PC to

CP6.  The CP-6 command to invoke this XEQ file is:

!XEQ $MODEMUP FID='cp6filename'

This may easily be represented as the Vip upload template:

XEQ $$MODEMUP FID='$*'

When sending files from the PC to CP-6 the "." that separates the

filename from the extension on the PC should be replaced by a

"-".  For instance, COMMAND.COM would become COMMAND-COM on CP-6.

$MODEMUP makes a decision on whether the file is binary or text

based on the file extension.  Extensions of -COM, -EXE and -BIN

are assumed to be binary files.  Any other extension implies a

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   40

text file.  Additionally, ",BIN" may be appended to the filename

to force binary mode.  $MODEMUP will give all binary files a type

of "BI".  Other files will have no type.

The XEQ file $MODEMDOWN is used to download a file from CP-6 to

the PC.  The CP-6 command to invoke this XEQ file is:

!XEQ $MODEMDOWN FID='cp6filename'

This may be represented as the Vip download template:

XEQ $$MODEMDOWN FID='$*'

Files with a type of "BI" will be downloaded in binary mode.

Other files in text mode.

7.3   Invoking MODEM from Vip

MODEM is invoked within Vip using the UPLOAD and DOWNLOAD

commands.  If no parameters are given along with the UPLOAD

command the user will be prompted for, first, the name of the

file on the PC and then the name the file is to have on CP-6.

The sequence looks like this:

Alt-Z gives:       Vip: UPLOAD

Alt-U gives:       Upload: pcfilename

To: cp6filename

Note that Alt-Z may be used to enter Vip command mode and then

the UPLOAD command may be given or the Alt-U key may be used to

get directly to the "Upload:" prompt.  The two parameters,

pcfilename and cp6filename, may be given at the "Vip:" prompt or

at the "Upload:" prompt, with the parameters separated by spaces.

If no cp6filename is specified then no upload command will be

sent to the host.  This is useful when using MODEM with a host

for which a proper upload template is not set up.  Following the

pcfilename with a semicolon will also cause this to occur.

If it is desired to upload several files to the host, a file of

UPLOAD commands may be built and the executed via the READ

command.  File contains:

UPLOAD pcfile1 cp6file1

UPLOAD pcfile2 cp6file2

...

UPLOAD pcfilen cp6filen

Then at the "Vip:" prompt:

Vip: READ commandfile

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   41

When the "To:" prompt is given the command editing buffer has

been set up to contain the same file name as was given to the

"Upload:" prompt, except that all DOS directory information has

been stripped off, and all "." characters are changed to "-".

For instance "c:dir1\dir2\file.ext" becomes "file-ext".  This

default name may be recalled by hitting F3 on PC-DOS.  This key

may vary on some other versions of MS-DOS.

Multiple parameters may be entered at the "To:" prompt, separated

by carets (^).  Each parameter will replace one occurrence of $*

in the template string.

The DOWNLOAD command is directly analogous to the upload command,

except the prompts become:

Alt-Z gives:       Vip: DOWNLOAD

Alt-D gives:       Download: pcfilename

From: cp6filename

7.4   MODEM Status Line

The status line is used during a MODEM file transfer to display

the status of the transfer.  The information on the status line

from left to right is:

o  A status message

o  Current data block

o  Total data blocks in file

o  Total error count

o  Consecutive error count

o  Instruction for aborting transfer

When downloading a file the current data block and total data

block counters count together as Vip has no way of knowing the

total number of blocks that will be sent.

7.5   Aborting Transfers

A file transfer may be aborted by hitting the Ctrl-Break key.

This does not send a break to the host.  A transfer will also

abort if more than 10 errors occur on a single block.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   42

8.0   Kermit File Transfer Protocol

The Kermit file transfer protocol was developed at the Columbia

University Center for Computing Activities.  Public domain

implementations for many computer systems are available from

Columbia.  For more information write to:

KERMIT Distribution

Columbia University Center for Computing Activities

7th Floor, Watson Laboratory

612 West 115th Street

New York, NY  10025

Unlike MODEM, Kermit does not require a fully transparent data

path.  The only control characters required are a start of packet

character (SOH in the Vip implementation) and an end of packet

character (CR in Vip).  All other data consists of printable

ASCII characters.  Kermit also contains provisions in the

protocol for transmitting file names and other control

information.  These provisions are missing from MODEM.  The

protocol also lends itself to implementation of packet windowing

algorithms that would eliminate the turnaround delay inherent in

the MODEM protocol.  Packet windowing is not currently

implemented in either Vip or CP-6 Kermit.

Because of the encoding of all data to printable characters the

Kermit protocol is less efficient than MODEM in terms of number

of characters sent to transfer a file.  The use of smaller blocks

and the inability to expand them beyond a 92 data byte limit

makes the protocol slower in most cases.  The availability of

repeat count prefixing makes up for these deficiencies on certain

files.

8.1   File Naming Convention

Since DOS filenames fit in with the naming convention established

for Kermit, Vip will send DOS filenames in the name packets.

When sending files from the PC to the host, any directory

information used to locate the file on the PC will be stripped

from the filename sent to the host.  For instance,

"c:\asm\termdat.asm" would become "termdat.asm".  Filenames

entered on a command to be sent to the host will be sent exactly

as entered.

When receiving a file Vip Kermit will change all periods, except

the last, in the filename to dashes.  Thus the name a.b.c.d would

become a-b-c.d.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   43

CP-6 Kermit has provisions for controlling what happens to period

and other illegal filename characters it receives.  Please refer

to the CP-6 Kermit documentation for more information.

8.2   Binary and Text Files

Vip does not care whether a file contains binary or textual data,

however, CP-6 does.  The CP-6 implementation of Kermit provides

facilities, using the "SET FILE MODE" command, to handle both

binary and text files.  Please refer to the documentation for

CP-6 Kermit for more information on this topic.

8.3   Vip Kermit Commands

Vip Kermit is intended to operate with a host Kermit running in

file server mode.  The available commands are described in this

section.

8.3.1    DEBUG

The debug command toggles a debug mode flag.  When debug mode is

on all packets sent and received by Vip Kermit will be displayed

on the screen.  Transmitted packets will be prefaced by "S: " and

received packets will be prefaced by "R: ".

8.3.2    END

Exits from Kermit back to the Vip command or terminal emulator

mode, depending on whether Kermit was called from command mode or

via a command key.

8.3.3    EXIT

Equivalent to END.

8.3.4    GET

Syntax:  GET fname1 fname2

This command is used to get a file from the Kermit server.

Fname1 is the name of the file on the server.  It may be

wildcarded if the server supports wildcarding.  Fname2 is

optional, and is the name the file will have on the PC.  If

fname2 is omitted the file will have the same name as it had on

the server.  If a wildcarded transfer causes multiple files to be

received, only the name of the first file received will be

affected by the presence of fname2.  Subsequent files will have

the name they had on the server.

8.3.5    QUIT

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   44

Equivalent to END.

8.3.6    RECEIVE

Syntax:  RECEIVE filename

This command prepares Vip Kermit to receive a file from the host

Kermit.  Filename is optional, and if omitted the file received

will have the same name it had on the host.  If multiple files

are received only the first one received will be affected by the

specification of filename.

This command is not used in conjunction with a Kermit server,

rather the get command is used to download files.

8.3.7    REMOTE

Syntax:  REMOTE command string

The REMOTE command sends a command string to be executed by the

remote server.  The implementation of the REMOTE command is not

compatable with all Kermits.

8.3.8    SEND

Syntax:  SEND fname1 fname2

This command is used to send a file from Vip to the host.  The

host must have been prepared to receive the file either by

placing the host Kermit in server mode or by issuing a RECEIVE

command to the host Kermit.

Fname1 is the name of the file on the PC.  It may be wildcarded

using DOS wildcarding conventions, in which case multiple files

may be sent.  Fname2 is the name the file will have on the host.

Fname2 is optional and if omitted the name of the file on the PC

is used.  If a multiple file transfer occurs, only the first file

is affected by the specification of fname2.

8.3.9    ?

The ? command lists the Kermit command keywords.

8.4   Kermit Status Line

Kermit uses the status line to display information regarding the

current or most recent file transfer.  The information displayed

on the status line, from left to right, is:

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   45

o  A status message.  May be blank.

o  Number of packets transferred thus far.

o  Number of characters transferred thus far.

o  Total error count.

o  Consecutive error count.

o  Packet type and sequence of most recently received

packet.

o  Packet type and sequence of most recently transmitted

packet.

8.5   Aborting Transfers

A file transfer may be aborted by hitting the Ctrl-Break key.

This does not send a break to the host.  A transfer will also

abort if more than 10 errors occur on a single block.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   46

Appendix A

ASCII Code Chart

0     1     2     3     4     5     6     7

+-----+-----+-----+-----+-----+-----+-----+-----+

0  | NUL | DLE |     |  0  |  @  |  P  |  `  |  p  |

+-----+-----+-----+-----+-----+-----+-----+-----+

1  | SOH | DC1 |  !  |  1  |  A  |  Q  |  a  |  q  |

+-----+-----+-----+-----+-----+-----+-----+-----+

2  | STX | DC2 |  "  |  2  |  B  |  R  |  b  |  r  |

+-----+-----+-----+-----+-----+-----+-----+-----+

3  | ETX | DC3 |  #  |  3  |  C  |  S  |  c  |  s  |

+-----+-----+-----+-----+-----+-----+-----+-----+

4  | EOT | DC4 |  $  |  4  |  D  |  T  |  d  |  t  |

+-----+-----+-----+-----+-----+-----+-----+-----+

5  | ENQ | NAK |  %  |  5  |  E  |  U  |  e  |  u  |

+-----+-----+-----+-----+-----+-----+-----+-----+

6  | ACK | SYN |  &  |  6  |  F  |  V  |  f  |  v  |

+-----+-----+-----+-----+-----+-----+-----+-----+

7  | BEL | ETB |  '  |  7  |  G  |  W  |  g  |  w  |

+-----+-----+-----+-----+-----+-----+-----+-----+

8  | BS  | CAN |  (  |  8  |  H  |  X  |  h  |  x  |

+-----+-----+-----+-----+-----+-----+-----+-----+

9  | HT  | EM  |  )  |  9  |  I  |  Y  |  i  |  y  |

+-----+-----+-----+-----+-----+-----+-----+-----+

10  | LF  | SUB |  *  |  :  |  J  |  Z  |  j  |  z  |

+-----+-----+-----+-----+-----+-----+-----+-----+

11  | VT  | ESC |  +  |  ;  |  K  |  [  |  k  |  {  |

+-----+-----+-----+-----+-----+-----+-----+-----+

12  | FF  | FS  |  ,  |  <  |  L  |  \  |  l  |  |  |

+-----+-----+-----+-----+-----+-----+-----+-----+

13  | CR  | GS  |  -  |  =  |  M  |  ]  |  m  |  }  |

+-----+-----+-----+-----+-----+-----+-----+-----+

14  | SO  | RS  |  .  |  >  |  N  |  ^  |  n  |  ~  |

+-----+-----+-----+-----+-----+-----+-----+-----+

15  | SI  | US  |  /  |  ?  |  O  |  _  |  o  | DEL |

+-----+-----+-----+-----+-----+-----+-----+-----+

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   47

Appendix B

CLKFIX.SYS

The PC hardware makes it's internal timer tick interrupts higher

priority than interrupts from the serial ports.  This means that

characters may not be received from the serial ports while the

code attached to the timer interrupts is executing.  If the timer

code executes for more than one character time this may cause

data from the serial ports to be lost.

The program CLKFIX.SYS is provided as a fix to this problem.

CLKFIX is installed as a device driver by naming it on a device

command in the CONFIG.SYS file.  CLKFIX should be installed prior

to any "real" device drivers.  The command that should appear in

the CONFIG.SYS file is:

DEVICE=CLKFIX.SYS

CLKFIX should be compatible with DOS versions 2.0 through 3.1.

It will not be compatible with systems that make use of the

multi-tasking features of the IBM AT Bios.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   48

Appendix C

The VIPMENU Program

VIPMENU is used to process a menu source file to create Vip's

menu file.  The file VIP.SRC contains the source for the

distributed VIP.MNU file.  Additional menus may be added to the

menu file, but extreme care should be taken in modifying any of

the existing menus.  The menu source file should only be modified

using a text editor that does not insert special control commands

into the edited text.  The menu facility described here is

somewhat experimental, and may be completly incompatible with a

future version of Vip.

User defined menus may be displayed using the HELP command in

Vip, as in, "HELP MYKEYS" call up a menu named "MYKEYS".  This

command may be assigned to a key using the DEFINE command.

In addition to the menu text the source file contains "dot"

commands that control the menu generation.  These commands are

denoted by a period in the first column.  The dot commands are

described below.

.N menuname

.E

The .N command is used to begin a menu and the .E command to end

it.  Any text in the source file appearing outside a .N, .E pair

is treated as commentary.  The menu name will be truncated at

eight characters.

.W col,row,width,height,attribute,flags

The .W command describes a window in which the menu is to be

displayed.  If no .W command appears, the menu will be displayed

in the current window, usually the terminal window.  The .W

command should immediately follow the .N command.  The col and

row parameters are the column and row of the upper left hand

corner of the window.  The width and height parameters are the

window dimensions in columns and rows, respectively.  The

attribute parameter is the screen attribute for the window.

These numeric parameters may be specified either as a decimal

number, or as a hexadecimal number preceeded by a dollar sign.

The flags are single character flags that enable special menu

features.  The currently defined flags are:

F Draw a frame around the window.  The window dimensions

should include the space occupied by the frame, one character

on each side.

AF Section:   32.5

Doc #     :   0988 B-0

Date      :   851025

Page      :   49

X Exclusive or the specified screen attribute with the

default screen attribute to arrive at the actual screen

attribute for this menu.

.B

The .B command causes a page break in multipage menus.  Each page

break causes the HELP command to wait for the enter key to be

struck, at which time the window is cleared and the next page of

the menu is displayed.

.Q char menuname

The .Q command causes a query for another menu to display.  A

single key is entered and if it matches the character, "char" the

corresponding menu will be displayed.  Multiple .Q commands are

used to describe the possible choices.  The esc or enter keys

will exit from the query.  For example, a menu that may call up

one of three other menus, might have the following .Q commands:

.Q A MENUA

.Q B MENUB

.Q Z MENUZ

.G menuname

The .G command is used to goto another menu after any key is

struck.  The esc key will cancel the .G command.