{ Qinit.doc - documentation for Qinit.inc                   ver 3.0, 08-31-87 }

This file will help you figure out how the new IBM PS/2 systems are config-

ured as well as original equipment.  First, it is helpful to have a list of

all the possible Alphanumeric (A/N or text) modes.  QWIK was not designed

for the All Points Addressable (APA or graphics) modes.

TABLE 1:  Hardware Specific Video Mode Characteristics

Mode Format Segment Display Box  MDA CGA EGA MCGA VGA PCjr PCC MaxPage

---- ------ ------- ------- ---- --- --- --- ---- --- ---- --- -------

0,1  40x25  B800:0  320x200 8x8       x   x   x    x   x    x     7

320x350 8x14          x        x              7

320x400 8x16              x                   7

360x400 9x16                   x              7

2,3  80x25  B800:0  640x200 8x8       x                x    x     3

640x200 8x8           x        x              7 *

640x350 8x14          x        x              7 *

640x400 8x16              x                   7

720x400 9x16                   x              7

7    80x25  B000:0  720x350 9x14  x                         x     0

720x350 9x14          x        x              7 *

720x400 9x16                   x              7

640x200 8x8                             x     3

Legend:

Format  - Characters per row by the number of rows in the data area.

Segment - Address of the first character on page 0 of the display buffer.

Display - The pixel resolution for the data area excluding the border,

horizontal by vertical.

Box     - The pixel resolution for each character, horizontal by vertical.

MDA     - Monochrome Display and Printer Adapter

CGA     - Color Graphics Adapter

EGA     - Enhanced Graphics Adapter

PGC     - Professional Graphics Controller

MCGA    - Multi-Color Graphics Array

VGA     - Video Graphics Array

PCjr    - PC Junior

PCC     - PC Convertible

MaxPage - 0-based highest page number; e.g. 7 means there are 8 pages.

MD      - 5151 Monochrome Display

CD      - 5153 Color Display

ECD     - 5154 Enhanced Color Display

Notes:

1. The 0 and 2 modes suppress color burst only on composite displays (not

RGB) only for CGA and EGA.

2. The PS/2 model 25 and 30 have an integrated MCGA.  The model 50 and

above have an integrated VGA.

3. The 8514 High Content Color Display along with the 8514/A adapter

produces a superset of the VGA for APA, but there is no differences in

the A/N modes to the VGA when the adapter is in "VGA" mode.  See IBM

documentation for Advanced Function Mode.

4. Reduce MaxPage to 3 if EGA only has 64K graphics memory installed for

modes marked "*".  MaxPage of 7 is for 128K or more.

5. The PCC can have either an alternate MDA or CGA.  The LCD (model 5140)

can emulate either the MDA or CGA modes, but the MDA mode is 640x200.

6. I currently have no information on the PGC.

7. To relieve cursor routine programming due to the multiple character box

sizes, the VGA has a cursor emulation mode.  Qinit turns on the mode

if PS/2 equipment is detected.  See CURSOR.DOC for emulation notes.

Display Combination Code (DCC) - The PS/2 video BIOS has a new function

that simplifies equipment detection called the Read/Write Display

Combination Code.  Using interrupt $10 with AH = $1A00, the call will

return the Active Display Device in BL and the Alternate Display Device in

BH.  If the function is supported, it also returns $1A to AL.  The possible

Display Device codes are:

TABLE 2:  Display Combination Code

CODE  DESCRIPTION                       QWIK30 Constant

----  --------------------------------  ---------------

$00 - No display                        NoDisplay

$01 - MDA with 5151 monochrome display  MdaMono

$02 - CGA with 5153/4 color display     CgaColor

$03 - reserved                          DCC3

$04 - EGA with 5153/4 color             EgaColor

$05 - EGA with 5151 monochrome          EgaMono

$06 - PGC with 5175 color               PgcColor

$07 - VGA with analog monochrome        VgaMono

$08 - VGA with analog color             VgaColor

$09 to $0A - reserved                   DCC9,DCC10

$0B - MCGA with analog monochrome       McgaMono

$0C - MCGA with analog color            McgaColor

$0D to $FE - reserved                   n/a

$FF - unknown                           Unknown

HavePS2 - Qinit sets HavePS2 to true if the DCC is supported.  This means

that the program has detected some kind of PS/2 equipment whether it is a

standard Model 30, a PS/2 Display Adapter on an IBM XT, or the like.  It

also means that either MCGA or VGA is present, but not necessarily

active.  To know which, just check the DCC.

Conforming to DCC - No results are obtained for the DCC on anything other

than PS/2 equipment.  However, to be consistent, Qinit was reprogrammed

to conform to the DCC for ALL equipment.  To see if a CGA is in use,

simply check to see if ActiveDD=CgaColor.  Qinit only sets the parameters

for the active display.  Although the alternate code is saved, it is only

for testing purposes.  If you change text modes in a running program,

Qinit should be executed again.

EGA Switches - By checking the value of this byte, you can determine the

monitor connected to the EGA, the alternate video system, and the start

up default.  The byte is a copy of how the dip switches are set on the

card where on=0 and off=1.  The primary is the default.  When the ECD is

set for 640x200, it is emulating the CD including the cursor mode.

TABLE 3: EGA dip switch configuration

SWITCH  VALUE  PRIMARY               SECONDARY

------  -----  --------------------  -------------------

0000    0      MDA + MD              EGA + CD - 40x25

0001    1      MDA + MD              EGA + CD - 80x25

0010    2      MDA + MD              EGA + ECD (640x200)

0011    3      MDA + MD              EGA + ECD (640x350)

0100    4      CGA + CD - 40x25      EGA + MD

0101    5      CGA + CD - 80x25      EGA + MD

0110    6      EGA + CD - 40x25      MDA + MD

0111    7      EGA + CD - 80x25      MDA + MD

1000    8      EGA + ECD (640x200)   MDA + MD

1001    9      EGA + ECD (640x350)   MDA + MD

1010   10      EGA + MD              CGA + CD - 40x25

1011   11      EGA + MD              CGA + CD - 80x25

EGA Information - The byte located at $0040:$0087 has the following

information when the EGA (or VGA) is present:

TABLE 4: EGA information

BITS  DESCRIPTION

----  -------------------------------------------------

7     (0) Clear, (1) Keep video data after mode set

6,5   (00) 64k, (01) 128k, (10) 192k, (11) 256k, memory

4     Reserved

3     (0) EGA active, (1) EGA not active

2     (0) Disable,    (1) Enable wait-for-display

1     (0) CD/ECD,     (1) MD attached

0     (0) Active,     (1) Inactive CD emulation

PCC Exceptions - Since the PCC also does not support the DCC, a little

hedging is done.  If the 5140 LCD is used in mode 7, the Active Display

Device is set to MdaMono which is close enough.  This set up can be

verified by testing if MaxPage=3.  The alternate display is found by using

interrupt $10 with AH=$15.  The result is saved in PCCAltDD.  Of course

the variable is undefined if a PCC is not used.  The possible types are:

TABLE 5: PC Convertible Alternate Display Adapters

VARIABLE   MODEL  DESCRIPTION

--------   -----  --------------------

PCCAltDD = $   0 - No alternate adapter

= $5140 - LCD

= $5153 - CGA type

= $5151 - Monochrome type

System Identification - To get both the model and submodel, you must use

interrupt $15 with AH=$C0 which only works on some computers.  If the

carry flag is set on exit, then that interrupt is not supported, because

there's no submodel number.  The alternative is to get the basic computer

model by directly accessing the byte in memory at $F000:$FFFE.  The

possible values are:

TABLE 6:  System Identification Codes

SysID Sub-Model  SYSTEM

----- ---------  ---------------------------

$FF    n/a      IBM PC

$FE    n/a      IBM PC XT

$FD    n/a      IBM PCjr

$FC    n/a      IBM PC AT (6 MHz, 01/10/84)

$00      IBM PC AT (6 MHz, 06/10/85)

$01      IBM PC AT (8 MHz, 11/15/85)

$02      IBM PC XT (286)

$04      IBM PS/2 model 50

$05      IBM PS/2 model 60

$FB    $00      IBM PC XT (256/640)

$FA    $00      IBM PS/2 model 30

$01      IBM PS/2 model 25

$F9    $00      IBM PC Convertible

$F8    $00      IBM PS/2 model 80 (16 MHz)

$01      IBM PS/2 model 80 (20 MHz)

?       IBM PS/2 model 70

The essential variables set by Qinit are:

VARIABLE      DESCRIPTION

------------  ---------------------------------------------------------

Page0seg      Segment for the base page, page 0.

Qseg          Segment for Q writing which is altered when changing

pages.

Qwait         Wait-for-retrace while Q writing.

MaxPage       Maximum possible 0-based page number.

The other variables set or located by Qinit are for your reference to

help make decisions in your programs:

VARIABLE      DESCRIPTION

------------  ---------------------------------------------------------

Vmode         Directly accesses the video mode code.

EgaRows       0-based number of rows on the screen for EGA or VGA.

EgaFontSize   1-based number of horizontal scan lines per character for

EGA or VGA.

EgaInfo       (Described above)

EgaSwithces   (Described above)

CRTcolumns    1-based number of columns on the screen for any card.

HavePS2       Detects the presence of some type of IBM PS/2 equipment.

CardWait      Requirement for wait-for-retrace for the video "card".

SystemID      Code for the IBM computer in use.

SubModelID    Code for the sub-model of the IBM computer in use.

ActiveDD      Code for the Active Display Device.

AltDD         Code for the Alternate Display Device.

PCCAltDD      Code for the Alternate Display adapter on the PC

Convertible.

A recent discovery has shown that CGA cards do not need wait-for-retrace

in 40 column modes 0 and 1.  Qinit was programmed to accommodate this.

For other hardware, you can change Qwait to suit your needs, but CardWait

should be left unchanged to save what Qinit detected.

References - For more information on the new IBM PS/2 system, you can get

the "IBM BIOS Interface Technical Reference Manual".  Other references

include:

IBM Personal System/2 Seminar Proceedings:

Volume 5, Number 2, Displays and Adapters, publication # G360-2678.

Volume 5, Number 4, Models 50, 60, 80, VGA, BIOS and Programming

Considerations, publication # G360-2747.

As always, the above information is subject to change without notice per

IBM.

IBM is the trademark for International Business Machines Corp. (but you

already know that!)