B A L L I S T I C

Version  3.10

(C) Copyright 1988 by William R. Frenchu

DISCLAIMER

Reloading  small arms ammunition can be a DANGEROUS pastime.  The

purpose of this program is NOT to teach the fundamentals of  this

fascinating hobby but rather to aid the hobbyist in assessing the

performance of the ammunition he has produced. It is assumed from

the outset that the user of this program is acquainted with basic

reloading  techniques. For review, a list of  references  wherein

may be found the necessary instruction is included in Appendix  B

of this document. DO NOT ATTEMPT TO PRODUCE AMMUNITION FOR  YOUR-

SELF  OR  OTHERS UNTIL YOU ARE COMPLETELY FAMILIAR WITH  ALL  THE

FUNDAMENTALS OF RELOADING. ALWAYS START WITH THE MINIMUM LOAD AND

WORK SLOWLY UP TO (NEVER EXCEED) THE MAXIMUM LISTED LOAD.

Although  every reasonable care was taken to insure the  accuracy

of this program, it is provided "AS IS" and without warranties as

to performance of merchantability or any other warranties whether

expressed  or implied, including, but not limited to warranty  of

fitness for a particular purpose. Good data processing  procedure

dictates that any program be tested with non-critical data before

relying on it. The user must assume the entire risk of using  the

program. The author will assume NO responsibility for any  damage

or injury that may occur.

User Supported Software

This  program may be copied and freely distributed for  any  non-

commercial use provided all accompanying files and  documentation

are  included  WITHOUT MODIFICATION. See Appendix E  for  further

information (and the usual pitch for cash).

TABLE OF CONTENTS

-----------------

1. About the Program ....................................... 1

A. General ............................................... 1

B. Program files ......................................... 1

C. System requirements ................................... 1

D. Program limitations ................................... 2

E. Notation .............................................. 2

F. What's new ............................................ 2

G. The future ............................................ 3

H. Acknowledgments ....................................... 3

2. Using the Program ....................................... 4

A. Starting the program .................................. 4

B. Command line arguments ................................ 4

C. Calculating the ballistic coefficient ................. 4

D. The ballistic table ................................... 5

E. Sight (trajectory) tables ............................. 5

F. Plotting the trajectory ............................... 5

G. Modifying the ballistic coefficient ................... 6

H. Changing the cross wind value ......................... 6

I. Changing the input function table ..................... 7

J. Miscellaneous functions menu .......................... 7

K. Calculating recoil .................................... 7

L. Calculating minimum twist ............................. 8

M. Calculating the point blank range ..................... 8

N. DOS Shell ............................................. 9

3. The Configuration File .................................. 10

A. Using the configuration file .......................... 10

B. Allowable parameters .................................. 10

4. Sources of Data ......................................... 12

A. Altitude .............................................. 12

B. Ballistic coefficient ................................. 12

C. Bullet weight, length and caliber ..................... 12

D. Gun and powder weight ................................. 12

E. Specific gravity of bullet alloys ..................... 13

F. Temperature, atmospheric pressure and wind ............ 13

G. Velocity .............................................. 13

5. Using GENTABLE and READTABLE ............................ 15

A. Using the pre-programmed functions .................... 15

B. Creating custom function tables ....................... 15

C. Checking the function file ............................ 16

Figures .................................................... 17

Appendix A, Glossary ....................................... 21

Appendix B, Information Sources ............................ 24

Appendix C, Coefficients for Several Factory Cartridges .... 25

Appendix D, Specific Gravities of Bullet Alloys ............ 28

Appendix E, User Supported Software ........................ 29

Registration Form .......................................... 30

1.  ABOUT THE PROGRAM

A. General

BALLISTIC  is a Turbo C (v1.5) program written  to  generate

ballistic  tables for shooters. Given certain  easily  obtainable

data,  it  calculates such useful information  as  the  ballistic

coefficient,  remaining  velocity, energy, momentum,  drop,  wind

deflection, maximum height, time of flight and lead (for a moving

target)  at a given range. In addition, it can be used to  deter-

mine the amount of recoil generated by a cartridge and gun, esti-

mate the minimum twist necessary for bullet stabilization, calcu-

late point blank ranges and produce trajectory tables for estima-

ting  "holdover". Finally, BALLISTIC will produce a plot  of  the

bullet's  trajectory versus range. It does all this by  comparing

the  bullet in question to a "standard bullet" with a well  known

trajectory.  As this data is calculated before use by a  separate

program (GENTABLE), the user's ballistic tables are produced very

quickly.

B. Program files

The  following files should be included in any  distribution

of BALLISTIC:

BAL-GEN.EXE  -GENTABLE  :Creates function tables for BALLISTIC.

BAL-READ.EXE -READTABLE :Optionally checks function table.

BAL-READ.ME! -Describes version, etc.

BALISTIC.EXE -BALLISTIC :Executable file.

BALISTIC.DOC -This file.

BALISTIC.REV -BALLISTIC Revision history.

C. System Requirements

BALLISTIC  should run on most IBM BIOS-compatible  computers

using PC/MS-DOS 3.0 or above. It requires 256k of free memory and

at  least one disk drive.  Non-graphic functions are directed  to

the  default  display. Graphic output  (for  Shareware  versions)

require  a  VGA, EGA, CGA, AT&T or  Hercules  compatible  display

card. (At this time graphic functions for the Toshiba 3100 series

are NOT supported due to incompatibilities between their  display

board  and the AT&T board it is supposed to emulate.  The  author

has  access to a patch that should allow a special version to  be

compiled, but no way to test it.) Registered versions can be sup-

plied  that additionally support the higher resolution  modes  of

the MCGA, IBM 8514, PC-3270 or Everex EGA+.

BALLISTIC  has been tested and found to operate properly  on

the following systems:

PC/XT with Hercules monochrome or "EGA Wonder" adapter

PC/AT with CGA or EGA adapter

NEC MultiSpeed Portable

BIOS Model G (V40 CPU)

AT&T 6300

Compaq Plus with Compaq CGA-type adapter

Compaq DeskPro 386 with Compaq EGA-type adapter

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 1

D. Program limitations

Any  program of this type will have certain limitations  re-

garding its use and accuracy and BALLISTIC is no exception. These

discrepancies  between  actual and calculated  performance  arise

primarily  because of the inability of the user to measure  input

values exactly and certain approximations the program uses in its

calculations. Assuming that the input values are known to 1%  the

average user should find agreement with published data to be 1-2%

for all velocity and trajectory calculations. The recoil section,

due to the more approximate nature of the calculation,  shouldn't

be  considered more than 10-15% accurate. In any case, while  the

numbers  produced  by  the program are believed  to  be  accurate

enough for all practical purposes, they are meant only as a guide

to ammunition performance. How a particular load performs with  a

given gun can ONLY be determined by actual firings.

While  Ingalls'  ballistic table is  historically  the  most

commonly used for this sort of work, several others are likely to

be encountered. (Many manufacturer, for instance, favor the  "G1"

table.) For the types of bullets most often used by reloaders the

results  will be very similar regardless of the table  used.  In-

deed, the variation between handloads or factory ammunition  ran-

domly  selected from the same lot will probably be  greater  than

the  discrepancies  noted between different  tables  over  normal

shooting ranges. For best results, one should try to use the same

input  function to produce the table that was initially  used  to

calculate  the ballistic coefficient. However, this isn't  always

possible,  as in the case where a published coefficient was  cal-

culated using a table to which the program doesn't have access.

E. Notation

Throughout  this document, keys to be pushed in response  to

program  prompts  are enclosed in angle  brackets.  For  example,

"press  <1>" means to press number 1 on the keyboard. For  "press

<ENTER>",  use the key labeled either "return" or  "enter".  "En-

tering" a value will require that the <ENTER> key be pressed when

finished, i.e., to "enter 10" press <1><0><ENTER>.

F. What's New

There have been several additions to the program since  ver-

sion 3.01. Some of these include:

1. Directory  under  New Table menu now prints  out  in  wide,

pause mode.

2. Shell  now  changes the environment by adding  a  temporary

"reminder" to the prompt string.

3. Plotting now has user definable axes.

4. Plot now labels range tic-marks and line of sight.

5. Added lead for moving targets to table.

6. Added Help screen to Plot function.

7. Added grid toggle to Plot function and configuration file.

8. Documentation now includes specific gravity information for

non-lead alloys.

9. Added ability to save and recall a plot for graphic compar-

ison of trajectories.

10. Added graphics screen dump for several printers. (And PTYPE

configuration option.)

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 2

11. Increased maximum number of intervals to 50.

12. Added new command line options to READTABLE. BALREMOV is no

longer necessary.

13. Improved configuration file. No more problems with  leading

or trailing blanks.

14. Added  Greenhill  number to configuration  file  and  twist

menu.

G. The Future

BALLISTIC  isn't perfect (or even necessarily finished).  It

is  hoped, for instance, that future releases will incorporate  a

"Powley  Computer"  type computation for  estimating  the  powder

charge necessary for a given velocity. The ability to make calcu-

lations based on distances given in meters or yards would also be

a  welcome improvement, as would an on-line database relating  to

commercially loaded cartridges. Additional features the author is

considering include: context sensitive help, reading/writing load

data  to  DBase  or Symphony  compatible  files,  uphill/downhill

shooting corrections, and recalling of multiple plots.

The  author is interested in adding new function  tables  if

reliable data can be found. He is currently on the lookout for  a

copy  of BRITISH TEXTBOOK OF SMALLARMS or listings of the "J"  or

"G5" tables. Also of interest is additional specific gravity data

for  commonly used bullet casting alloys, and information on  the

types  of materials being used in the "new" non-lead  bullet  de-

signs (i.e., bronze) AND the "old" ones (steel). All things  will

come  to  those users who wait (and support  BALLISTIC  with  the

voluntary contribution!)

All in all, BALLISTIC seems likely to fill the needs of many

shooting  buffs.  Included  in the documentation   is  a  section

listing sources for further reading on small arms ballistics  and

reloading. The local gun shop or public library may have  others.

READ some of them!.. AND the disclaimers. Then enjoy the program.

H. Acknowledgments

The  author  is indebted to the following people  for  their

help, support, and the many suggestions they made. Without  dedi-

cated  users such as these, the program would never have  reached

its current level of sophistication:

Bart Carfizzi

Jim Kreiser

James Roberts

Donald Terwilliger

Dave Zarodnansky

Special  thanks of course, are due to the author's wife  and

family,  Susan, Billy and Karen, for understanding (or  at  least

putting  up  with!) the author's wild mood  swings,  ranting  and

raving  as  various parts of BALLISTIC (which were  working  just

fine  yesterday!) stopped working  for no apparent reason.  Thank

you.

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 3

2.  USING THE PROGRAM

The goal in writing BALLISTIC was to produce a program which

was at once versatile and relatively easy to use. While there  is

still room for improvement, some success in both of these  direc-

tions has been achieved. The program is primarily menu driven and

prompts  whenever input data is required. Additionally, data  en-

tered  (or calculated) in one section of the program need not  be

re-entered  unless the user wants to change the value.  Data  the

program  knows about are displayed during the prompts. To  accept

the displayed value press <ENTER>. To change it, simply type  the

new data over the old, then press <ENTER>. (The table title  must

ALWAYS  be entered explicitly.) To correct typing errors  on  the

current line, use the <BACKSPACE> key, NOT the <DELETE> key.  Im-

proper  entries on previous lines can be corrected by  using  the

<ESC>  key to "back up" to the bad line, then retyping the  data.

In  most  cases, obviously improper data will be refused  and  an

acceptable  range displayed. Input filenames will be checked  for

existence before they are accepted.

A. Running the Program for the First Time

Prior  to  running BALLISTIC for the first  time,  the  file

containing  the  function table required by the program  must  be

generated  using GENTABLE (v3.00 or above, see Section  5).  This

file  will  contain the version number of the  GENTABLE  used  to

create it, the function name, some atmospheric data, the  maximum

and minimum velocity and S and T value pairs. Once this file  has

been  created (and optionally checked using READTABLE)  BALLISTIC

can  be invoked. Only BALISTIC.EXE and a function file  need  ac-

tually be present on the disk when running the program.

B. Command Line Arguments

BALLISTIC is started by simply typing its filename (BALISTIC)

at  the DOS prompt, optionally following it with one or  both  of

the  following arguments. "-BIOS" will cause all video output  to

be  done using BIOS calls rather than direct screen  writes.  The

second argument, "-F" followed (with NO spaces) by the name of  a

BALLISTIC  function  file, causes the named function file  to  be

loaded instead of the default file BALISTIC.TAB.

BALLISTIC will first attempt to read the configuration  file

(BALISTIC.CFG, see Section 3), then search for the default  func-

tion  file  (either BALISTIC.TAB or the  command  line  specified

file),  prompting the user for the name of an alternate  if  it's

not  found.  The default file type is ".TAB" which  need  not  be

entered. (To enter a filename with no extension enter "filename."

with the trailing period.) BALLISTIC will next read the  function

file,  make a check to determine its validity (a simple  checksum

procedure), and present the user with the main menu. (Fig. 1)

C. Calculating a Ballistic Coefficient

The first step in developing a ballistic table is to  deter-

mine  the  ballistic coefficient. If this value cannot  be  found

explicitly  in one of the sources described in section  4B,  main

menu  choice  2 may be used to calculate it. Press  <2>  and  the

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 4

program will prompt for an initial and final velocity, the  range

and some atmospheric data. After this has been entered, the  cal-

culated value of the effective ballistic coefficient (as well  as

the  coefficient at standard conditions for the function  chosen)

will be displayed on the screen (Fig. 2). Having found the  coef-

ficient, BALLISTIC can now calculate the table. Press the <ENTER>

key  to  redisplay the main menu then <1> to bring up  the  input

section.

D. The Ballistic Table

Figure  3 shows the input data for main menu choice 1,  cal-

culating  the ballistic table. Since the initial velocity,  range

and  ballistic coefficient have been entered or calculated  on  a

prior  screen, these values needn't be re-entered unless  a  dif-

ferent value is required. In this case, entering a range value of

500  will  create a table to 500 yards based on  the  performance

noted over the first 100. Entering a value of zero for the  range

will cause the program to switch to metallic silhouette mode. The

user will then be prompted for his choice of standard  silhouette

rifle  or pistol ranges rounded to the nearest yard. As  long  as

the  input  value for range remains zero these  alternate  ranges

will  be  used. The final necessary data item is  the  number  of

intervals  into  which  the total range should  be  divided.  The

maximum  number allowed is the smaller of 50 or the total  range.

Data entry is concluded with an optional table title.

Figure 4 shows the table itself and the program's prompt for

generation  of a sight table. At this point, should it be  desir-

able to get a printed copy, press <1>. The program will  redirect

ballistic table output to the system's LST: device and return  to

the  prompt. A form feed (page eject) can be sent to the  printer

by pressing <2>.

The "New Data" choice, <4>, will return the user to the data

entry  screen to calculate another ballistic table,  while  "Main

Menu"  (<5>  or <ESC>) will jump directly back to  the  program's

initial screen.

E. Sight (Trajectory) Tables.

Sight (trajectory) tables can be produced by pushing <3>  in

response to the prompt at the bottom of the ballistic table.  The

program  will  require  that distance for  which  the  weapon  is

"sighted  in"  (this value need NOT have appeared  in  the  range

column  of  the original ballistic table) and the height  of  the

front sight above the centerline of the bore. In silhouette mode,

entering  a value of zero for the targeted range will  cause  the

prompt to take the form of which silhouette (chicken, pig, turkey

or  ram) the weapon is zeroed for. The user should  respond  with

the first letter of the appropriate target. Output of this  table

can  be  diverted  in exactly the same manner  as  the  ballistic

table. Pressing <3> allows the user to enter new data for another

sight  table  (using the current ballistic conditions),  and  <5>

(or <ESC>) returns to the ballistic table and prompt.

F. Plotting the Trajectory.

Pressing <4> at the sight table prompt will present the user

with a graphic representation of the bullet's trajectory on  sys-

tems  with an appropriate graphics card. Range, in yards will  be

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 5

indicated along the bottom, with tic marks appearing at distances

corresponding to the entries in the ballistic table. Dotted lines

will  appear vertically at the distance for which the  weapon  is

zeroed and horizontally at the line of sight. The trajectory  (in

inches above or below the line of sight) is indicated on the left

scale.  Pressing  <A>  (for axes) will allow the user  to  set  a

"viewport".  The  program  will prompt for  starting  and  ending

range,  and  the maximum and minimum trajectory, then  plot  only

that portion of the trajectory that falls in the described  "win-

dow". Pressing <S> (for start) will reset the plot to the initial

(program chosen) limits. <G> will toggle the appearance of a grid

on  the plot and <H> will provide a short help screen. If <W>  is

pressed, trajectory data (for the current viewport ONLY) will  be

written  to  a file. This data may be "merged" with  the  current

trajectory plot at any time by pressing <R>. Hardcopy of the plot

(actually,  a screen dump) may be obtained by pressing <P> if  an

appropriate graphics printer is present. (Supported printers  are

discussed  in  the configuration file section under  PTYPE.)  De-

pending  on  the display adapter and printer,  this  may  require

several  minutes.  (A typical EGA graphics screen dump  is  about

30k,  roughly the size of a ten page document, and takes just  as

long to print.) Printing may be abandoned at any time by pressing

a key. Users with a printer not listed may want to try using  the

<PRINT  SCREEN> key which will usually require that  the  program

GRAPHICS.COM (from the DOS system diskette) be run before BALLIS-

TIC  is  started. Neither method will work with all  display  and

printer  combinations. (Registered users may, of course,  contact

the  author  and request support for their  particular  printer.)

Pressing  <X> or the <ESC> key will return the user to the  sight

table prompt.

From  the sight table press <5> (or <ESC>) to return to  the

ballistic table, (Figure 4 again) then a <5> (or <ESC>) to return

to the main menu.

G. Modifying the Ballistic Coefficient

Those  living at high altitude or in areas of extreme  temp-

erature  will want to adjust published (sea level) values of  the

ballistic  coefficient to more accurate local values.  Main  menu

choice  3 may be used to accomplish this (Figure 5). The  program

will  need to know the altitude, barometric pressure and  temper-

ature  in addition to the value of the ballistic  coefficient  at

STP (Standard Temperature and Pressure, the published value).

Bear  in mind that these meteorological  values  (especially

temperature)  ALSO affect the initial velocity of a  given  load.

This  effect is beyond the ability of this program  to  calculate

but descriptions of it may be found in many reloading manuals.

H. Changing the Cross Wind Value and Angle

Menu choice 3 also allows for adjustment of the speed of the

cross  wind  used to calculate wind deflection. By  default,  the

program  sets this value to 10 miles per hour,  blowing  directly

across  the  path of the bullet (0 degrees). It should  be  noted

that  deflection  is a linear function, i.e., a 5 mph  wind  will

deflect a bullet by exactly half the amount of a 10 mph wind. The

angle  of the cross wind may be changed here as well. In  program

coordinates,  0 degrees (the default) is blowing directly  across

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 6

the  bullet's  path, either left to right or right  to  left.  90

degrees  would be either a head or tail wind with no  cross  wind

component. These values are used ONLY for calculating deflection.

Their  effect  on other aspects of the  trajectory  is  currently

ignored.

I. Changing the Input Function Table

Starting  with version 2.00, it is possible to switch  input

function tables without restarting the program. Pressing <4> from

the  main menu will cause a prompt for the new file name.   If  a

filename containing wildcard characters ("*" or "?") is  entered,

the  directory  of all matching filenames will be  displayed.  As

noted  in section A, the default table type is ".TAB" which  need

not  be entered. Simply press <ENTER> to abort and return to  the

main menu.

This  option doesn't affect the fact that  BALISTIC.TAB  (or

the filename supplied with the "-F" command line option) will  be

searched  for (and loaded if present) when the program  is  first

started.  It  is provided only for users who routinely  use  more

than one function table. Even so, program startup will be simpli-

fied if the table most commonly used is copied to BALISTIC.TAB.

It  should  be noted here that loading a new  function  file

will  not reset the temperature and pressure to the defaults  for

the  new table. Thus, some care should be taken if switching  be-

tween two tables that use different standard conditions.

J. Miscellaneous Functions Menu

Pressing  <5> at the main menu prompt will present the  user

with  the miscellaneous functions menu. From this menu  the  user

may  elect to calculate recoil, the minimum necessary  twist  for

bullet stabilization, or the point blank range for a given target

size.

K. Calculating Recoil

The recoil energy and velocity of a weapon can be calculated

using  miscellaneous menu choice 1, provided its weight  and  the

weight of the powder charge are known (Figure 6). This number can

be deceptive as the perception of recoil by different shooters is

VERY subjective. In addition to the physics involved, there  must

be some consideration given to the fit of the gun to the individ-

ual,  shape  and  material  of the  stock  or  grips,  etc.  Many

shooters,  for  instance, fear the recoil of a 12  gauge  shotgun

even with target loads while thinking nothing of using a  6 pound

30-06 despite the fact that the "calculated" recoil can be  quite

similar.  Such  recoil is often in the eye (or shoulder)  of  the

beholder.

This  section also calculates whether a "muzzle brake"  type

device  will  be useful in reducing felt recoil by  allowing  the

user  to determine the rough percentage of recoil caused  by  the

bullet  itself  (which a muzzle brake cannot affect).  The  value

indicated is the level to which recoil energy would be reduced if

there  were no powder at all. This is a crude estimate of what  a

shooter  might achieve with a perfect brake. (Bear in mind,  how-

ever,  that  NO  muzzle brake is 100% effective  and  the  actual

amount of reduction will be less.)

The  recoil  calculation used by BALLISTIC is a  very  rough

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 7

approximation, similar to that discussed in the March 1988  Amer-

ican Rifleman. It would be extremely challenging to write a  com-

pletely accurate description of the recoil process. It would also

be  very  difficult  to use as many variables  are  involved  in-

cluding, but not limited to bore cross sectional area, length  of

barrel,  atmospheric  pressure and temperature, type  of  powder,

loading  density, etc. The values calculated here should  not  be

considered  accurate to more than 10-15%. Barrel/powder  combina-

tions that produce excessive muzzle flash will be even less accu-

rate.

L. Calculating Minimum Twist

The slowest twist that will stabilize a bullet in flight may

be calculated using miscellaneous menu choice 2. The formula used

here is the approximation credited to Sir Alfred George Greenhill

in 1879. Necessary input data includes the bullet's  diameter and

length  in  inches and specific gravity. The  default  value  for

specific  gravity  is the average density of a  typical  jacketed

bullet.  Also listed on the input screen (for reference)  is  the

specific gravity of pure lead. Specific gravities (or  densities)

of  other  bullet  alloys seem difficult to find.  It  should  be

pointed out that these specific gravities can NOT be added algeb-

raically. That is, an alloy containing 50% lead and 50% tin  will

NOT  have a specific gravity that is the average of the two  pure

metals.  Appendix  D of this document contains  specific  gravity

data for a small selection of common alloys.

It has been pointed out to the author, that this  approxima-

tion was originally intended for use with artillery  projectiles.

More  recent findings indicate that, for typical small  arms  am-

munition,  a Greenhill number smaller than the default  of  150.0

(100  has  been suggested)  may be appropriate  especially  under

adverse  atmospheric conditions such as rain and snow. A  discus-

sion may be found in the June 1988 issue of Rifle. ("Shooting the

Kalashnikov",  pp.  28 - 31.) This number  may  be  "permanently"

changed using the configuration file.

M. Calculating the Point Blank Range

Given the velocity, sight height and ballistic  coefficient,

a  point blank range may be determined for any target  size.  The

point blank range is the maximum range for which a bullet can  be

counted upon to strike a target by simply aiming the weapon  dir-

ectly  at it. For example, given a 6 inch target, a  point  blank

range may be calculated so that the bullet is always less than  6

inches  from  the line of sight. BALLISTIC  will  calculate  this

distance for a given target size and tell the user for what range

the weapon should be sighted and what trajectory the bullet  will

have at 100 yards. (If the targeted range is less than 100 yards,

the  trajectory at 25 yards will be given instead.)  This  figure

may be used to sight the weapon when the targeted range is either

unrealistically  far  away, or an unusual number  of  yards.  For

instance,  in the example screen, sighting the weapon 5.0  inches

high  at 100 yards will allow the shooter to hit a 6 inch  target

out to 337 yards, without allowing for any holdover. However, the

bullet will only strike the CENTER of the target at 287 yards.

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 8

N. DOS Shell (Escape to DOS)

Users operating under MS/PC-DOS version 3.00 or higher  have

the option of shelling out to DOS from either the main or miscel-

laneous menus. The only other requirement for this option is that

there  be  a copy of COMMAND.COM in the place  indicated  by  the

COMSPEC path variable.

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 9

3.  THE CONFIGURATION FILE

Version  3.10  of BALLISTIC will allow certain  program  de-

faults  to be overridden with the aid of the  configuration  file

BALISTIC.CFG.  This option was originally provided for  users  in

the  CP/M environment. The file is searched for only in the  cur-

rent directory when BALLISTIC is first started and is NOT  neces-

sary for the operation of the program.

A. Using the configuration file

The  configuration file is an ascii text file each  line  of

which  contains the name of the parameter to be changed,  one  or

more  spaces,  and the new value. Parameters listed as  "must  be

integer" may NOT contain a decimal point or exponential notation.

Certain parameters may only accept ON and OFF (or similar  string

choices) as values. Entries are NOT case specific.

The following file would turn off the bell, set the default

altitude to 2000 feet and set the video write mode to BIOS:

BELL OFF

Altitude 2000

video bios

B. Allowable parameters

The following parameters are allowable for BALLISTIC v3.10:

ALTITUDE - The  altitude to be used in the ballistic  table

and  ballistic coefficient calculations. May  be  REAL.

The default value is zero.

BELL - Controls whether the console bell is rung for errors

detected  by the program. Allowable values are  ON  and

OFF. The default is BELL ON.

BKG - Determines  the background text screen color.  Allow-

able  values are 0 through 7. Must be INTEGER. The  de-

fault value is 0. This parameter is illegal for systems

having only a monochrome text/graphics card.  Following

are the colors corresponding to each value:

0 - Black         4 - Red

1 - Blue          5 - Magenta

2 - Green         6 - Brown

3 - Cyan          7 - Lt. Gray

COLOR - The  text color. Default is 7. See BKG for  a  des-

cription of allowable values and restrictions.

GREENHILL - This  option specifies a new  Greenhill  number

for  use in the minimum twist  calculation.  Acceptable

values range from 75.0 to 200.0. The default is 150.0.

GRID - This  option  informs the program whether  the  grid

should initially appear in the Plot function. This  may

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 10

also  be toggled from the Plot function itself.  Allow-

able values are ON and OFF.

PRESSURE - The atmospheric pressure used for the  ballistic

table  and ballistic coefficient calculations.  May  be

REAL.  The default value is dependent on  the  function

file chosen.

PRINTER - Line  width in characters of the system  printer.

Must be an INTEGER value from 80 to 200. Default is 80.

PTYPE - Type of graphics printer connected to system.  This

option is only used by the graphic screen dump routine.

Default is 1, for the HP Laserjet. Currently  supported

values are:

1 - Hewlett Packard Laserjet

2 - Star SG-10 (in IBM mode)

3 - IBM Proprinter

4 - IBM Graphics Printer

SCREEN - The  output screen width. This is for  future  re-

visions of BALLISTIC. Currently, only 80 is  supported.

Must be an INTEGER value.

TEMPERATURE - The  temperature  to be  used  for  ballistic

table  and ballistic coefficient calculations.  May  be

REAL.  The default value is dependent on  the  function

file chosen.

VIDEO - Determines the method used to write to the  screen.

Allowable  values are BIOS and DIRECT. BIOS causes  all

output to be done through the use of system BIOS calls.

This mode may be useful for computers/graphics adapters

that are not 100% compatible with the IBM PC family. It

does,  however,  significantly slow output.  Using  the

DIRECT  option causes all screen output to  be  written

directly to video memory. Default is DIRECT.

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 11

4.  SOURCES OF DATA

Much  of  the data necessary for ballistic  calculations  is

readily available. Even the ballistic coefficient can usually  be

calculated from the information at hand. Following are  suggested

sources for specific input data to the program.

A. Altitude

The  calculation  doesn't depend on altitude  directly,  but

rather  on the way air temperature and pressure vary  with  alti-

tude.  The program uses this value to determine what  the  "stan-

dard"  temperature and pressure are for a given  ballistic  func-

tion. Users can look up the actual altitude of their location  in

an  almanac or one of the U.S. Department of the Interior  "Quad-

rangle Maps" available at many sporting goods or camping  stores.

Either  jot it down near the computer so it can be  entered  when

the  program  is run or add it to the configuration file  as  de-

scribed in section 3.

B. Ballistic Coefficient

This  will  probably be the most difficult number  to  find.

Those who reload will usually be able to get this value from  the

manufacturer  of their bullets. (Speer and Sierra, for  instance,

print this data in their reloading manuals.) The value for  other

bullets  may sometimes be estimated by comparing  bullet  shapes,

provided the bullet is of the same caliber. Comparing the  bullet

in question to a specially prepared chart of shapes is often more

accurate.  The  DuPont Company at one time produced a  series  of

these  and  other useful charts, called "A Short Cut  To  Ballis-

tics".  The complete set is supplied with Ackley's  handbook  and

the  specific chart for estimating ballistic coefficients is  re-

printed in HATCHER'S NOTEBOOK.

The program itself will calculate the ballistic  coefficient

in  certain cases. Both the initial and remaining velocity  at  a

given  range  must be known. This data is available  for  factory

loaded  ammunition in manufacturers' literature and sources  such

as  SHOOTERS BIBLE. (A short section of coefficients for  several

factory loadings calculated using this program and Ingalls' table

may be found in Appendix C.)

C. Bullet Weight, Length, and Caliber

These are easy. On factory-loaded ammunition or  components,

bullet weight (in grains) and caliber will be written on the box.

Those who cast their own bullets will need to weigh several  sam-

ples  prepared  with the actual alloy used. Take the  average  of

half  a dozen or so and use that value. A  sufficiently  accurate

value for bullet length may be found with a ruler.

D. Gun, Powder Weights

These two are needed only to calculate the amount of  recoil

generated  by a given load. The weight of a rifle can usually  be

found accurately enough on a bathroom-type scale. Actual  weights

of  most  firearms can be found in manufacturers'  literature  as

well as in such sources as SHOOTER'S BIBLE, GUN DIGEST, and  pro-

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 12

duct reviews in many magazines. Don't forget to add in the weight

of scopes, slings, custom stocks or any other accessories.

Powder weights will be known if performing calculations  for

handloads. If using factory loads, try estimating by comparing to

available  reloading  data. Find a load that moves  a  bullet  of

similar  construction  (jacketed, soft lead, etc.) to  about  the

same muzzle velocity.

E. Specific Gravity

This  value  is only required for  calculating  the  minimum

necessary  twist  for stabilization. Two more  or  less  standard

values are 11.34 for pure lead, and 10.90 for a typical  jacketed

bullet.  Other values are very difficult to find in  the  litera-

ture.  Hatcher,  describes a fairly simple way  to  measure  this

quantity,  but the two values above should be sufficiently  accu-

rate for most practical purposes.

Appendix D of this document contains a short compilation  of

specific  gravity data for some common Lead/Tin/Antimony  alloys.

These were taken primarily from volume one of the American  Soci-

ety of Metals METALS HANDBOOK.

F. Temperature, Atmospheric Pressure, Wind Speed and Direction

Suitably accurate values for temperature and pressure can be

obtained  from a weather report or newspaper. As  with  altitude,

the  calculation  isn't terribly sensitive to these  factors,  so

unless  the range is in the Sahara Desert or Antarctica they  can

probably be ignored. (It CAN be interesting to set up a ballistic

table and vary them just to see what happens... Experiment!)  The

same  suggestion applies; try the program with and  without  cor-

recting and see if the difference is big enough to  realistically

notice.  Bear  in mind that this program examines the  effect  of

these  variables only on the ballistic coefficient.  Temperature,

for instance, will also change the initial velocity of a load,  a

factor  beyond the ability of this program to calculate.  (Tables

describing  this effect can be found in both the Speer  reloading

manual and Ackley's handbook.) Temperature and pressure are  also

valid parameters for use in the configuration file. (Section 3)

Wind direction can only be estimated at the actual  shooting

site.  Zero degrees indicates a wind blowing across the  bullet's

path to either the right or left. Ninety degrees indicates a wind

blowing  either  directly behind or directly  into  the  bullet's

path. As this value is only used for calculating wind deflection,

a signed value isn't necessary. Also, since deflection is  linear

with  wind  speed, leaving the value set for the default  10  mph

allows for easy interpolation at the range.

G. Velocities

Muzzle velocities of most factory ammunition can be found in

periodicals  such  as the SHOOTER'S BIBLE. Those who  reload  are

aware  that estimated or reported velocities are  usually  listed

with the loading data for a particular cartridge.

The most effective way to find the velocity for a completely

unknown  load is, of course, the chronograph. Unfortunately,  few

people  have access to one and their price puts them  beyond  the

reach of the casual buyer. (The author once built an  inexpensive

attachment  that allowed a Commodore-64 home computer to be  used

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 13

as  a simple chronograph, but its usefulness was limited since  a

source of house current was still needed nearby... also, the C-64

isn't exactly portable. The technique SHOULD be adaptable to many

other home computers and the author would be happy to discuss the

project with registered users.) Ackley, in his book HANDBOOK  FOR

SHOOTERS  AND RELOADERS, describes a simple "ballistic  pendulum"

that  can be used to estimate projectile velocities. No  external

power is necessary; it relies on the kinetic energy of the bullet

to swing a heavy mass.

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 14

5.  USING GENTABLE AND READTABLE

A. Using the pre-programmed functions

Before  running  BALLISTIC for the first time  a  file  con-

taining  the chosen set of function values the program  will  use

during  its calculations must be produced. GENTABLE Version  3.00

makes the following functions available:

FUNCTION           VELOCITY         ATMOSPHERIC

max.   min.     temp.  pressure

British 1909          4000    0        59      29.53

"G1"                  4094   300       59      29.53

Ingalls'              3600    0        60      30.00

Ingalls' (extended)   4094    0        60      30.00

The  British  1909 and Ingalls'  functions  are  practically

identical until low velocities are reached. Both are well  suited

to  calculations  involving typical hunting  or  pistol  bullets,

i.e.,  rounded or slightly pointed (roughly two  caliber  radius)

spitzers. Results for sharply pointed spitzers and boattails will

not  be  as accurate with these functions as they might  be  with

some others. Less accurate results will also be obtained if  bal-

listic coefficients calculated with one set of functions are used

with  another, i.e., using the coefficients listed in Appendix  C

(calculated with Ingalls' table) with the British 1909 tables.

The  extended  Ingalls' table is simply the  standard  table

continued to the maximum allowable program velocity following the

method  used in Hatcher. As values above 3600  are  extrapolated,

they may not be as accurate as those below.

Many  of the companies producing reloading  components  feel

that the G1 table is the best choice for all around use.  Indeed,

most  of  the major ammunition manufacturers seem to use  the  G1

drag function for calculating their own "factory" ballistics  and

duplication  of  these tables is most easily  accomplished  using

this selection. This function is used by SAAMI in calculating the

tables they supply to the manufacturers as well.

GENTABLE is started by typing its name (BAL-GEN) at the  DOS

prompt, optionally following it with a command line argument. The

user will be presented with a short menu to which he should  res-

pond with the number of the function he desires. The program will

next  prompt for a filename to contain the new data, then  create

the  table, overwriting any previous file with that  name.  Users

that  plan to routinely use more than one table  should  probably

create  them all ahead of time, renaming them to  something  more

descriptive. Copy the one used most often to BALISTIC.TAB.

GENTABLE will accept only one argument on the command  line.

If the flag "-BIOS" is used all video output will be sent to  the

screen via BIOS calls. The default is to write directly to screen

memory.

B. Creating custom function tables

The  user is not limited to using the functions produced  by

GENTABLE. Tables may be produced by any program or language  that

stores  numbers  in IEEE format, including Microsoft  C  v5.0  or

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 15

Quick  C.  (There will be a problem  calculating  checksums  when

using  Microsoft  5.0 unless the -Op optimization is  used.)  The

file format and data types used are as follows:

1.  Truncated  revision number for version of  GENTABLE  used

multiplied by -1. (int)

2.  The table name. (char[32])

3.  Standard temperature for table data. (double)

4.  Standard pressure for table data. (double)

5.  Maximum velocity in table (<=4094). (unsigned)

6.  Minimum velocity in table. (unsigned)

7.  S_table, T_table pairs as described in HATCHER'S NOTEBOOK

for velocities (max >= v >= 0). (float)

8.  Checksum  calculated  as the sum of (S_table  -  T_table)

values. (float)

C. Checking the function file

After  creating  the  function file  it  may  be  optionally

checked  with  READTABLE. READTABLE was originally written  as  a

debugging aid and is included only to satisfy user's curiosity as

to what is contained in a ballistic function table. It might also

be of some use to those contemplating creation of their own func-

tion files. Once started, it prompts the user for the name of the

function  file to be examined, displays the  header  information,

then  prints the values to the screen. The speed of  the  display

may  be altered by pressing either the <-> or <+> keys.  Pressing

any other key will cause the display to pause at the next  value.

Once paused, pressing <ESC> causes execution to abort. Any  other

key will cause the program to resume.

As  with GENTABLE, READTABLE is started by typing  its  name

(BAL-READ)  at the DOS prompt, optionally followed by any of  the

following flags:

-BIOS - Causes  the program to perform all writes using  BIOS

calls. This will significantly slow output.

-F - (From) - Start listing the table to the screen with  the

value  immediately following the argument, i.e.,  -F2000

will start the listing at 2000 fps.

-T - (To) - End the listing at the value following this argu-

ment.

-N - (Noprint) - Don't print the table, calculate the  check-

sum only.

-G - (Graphics) - Don't  use the IBM graphic  character  set.

(Use ASCII characters instead.)

Flags  requiring numeric arguments should NOT have  a  space

between the flag and input value.

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 16

--==<  B A L L I S T I C  v3.10  >==--

(C) Copyright 1988 by W.R. Frenchu

- MAIN MENU -

1 ........... Calculate Ballistic Table.

2 ..... Calculate Ballistic Coefficient.

3 ....... Adjust Atmospheric Conditions.

4 ........... Select New Function Table.

5 ............. Miscellaneous Functions.

6 ........................... DOS Shell.

7 .................... Return to System.

Your Choice ? 1

---------------------( Fig. 1 - MAIN MENU )----------------------

--==<  B A L L I S T I C  v3.10  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE BALLISTIC COEFFICIENT -

Please input:

Initial velocity ....(fps)  2820

Final velocity ......(fps)  2507

Range .............(yards)  100

Altitude ............(Ft.)  0

Temperature ......(Deg. F)  59.0

Atmos. Pressure ..(In. Hg)  29.53

Effective Bal. Coeff. =  0.288

(Bal. Coeff. at STP = 0.288

-------( Fig 2. - Calculating the Ballistic Coefficient )-------

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 17

--==<  B A L L I S T I C  v3.10  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE BALLISTIC TABLE -

Please input:

Initial velocity ........(fps)  2820

Bullet weight ........(grains)  165

Effective Bal. Coeff. ........  0.288

Range .................(yards)  500

Number of intervals .(10 max.)  10

Chart title ?

.308 Winchester - 165 gr. Spire pt.

-----------------( Fig. 3 - MAIN MENU choice 1 )-----------------

.308 Winchester - 165 gr. Spire pt.

(Calculated using G1 table)

Bullet weight ......... 165 grains   IPSC Power Factor ..... 465.3

Cross wind ............ 10.0 m.p.h.  Altitude .............. 0    Ft.

Atmospheric pressure .. 29.53 in.    Temperature ........... 59.0 F

Effective Bal. Coeff... 0.288        Bal. Coeff. at STP .... 0.288

Range  Velocity  Energy   Momentum  Mx. Ht.  Defl.   Drop   Lead   Time

yards   f.p.s.   ft-lbs   lb.-secs    in.     in.     in.  in/mph  secs

0     2820    2913.2    2.0661     0.0     0.0     0.0    0.0   0.000

50     2661    2593.5    1.9494     0.1     0.3     0.6    1.0   0.055

100     2507    2302.4    1.8368     0.6     1.1     2.4    2.0   0.113

150     2358    2037.4    1.7278     1.5     2.6     5.5    3.1   0.175

200     2215    1797.1    1.6228     2.8     4.8    10.3    4.2   0.240

250     2076    1579.4    1.5213     4.7     7.8    16.8    5.5   0.310

300     1942    1382.2    1.4231     7.2    11.6    25.3    6.8   0.385

350     1814    1205.8    1.3292    10.5    16.3    36.2    8.2   0.465

400     1693    1050.3    1.2406    14.7    22.0    49.7    9.7   0.550

450     1578     912.5    1.1563    20.0    28.8    66.2   11.3   0.642

500     1470     791.2    1.0767    26.7    36.7    86.2   13.0   0.741

1=Print  2=Form Feed  3=Sight table  4=New data  5=Main menu  ? 3

------------( Fig. 4 - The completed ballistic table )-----------

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 18

--==<  B A L L I S T I C  v3.10  >==--

(C) Copyright 1988 by W.R. Frenchu

- ADJUST ATMOSPHERIC CONDITIONS -

(Modify ballistic coefficient)

Please input:

Bal. Coeff. at STP ........ 0.288

Altitude .............(Ft.) 5000

Temperature .......(Deg. F) 41.2

Atmos. Pressure ...(In. Hg) 25.44

Wind Speed ........(M.P.H.) 10.0

Angle from Broadside (deg.) 0

Effective Ballistic Coefficient = 0.323

--------( Fig. 5 - Modifying the ballistic coefficient )--------

--==<  B A L L I S T I C  v3.10  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE RECOIL -

Please input:

Initial velocity .....(fps) 2820

Weight of gun ........(lbs) 9.50

Weight of bullet ..(grains) 165

Weight of powder ..(grains) 42.0

Recoil Velocity =  9.5 fps

Recoil Energy   = 13.4 ft-lbs

(Recoil due to bullet =  7.2 ft-lbs or 54%)

-----------------( Fig. 6 - Recoil calculation )----------------

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 19

--==<  B A L L I S T I C  v3.10  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE POINT BLANK RANGE -

Please input:

Initial Velocity ............(fps)  2820

Sight Height .............(inches)  0.90

Max. Dist. from Sight Line ..(in.)  6.00

Effective Bal. Coeff. ............  0.288

Targeted range for +/-6.00 inch path is 287 yds.

Point-blank range is 337 yds.

(100 yd. trajectory = +5.0 in.)

-----------( Fig. 7 - Calculating Point Blank Range )-----------

--==<  B A L L I S T I C  v3.10  >==--

(C) Copyright 1988 by W.R. Frenchu

- CALCULATE TWIST -

Please input:

Bullet Diameter ......(in.)  0.308

Bullet Length ........(in.)  1.20

Greenhill Number ..........  150.0

Spec. Gravity of Bullet ...  10.90

Slowest twist allowing for stabilization is

1 turn in 11.9 inches.

------------------( Fig. 8 - Calculating twist )----------------

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 20

Appendix A

Glossary

BALLISTICS - The  study of moving projectiles. Commonly, what  is

meant is "exterior" ballistics, that portion of the bullet's

flight  between  muzzle and  target.  "Interior"  ballistics

refers  to  the  portion between primer  ignition  and  free

flight  and "terminal" ballistics to what happens  when  the

bullet strikes its target.

BALLISTIC COEFFICIENT - The  ratio of the sectional density of  a

projectile to its coefficient of form. A measure of how well

a  bullet  retains its velocity. Commonly  supplied  by  re-

loading manufacturers, or it may be derived from their  bal-

listic tables.

BULLET - The actual projectile in small arms ammunition. Commonly

corrupted to mean the entire cartridge.

BULLET WEIGHT - Weight of the actual projectile. Usually given in

grains.

CALIBER - Technically, the diameter of the barrel measured across

the lands. In practice, the diameter of the bullet.

CARTRIDGE - Fixed ammunition, including case, powder, primer, and

bullet, for small arms.

COEFFICIENT OF FORM - A number relating the ballistic  efficiency

of  a  given shape to the shape of the  projectile  used  to

calculate the ballistic table.

CROSS WIND ANGLE - The  angle  between the wind and line  of  de-

parture.

DRIFT - Technically,  the distance a bullet will travel  horizon-

tally  due to its spin. This effect is usually quite  small,

amounting to about 7 inches at 1000 yards for a military  M2

(30-06) cartridge. Often confused with wind deflection.

DEFLECTION - See wind deflection.

DROP - The  distance a bullet will fall due to the  influence  of

gravity.  Measured from the line of departure, not the  line

of sight.

ENERGY - Here,  the  kinetic energy of a moving  bullet,  usually

given  in ft-lbs. Equal to one half the mass of  the  bullet

multiplied by the square of the velocity.

F.P.S. - Feet per second. A unit of velocity.

FT.-LBS. - Foot  pounds.  A unit of energy equal  to  the  effort

required to raise one pound to a height of one foot.

GRAINS - A unit of weight equal to 1/7000 of a pound.

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 21

INGALLS' TABLES - A  set of ballistic tables first calculated  by

Col.  J. M. Ingalls in 1918. Probably the most  widely  used

tables  for small arms calculations. Other  tables  commonly

used include the British tables of 1909, 1929,  Winchester's

G tables and the Ordnance Department's J table.

I.H.M.S.A. - International  Handgun  Metallic  Silhouette  Assoc-

iation.

I.P.S.C - International Practical Shooting Confederation.

LEAD - How far ahead of a moving target a shooter must aim to  be

assured  of hitting it. In BALLISTIC, the lead is  given  in

inches/m.p.h. of target speed. If the entry in the table  is

"1.2",  the lead for a target moving at 5  m.p.h.,  (perpen-

dicular to the bullet's path) would be (5 * 1.2) 6.0 inches.

Wind speed is NOT taken into account in this calculation.

LINE OF DEPARTURE - An  imaginary  line formed by  extending  the

muzzle  of the weapon. It coincides with the  bullet's  path

only while the bullet is actually in the barrel.

LINE OF SIGHT - An  imaginary line from the center of the  sights

to the point of aim. The bullet typically crosses this  line

only twice; once, a few yards from the muzzle, and again  at

the targeted distance.

MAXIMUM HEIGHT - The highest vertical distance above the line  of

sight attained by a bullet. Sometimes confused with the mid-

range  trajectory due to the fact that it  typically  occurs

about  halfway between muzzle and target. Maximum height  is

usually somewhat higher and later than the MRT.

M.O.A. - Minute of Angle. Equal to 1/60 of an angular degree.  At

100 yards, 1 M.O.A. is approximately 1.047 inches.

MOMENTUM - Equal to the mass of a bullet multiplied by its  velo-

city.  Indicative  of the striking or knockdown power  of  a

projectile.

N.R.A - National  Rifle  Association. If you  shoot,  you  SHOULD

belong to this organization.

POINT-BLANK  RANGE - That  range  for  which  the  bullet's  path

doesn't  vary  from the line of sight by more than  a  given

amount.

POWER FACTOR - A  number  used in IPSC competition  to  determine

whether  a handgun cartridge may be scored as a  "major"  or

"minor"  caliber. Defined as the bullet weight  (in  grains)

divided  by 1000 and multiplied by the velocity.  Cartridges

with  a power factor equal to or greater than 175  are  con-

sidered "major". Cartridges ranking between 125 and 175  are

"minor"  and those below 125 are not legal for  IPSC  compe-

tition.

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 22

S.A.A.M.I - Sporting Arms Ammunition Manufacturers Institute.

SECTIONAL DENSITY - The  weight of a projectile (in  pounds)  di-

vided by the square of its diameter (in inches).

SIGHT HEIGHT - The  measured distance between the centerlines  of

the sights and muzzle of a gun.

SPECIFIC GRAVITY - The ratio of the mass of a material to that of

an equal volume of water.

TARGETED RANGE - That  distance  where  the path  of  the  bullet

crosses the line of sight. (The "sighting in" distance.)

TIME OF FLIGHT - The  time necessary for a bullet to travel  from

the muzzle of a gun to a given range.

TRAJECTORY - The path of a moving projectile. Here, the number of

inches above or below the line of sight.

TWIST - Here,  the rate of twist in the rifling of a gun  barrel.

The minimum amount of twist necessary to stabilize a  bullet

in  flight can be determined from its length,  density,  and

caliber. Usually expressed as "1 turn in xx inches" where xx

may range from about 8 inches up to over 30. "Faster"  twist

translates  to  a  smaller  number.  Minimum  twist  is  the

"slowest" twist that will stabilize a given bullet.

WIND DEFLECTION - The amount of horizontal motion attributable to

the action of the wind. Often mistakenly called drift.

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 23

Appendix B

Information Sources

BALLISTICS:

Any intermediate physics (mechanics) text.

BALLISTICS ON THE HOME COMPUTER. American Rifleman, June

1983

BALLISTICS ON YOUR POCKET CALCULATOR. American Rifleman,

June 1987

BALLISTIC TABLES AND HOW TO USE THEM. American Rifleman,

December 1963

CALCULATING RECOIL. American Rifleman, March 1988

EXPLORING BALLISTICS WITH YOUR COMPUTER, Byte Magazine

September 1980

EXTERIOR BALLISTICS, McShane, Kelly and Reno, University of

Denver Press, 1953

HATCHER'S NOTEBOOK, Julian S. Hatcher, Harrisburg: Stackpole

Books, 1962

TRAJECTORY COMPUTATION SYSTEM FOR DIGITAL COMPUTER, SAAMI

publications, 1976

RELOADING:

CARTRIDGES OF THE WORLD, J.T. Amber, Northfield: Digest

Books, 1972

COMPLETE GUIDE TO HANDLOADING, Philip B. Sharpe, New York:

Funk & Wagnells Co., 1953

HORNADY HANDBOOK OF CARTRIDGE RELOADING, RIFLE-PISTOL, Grand

Island: Hornady Manufacturing Co., 1973

HANDBOOK FOR SHOOTERS AND RELOADERS, VOL I & II, P.O. Ackley,

Salt Lake City: Publisher's Press, 1965

LYMAN RELOADING HANDBOOK #46, Middlefield: Lyman Products

for Shooters, 1970

NRA HANDLOADER'S GUIDE, Wash. D.C.: The National Rifle

Association of America, 1969

PRINCIPLES AND PRACTICE OF HANDLOADING, Georgetown: Small

Arms Technical Publishing Co., 1954

SIERRA BULLETS RELOADING MANUAL, Santa Fe Springs: Sierra

Bullets, 1971

SPEER RELOADING MANUAL #10, Lewiston: Speer, Inc., 1979

ASSOCIATIONS:

THE NATIONAL RIFLE ASSOCIATION - 1600 Rhode Island Ave. NW,

Washington, D.C. 20036

S. A. A. M. I. (SPORTING ARMS AMMUNITION MANUFACTURERS

INSTITUTE) - PO Box 838, Branford, Conn. 06405

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 24

Appendix C

Ballistic Coefficients for Factory Bullets

The  coefficients  listed here were calculated  from  tables

found  in  SHOOTER'S BIBLE No. 75 (1984) using  the  program  and

Ingalls'  function.  No attempt was made to include  every  load,

instead, representative loads for each caliber were chosen.

Part 1 - Centerfire Pistol and Revolver

Cartridge      Bullet       Coefficient    Manufacturer

================================================================

.25 ACP        50 FMC          .174            F

.25 ACP        50 FMC          .097            R,W

.380 ACP       85 STHP         .105            W

.380 ACP       90 JHP          .073            F

.380 ACP       95 FMC          .082            F,R,W

9 mm          95 JHP          .101            F,R,W

9 mm         115 FMC          .128            W

9 mm         115 JHP          .131            R,F

9 mm         115 STHP         .131            W

9 mm         123 FMC          .140            F

.38 SPEC.     110 STHP         .147            W

.38 SPEC.     148 WC           .062            F,R,W

.38 SPEC.     158 LRN          .162            F,R,W

.357 MAG.     110 JHP          .093            F,R,W

.357 MAG.     125 JHP          .122            F,R,W

.357 MAG.     158 JSP          .135            F,R,W

.44 MAG.      180 JHP          .120            F,R

.44 MAG.      210 STHP         .125            W

.44 MAG.      220 MCP          .193            F

.44 MAG.      240 JHP          .156            R

.44 MAG.      240 LGC          .139            R

.45 ACP       185 MCWC         .082            R,W

.45 ACP       185 JHP          .152            F

.45 ACP       185 STHP         .137            W

.45 ACP       230 FMC          .165            R,W

.45 COLT      225 STHP         .167            W

.45 COLT      225 SWCHP        .172            F

.45 COLT      250 LRN          .157            R

---------------

ABBREVIATIONS

Manufacturers: F=Federal, R=Remington, W=Winchester

Bullets:  FMC=Full  Metal  Case, STHP=Silver  Tip  Hollow  Point,

JHP=Jacketed  Hollow Point, MCWC=Metal Case Wadcutter,  WC=Wad-

cutter,  LRN=Lead Round Nose, LGC=Lead Gas Check,  JSP=Jacketed

Soft Point, MCP=Metal Case Profile, SWCHP=Semiwadcutter  Hollow

Point

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 25

Part 2 - Centerfire Rifle

Cartridge      Bullet       Coefficient    Manufacturer

=================================================================

.22-250 REM.     55 PSP          .231            F,W

.22-250 REM.     55 PSP          .253            R

.223 REM.        55 FMC          .207            R

.223 REM.        55 FMC          .278            W

.223 REM.        55 MCBT         .350            F

.243 WIN.        80 SP           .259            F,R,W

.243 WIN.       100 PSP          .371            F,R,W

.270 WIN.       130 PSP          .378            F,W

.270 WIN.       130 PSP          .345            R

.270 WIN.       150 SP           .267            F,R

.270 WIN.       150 PSP          .356            W

7mm MAUSER     140 PSP          .433            F,R

7mm MAUSER     175 SP           .280            F,W

7mm REM.MAG.   150 PSP          .354            F,R,W

7mm REM.MAG.   175 PSP          .453            F,R,W

.30 CARBINE     110 FMC          .180            F,W

.30 CARBINE     110 FMC          .164            R

30-30 WIN.     150 SP           .221            F,W

30-30 WIN.     150 SP           .195            R

30-30 WIN.     170 SP           .259            F,R,W

30-06          150 PSP          .329            F,R,W

30-06          180 PSP          .398            F,R,W

30-06          200 BTSP         .592            F

30-06          220 SP           .299            R,W

.300 WIN.MAG.   180 PSP          .462            F,R,W

.308 WIN.       150 PSP          .323            F,R,W

.308 WIN.       180 PSP          .390            F,R,W

.35 REM         200 SP           .193            F,R,W

.45-70          300 JHP          .302            F,W

.45-70          405 SP           .269            R

---------------

ABBREVIATIONS

Manufacturers: F=Federal, R=Remington, W=Winchester

Bullets:  FMC=Full  Metal Case, PSP=Pointed Soft  Point,  SP=Soft

Point, BTSP=Boat Tail Soft Point, MCBT=Metal Case Boat Tail

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 26

Part 3 - Rimfire

Cartridge              Bullet   Coefficient  Manufacturer

==================================================================

.22 LR "VIPER"           36 TC       .110           R

.22 LR "YELLOW JACKET"   33 TCHP     .101           R

.22 LR HV                40 LRN      .128           R

.22 LR HV                36 LHP      .115           R

.22 LR TARGET            40 LRN      .136           R

.22 LONG                 29 LRN      .095           R

.22 SHORT                29 LRN      .098           R

.22 SHORT                27 LHP      .091           R

.22 SHORT TARGET         29 LRN      .097           R

.22 WIN. MAG.            40 JHP      .108           W

.22 WIN. MAG.            40 FMC      .108           W

.22 LR "XPEDITER"        29 LHP      .079           W

.22 LR "H.P."            37 LRN      .117           W

.22 LR "DYNAPOINT"       40 LDP      .127           W

.22 LR "T22"             40 LRN      .135           W

.22 LR                   40 LRN      .127           W

.22 LONG                 29 LRN      .095           W

.22 SHORT "H.P."         27 LRN      .091           W

.22 SHORT "T22"          29 LRN      .097           W

.22 SHORT                29 LRN      .098           W

.22 LR "HI POWER"        40 LRN      .126           F

.22 LR "HI POWER"        38 LHP      .122           F

.22 LR "CHAMPION"        40 LRN      .135           F

.22 LONG "HI POWER"      29 LRN      .094           F

.22 SHORT                29 LRN      .100           F

.22 SHORT                29 LHP      .092           F

---------------

ABBREVIATIONS

Manufacturers: F=Federal, R=Remington, W=Winchester

Bullets:  LRN=Lead  Round Nose, LHP=Lead Hollow  Point,  LDP=Lead

Dynapoint, FMC=Full Metal Case, TC=Truncated Cone, TCHP=  Trun-

cated Cone Hollow Point

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 27

Appendix D

Specific Gravities of Bullet Alloys

Part 1 - Lead/Tin/Antimony Alloys

Percent Composition      Specific Gravity      Trade or

Pb      Sn       Sb         of alloy          Common Name

=================================================================

100                           11.340        pure lead

100                    7.298        pure tin

100           6.62         pure antimony

30      70                    8.32         soft solder

37      63                    8.42         eutectic solder

50      50                    8.89         50-50 solder

75      10       15           9.73         lead babbit-SAE 14

80      20                   10.20         20-80 solder

80       5       15          10.04         alloy 8

83       2       15          10.09         alloy 10

85               15          10.28         alloy 11

85       5       10          10.24         lead babbit-SAE 13

91                9          10.66         9% antimonial lead

92                8          10.74         8% antimonial lead

94                6          10.88         hard lead (bullets)

95       5                   11.00         5-95 solder

96                4          11.04         hard lead

99                1          11.27         1% antimonial lead

92        8           7.28         white metal

Part 2 -  Other Alloys

Composition         Specific Gravity      Trade or

of alloy          Common Name

=================================================================

100 Cu                         8.96         pure copper

100 Fe                         7.87         pure iron

95 Cu, 5 Zn                   8.86         gilding

90 Cu, 10 Zn                  8.80         commercial bronze

70 Cu, 30 Zn                  8.53         cartridge brass

98.75 Cu, 1.25 Sn             8.89         1.25% phos. bronze

90 Cu, 10 Sn                  8.78         10% phosphor bronze

97 Cu, 3 Si                   8.53         silicon bronze (A)

98.5 Cu, 1.5 Si               8.75         silicon bronze (B)

99.5 Fe,.06 C,.38 Mn,.01 Si   7.87         .06% carbon steel

99.0 Fe,.23 C,.64 Mn,.11 Si   7.86         .23% carbon steel

98.7 Fe,.44 C,.69 Mn,.20 Si   7.84         .435% carbon steel

---------------

SOURCES:

Metals Handbook, Vol. 1, "Properties and Selection of Metals"

(8th ed.), American Society for Metals

Metal and Alloys Data Book, Samuel L. Hoyt

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 28

Appendix E

User Supported Software

User  Supported Software is based on the premise that  if  a

someone  gets a chance to try out a program, likes it, and  finds

he uses it on a regular basis, he will voluntarily contribute  to

the  support of that program. (Where else do you get  the  oppor-

tunity  to check out a product BEFORE you buy it?) Users are  en-

couraged to copy and distribute User Supported Software for  non-

commercial  purposes. Commercial users should be aware  that  the

author of the program often retains commercial rights.

BALLISTIC  is being distributed as User Supported  Software.

If after an initial trial period you find you're using  BALLISTIC

and  you like it, please send the registration form,  check  (and

diskette if indicated) to:

William R. Frenchu

79 Taylor Terrace

Hopewell, NJ  08525

(Please label the diskette with your name and address.)

Contributors  will receive a copy of the latest revision  of

the  program,  as well as the TURBO C source code.  In  addition,

they  will be informed by mail of any major updates  or  improve-

ments. Major revisions will probably not be undertaken without  a

positive  user response. If you have any questions,  problems  or

suggestions  about  the program, (or need support for  a  special

printer, etc.) please contact the author.

All  commercial  rights  to BALLISTIC are  reserved  by  the

author. Anyone wishing to make any commercial use of this program

(or the tables it produces) is urged to contact the author at the

above  address. This includes people who use the program  in  any

commercial  environment. Site and commercial licenses are  avail-

able at very reasonable rates.

Non-commercial  users are encouraged to make and  distribute

as  many UNMODIFIED copies as they like, provided  the  following

files are included:

BAL-GEN.EXE

BAL-READ.EXE

BAL-READ.ME!

BALISTIC.EXE

BALISTIC.DOC

BALISTIC.REV

Comments  or  suggestions may be left for the  author  (Bill

Frenchu) at any of the following places:

Bullet 'n Board BBS    (703)-971-4491

Road To Utopia BBS     (609)-799-5418 (CoSysop)

CompuServe                   74575,61

BALLISTIC v3.10 - (C) Copyright 1988  W.R. Frenchu         Page 29

REGISTRATION FORM FOR BALLISTIC v3.10

Name _____________________________________   Date _______________

Mailing Address:

Street __________________________________   Phone ______________

City  ________________________  State ___   Zip ________________

Computer type (& BIOS)________________________ Printer __________

Display and adapter type ________________________________________

Where did you get your copy of BALLISTIC? _______________________

Please register my copy of BALLISTIC and send the latest version

(as well as the TURBO-C source code) to the above address. (If

the version being registered IS the latest, the next upgrade will

be sent at half price.) Check:

_____ I've enclosed $15.00 and a 5 1/4" DSDD (360k) or 3.5"

(720k) diskette. (No 1.2 or 1.44 Meg diskettes please!)

_____ I've enclosed $20.00. Please send  5 1/4"  3.5" (circle)

The author is always trying to improve BALLISTIC. You can help!

Comments about BALLISTIC v3.10:

Comments about the documentation:

Suggestions for future versions of BALLISTIC:

Types of shooting you do:

Mail to: William Frenchu, 79 Taylor Terrace, Hopewell, NJ  08525