HP LASERJET/LASERJET PLUS/SERIES II SETUP STRINGS

=================================================

CONTROL CODES AND                 |

ESCAPE SEQUENCES                  |  DEFINITION

----------------------------------|-----------------------------------------

BS  (ASCII  8/HEX 08)             |  BACKSPACE

CR  (ASCII 13/HEX 0D)             |  CARRIAGE RETURN

ESC (ASCII 27/HEX 1B)             |  ESCAPE

FF  (ASCII 12/HEX 0C)             |  FORM FEED

LF  (ASCII 10/HEX 0A)             |  LINE FEED

SI  (ASCII 15/HEX 0F)             |  SHIFT IN

SO  (ASCII 14/HEX 0E)             |  SHIFT OUT

|

<ESC>&F#S                         |  PUSH/POP POSITION

<ESC>=                            |  HALF LINE FEED

<ESC>&10O                         |  PORTRAIT ORIENTATION

|

<ESC>&a#C                         |  HORIZONTAL CURSOR POSITIONING (COLUMNS)

<ESC>&a#H                         |  HORIZONTAL CURSOR POSITIONING (DECIPOINTS)

<ESC>&a#L                         |  LEFT MARGIN

<ESC>&a#M                         |  RIGHT MARGIN

<ESC>&a#R                         |  VERTICAL CURSOR POSITIONING (LINES)

<ESC>&a#V                         |  VERTICAL CURSOR POSITIONING (DECIPOINTS)

|

<ESC>&d@                          |  DISABLE AUTO-UNDERLINING

<ESC>&dD                          |  ENABLE AUTO-UNDERLINING

|

<ESC>&f#S                         |  PUSH/POP POSITION

<ESC>&f#X                         |  MACRO CONTROL

<ESC>&f#Y                         |  MACRO ID

|

<ESC>&k#G                         |  LINE TERMINATION

<ESC>&k#H                         |  HORIZONTAL MOTION INDEX (HMI)

<ESC>&k#S                         |  FONT PITCH (PRIMARY AND SECONDARY)

|

<ESC>&l#C                         |  VERTICAL SPACING (VMI)

<ESC>&l#D                         |  VERTICAL SPACING (LPI)

<ESC>&l#E                         |  TOP MARGIN

<ESC>&l#F                         |  TEXT LENGTH

<ESC>&l#H                         |  PAPER INPUT CONTROL

<ESC>&l#P                         |  PAGE LENGTH

<ESC>&l#X                         |  SELECT NUMBER OF COPIES

|

<ESC>&l0L                         |  DISABLE PERFORATION SKIP MODE

<ESC>&l1L                         |  ENABLE PERFORATION SKIP MODE

<ESC>&l1O                         |  LANDSCAPE ORIENTATION

|

<ESC>&p#X[transparent print data] |  TRANSPARENT PRINT DATA

|

<ESC>&s0C                         |  ENABLE WRAP-AROUND MODE

<ESC>&s1C                         |  DISABLE WRAP-AROUND MODE

|

<ESC>(#@                          |  PRIMARY FONT DEFAULT

<ESC>(#X                          |  DESIGNATE DOWNLOADED FONT AS PRIMARY

|

<ESC>(0A                          |  MATH SYMBOL SET (PRIMARY)

<ESC>(0B                          |  LINE DRAW SYMBOL SET (PRIMARY)

<ESC>(0U                          |  USASCII SYMBOL SET (PRIMARY)

<ESC>(8K                          |  KANA-8 SYMBOL SET (PRIMARY)

<ESC>(8U                          |  ROMAN-8 SYMBOL SET (PRIMARY)

<ESC>(9U                          |  ANSI SYMBOL SET (PRIMARY)

|

<ESC>(s#B                         |  STROKE WEIGHT (PRIMARY)

<ESC>(s#H                         |  FONT PITCH (PRIMARY)

<ESC>(s#T                         |  TYPEFACE (PRIMARY)

<ESC>(s#V                         |  POINT SIZE (PRIMARY)

<ESC>(s#W[descriptor and data]    |  DOWNLOAD CHARACTER

|

<ESC>(s0P                         |  FIXED SPACING (PRIMARY)

<ESC>(s0S                         |  UPRIGHT CHARACTER STYLE (PRIMARY)

<ESC>(s1P                         |  PROPORTIONAL SPACING (PRIMARY)

<ESC>(s1S                         |  ITALIC CHARACTER STYLE (PRIMARY)

|

<ESC>)#@                          |  SECONDARY FONT DEFAULT

<ESC>)#X                          |  DESIGNATE DOWNLOADED FONT AS SECONDARY

|

<ESC>)0A                          |  MATH SYMBOL SET (SECONDARY)

<ESC>)0B                          |  LINE DRAW SYMBOL SET (SECONDARY)

<ESC>)0U                          |  USASCII SYMBOL SET (SECONDARY)

<ESC>)8K                          |  KANA-8 SYMBOL SET (SECONDARY)

<ESC>)8U                          |  ROMAN-8 SYMBOL SET (SECONDARY)

<ESC>)9U                          |  ANSI SYMBOL SET (SECONDARY)

|

<ESC>)s#B                         |  STROKE WEIGHT (SECONDARY)

<ESC>)s#H                         |  FONT PITCH (SECONDARY)

<ESC>)s#T                         |  TYPEFACE (SECONDARY)

<ESC>)s#V                         |  POINT SIZE (SECONDARY)

<ESC>)s#W[font descriptor]        |  CREATE FONT (FONT DESCRIPTOR)

|

<ESC>)s0P                         |  FIXED SPACING (SECONDARY)

<ESC>)s0S                         |  UPRIGHT CHARACTER STYLE (SECONDARY)

<ESC>)s1P                         |  PROPORTIONAL SPACING (SECONDARY)

<ESC>)s1S                         |  ITALIC CHARACTER STYLE (SECONDARY)

|

<ESC>*b#W[rater data]             |  TRANSFER RASTER GRAPHICS

|

<ESC>*c#A                         |  HORIZONTAL RULE/PATTERN SIZE

<ESC>*c#B                         |  VERTICAL RULE/PATTERN SIZE (DOTS)

<ESC>*c#D                         |  SPECIFY FONT ID

<ESC>*c#E                         |  SPECIFY CHARACTER CODE

<ESC>*c#F                         |  FONT AND CHARACTER CONTROL

<ESC>*c#G                         |  PATTERN ID

<ESC>*c#H                         |  HORIZONTAL RULE/PATTERN SIZE (DECIPOINTS)

<ESC>*c#P                         |  PRINT RULE/PATTERN

<ESC>*c#V                         |  VERTICAL RULE/PATTERN SIZE (DECIPOINTS)

|

<ESC>*p#X                         |  HORIZONTAL CURSOR MOVES (DOTS)

<ESC>*p#Y                         |  VERTICAL CURSOR MOVES (DOTS)

|

<ESC>*r#A                         |  START RASTER GRAPHICS

<ESC>*rB                          |  END RASTER GRAPHICS

<ESC>*t#R                         |  RASTER GRAPHICS RESOLUTION

|

<ESC>9                            |  CLEAR SIZE MARGINS

<ESC>Y                            |  ENABLE DISPLAY FUNCTIONS MODE

<ESC>Z                            |  DISABLE DISPLAY FUNCTIONS MODE

|

----------------------------------|-----------------------------------------

Z

NOTE:

1.   <ESC> is ASCII 27 / HEX 1B

2.   ANYTHING IN BRACKET [ ] MUST BE SUPPLIED

SOURCE:  HP LASERJET TECHNICAL REFERENCE MANUAL

PROVIDED FOR YOUR REFERENCE BY:

S. NOH (SYSOP - FORM FEED LASERJET BBS)

(201) 869-1327

11/15/87

#########################################################################