USING THE LASERJET/THINKJET PRINT SCREEN UTILITY

FOR THE IBM PC OR IBM PC/XT

The PRINT SCREEN utility allows you to print the contents of your

IBM PC or PC/XT display screen using the LaserJet or ThinkJet printers.

The PRINT SCREEN utility also operates with the HERCULES high-

resolution graphics board.

These instructions explain how to install and use this

utility.  Before performing the instructions, be sure to make a copy

of your current DOS disc so that you will have a backup available for

printing with other printers.  (When your operating system disc is

modified for use with the PRINT SCREEN program, it may not operate

with other printers.)  Notice that there are two sets of installation

instructions--one for LaserJet printers and one for ThinkJet printers.

If you have any problems, there is an "IN CASE OF DIFFICULTY" section

at the end of these instructions.

Now proceed to the installation instructions for your printer.

*************************

**LASERJET INSTALLATION**

*************************

The LaserJet printer is configured at the factory to communicate

at 9600 baud.  If you wish to increase the speed of

the PRINT SCREEN utility, you can set the printer's baud rate to

19200 by following steps a. through g. below.  If you wish to leave the

baud rate at 9600, skip to step h.

a.  Switch the LaserJet power switch to the OFF (0) position and

remove the printer power cord from the wall socket.

____________________________________________________

WARNING

Removing the back panel of the LaserJet printer can

expose hazardous voltages--be sure the printer power

switch is OFF and the power cord unplugged before

removing the back panel.

____________________________________________________

b.  Remove the back panel from the LaserJet by removing the 4

screws.

c.  Remove the vertical support at the center of the cabinet (1

screw).

d.  Set the switches on switch bank #1 (SW1) to the following

settings:

1     2     3     4     5     6     7     8

OFF   ON    ON    OFF   OFF   OFF   OFF   ON

e.  Replace the vertical support and the back panel.

f.  Plug in the printer power cord and set the printer power

switch to the ON (1) position.

g.  Reboot the IBM PC or PC/XT by pressing the "Ctrl", "Alt", and "Del" keys

simultaneously.  (If you have an IBM PC, your operating system (DOS) disc

must be installed in drive A.)  NOTE:  If an error number 40 appears

on the LaserJet display, press the CONT. key on the LaserJet Operator

Control Panel--this is caused by booting up the computer while the

printer is on-line.

h.  Copy the LASERJET.EXE file from the PRINT SCREEN disc to the operating

system (DOS) disc.  To do this, follow the procedure below for your computer.

(NOTE:  If you are using a HERCULES high-resolution graphics board,

substitute HERCULES.EXE for LASERJET.EXE in this step.)

For the IBM PC:

1.  With your system (DOS) disc in drive A, load your PRINT SCREEN

disc in drive B.

2.  From the A> prompt, type:   COPY B:LASERJET.EXE A:   then Enter.

For the IBM PC/XT:

1.  Place your PRINT SCREEN disc in drive A.

2.  From the C> prompt, type:   COPY A:LASERJET.EXE C:   then Enter.

i.  Next, you need to create or add some lines to your AUTOEXEC.BAT file so that

every time you boot up your computer, the PRINT SCREEN program will

automatically load.  To see if you already have an AUTOEXEC.BAT file

on your system disc, type DIR and then Enter from the A> prompt (or

from the C> prompt if you have an IBM PC/XT).  When you press the Enter

key, a file directory listing will be printed on your display screen.

Check the listing for an AUTOEXEC.BAT file.  If you do not see a file

with this title, proceed to step j.  If you already have an AUTOEXEC.BAT

file, do the following:

1.  For the IBM PC:

From the A> prompt, type:   EDLIN AUTOEXEC.BAT    then Enter.

The screen will read "end of input file" and the * prompt will appear.

For the IBM PC/XT:

From the C> prompt, type    EDLIN AUTOEXEC.BAT    then Enter.

The screen will read "end of input file" and the * prompt will appear.

2.  For both the IBM PC and PC/XT, type L and then press Enter from the

* prompt.  The contents of the AUTOEXEC.BAT file will be listed on

your screen and the * prompt will reappear.

3.  Notice that each line in the AUTOEXEC.BAT file has a line number

to the left of it.  Add "1" to the largest line number in your

AUTOEXEC.BAT file and type the resulting number, followed by an I,

and then press the Enter key.  This step allows you to add information

to the end of your AUTOEXEC.BAT file (see step 4).

For example, if the largest line number in your AUTOEXEC.BAT file

is 4, you would type 5I and then press the Enter key.  This would

allow you to add information on line number 5.

4.  Add the following lines to the AUTOEXEC.BAT file by typing

the following (choose the correct baud rate--9600 is standard).

(NOTE:  If you have a HERCULES high-resolution graphics board installed

in your IBM PC or PC/XT, replace LASERJET A (or B) in the lines below

with HERCULES A or HERCULES B (depending on the desired baud rate).

For 9600 Baud

MODE COM1:9600,N,8,1,P                    then Enter

MODE LPT1:=COM1                           then Enter

LASERJET A                                then Enter

^Z   (Hold the "Ctrl" key and press "Z")  then Enter

(The * prompt will appear)

For 19200 Baud

MODE COM1:9600,N,8,1,P                    then Enter

MODE LPT1:=COM1                           then Enter

LASERJET B                                then Enter

^Z   (Hold the Ctrl key down and press Z) then Enter

(The * prompt will appear)

___________________________________________________________

NOTE

If there are any commands in the AUTOEXEC.BAT file for

other screen dump programs (such as IBM "GRAPHICS"),

remove them from the file.

Some application programs may automatically load "GRAPHICS"

or a similar program.  Such applications are not compatible

with this PRINT SCREEN utility without modification to

prevent them from loading "GRAPHICS".

To delete a line from the AUTOEXEC.BAT file, type the line

number you want deleted, followed by the letter d, and then

press Enter.  For example, if you list your file and see that

line number 3 loads a GRAPHICS program (which is

incompatible with the PRINT SCREEN utility), you may delete

that line by typing 3D and then pressing Enter.

To return to the A> prompt, type END and then press Enter.

___________________________________________________________

5.  Type END and press the Enter key to return to the A> prompt.

j.  If you do not already have an AUTOEXEC.BAT file on your

system disc, type the following lines from the A> prompt (for IBM

PC) or C> prompt (for IBM PC/XT).  This will create an AUTOEXEC.BAT

file for you.  (If you have a HERCULES high resolution graphics board,

type HERCULES A or HERCULES B in place of the LASERJET A (or B)

below.)

For 9600 Baud

COPY CON: AUTOEXEC.BAT      then Enter  (no prompt will appear)

MODE COM1:9600,N,8,1,P      then Enter

MODE LPT1:=COM1             then Enter

LASERJET A                  then Enter

^Z    (Hold the Ctrl key down and press Z) then Enter

For 19200 Baud

COPY CON: AUTOEXEC.BAT      then Enter  (no prompt will appear)

MODE COM1:9600,N,8,1,P      then Enter

MODE LPT1:=COM1             then Enter

LASERJET B                  then Enter

^Z    (Hold the Ctrl key down and press Z) then Enter

k.  Reboot the system by pressing the "Ctrl", "Alt", and "Del" keys

simultaneously.

*************************

**THINKJET INSTALLATION**

*************************

Perform the following steps to install the PRINT SCREEN utility:

a.  Switch the ThinkJet power switch to the OFF (0) position.

b.  Set the switches in the back of the ThinkJet printer as

follows:

SWITCH 1                           SWITCH 2

(Parallel & Serial Printers)            (Serial Printers Only)

1    2    3    4    5    6    7    8      1    2    3    4    5

DOWN DOWN DOWN DOWN DOWN  UP   UP  DOWN    UP  DOWN DOWN DOWN DOWN

c.  Switch the ThinkJet power switch to the ON (1) position.

d.  Reboot the IBM PC by pressing the "Ctrl", "Alt", and "Del" keys

simultaneously.  (If you have an IBM PC, your operating system disc

must be installed in drive A.)

e.  Copy the THINKJET.EXE file from the PRINT SCREEN disc to the operating

system (DOS) disc.  To do this, follow the procedure below:

(NOTE:  If you are using a HERCULES high-resolution graphics board,

substitute HERCULES.EXE for THINKJET.EXE in this step.)

FOR THE IBM PC:

1.  With your system (DOS) disc in drive A, load your PRINT SCREEN

disc in drive B.

2.  From the A> prompt, type:   COPY B:THINKJET.EXE A:   then Enter.

FOR THE IBM PC/XT:

1.  Place your PRINT SCREEN disc in drive A.

2.  From the C> prompt, type:   COPY A:THINKJET.EXE C:   then Enter.

f.  Next, you need to add some lines to your AUTOEXEC.BAT file so that

every time you boot up your computer, the PRINT SCREEN program will

automatically load.  To see if you already have an AUTOEXEC.BAT file

on your system disc, type DIR from the A> prompt (or from the C> prompt

if you have an IBM PC/XT).  When you press the Enter key, a file directory

listing will be printed on your display screen.  Check the listing for

an AUTOEXEC.BAT file.  If you do not see a file with this title, proceed

step g.  If you already have an AUTOEXEC.BAT file, do the following:

1.  For the IBM PC:

From the A> prompt, type:   EDLIN AUTOEXEC.BAT    then Enter.

The screen will read "end of input file" and the * prompt will appear.

For the IBM PC/XT:

From the C> prompt, type    EDLIN AUTOEXEC.BAT    then Enter.

The screen will read "end of input file" and the * prompt will appear.

2.  For both the IBM PC and PC/XT, type L and then press Enter from the

* prompt.  The contents of the AUTOEXEC.BAT file will be listed on

your screen and the * prompt will reappear.

3.  Notice that each line in the AUTOEXEC.BAT file has a line number

to the left of it.  Add "1" to the largest line number in your

AUTOEXEC.BAT file and type the resulting number, followed by an I,

and then press the Enter key.  This step allows you to add information

to the end of your AUTOEXEC.BAT file (see step 4).

For example, if the largest line number in your AUTOEXEC.BAT file

is 4, you would type 5I and then press the Enter key.  This would

allow you to add information on line number 5.

4.  Add the following lines to the AUTOEXEC.BAT file.  (If your ThinkJet

printer has a parallel interface, skip the first two lines [start

with the THINKJET line].)  NOTE:  If you have a HERCULES high-resolution

graphics board installed in your IBM PC or PC/XT, type HERCULES A

in place of THINKJET in the lines below.

MODE COM1:9600,N,8,1,P    then Enter  (serial interface only)

MODE LPT1:=COM1           then Enter  (serial interface only)

THINKJET                  then Enter

^Z   (Hold the "Ctrl" key and press "Z")  then Enter

(The * prompt will appear)

___________________________________________________________

NOTE

If there are any commands in the AUTOEXEC.BAT file for

other screen dump programs (such as IBM "GRAPHICS"),

remove them from the file.

Some application programs may automatically load "GRAPHICS"

or a similar program.  Such applications are not compatible

with this PRINT SCREEN utility without modification to

prevent them from loading "GRAPHICS".

To delete a line from the AUTOEXEC.BAT file, type the line

number you want deleted, followed by the letter d, and then

press Enter.  For example, if you list your file and see that

line number 3 loads a GRAPHICS program (which is

incompatible with the PRINT SCREEN utility), you may delete

that line by typing 3D and then pressing Enter.

To return to the A> prompt, type END and then press Enter.

___________________________________________________________

5.  Type END and press the Enter key to return to the A> prompt.

g.  If you do not already have an AUTOEXEC.BAT file on your

system disc, type the following lines from the A> prompt (for IBM

PC) or C> prompt (for IBM PC/XT).  This will create an AUTOEXEC.BAT

file for you.  (If your ThinkJet printer has a parallel interface,skip

the two lines beginning with MODE.  (If you have a HERCULES high resolution

graphics board, type HERCULES A in place of THINKJET below.)

COPY CON: AUTOEXEC.BAT      then Enter  (no prompt will appear)

MODE COM1:9600,N,8,1,P      then Enter  (Serial interface only)

MODE LPT1:=COM1             then Enter  (Serial interface only)

THINKJET                    then Enter

^Z  (Hold the Ctrl key down and press Z)  then Enter

h.  Reboot your IBM system by pressing the "Ctrl", "Alt", and "Del" keys

simultaneously.

USING THE PRINT SCREEN UTILITY

Once you have performed the installation procedure, the PRINT

SCREEN utility will be loaded every time you boot your computer.

To print the contents of the IBM PC or PC/XT display, read and then

perform the following two-step procedure:

a.  Press and hold the "SHIFT" key (key with upward-pointing arrow)

and then press the "PrtSc" key.  After a 5-second delay, your display

screen will be dumped to the printer.  Note that if the display is

in Alpha mode, an "A" will flash in the upper left corner of the

screen.  If the screen is in Graphics mode, a short horizontal line

will appear in the upper left corner of the screen.

b.  You may select any of the options from the list below by pressing

the key of the desired option within 5 seconds of pressing the "SHIFT"

and "PrtSc" keys.  One option from each group (orientation, print

positioning, or page ejection) may be selected.  Printing will begin

5 seconds after the last key is pressed.  The Enter key may be

pressed to terminate the 5-second delay.  NOTE:  If you select any

of the options listed below, you do not have to select one from each

group--those that are not specified will default automatically.

KEY              OPTION

ORIENTATION            * P -- PORTRAIT ORIENTATION  (LaserJet only)

L -- LANDSCAPE ORIENTATION (LaserJet only)

PRINT POSITIONING      * T -- TOP OF PAGE (PORTRAIT MODE ONLY)

M -- MIDDLE OF PAGE (PORTRAIT MODE ONLY)

B -- BOTTOM OF PAGE (PORTRAIT MODE ONLY)

PAPER EJECTION         * Y -- AUTO FORM FEED AT END OF PAGE

N -- NO AUTO FORM FEED AT END OF PAGE

ENTER KEY -- TERMINATES 5-SECOND TIME-OUT PERIOD

*  Default values

EXAMPLE:  To print a page of graphics, centered on the page, press

the "SHIFT" key and "PrtSc" key, immediately followed by

pressing the "M" key.  Your graphics will be dumped from

the display and printed at the center of the page.

______________________________________________________

NOTE

The PRINT SCREEN utility can print one graphics

screen and one alphanumeric screen on the same

page, but cannot print two graphics screens on

the same page because of limited LaserJet memory.

However, two graphs may be printed

on the same page by printing one graph using the

form feed option (to eject the page) and then manually

feeding the same page through the printer to print

the second page.

______________________________________________________

**********************************************************

**USING THE PRINT SCREEN UTILITY WITH THE HERCULES BOARD**

**********************************************************

If you have a HERCULES high-resolution board installed in your IBM

PC, the PRINT SCREEN utility operates almost the same as without

the HERCULES board.  The exception is that the HERCULES version does

not have the "C/W (Color/Black-White) option.  Instead, a "1/2" command

can be used to specify the first or second page of graphics memory on

the HERCULES board.  For example, to print the second screen (HERCULES

allows you to store two display screens), you simply press the "SHIFT"

key and "PrtSc" key, and then press the "2" key.  (NOTE:  Attempting to

print a large graphics image may cause the LaserJet graphics buffer to

overflow.  If this happens, elimination of graphics near the right side

of the page (borders, for example) may alleviate the problem.

*************************

**IN CASE OF DIFFICULTY**

*************************

If you have any problems printing a screen, be sure that the printer

is ready to print (the LaserJet printer must be on-line--ON LINE indicator

illuminated; the ThinkJet printer PWR light must be illuminated and the

yellow light [located below the blue box near the PWR light] must NOT be

blinking).  The IBM PC or PC/XT must be correctly configured for the

specified printer (LaserJet or ThinkJet).

If you are using a software package that automatically loads an

AUTOEXEC.BAT file, you may have problems with the PRINT SCREEN utility.

In this case, check to make sure that the AUTOEXEC.BAT file does not

load another screen dump program, such as GRAPHICS.  If it does, you may

have to eliminate that line from the AUTOEXEC.BAT file to operate the

PRINT SCREEN program successfully.

__________________________________________________________

NOTE

Characters which can be printed on the IBM display screen,

but are not part of the printer's available font, will not

be dumped from the display as intended.

__________________________________________________________

When using some software packages, the screen will not print as

desired.  In these cases, try the following options to enable you

to print the screen.  Use these options only after you have already

tried the PRINT SCREEN utility unsuccessfully.  These options are accessed

the same way as the other options (by pressing the key associated with

the option within 5 seconds of pressing the "SHIFT", "PrtSc" keys).

KEY             OPTION

A    -- ALPHA SCREEN DUMP

G    -- GRAPHICS SCREEN DUMP

C    -- COLOR GRAPHICS

W    -- BLACK & WHITE GRAPHICS

4    -- 40 CHARACTERS/LINE ALPHA

8    -- 80 CHARACTERS/LINE ALPHA

For example, when printing a color bar chart using LOTUS 1-2-3,

you must first press "SHIFT", "PrtSc", and then "G"

and "C" (for color graphics).

If you have difficulty printing your display screen with a

particular software package, try the above options until the

screen prints correctly.

______________________________________________________________

IBM is a registered trademark of International Business

Machines Corp.

LOTUS and 1-2-3 are registered trademarks of Lotus Development

Corp.

______________________________________________________________