ANADISK - The Ultimate Diskette Utility

Charles P. Guzis

SYDEX

153 North Murphy Ave.

Sunnyvale, CA  94086

(408) 739-4866

September 1, 1988

Version 1.32

Copyright 1987, 1988, All Rights Reserved.

Charles P. Guzis / SYDEX

THIS IS NOT FREE SOFTWARE and may not  be	 sold  without

the  author's  express written permission.   If you have

paid a "public domain" vendor for this program, rest as-

sured that not one bit of what you paid ever  gets  back

to  the  author.	  If  you  use this program,  YOU MUST

REGISTER.	 Neither SYDEX nor Mr. Guzis will answer tech-

nical support inquiries from unregistered users.

To register,  send $15.00 with your name,	 company  name

(if any) and address to -

SYDEX

153 North Murphy Ave.

Sunnyvale, CA  94086

Upon receipt of your registration fee,  you will be sent

the most current version of ANADISK and its accompanying

documentation.

ANADISK and this document are  protected	by  copyright.

The receipt or possession of this document does not con-

vey   any	 rights	 beyond	 those	enumerated  herein  to

reproduce or distribute its contents.    If  ANADISK  or

this  document are copied,  a fee may be charged for the

service of copying only, and not for ANADISK itself.  If

such a charge is made,  it must be clearly identified as

a service charge only by the vendor.

Table of Contents

INTRODUCTION.................................................1

ANADISK IN BRIEF.............................................1

ANADISK OPERATION............................................1

Menus.......................................................2

The Main Menu...............................................2

SCAN Operation.............................................3

Searching for Data........................................5

SECTOR EDIT Operation......................................6

FILE EDIT Operation........................................8

REPAIR Operation...........................................9

COPY Operation............................................10

DUMP Operation............................................11

Accessing External Drives..................................11

Notes on Media Types.......................................12

THE SYDEX PRODUCT LINE......................................13

i

INTRODUCTION

This document is comprised of major excerpts of the full  ANADISK

reference  material.	 If you wish to get a copy of the complete

documentation for  ANADISK  -	you  guessed  it  -  you  have	to

REGISTER.  See the preceding page for information on registering.

ANADISK IN BRIEF

ANADISK is a general-purpose utility package to analyze, inspect,

edit, repair and copy diskettes.  It is intended for IBM Personal

Computer-compatible  machines	and  makes  extensive  use  of the

hardware facilities of this series of machines.  ANADISK does not

support the "hard" disk, just diskettes.

ANADISK contains  support  for,  but  is  not	limited	 to,  DOS-

compatible diskettes.	 Almost any diskette that can successfully

be read  by  the  hardware  contained	in  your  machine  can	be

processed by ANADISK.

Menus,	 windows  and  use  of	color  where available are part of

ANADISK's implementation and make it easy to use, even by the in-

experienced.

ANADISK OPERATION

As distributed,  ANADISK consists of this  file  (usually  called

READ.ME or ANADISK.DOC) and the file ANADISK.EXE.  To begin using

ANADISK, just place the ANADISK.EXE file where it can be executed

as a program and enter -

ANADISK

at the DOS prompt.

If you have a monochrome display adapter, ANADISK's displays will

be  light  on	a dark background with occasional reverse video or

highlighting.

If you have a monochrome monitor connected to	a  color  graphics

adapter,  or  you find that you would rather have a light-on-dark

display rather than a color display,  simply invoke ANADISK  with

the following line -

ANADISK M

All displays will be light on a dark background.

Page 1

If  all goes well,  you'll get what a friend calls the "elevator"

display - a graphic ANADISK floats down to  the  bottom  of  your

screen,  and the registration information appears.  After a short

pause, the main menu appears.

During the operation of ANADISK,  there are two keys whose  func-

tion does not change:

ESCape  exits from whatever ANADISK is currently doing.	If

ANADISK is at the Main Menu,  ESCape exits to DOS.    Other-

wise,  ESCape  takes ANADISK back one menu or display.	In

general, ESCape takes ANADISK  back to whatever it was doing

before the current operation.

F1 gets a help display if one is available.   Beginning with

version 1.31, a help display is available everywhere ANADISK

requests keyboard input.	 This help is "context sensitive";

that is, it relates directly to the current state of ANADISK

execution.   For example,	 pressing F1 at the main menu will

display  a  short	 explanation  of  the choices shown on the

menu.   Pressing any other key will then bring you  back	to

the main menu.

Menus

Menus are used to select ANADISK's operating modes.   The current

choice for an item is indicated by displaying that item in a  in-

verse video field;  the current position within the menu is indi-

cated by blinking that item.	To select a different choice,  use

the right- and left-arrow cursor keys.	 To go to the next line in

the  menu,  use  the  up-  or	down-arrow cursor keys.	  When all

choices have been made,  press the ENTER  key	and  ANADISK  will

progress to the next menu.  To exit to DOS, press the ESCape key.

The Main Menu

ANADISK's  main  menu  selects  what function is to be performed.

The following operations are available:

SCAN - reads every sector on a diskette, indicates the find-

ings,  and optionally pauses	 for  examination.    This

function  can  be  used to determine the type of a dis-

kette, or just to determine its basic readability.	In

addition,  SCAN is used to search for a data pattern on

a diskette.

Page 2

EDIT SECTORS - allows editing of the  data  on  a	 diskette,

sector by sector.   Many other operations are possible;

the diskette under examination need not be a	 DOS  dis-

kette.  SECTOR mode is often used to repair data errors

or to change the structure of data not normally  acces-

sible with the normal DOS utilities.

EDIT  FILES  -  edits data contained in a DOS file on a dis-

kette.   Because a file structure is essential to  this

mode, it may be used only with DOS diskettes.  The dis-

play and operation is similar to that of EDIT SECTORS.

REPAIR  -	 reconciles the File Allocation Tables,	 moves bad

sectors and scans the diskette for readability.  REPAIR

can be used to salvage a diskette containing unreadable

files.   Again,  because the REPAIR function requires a

DOS	file structure,	 it may be used only with DOS dis-

kettes.

COPY - copies one diskette to another, regardless of format.

While not intended  as  a  method  for  defeating  copy

protection schemes,	the COPY function will make a true

copy of just about any diskette,  regardless of format.

This	 operation also verifies the integrity of the copy

by performing a read-after-write check of each sector.

DUMP - copies a range of cylinders to a file.  This function

allows "off-line" processing of data read by  the  SCAN

operation.

Where	the  integrity or format of a diskette is unknown,  a SCAN

operation should first be performed.  If data errors are reported

during the progress of a SCAN,	 it is best to	attempt	 a  repair

first by using the SECTOR or FILE Edit modes.	REPAIR mode can be

used as a "last resort" for correcting DOS diskettes, or to iden-

tify a particular file containing erroneous data.

After	the  operating	mode has been selected from the main menu,

depressing the ENTER key will cause  one of several sub-menus	to

appear.      Each sub-menu contains specifications for parameters

pertaining to the operating mode selected.

SCAN Operation

The SCAN mode sub-menu displays the following options;

Page 3

DISKETTE UNIT - Selects which diskette unit will be scanned.

Normally, these appear as A: through D:.   However,	if

physical unit access has been specified (see "Accessing

Other  Drives"),  the diskette unit choices will appear

as 0: through 3:.

PRINTER OUTPUT - If selected,  causes a log to be printed on

device  LPT1:  during  the SCAN operation.	Otherwise,

results of the scan are only displayed and not printed.

PAUSE AFTER ANOMALIES -  If  selected,  ANADISK  will  pause

after an anomalous condition or error occurs.    Press-

ing any key will cause execution to resume.

SHOW DETAIL MAP AFTER ANOMALIES - If selected,  a map of the

track will be displayed showing cylinder, head and sec-

tor	addresses as they occur on the track.	Errors and

deleted data marks are also noted.	Pressing  any  key

will cause SCAN operation to resume.

SEARCH  FOR A DATA PATTERN - If selected,	 allows specifica-

tion of a hexadecimal or ASCII  data	 value	up  to	20

bytes in length.   In addition, a hexadecimal selection

mask may also  be  specified;  '1'  bits  in  the  mask

specify  positions that are to be considered during the

search; '0' bits are "don't cares".

For more information on this mode,  see "Searching  for

Data", below.

SCAN  mode first attempts to determine the characteristics of the

diskette drive and controller.	 As  the  characteristics  of  the

equipment  found are determined,  they are reported in either the

left (General Information) or right (Detailed	Information)  win-

dows.	An attempt is made to determine the following:

1.	 If the diskette contains any readable information.

2.	 The  data rate (250Kbps,  300Kbps or 500Kbps) used

to record the information, if applicable.  This is

reported in terms of the drive type and the  terms

"high" or "low" density.

3.	 The  track density or spacing of the diskette ver-

sus the normal	recording  track  density  of  the

drive (48 or 96 tracks per inch).  If the diskette

appears to be blank,  this is reported and ANADISK

exits to the main menu.

4.	 If the diskette contains 512 byte sectors,  sector

0,  side 0,  cylinder 0 is read and examined for a

DOS boot sector.  If one is found, the information

Page 4

is saved and used in operations involving DOS file

structure.   Additionally,  a "pop-up" window  ap-

pears,	showing the information read from the boot

sector.	  Pressing any key causes this	window	to

disappear.

If no valid boot sector data can be found, a check

is  made for a File Allocation Table (FAT) present

on the first track of the diskette.    If  one	is

found,	the  diskette  is assumed to be a DOS dis-

kette,	and analysis is conducted based on the FAT

identification byte located.   Note that either of

the two FATs may be  intact  (something	 that  DOS

will not look for).

5.	 If  a	DOS diskette is detected,  all FAT clusters

are checked for correct range,	and if both copies

of  the	 FAT can be read,  the two copies are com-

pared  for  consistency.     Any  differences  are

reported  and  may  be	repaired  by  means of the

REPAIR mode or the SECTOR edit mode.

If a DOS diskette cannot be identified, a check is

made for a few other common diskette  types,  such

as CP/M-86 system diskettes.

Regardless  of	the  type of diskette,	the entire

diskette is scanned for structure and readability,

reporting changes in the overall  format  of  each

track as well as any errors detected.

Searching for Data

A  SCAN  mode	option	allows	a  diskette  to	 be searched for a

specified data value.	 If this option is selected  on	 the  SCAN

sub-menu, another menu will appear with the following choices:

SEARCH KEY MODE specifies whether the search key will be en-

tered in ASCII or hexadecimal.   In either case,  up to

20 bytes of data may be entered.

SEARCH QUALIFICATION specifies how much 'leeway'  should  be

given  in  the search.   if NONE is selected,  the data

value will be located exactly as it is entered.  IGNORE

CASE will cause upper- and lower-case ASCII letters	to

be considered as being equivalent.  SPECIFY MASK allows

specification  of a hexadecimal mask value.	 This mask

value contains '1' bits in those positions of the  data

value  to be considered in the search.  '0' bits denote

"don't care" positions.

Page 5

When either the search data value or mask is being entered, Alt-X

(the Alt and 'X" keys depressed simultaneously),  will cause  the

current  position  to	be  regarded as a "don't care".   That is,

Alt-X specifies a "wild card" which will not be considered in the

search.   The first byte of the search data value must not  be	 a

"don't care".

After	the  search  data  value and the qualifying mask have been

specified,  a normal track-by-track scan is performed of the dis-

kette.	  When a match is found with the specified value,  a "pop-

up" window gives the location and, if applicable,  the file where

the match occurred.  ANADISK then pauses until one of the follow-

ing keys has been pressed:

C	    to continue and find the next occurrence of the value.

S	    to skip the remainder of the track and continue search-

ing with the next track.

E	    to	enter  SECTOR  EDIT mode with the sector containing

the data.   When an exit from SECTOR EDIT mode is made,

the "pop up" window reappears.

ESCape to exit SCAN mode and return to the main menu.

The search for a match on a specified data value continues to the

end of the diskette.

SECTOR EDIT Operation

The  SECTOR  Edit  mode allows diskette data to be altered or in-

spected on a sector address basis,  as contrasted with	 the  FILE

edit  mode,  which  allows editing of data within a specified DOS

file.

The SECTOR EDIT sub-menu selects the diskette unit to be used.

Like SCAN mode,  SECTOR Edit mode begins operation by determining

the diskette type and drive characteristics.  After this is done,

a prompt is displayed to select the track and side containing the

sector(s) to be inspected or modified.

The right- and left-cursor keys are used to select the track, the

up- and down-cursor keys are used to select the head or side con-

taining the sector.   The "ruler" line shows this position change

correspondingly.     When the ENTER key is pressed,  the track is

read for analysis and a track map is displayed.

The desired sector is selected from the displayed  track  map	by

use of the up- and down-cursor keys. When the sector to be edited

is highlighted,  the ENTER key is pressed to display the data for

Page 6

that sector.	Note that the sectors are listed in the	 order	in

which they occur on the track,	 but that the selection is done in

strictly numerical order.

The sector data is displayed in hexadecimal  and  ASCII  formats;

the  current  position	 within	 the sector data is indicated by a

highlighted, inverse video block.   Sector data may be changed by

simply entering new data at the current position or by making use

of  one of the predefined function keys.   If the sector data has

been changed,	but not re-written to the disk,	 the  word  "SAVE"

will  appear  in  the	editing menu window as a reminder that the

data displayed does not yet reflect the actual data on the disk.

The function keys are as follows:

F1    Help key, brings up a help display for editing.

F2    Re-reads the current sector and display its data.

F3    Writes the currently displayed data back  to  the  dis-

kette.

F4    Toggles the deleted data mark indicator.   If set,	the

data in the buffer will be written (when  F3	 is  used)

back	 to  the  diskette  with a Deleted Data Mark.	If

clear, no deleted data mark will be set.

F5    Shifts all the currently displayed data  left  one	bit

from	 the  current  position	 to the end of the sector.

High-order bits shifted out of the current position are

discarded;  zeroes are shifted into the low-order  bits

of the last byte of the sector.

F6    Shifts  all	 the currently displayed data right one bit

from the current position to the  end  of  the  sector.

High-order  zeroes  are  shifted into the current posi-

tion, with low order bits being discarded from the last

byte of the sector.

F7    Takes the ones' complement (logical NOT)  of  the  cur-

rently  displayed data from the current position to the

end of the sector.

F8    Fills the remainder of the currently  displayed  sector

with the value of the byte at the current position.

F9    Reads the next sector of the current track; if the last

sector  has	just been read,	 "wraps" back to the first

sector on the track.

Page 7

F10   Changes  the  display  mode	  and	data   entry   from

hexadecimal	to  ASCII  or  vice-versa.   When in ASCII

entry mode,	all printable characters (value of a space

or above) can be entered.

Shift-F2	 Performs  a  "diagnostic  read"  of the track.   A

diagnostic read involves reading the data field of  the

first  sector  and continues with all fields until 8192

bytes have been read.    ID fields,	gap bytes and CRCs

are read indiscriminately; no attempt to re-synchronize

the	data  separator is made after the first sector has

been read.	This function allows viewing of	 raw  data

and	may be used to determine the value of a data field

whose address ID field has been corrupted.

Shift-F3	 Performs a read of every  sector  on  the  current

track and side;  the data is accumulated.  The track is

then re-formatted and the data rewritten to the  track.

Before the formatting occurs,  you will be asked to ap-

prove the formatting - if ESCape is pressed, the opera-

tion will not be performed.	 This function may be used

to transfer just one track from one diskette to another

by swapping the source and destination after the prompt

appears, but before a response is given.

Shift-F9	 Reads the previous sector on the track; if already

at the beginning of the track, "wraps" to the last sec-

tor on the track.

Shift-F10	 Reads the next side; equivalent to exiting back to

the main display and advancing to the next side.

ESCape - Backs up to the previous display.

In  case  of sector data errors,  ANADISK will attempt to present

all data recoverable from the diskette.   Some of  the	 functions

just described are intended to assist in recovering data that has

become	 corrupted.   The Shift Right,	Left and NOT functions are

especially useful in this respect.

If a sector shows up as "missing" or "added" on the track map, it

cannot be read,  nor  will  it	 be  written.	 If  any  data	is

recoverable when a sector is read however,  the sector can be re-

written and the error condition generally will disappear.

FILE EDIT Operation

FILE Edit mode is similar to the SECTOR Edit mode  of	operation,

with  the  exception that DOS file structure is used as the basis

for organizing information, rather than the physical organization

of sectors on the diskette.

Page 8

The opening display for FILE Edit mode presents a  list  of  file

names contained in the root directory of the diskette.	  The cur-

sor up- and down-arrow keys are used to select	 the  file  to	be

edited;  the  file  name  display may be "paged"  by means of the

Page Up (PgUp) or Page Down (PgDn) cursor keys.

To examine the files in a particular subdirectory,  position  the

cursor bar to the subdirectory name and press ENTER.	To go back

to the parent directory of a subdirectory,  position to the  ".."

entry and press ENTER.	  To begin editing a file, position to the

file name and press ENTER.

The  editing  display	is  almost identical to that of the SECTOR

Edit mode, with the following exceptions:

F9    Reads the next sector in the file, rather than the next

sector on the track.

Shift-F9	 Reads the previous sector in the file, rather than

the previous sector on the track.

Shift-F10	 Does not operate in this mode.

Note that the file name and offset from the beginning of the file

appear in the editing menu window, along with a "SAVE" indication

if data has been altered.

REPAIR Operation

REPAIR mode can be considered to be two functions.   The first is

a rudimentary DOS diskette check and verify mode; the second is a

bad  sector  remapping mode.  The bad sector remapping operations

should be viewed as a "last resort" type of repair as the overall

structure of the diskette is affected.

REPAIR mode performs the following tasks:

1.    Checks for the presence and agreement of two  File	Al-

location tables on the diskette.   If a disagreement is

found,  or one table is absent,  a new copy of the  FAT

may optionally be rewritten to the diskette.

2.    Scans the entire diskette for unreadable data.   If any

is found,  the file containing the bad data is located,

the	cluster	 (a  grouping  of sectors used by DOS as a

"unit" of diskette space) is reassigned to a  different

area	 of  the  diskette and the data (or as much of the

data that can be read) is written to the newly assigned

cluster.   The original cluster containing the data	is

marked as unavailable for future allocation.

Page 9

The  object of REPAIR mode is to produce a readable DOS diskette.

Missing data or data in error still must  be  corrected  manually

but  the  structure  and size of files will be correct.   The DOS

utility CHKDSK can  be useful when checking  the  structural  in-

tegrity  of  a	 diskette and should be used after REPAIR mode has

been run for a "second opinion".

COPY Operation

COPY mode allows the contents of one diskette	to  be	copied	to

another,  regardless  of  format or DOS orientation.	While COPY

mode will successfully produce copies of  many	 "copy  protected"

diskettes,  the intent of its function is to enable one to copy a

diskette which is not copyable using the DOS DISKCOPY function.

A two-drive machine is not necessary for this function;  that is,

the source and target drives may be the same unit.  If one of the

drives	 is a high-capacity (PC-AT type) drive and normal 40 track

diskettes are being copied,  the high-capacity	 drive	should	be

designated  as	 the source drive and the normal-capacity drive as

the destination to ensure an accurate, readable copy.

The COPY sub-menu requests specification of the source	 and  des-

tination  drives.    When ENTER is depressed,	the COPY operation

begins by reading the source diskette.

If the type of the source drive and diskette  is  not	compatible

with the type of the destination drive,  an error message will be

issued and ANADISK will exit COPY mode.   For	instance,  a  1.2M

byte  high-density  diskette  in  a  high-density drive cannot be

copied to a 360K byte normal-density drive.

A scan of the diskette is performed as the copy proceeds;  errors

are  noted  in	 exactly the same manner and format as in the SCAN

mode.	 Sectors containing data errors are copied  with  the  er-

roneous  data written as read.	  Each track is read back after it

is written for verification.	The copy process completes when no

more data can be found on the source diskette.

If the destination and source drives have been selected to be the

same  unit,  ANADISK  will  prompt  for  disk	changes as needed.

ANADISK attempts to read as much data as is possible into  memory

from  the  source diskette before writing to the destination dis-

kette.	  With a 640K system under DOS 3.3,  about 46 double-sided

tracks can be copied before a disk change is necessary.

Page 10

DUMP Operation

The  DUMP operation specifies that data read during a SCAN opera-

tion be written to a file.   Either the entire disk or a part	of

it can be written to the specified file.

The DUMP operation first requests specification of the drive con-

taining  the diskette to be scanned.	A prompt is then displayed

for the name of the file  to  contain	the  data  read	 from  the

scanned diskette.   If the ESCape key is pressed at this time, an

exit is made to the main menu.	  Otherwise,  the  named  file	is

created  and  prompts	are displayed for the cylinder range to be

written to the file.  If the starting cylinder is specified to be

greater than the ending cylinder,  no data will be written to the

file.	 Otherwise, a series of records of the following form will

be written:

1 Byte	 The actual (physical) cylinder number.

1 Byte	 The actual head (or side) number; either 0 or 1.

1 Byte	 The cylinder number as read.

1 Byte	 The head or side number as read.

1 Byte	 The sector number as read.

1 Byte	 The length code as read.  Thus, a length code of 1

implies a sector length of 256 bytes; a code of 2,

512 bytes; 3, 1024 bytes, and so on...

2 Bytes	 The length of the data read.	This  a	 an  Intel-

style binary word;  that is, the least significant

byte of the number precedes the	 most  significant

byte.  Thus, a length of 512 bytes (200 hex) would

appear	as 00 02.   If this quantity is zero,  the

sector data could not be read.

Then follows the sector data.

All sectors  occurring	 on  a	side  will  be	grouped	 together;

however,  they	 will appear in the same order as they occurred on

the diskette.	Therefore,  if an 8 sector-per-track diskette were

scanned which had a physical interleave of 2:1, the sectors would

appear in the order 1,5,2,6,3,7,4,8.

Accessing External Drives

In  its  normal  operating  mode,  ANADISK uses BIOS interrupt 11

(hex) to determine the number of diskette drives attached to  the

computer.    The  ROM BIOS gets this information from the mother-

board DIP switches (on PC's and XT's), or from the CMOS RAM writ-

ten by the SETUP program (on AT's).   If a  drive  has  been  in-

stalled  which	 is  not  indicated  by	 either of these settings,

Page 11

ANADISK will need to be told about the total number  of  diskette

drives	 that are actually installed.	This is done by specifying

the number of drives on the ANADISK command line.  For example,

ANADISK 4

indicates a four-drive system is present.   ANADISK will refer to

these drives as 0:, 1:, 2: and 3:.

Notes on Media Types

ANADISK  contains  code  to recognize single density (FM encoded)

diskettes if the host diskette controller supports that facility.

The notation "FM",  rather than "MFM" will appear in the  GENERAL

INFORMATION window.   All operations (read, write and format) are

supported.

Additional code has been incorporated in Version 1.31 to  provide

support  for ATARI ST 3.5" diskettes.   Strictly speaking,  these

are not DOS diskettes,	 but their structure is close  enough  for

them to be treated as such.

High-density  modes  (1.2Mbyte 5.25" and 1.44Mbyte 3.5") are sup-

ported only on AT-class computers and Personal System/2 models 50

and above.

Some  of  the	terms  used  in	 this  document	 are   proprietary

trademarks of one or more of the following:

International  Business Machines - IBM,  PS/2,  Personal

System/2, PC AT.	Microsoft - MS-DOS

Page 12

THE SYDEX PRODUCT LINE

SHAREWARE

22DISK - Transfer files, format,	examine and erase files on

"foreign"  CP/M  diskettes  on  your  PC.    ASCII  diskette

description file allows "roll your own"  specifications  for

really hard-to-find systems.  Includes tips on supporting 8"

and  5.25"  single-density diskettes.   Bundled with 22NICE.

$15.00 Shareware registration fee.

22NICE - A CP/M 2.2 emulation package.	Supports  the  NEC

V-series	chips  or  performs emulation by software for both

the 8080 and Z80 processors.   Includes  terminal	 emulation

and   diskette   handling	 for  Osborne,	 Kaypro,   Morrow,

Heath/Zenith and Actrix.	 Allows use of CP/M utilities  in-

terchangeably with DOS programs.	 Includes 22DISK.   $30.00

Shareware registration fee.

TELEDISK - Turn any diskette into a compressed data file and

vice-versa.  Allows you to send and receive entire diskettes

via modem.   Great for those bootable game diskettes and for

diskettes	 with  subdirectories.	 Works	with  most  "copy-

protected" diskettes.  $20.00 Shareware registration fee.

COPYQM - Cheap diskette duplicator.  Format, copy and verify

multiple diskettes from a single master.	Implements "smart"

drive status determination,  freeing  the	 operator's  hands

from keyboard interaction.  Handles all standard DOS formats

including	 3.5" 1.44M files.   $15.00 Shareware registration

fee.

FORMATQM - Mass diskette formatter - format a  box  of  dis-

kettes at a single sitting.  Implements "smart" drive status

determination,  freeing  the  operator's hands from keyboard

interaction.   Format a 360K diskette in 41 seconds.    Sup-

ports all standard DOS formats.	$10.00 Shareware registra-

tion fee.

RETAIL SOFTWARE

SIMCGA 4.2 - Color Graphics Adapter simulation on monochrome

(Hercules-compatible) graphics-equipped machines.	 Available

from better software retailers and distributed by CSS,  ABCO

and American Software Distributors.

SIM-EMS  -  A  "must"  for AT users -- emulates Lotus/Intel/

Microsoft Expanded  memory  using	 your  machine's  extended

memory.	Allows sharing with VDISK and has extremely simple

installation.

Page 13