1.            AT  HARD DISK PREPARATION program 1.0

KFH ASSOCIATES,           1985

2771, Middleborough

SAN JOSE, CA 95132

FOR IBM AT & COMPATIBLES

PC and XT needs HDPREP.exe

DOS 2.0/2.1/3.0/3.1

** If you think the program is useful  to you,

$35.00 donation to    H. King   at above

address will register for updating service

2.  The program READ in the hard disk drive parameters from the

controller to determine the available number of heads and cylinders

for each attached drive, if the controller follows the IBM BIO

convention. Otherwise, the program defaults to 2 head and 100

cylinders for each drive. When this happens, the controller usually

has problem.

3.  Hard disk preparation uses the controller default parameters in the

BIOS.

Instructions:

The program is manual driven and self explanatory. The following is a

summary.

Options:

1. Select the drive to be operated on. Drive 0 is the first hard disk

drive attached to the controller (usually the C:), Drive 1 is the

second one (usually the D:).

2. Perform the controller FORMAT of the specified drive by option 1.

See option 3. Run FDISK and Dos FORMAT after this step.

Bad track locations can be input at this step and the program

will use the input to mark the bad ones accordingly.

3. Perform the controller FORMAT of the specified drive by option 1

and scan the disk for bad tracks and mark them bad. Run FDISK and

DOS FORMAT after this step. This option 3 is preferred to option 2.

In the begining of this step, you can enter the bad tracks if you

already know them.  This step marks  any bad track found,

the ones you entered,  at the end of the process.

4. Move the head to track 0 for the specified drive.

5. Exit this program.

6. Set the interleave number for the controller FORMAT. The default is

6 which is used by IBM PC/XT. Do not use other number unless you

have enough knowedge on the controller. Adaptec controller can use

interleave 2.

7. If you know a specific track and head location is bad, you can use

this option to mark the track bad. Use DOS FDISK AND FORMAT after

this step.

8. If you want to remark a bad track to reuse the track, you can use this

option. This option can not physically correct the defect track.

9. The same operation as the option 3, except during the verify process

more data patterns are used. The process can take twice longer then

the option 3. Usually, the option 3 is good enough.