DANCAD3D.COM  v1.10

Daniel H. Hudgins, 466 Diamond St., San Francisco, CA  94114

Rights to trade secrets reserved, some of the code is Borland Int.'s.

Read the notice in this manual before using this program.

[1]

[                 T A B L E   O F   C O N T E N T S                  ]

Notice .................................... 3

Disclaimer ................................ 3

Files on the Disk ......................... 4

How to use DANCAD3D ....................... 5

How to Program ............................ 7

Error Messages ........................... 10

Programming Commands ..................... 11

Outline of Main Menu ..................... 33

Drawing Commands ......................... 36

Editing DANCAD3D Programs ................ 38

ASCII File Formats ....................... 41

Changes .................................. 42

Index .................................... 43

[2]

[                            N O T I C E                             ]

Read the disclaimer before using the program.

DISCLAIMER

DANCAD3D disk including programs and documentation is  distributed  as

is,  with  no  guarantee  that  it  will  work correctly in any or all

situations.  This manual may contain errors and omissions. In no event

will Daniel H.  Hudgins be liable  for  any  damages,  including  lost

profits,  lost  savings  or  other incidental or consequential damages

arising out of the use or inability  to  use  these  programs  and  or

files,  even if the Author has been advised of the possibility of such

damages,  or for any claim by any other party.  The  program  can  and

will crash resulting in loss of data when operated, Daniel H.  Hudgins

shall not be held liable or responsible for any such loss.  Should the

program prove defective,  you (the user) assume the entire cost of all

necessary repair, servicing, or correction.  All offers are subject to

change  or withdrawal without notice or notification of any kind.  The

use of the program constitutes total agreement and acceptance of  this

disclaimer.

USER SUPPORTED PROGRAM

Please  help  distribute  DANCAD3D.COM by sharing unmodified copies of

this  diskette  including  all  the  original files with other people.

Send a  S.A.S.E. to receive information on up-dates and library disks.

A  fast  8087  version  is  under  development,  send  a  request  for

information and include a long S.A.S.E..

Send $10.00 for some support and a postcard good for an up-date.  I do

not  receive  any money from distributors of this program.  If you use

this program please let me know which parts are most useful to you and

which parts you would like to see changed  (and  how  they  should  be

changed).  If  you  find  any  bugs  let me know about them so they be

taken care of.   When you write state which version of DANCAD you use.

If you have line data files in some other format and want me to add  a

program  to  convert them to an update send me the name of the program

that produced the file,  a sample file on disk,  a description of  the

data  format  (if  you  have it),  and a print out of what the drawing

looks like from the source program.  I can probably convert ASCII form

data most easily so send that if you can (redirect plotter  output  if

that  is  all  you have).  I will see what I can do (no promises).  Be

sure to let me know exactly what equipment you have and  what  version

of  what  program(s)  you  are  using  so I can pass it on  to  others.

[3]

[                 F I L E S   O N   T H E   D I S K                  ]

DANCAD3D.COM  is the main program file.

DANCAD3D.00?  are the overlay files.

DANCAD3D.MSG  message file for editor.

DANCAD3D.COL  color 80x25 resolution installation.

DANCAD3D.B&W  for monochrome from CGA video board.

(rename DANCAD3D.COL for color).

DANCAD3D.P??  plotter installation data files

(may be renamed to swap plotters).

These  and  any other files named DANCAD3D.???  must be in the current

DOS directory for the program to run.

DEMO.???      are program files written with  DANCAD 3D  commands that

you can  run  and work with (on your work copy) to learn

how to use DANCAD 3D's macro commands.

????????.MAC  files are DANCAD 3D macro files called by the demos.

????????.2D   files are 2D line files for the demos.

????????.3D   files are 3D line files for the demos.

INSTALL.BAT   utility to install DANCAD3D.

README.BAT    help for first time running DANCAD3D.

If you are using a hard disk I recommend that you use separate sub-sub

directorys for each data and macro file type in order to avoid program

crashes resulting  from confusing the use  of  different  file  types.

[4]

[              H O W   T O   U S E   D A N C A D  3 D                ]

This program was written to do stereoscopic 3D wire  frame  animation.

It  is  also  useful  in  the  production of line drawings and graphs.

Automatic Dimensioning can be done in all three  axis  while  drawing.

Provision  has  been made to trigger a movie camera or stop motion VCR

through the computers audio output so rapid "pencil tests" can be done

on the animation.  After the DANCAD program has  been  verified,  high

resolution print-out can be made for photography on an animation stand

in  35mm.  You  can  also use a hard disk to save the screen pixels in

frame files to be read back as a sequence  by a loop  in a macro file.

The data is stored internally as a set of primitive elements  made  of

lines.  When  perspective  display is called the data is duplicated so

that rotations do not cause cumulative errors and so that  a  constant

relationship  of the elements to the axes is maintained.  Each element

has  its own center and can be rotated, magnified, flipped, or  offset

in  relation  to  its  center  or  the  center  of  the  whole  world.

The program only works with  the  CGA  (Color  Graphics  Adapter),  or

equivalent,  video  board.  Your  computer  must  have 640K DOS system

memory.  If you have enough extra memory above the 640K to put all the

files named DANCAD3D.???  in a RAM disk the program will  run  better.

Copy  COMMAND.COM  and GRAPHICS.COM from your DOS disk to your working

disk (or sub-directory) to reduce  disk  swapping  (you  may  need  to

delete some *.DOC or DEMO????.* files to make room).

A mouse can be used by installing it so that  movement  simulates  the

cursor keys and the buttons are assigned to [0/INS], [./DEL], and both

buttons assigned to [*].   This will also help in the program  editor.

Please do not judge the quality of this program by the poor resolution

of  the  CGA video display (640x200).  If you have a IBM graphics code

type printer such as the LX-80 the  print  out  using  the  DANCAD  3D

command Dot_printer will be 30 to 60 times as sharp (more pixels) as a

simple  screen dump (photocopy the printout at 0.5x and you get 207360

dots per square inch).  If you have a good plotter you can  do  better

than  that  and  also  get color by rotating files through a DOS batch

file to change pens.  The zoom range in drawing (F, ^S,  or T from the

main  menu)  has  a range over a trillion to one,  so zoom in to pixel

size to join lines when the location is uncertain.

The graph file generating utility is accessed from the main  menu.  It

produces a 2D line file (from point data files) that can be loaded and

printed out as is, or distorted and/or combined with text and graphics.

[5]

To run DANCAD3D programs automatically from DOS or DOS batch files

enter: DANCAD3D PATH\FILENAME.EXT

where PATH\FILENAME.EXT is the name of a DANCAD3D command (macro) file.

The program will then start automatically in two seconds.

Press ^C to Break and return to the main menu.

In DOS type DANCAD3D DEMO.1 to see the demo No. 1.  Use the demo files

on  your  working  disk  to  learn how DANCAD3D works.  From DOS enter

READ-ME for help in doing this.

If there is a file named DANCAD3D.B&W in the directory with DANCAD3D.COM

the program will run in black and white otherwise the text  is in color.

Use  the file MAKE_B&W.BAT to install as CGA monochrome.  Use the file

MAKE_COL.BAT to reinstall as CGA color.  Use the file INSTALL.BAT  for

hard disk and other installations.

You  may  write  your  ideas  in  a  macro  file  using  the DANCAD 3D

programming commands or just press a key from the menu when you see:

----------  S E L E C T   C O M M A N D   F R O M   M E N U  ---------

at the bottom of the main menu's screen.

[6]

[                   H O W    T O    P R O G R A M                    ]

Loops are provided to allow you to save/load large sets of files.  The

effects  of  Rotate,  Offset,  and Magnification are cumulative so the

loops can also animate the elements individually using these commands.

; example of how to loop

NAME 1 = c:myfile i   ; assign path/filename starting with i = -99..999

; as .ext

[label]               ; put a label at the top of the loop

Run NAME 1 HERE       ; use automatic name substitution

NAME 1 + 3            ; use automatic name advance

Goto [label] 15       ; loop up to label 15 times (16 passes)

Files run inside loops can contain loops but do not nest loops in one

file.  That is, do

put in file "MAIN":

;--------------------------------------------------------------------

; START file MAIN

;--------------------------------------------------------------------

Run loop.1

Run loop.2                   ; it should print 10 times after 25 beeps

;---------------------------------------------------------------------

; END file MAIN

;---------------------------------------------------------------------

put in file "LOOP.1":

;--------------------------------------------------------------------

; START file LOOP.1

;--------------------------------------------------------------------

[label1]

RUN b:sub.001

goto [label1] 4

;--------------------------------------------------------------------

; END file LOOP.1

;--------------------------------------------------------------------

[7]

put in file "LOOP.2":

;--------------------------------------------------------------------

; START file LOOP.2

;--------------------------------------------------------------------

[label3]

Echo LOOP.2 running ...

goto [label3] 9

;--------------------------------------------------------------------

; END file LOOP.2

;--------------------------------------------------------------------

put in file "SUB.001":

;---------------------------------------------------------------------

; START file SUB.001

;---------------------------------------------------------------------

[label2]

RUN b:sub.002

goto [label2] 4              ; run sub.002 5 times on each call to

; to sub.001, 25 times total.

;---------------------------------------------------------------------

; END file SUB.001

;---------------------------------------------------------------------

put in file "SUB.002":

;---------------------------------------------------------------------

; START file SUB.002

;---------------------------------------------------------------------

Beep                         ; do anything...

;---------------------------------------------------------------------

; END file SUB.002

;---------------------------------------------------------------------

[8]

Automatic generation of filename .ext allows looping to save  or  load

1099 files from brief code in a program file.  Do not use with Save or

Load a set of elements since it adds  its  own  .ext  to the filename.

NAME i = path\filename 0 : the = assigns path\filename to name index

i (i=0..7 or up to 8 names at a level).

NAME i HERE : HERE uses name to Save or Load a numeric .ext is added.

NAME i NEXT : NEXT adds one to numeric .ext before using in Save or Load.

NAME i + 2  : adds 2 to numeric .ext.

NAME i - 1  : subtracts 1 from numeric .ext.

Be  sure  to  leave  a  space  between  all  commands  and  parameters

(eg.  #  0  Flip  current_center  X ).   Use  the  underline  where  a

space  is  not  allowed  (eg.  Graph_mode).   Look  at  the  demo  and

macro   files   that   came   on   the   disk   for   more   examples.

Use the source code text editor's windows to display and edit the main

program and the sub programs or macros  it  runs.  ^O^O  opens  a  new

window.  You  can save to the file of the window the cursor is in with

^K^S.  The syntax of DANCAD3D is in the editor help screen,  press ^6.

To trace a program echo a duplicate of each line.

[9]

[                     E R R O R   M E S S A G E S                    ]

DANCAD 3D has been tested to confirm that the Demo files run  properly

demonstrating that the program will work when used properly.  When the

program crashes it is most often due to improper input.  After a crash

you  should  see  a  error  number  (in  hexadecimal)  on  the screen.

The meaning of the numbers that you are likely to see are given below.

Run-time error messages.

01  Floating point overflow, n>1E18 or n<-1E18.

02  Division by zero, try entering 1E-18 instead of 0.

90  Array index out of range.

92  Integer out of range, n>32767 or n<-32767.

F0  Overlay  file  not found,  all files named DANCAD3D must be in the

same  and default dir  (or sub directory) for the program to work.

FF  Heap/Stack collision, you are not leaving enough of your 640k free

for  your  macro (or just DANCAD3D) to run.  Look at bytes-free in

the main menu it should be positive.  Use fewer resident programs.

I/O error messages.

01  No file with the name used or wrong drive named.

10  Error  in  numeric  format.  Be  extremely  careful  to  state all

parameters for a command when used in a macro.  Also put  a  space

between  all  groups  of  characters.  Do not use real number form

where an integer is called for.  Do not enter letters  or  symbols

where a number is requested.

99  Unexpected end of file.  You got the file types mixed up.

F0  Disk write error.

F1  Directory is full, no more room on a disk.

F3  Too many files open, add files=20 to your DOS config.sys file.

FF  File disappeared, you took the disk out.

[10]

[     D A N C A D  3 D   P R O G R A M I N G   C O M M A N D S       ]

The ^ in front of a menu letter tells you to hold down [CTRL] key like

you would the shift key and press the letter key shown.

You must leave a space between all commands and parameters !!!!!!!

You must state all of the parameters that go with a command in a program.

Press ^C to Break a Running DANCAD 3D file and return to main menu.

Only the first three letters of most of the program commands are used.

So you can use the abbreviations shown after PROGRAM: below.

A  label  used  in  a  program  must  be a continuous bunch of letters

without spaces.  To avoid a label being  mistaken  for  a  command  it

should start with an unused symbol such as the [ (eg. [label1]).

To  run  a  program  file  with  the  Run  file command used in it add

files=20 to your DOS config.sys file.

All program files should start and end with remarks on the  first  and

last lines.

[11]

[                              A L T E R                             ]

PURPOSE:      To change a x or y or z value by line number.

Use = inside of F or ^S of T to find line number.

You can also use C (change) while drawing manually.

MENU:         A

PROGRAM:      NA

PARAMETERS:   NA

EXAMPLE:      NA

[                               B E E P                              ]

PURPOSE:      For computer to make an audio beep.

MENU:         NA

PROGRAM:      Beep  beep BEE

PARAMETERS:   none

EXAMPLE:      Beep                ; alert operator

Echo PUT DATA DISK No. 6 IN DRIVE B:

Wait                ; stop for action

[12]

[                              B L O C K                             ]

PURPOSE:      To read a ASCII text file in and draw it with upper left

at 0 0 0 with normal size and spacing.

MENU:         B

PROGRAM:      Block_of_text  block  BLO

PARAMETERS:   path\filename.ext

EXAMPLE:      BLO c:\TEXT\corner.jx2   ; be careful that the number

; of lines generated will fit in the work space eg.<6000

[                              C L E A N                             ]

PURPOSE:      To remove fractional part of x y z point values.

MENU:         C

PROGRAM:      Clean  clean  CLE

PARAMETERS:   none

EXAMPLE:      # 0 CLE             ; round all points

[13]

[                              D E L A Y                             ]

PURPOSE:      Delay progress of program for 1 to 32766 milliseconds.

MENU:         NA

PROGRAM:      Delay  delay  DEL

PARAMETERS:   delay_of_milliseconds_as_an_integer

EXAMPLE:      DEL 1000            ; a one second delay

[                             D E L E T E                            ]

PURPOSE:      To delete a line in an element.

MENU:         ^D

PROGRAM:      NA

PARAMETERS:   NA

EXAMPLE:      NA

[14]

[                            D I S P L A Y                           ]

PURPOSE:      To display the lines on the video screen.

MENU:         P  or  F  or  ^S  or  T  (see also Load Pixel)

PROGRAM:      Display  display  DIS

PARAMETERS:   xdeg ydeg zdeg zvp scale mode x_shift y_shift

EXAMPLE:      # 0 DIS 0 0 0 -1e6 240 1 0 0  ; video display

SAV PIX b:frame.036           ; and save it

; x_shift and y_shift move the vanishing point off center

[                                D O T                               ]

PURPOSE:      To print out at 51840 dots per square inch on 5152  type

graphics  printer  select  number  of strips for drawing

height (paper feed) 12 strips = 10.8 inches  of  drawing

on 11 inch paper enter number of horizontal strips (9/10

inch high ea.).

MENU:         H D

PROGRAM:      Dot_printer  Dot  DOT

PARAMETERS:   strips xdeg ydeg zdeg zvp scale mode x_shift y_shift

EXAMPLE:      # 0 DOT 12 0 0 0 -1e6 240 1 0 0    ; very sharp hard copy

; x_shift and y_shift move the vanishing point off center

[15]

[                               D U M P                              ]

PURPOSE:      To print out screen dump using DOS graphics.com.

MENU:         [Shift][PrtSc]

PROGRAM:      Dump  dump  DUM

PARAMETERS:   none

EXAMPLE:      # 0 DIS 0 0 0 -1e20 1 0 0 0   ; put it in video screen

DUM                 ; use DOS print out

; use DOT command instead if you can

[                               E C H O                              ]

PURPOSE:      To display a message on the screen.

MENU:         NA

PROGRAM:      ECHO  Echo  ech

PARAMETERS:   text

EXAMPLE:      Echo the text goes to the end of the line

[16]

[                              E N T E R                             ]

PURPOSE:      To manually enter line data by value of end-points.

MENU:         E

PROGRAM:      Enter  enter  ENT

PARAMETERS:   (new line) x1 y1 z1 z2 y2 z2

(new line) x1 y1 z1 z2 y2 z2 (and so on)

(new line) 0 0 0  0 0 0

EXAMPLE:                          ; do not put comments inside enter

ENT

100.00      0.00      0.00     50.00     86.60      0.00

-50.00     86.60      0.00   -100.00      0.00      0.00

-100.00      0.00      0.00    -50.00    -86.60      0.00

49.99    -86.60      0.00    100.00     -0.00      0.00

0 0 0  0 0 0

; 0 0 0  0 0 0 ends list of points

[                               F L I P                              ]

PURPOSE:      To flip (invert) the points across an axis.

MENU:         ^F C  or  ^F Z

PROGRAM:      Flip Zero   flip current   FLI Z    fli c

PARAMETERS:   center_to_use letter_of_axis

(center to use = C, elements current or Z, zero)

(letter of axis = X or Y or Z)

EXAMPLE:      # 1 Flip c Z          ; flip z on # 1's center

# 2 Flip c X          ; flip x on # 2's center

# 3 Flip c Y          ; flip y on # 3's center

# 1 Flip z Z          ; z = -z in element 1

# 2 Flip z X          ; x = -x in element 2

# 3 Flip z Y          ; y = -y in element 3

[17]

[                               G O T O                              ]

PURPOSE:      To loop in a DANCAD3D program file.

MENU:         NA

PROGRAM:      GOTO  goto  (only spelled out in full)

PARAMETERS:   [Label] times

EXAMPLE:      [label1]            ; declare a label.

Beep                ; dummy anything.

GOTO [label1] 11    ; loop up to [label1] 11 times for

; 12 passes through the loop.

[                         G R A P H__M O D E                         ]

PURPOSE:      To clear and set graphics screen before calling Display.

MENU:         NA

PROGRAM:      Graph_mode  Graph  graph  GRA

PARAMETERS:   none

EXAMPLE:      GRA                 ; clear screen for Display

[18]

[                               H A L T                              ]

PURPOSE:      To stop program and return error level to DOS.

MENU:         ^Q

PROGRAM:      Halt  HAL

PARAMETERS:   error_level

EXAMPLE:      Halt 0         ; go back to DOS and report no error (0)

; useful in DOS batch files

; start program with DANCAD3D P\FN.E

[                         I N I T I A L I Z E                        ]

PURPOSE:      To destroy all data in memory, reset elements and lines

to 0.

MENU:         ^I

PROGRAM:      Init  initialize  INI

PARAMETERS:   none

EXAMPLE:      INI                 ; make work-space like new

[19]

[                             I N S E R T                            ]

PURPOSE:      To insert a line in an element.

MENU:         I

PROGRAM:      NA

PARAMETERS:   NA

EXAMPLE:      NA

[                          L E T T E R I N G                         ]

PURPOSE:      To put lettering in the drawing (see also Block).

MENU:         ^L

PROGRAM:      Lettering  lettering  LET

Center or c, First or f, Last or l ...

PARAMETERS:   text   ( on its own line )

xoff yoff zoff xrot yrot zrot height length spacing center

(center = C, centering or F, first_letter, L, last_letter)

EXAMPLE:      LETTERING_on_next_line

This is some text to end of line

-800 0 0 0 0 90 50 50 1 C ; parameters on line after text

; normal spacing is 1.0

; see also drawing commands:

; [L] lettering at cursor  and  [A] automatic dimensioning

LET

Message.

0 0  0 0 0  7 5  3 First  ; puts first letter at 0 0 0

[20]

[                            L O A D   2 D                           ]

PURPOSE:      To load an element from a 2D integer disk file with  the

same  perspective  it  had  when saved.  can be used for

shadows, multiple views, and special effects.

MENU:         L 2

PROGRAM:      Load 2D  load 2  LOA 2

PARAMETERS:   path\filename.ext

EXAMPLE:      LOA 2 c:\2D\star.001     ; load from sub dir 2D

[                            L O A D   3 D                           ]

PURPOSE:      To load an element from a 3D real disk file

this can also load elements saved with Save Elements.

MENU:         L 3

PROGRAM:      Load 3D  load 3  LOA 3

PARAMETERS:   path\filename.ext

EXAMPLE:      LOA 3 c:\3D\star.001     ; load from sub dir 3D

[21]

[                         L O A D   A S C I I                        ]

PURPOSE:      To load ASCII disk file as an element (see also Enter).

MENU:         L A

PROGRAM:      Load ASCII  load a  LOA A

PARAMETERS:   path\filename.ext

EXAMPLE:      LOA A c:\ASCII\star.001  ; load from sub dir ASCII

[                      L O A D   E L E M E N T S                     ]

PURPOSE:      To reload a set of elements from 3D-real disk files

by numbered .ext.

MENU:         L E

PROGRAM:      Load Elements  load e  LOA E

PARAMETERS:   path\filename     (only, no .ext)

EXAMPLE:      LOA E c:\WORK\job-1 ; load all elements as they were

; DO NOT use with NAME command

[22]

[                         L O A D   P I X E L                        ]

PURPOSE:      To load the video screen from  disk,  can  be  used  for

display  or  backgrounds for drawing over or building up

unlimited lines.

MENU:         L P

PROGRAM:      Load Pixel  load p  LOA P

PARAMETERS:   path\filename.ext

EXAMPLE:      LOA P c:\PIXEL\frame.001 ; load video display only

[                      M A G N I F I C A T I O N                     ]

PURPOSE:      To make elements big and small or both.

MENU:         M C  or  M Z

PROGRAM:      Magnify Current  Mag Zero  MAG C  mag z

PARAMETERS:   center  x_mag_real  y_mag_real  z_mag_real

EXAMPLE:      # 2 MAG C 0.1 1 10  ; note that if you wont an axis to

; stay the same you use mag 1 not 0.

# 5 mag zero 2 2 2  ; grows away from x y z = 0 0 0

# 0 mag zero 2 2 2  ; scale everything

[23]

[                               N A M E                              ]

PURPOSE:      To substitute for path\filename.ext and automatically

generate numbered files for looping.

MENU:         NA

PROGRAM:      NAME  name  (only spelled out in full)

PARAMETERS:   index = path\filename  ext_starting_value

index HERE  or  index here

index NEXT  or  index next

index + value

index - value

(index = 0..7, ext_starting_value = -99..999, value = 1..1098)

EXAMPLE:      NAME 1 = c:\sub\myfile 0 ; assign name 1 c:\sub\myfile.0

LOAD 3D NAME 7 HERE ; use name No. 7 to load a 3D file

LOAD 3D NAME 7 NEXT ; advance .ext before using

NAME 3 + 10         ; add 10 to name 3's .ext

NAME 4 - 25         ; subtract 25 from name 4's .ext

[                            N U M E R I C                           ]

PURPOSE:      To get a listing of the point values.

MENU:         N   (reads in exponential notation, will not echo to prn)

PROGRAM:      Save ASCII

PARAMETERS:   PRN. or AUX.

EXAMPLE:      SAV A PRN           ; fixed point only

[24]

[                             O F F S E T                            ]

PURPOSE:      To move the main image of an element,

x = x+xoff, y = y+yoff, z = z+zoff.

MENU:         O C  or  O Z  or  O R

PROGRAM:      Offset Current   offset zero  OFF C  off z  off recorded

PARAMETERS:   center_to_use x_offset_real y_offset_real z_offset_real

(center to use = C, elements current or

Z, absolute from x y z equal to zero or

R, x y z recorded in Front, ^Side or Top.)

EXAMPLE:      # 5 OFF C 97.3 0 1.0E-5  ; move element No. 5

; relative to present position

; x=x+97.3

; z=z+1.0E-5

# 1 off R 0 0 0          ; the 0 0 0 is a necessary

; dummy argument for offset to a recorded x y z

[                            P L O T T E R                           ]

PURPOSE:      To print out on an installed plotter

(use menu H I to install).

MENU:         H P

PROGRAM:      Plotter  plot  PLO

PARAMETERS:   xdeg ydeg zdeg zvp scale mode x_shift y_shift

EXAMPLE:      # 0 PLO 0 0 0 -1e6 240 1 0 0  ; super sharp hard copy

; x_shift and y_shift move the vanishing point off center

[25]

[                            P O L Y G O N                           ]

PURPOSE:      To draw all or part of a regular polygon as an element.

MENU:         ^P

PROGRAM:      Polygon  polygon  POL

PARAMETERS:   total_sides_integer  sides_drawn_integer  radius_real

EXAMPLE:      POL 36 18 0.1       ; draw half a small "circle"

[                             R E M A R K                            ]

PURPOSE:      Remark the remainder of the line is skipped.

MENU:         NA

PROGRAM:      ; or | or Rem

PARAMETERS:   text

EXAMPLE:      ; this is a reminder

| so is this

REM and this, so no commands go here

[26]

[                             R O T A T E                            ]

PURPOSE:      To turn the main image of an element around a center.

MENU:         R C  or  R Z

PROGRAM:      Rotate Current  rotate zero  ROT C  rot z

PARAMETERS:   center  x_degrees_real  y_degrees_real  z_degrees_real

EXAMPLE:      # 4 ROT Z 22.5 45  90    ; turn element 4 on all 3 axis

# 7 ROT Z  0   0  120    ; turn element 7 on z axis only

# 1 ROT C  0   0  22.5   ; turn element 7 on its center

[                                R U N                               ]

PURPOSE:      To run a file of DANCAD3D commands, they can be nested.

MENU:         ^R

PROGRAM:      Run  run  RUN

PARAMETERS:   path\filename.ext

EXAMPLE:      RUN c:\programs\frame-it.001

RUN NAME HERE            ; see NAME

RUN NAME NEXT            ; see NAME

(Also you can type DANCAD3D PATH\FILENAME.EXT from DOS or batch file).

[27]

[                           S A V E    2 D                           ]

PURPOSE:      To save one or all element(s) to an 2D integer disk file

with perspective.  Can be  used  for  shadows,  multiple

views,  or saving disk space by preserving a perspective

view in 2D.  Point x y z values can range from -32767 to

32767.

MENU:         S 2

PROGRAM:      Save 2D  save 2  SAV 2

PARAMETERS:   xdeg ydeg zdeg zvp scale mode x_shift y_shift p\fn.ext

EXAMPLE:      SAV 2 0 0 0 -1e20 1 0 0 0 c:\2D\star.001     ; save in

; sub dir 2D

[                           S A V E    3 D                           ]

PURPOSE:      To save one or all element(s) to an 3D real disk file

point x y z values can range from 1.0E-18 to 1.0E+18.

MENU:         S 3

PROGRAM:      Save 3D  save 3  SAV 3

PARAMETERS:   path\filename.ext

EXAMPLE:      SAV 3 c:\3D\star.001     ; save in sub dir 3D

[28]

[                        S A V E    A S C I I                        ]

PURPOSE:      To save one or all element(s) to an ASCII disk file

make a print-out of point x y z values.

MENU:         S A

PROGRAM:      Save ASCII  save a  SAV A

PARAMETERS:   path\filename.ext

EXAMPLE:      SAV A c:\ASCII\star.001  ; save in sub dir ASCII

SAV A prn.               ; send to parallel printer

SAV A aux.               ; send to serial printer

[                     S A V E    E L E M E N T S                     ]

PURPOSE:      To save all elements to a 3D-real disk file

by numbered .ext for reloading as a set.

MENU:         S E

PROGRAM:      Save Elements  save e  SAV E

PARAMETERS:   path\filename     (only, no .ext)

EXAMPLE:      SAV E c:\WORK\job-1 ; save all elements as they are

; DO NOT use NAME command

[29]

[                        S A V E    P I X E L                        ]

PURPOSE:      To save the video screen to  disk, can be used for later

display or backgrounds for drawing over or  building  up

unlimited lines.

MENU:         F P  or  ^S P  or  T P  or  P P

(type filename after beep)

PROGRAM:      Save Pixel  save p  SAV P

PARAMETERS:   path\filename.ext

EXAMPLE:      SAV P c:\PIXEL\frame.001 ; save video display only

[                     S E L E C T__E L E M E N T                     ]

PURPOSE:      To select an element before a command is to act on it.

MENU:         You are given a prompt when needed.

PROGRAM:      # 0  # 666

PARAMETERS:   number_of_element

EXAMPLE:      # 1  MAG 2 1 0.5    ; x=x*2:y=y:z=z*0.5 in element 1

; remember to space between # & num

[30]

[                             S I G N A L                            ]

PURPOSE:      Electronic  signal  to  trigger camera or other external

device.  Output is available from computer audio  output

connector.

MENU:         NA

PROGRAM:      Signal  signal SIG

PARAMETERS:   frequency_integer  duration_in_milliseconds_integer

EXAMPLE:      SIG 20000 100       ; 20,000 HZ for 0.1 seconds

[                          T E X T__M O D E                          ]

PURPOSE:      To clear and set text screen for Echo

(Echo works in graphics as well)

MENU:         NA

PROGRAM:      Text_mode  Text  text  TEX

PARAMETERS:   none

EXAMPLE:      TEX                 ; clear screen to text mode

[31]

[                               W A I T                              ]

PURPOSE:      Pause until [Return] key is pressed.

MENU:         NA

PROGRAM:      Wait  wait  WAI

PARAMETERS:   none

EXAMPLE:      Echo >>SWAP DATA DISKS<<  press [Return] when done...

Wait

[                              W O R L D                             ]

PURPOSE:      To set world scale.  With a World scale of 1.000: one  x

drawing unit = one printer dot. 1920 dots at 1/240" each

=  8"  of  print  out  width  so a scale of 240 reads in

inches and a scale of 9.449 in mm for the  printer.  Use

Install_plotter  from  the  Hard menu to adjust scale to

your plotter.

MENU:         ^W

PROGRAM:      World

PARAMETERS:   scale

EXAMPLE:      World 9.449    ; scale will be set on return to menu

[32]

[               O U T L I N E   O F   M A I N   M E N U              ]

1. What the status line shows.

A. nnnnn:bytes-free, should always be positive for program to run.

B. nnnn:lines, 0..6000 lines may be in memory.  Plotter and screen

can  build  up  any  number  of  lines by reloading the memory.

C. nnn:elements, 0..256 independent groups of lines.

2. Commands that accesses the disk or RAM disk.

A. Write_program_file.

B. ^Run_program_file.

C. Graph_data_file.

D. Directory.

E. Save_data, with options.

1. 2D-integer.

2. 3D-real, normal choice.

3. ASCII-text, to edit in text editor or interchange of data.

4. Elements_by_number_as_3D, to save and continue later.

5. Pixel-video_display.

F. Load_data, file type must match the type used to save the data.

1. 2D-integer.

2. 3D-real.

3. ASCII-text.

4. Elements_by_number_as_3D.

5. Pixel-video_display.

G. Block_text, to read in an ASCII text file for lettering.

[33]

3. Set up the working environment.

A. ^World_scale, for drawing 1:1 on print out.

B. ^Initialize, to reset memory work-space.

C. ^Quit_all, return to DOS environment.

4. Manipulating the main data image.

A. Mag, make bigger or smaller or both.

1. From x = 0, y = 0, z = 0.

2. From element's center.

B. Offset, move in 3D or shift in 2D.

1. From x = 0, y = 0, z = 0.

2. From element's center.

3. To x y z recorded if Front, ^Side or Top.

C. Rotate, turn relative to constant space.

1. Around world x = 0, y = 0, z = 0.

2. Around element's center.

D. ^Flip, mirror image.

1. Across world axis x = 0, y = 0, z = 0.

2. Across element's center.

E. Alter, change a line by data.

F. Clean, round off.

G. ^Delete, delete a line from an element.

H. Insert, insert a line into an element.

I. Enter, make an element with line point data.

J. Numeric, list line point data to console.

[34]

5. Drawing lines as elements.

A. Front, view of 3D work-space with zoom.

1. Draw lines.

2. Lettering, at cursor.

3. Dimensioning, automatic with options.

B. ^Side, view of 3D work-space with zoom.

1. Draw lines.

2. Lettering, at cursor.

3. Dimensioning, automatic with options.

C. Top, view of 3D work-space with zoom.

1. Draw lines.

2. Lettering, at cursor.

3. Dimensioning, automatic with options.

D. ^Polygon, use to make whole or partial circles and polygons.

E. ^Lettering, put lettering in your drawing.

F. Block_text, to read in an ASCII text file for lettering.

6. Display of drawing.

A. Perspective, use to see how data will be effected before action.

B. Hard_copy, with options.

1. Dot_print, very hi-res print out (approx. 30x screen dump).

2. Plotter, output to your plotter or disk spooler.

3. Install_plotter, set up for your plotter.

C. Load with Pixel-video_display option.

D. Front, view of 3D work-space with zoom.

E. ^Side, view of 3D work-space with zoom.

F. Top, view of 3D work-space with zoom.

[35]

[                   D R A W I N G   C O M M A N D S                  ]

From Main Menu

Use T (top view), F (front view), or ^S (side view) to draw 3D.

Use F (front view) for 2D drawing.

[4] or [LEFT]  =  x - move, move "left".

[6] or [RIGHT] =  x + move, move "right".

[8] or [UP]    =  y - move, move "up".

[2] or [DOWN]  =  y + move, move "down".

[7] or [HOME]  =  z - move, move "in".

[3] or [PgDn]  =  z + move, move "out".

[Z]  = zero x y z.

[O]  = Sets move to 1.

[T]  = Sets move to 10.

[H]  = Sets move to 100.

[K]  = Sets move to 1000.

[M]  = Set move to any number, see also G goto x y z.

[R]  = Record cursor x y z with an index 0..9 to jump to later.

[J]  = Jump to recorded cursor x y z,

works when going from Top to ^Side to Front.

[G]  = Goto, entered x y z.

[1] or [END]   =  Toggle scale border on/off.

[*]  = Displays area within scale border.

[+]  = Zoom in, makes scale border smaller. Press * to see up-date.

[-]  = Zoom out, makes scale border bigger. Press * to see up-date.

[S]  = Set scale manually, zoom in or out.  Press * to see up-date.

[=]  = Search for (first match) array number of a displayed line.

Use this information for main menu Alter and ^Delete.

[P]  = Pixel, save the drawing you see to disk, type filename at beep.

[D]  = Delta, toggle writing of x y z cursor position on top line in

delta form from the last point marked with [0/INS] or [./DEL].

[I]  = Initialize draw, toggle the automatic resetting of scale and

zeroing of x y z on and off so you can go to the menu and return

to where you were.  You can then zoom in once and go from Front

to ^Side to Top without having to relocate each time.

[N]  = Number of digits to the right of the decimal point in x y z.

[SpaceBar] =  display graphics cursor. press [1/End] twice to remove.

[Return]   =  Return to main menu.

[36]

[L]  = Lettering at graphics cursor with centering options.

[A]  = Automatic dimensioning, inside and outside with options.

[.] or [DEL]   =  Mark new start of line otherwise the line starts

with last point marked with [0]/[INS].

[0] or [INS]   =  Marks end point, draws and records a line.

[?]  = Test appearance of line before keeping it (faster than Undo).

[U]  = Undo, removes lines in reverse order, (latest element only,

you can rearrange elements] also works on text).

[E]  = Erase the elements in reverse order.

[C]  = Change,  find and move  a  point  where connecting  lines,

meet use it to "pull"  points around and make corrections.

^[C] = Change center, move the rotation center of an element.

When you draw using F or ^S or T the lines are appended to the element

shown on the main menu's status line.  If no elements have been loaded

the  lines will belong to element "0".  Since element 0 stands for all

elements you can only save what was drawn before element 1 was  loaded

by  saving  the  drawn  lines  and  element  1  (along  with any other

elements) as element 0.  To save what you draw separately save  it  as

element  0  before loading any elements.  Then reload what you drew as

element 1  (or a higher number)  so it can be manipulated  separately.

The graphics cursor has a "clear" pixel at  its  center  to  help  you

align it with points on lines in the drawing.

Try  to draw simple shapes that you can save and load later to combine

to build bigger shapes.  The R(ecord x y z) / J(ump to recorded x y z)

combined with C(hange point x y z) are useful when drawing  3D  shapes

since  they  can be drawn 2D then "pull" points in z axis with C while

viewing in ^S(ide) or T(op).

To save time write macros for complex shapes so you can edit the macro

rather than re-enter commands manually.

[37]

[         E D I T I N G   D A N C A D  3 D   P R O G R A M S         ]

Use W from main menu to read and write macro program files.

[       C O N T R O L L   L E T T E R  K E Y   C O M M A N D S       ]

^A = left word.

^S = left character.

^E = up line.

^R = up page.

^W = scroll up.

^I = tab.

^D = right character.

^F = right word.

^X = down line.

^C = down page.

^Z = scroll down.

^J = jump to beg/end of line.

^G or [DEL] = delete right char.

^H or [BKSP] = delete char left.

^P = insert character by ASCII into text.

[ESC] = undo last deletion.

^Y = delete line.

^T = delete right word.

^N = insert line.

^M or [RETURN] = new line in text buffer.

^B = reformat paragraph.

^L = repeat last find/replace.

^V = toggle insert mode.

^U = abort.

^\ = disk directory.

^6 or ^^  = help (editor help, and a quick DANCAD3D command reference).

The keyboard cursor keys as well as [Ins] and [Del] also work.

[38]

[               C O N T R O L L    Q    C O M M A N D S              ]

^Q^A = find and replace.

^Q^B = cursor to beg. of block.

^Q^C = bottom of window.

^Q^D = end of current line.

^Q^F = find pattern.

^Q^I = toggle autoindent mode.

^Q^J = jump to marker (prompt).

^Q^K = cursor to end of block.

^Q^R = top of window.

^Q^S = beg. of current line.

^Q^Y = delete text to end of line.

^Q 0..9 = jump to numbered marker.

[               C O N T R O L L    K    C O M M A N D S              ]

^K^B = begin block.

^K^K = end block.

^K^C = copy block.

^K^V = move block.

^K^Y = delete block.

^K^H = hide/display toggle block.

^K^R = read file.                        <--

^K^W = write file.                       <--

^K^S = save file, use after read file.

^K^T = set tab width.

^K^M = set marker (prompt).

^K 0..9 = set marker #.

^K^X = exit and return to main menu.     <-----

[39]

[               C O N T R O L L    O    C O M M A N D S              ]

^O^C = center text.

^O^E = up window.

^O^G = goto window (prompt).

^O^I = goto column #.

^O^J = link (join) window.

^O^K = change case.

^O^L = set left margin.

^O^N = goto line #.

^O^O = open new window.

^O^R = set right margin.

^O^S = set undo limit.

^O^W = toggle word wrap mode.

^O^X = down window.

^O^Y = destroy window.

^O 0..9 = jump to window #.

Use your key board macro program to install the function keys as:

[F1]  = ^K^X                 : Quit and return to main menu.

[F2]  = ^Q^R^K^B^Q^C^K^K^K^Y : Clear editor work space.

or  ^O^O^M^M^O^Y 1 ^M    : Clear everything from window No.1

[F3]  = ^K^C                 : Block copy.

[F4]  = ^K^Y                 : Block delete.

[F5]  = ^K^H                 : Hide/display marked block.

[F6]  = ^K^V                 : Block move.

[F7]  = ^K^B                 : Mark block begin.

[F8]  = ^K^K                 : Mark block end.

[F9]  = ^K^R                 : Read file.

[F10] = ^K^W                 : Write file.

[40]

[                A S C I I    F I L E    F O R M A T S               ]

The Save ASCII file looks like:

data as x1 y1 z1 x2 y2 z2, 0 0 0  0 0 0 ends file: b:ascii.exm

100.0000      0.0000      0.0000     50.0000     86.6025      0.0000

50.0000     86.6025      0.0000    -50.0000     86.6026      0.0000

-50.0000     86.6026      0.0000   -100.0000      0.0000      0.0000

-100.0000      0.0000      0.0000    -50.0001    -86.6025      0.0000

-50.0001    -86.6025      0.0000     49.9999    -86.6026      0.0000

49.9999    -86.6026      0.0000    100.0000     -0.0001      0.0000

0 0 0  0 0 0

If the plotter output is sent to the printer or a disk file it will

look like:

P 1

M -200 -200

L -200 -200 200 -200

L 200 -200 200 100

L 200 100 -200 100

L -200 100 -200 -200

M 0 -100

L 0 -100 100 0

L 100 0 -100 0

L -100 0 0 -100

H

The commands can be installed for many plotters.  For example the line

command could look like " LINE(-200,-200)-(200,-200);  ".  If you  are

sure  your plotter cannot be installed write me explaining the precise

difficulty so I can try to correct the problem.

The data file format used by the graphing part of the program has the

fallowing form.

b:data

5

1.0000000000E+00  1.0000000000E+00

2.0000000000E+00  2.0000000000E+00

3.0000000000E+00  3.0000000000E+00

4.0000000000E+00  4.0000000000E+00

5.0000000000E+00  5.0000000000E+00

The number on the second line is the number of points.

[41]

[                           C H A N G E S                            ]

Note these changes if you have used an older version of DANCAD3D.

v1.00    Original program.

v1.10    Main menu and macro commands:

old: OFF 0 0 0           new: OFF Z 0 0 0 or OFF C 0 0 0

OFF R 0 0 0

old: MAG 0 0 0           new: MAG Z 0 0 0 or OFF C 0 0 0

old: ROT 0 0 0           new: ROT Z 0 0 0 or ROT C 0 0 0

old: FLIP (AXIS)         new: FLIP Z (AXIS) or FLIP C (AXIS)

old: LET, text, 0 0 0 0 0 0 0 0 0

new: LET, text, 0 0 0 0 0 0 0 0 0 center

new: LET, text, 0 0 0 0 0 0 0 0 0 first

new: LET, text, 0 0 0 0 0 0 0 0 0 last

Drawing commands added:

[A], automatic dimensioning

[L], lettering at cursor

^[C], change element center

[E], erase elements

[I], initialization of drawing display toggle

[42]

[                              I N D E X                             ]

2D file, save ......................... 27

2D files, load ........................ 20

3D file, save ......................... 28

3D files, load ........................ 21

8087 chip .............................. 3

alter data in memory .................. 12

animation .............................. 5

arcs .................................. 25

ascii 3D line file, load .............. 21

ascii 3D line file, save .............. 28

ascii file formats .................... 41

audio beep ............................ 12

audio output .......................... 30

audio output ........................... 5

b&w .................................... 6

block text ............................ 12

break ................................. 11

bug reports ............................ 3

bytes free ............................ 33

camera trigger ........................ 30

centers ................................ 5

CGA video board ........................ 5

circles ............................... 25

clear memory .......................... 19

coded data ............................ 16

color .................................. 6

command alter ......................... 12

command beep .......................... 12

command block ......................... 12

command clean ......................... 13

command delay ......................... 13

command delete ........................ 14

command display ....................... 14

command dump .......................... 15

command echo .......................... 16

command enter ......................... 16

command flip .......................... 17

command goto .......................... 17

command graph_mode .................... 18

command halt .......................... 18

command initialize .................... 19

command insert ........................ 19

command lettering ..................... 20

command load 2D ....................... 20

command load 3D ....................... 21

command load ascii .................... 21

command load elements ................. 22

command load pixel .................... 22

command magnification ................. 23

[43]

command name .......................... 23

command numeric ....................... 24

command offset ........................ 24

command plotter ....................... 25

command polygon ....................... 25

command remark ........................ 26

command rotate ........................ 26

command run ........................... 27

command save 2D ....................... 27

command save 3D ....................... 28

command save ascii .................... 28

command save elements ................. 29

command save pixel .................... 29

command select_element ................ 30

command signal ........................ 30

command text_mode ..................... 31

command wait .......................... 31

command world ......................... 32

commands, drawing ..................... 36

CONFIG.SYS ............................ 11

control K commands .................... 39

control O commands .................... 39

control Q commands .................... 39

control letter key commands ........... 38

ctrl key .............................. 11

DANCAD3D.B&W ........................... 4

DANCAD3D.COM ........................... 4

DANCAD3D.MSG ........................... 4

data listing .......................... 24

data magnification .................... 23

data structure ......................... 5

delay dancad3d program ................ 13

delete line data ...................... 14

demo files ............................. 4

disclaimer ............................. 3

disk directory ........................ 33

display lines as video ................ 14

dos error level ....................... 18

DOS GRAPHICS .......................... 15

dot printer ........................... 15

draw lettering ........................ 20

drawing a graph ....................... 33

drawing commands ...................... 36

drawing from menu ..................... 34

drawing text files .................... 12

editing blocks ........................ 39

editing dancad3d programs ............. 38

editing windows ....................... 39

electronic signal ..................... 30

element 0 ............................. 37

element selection ..................... 30

element set, load ..................... 22

element set, save ..................... 29

[44]

elements ............................... 5

elements, center ...................... 37

elements, erase ....................... 37

error messages ........................ 10

file conversion ........................ 3

files on the disk ...................... 4

free memory ........................... 33

function keys ......................... 40

graph data format ..................... 41

graphics printer ...................... 15

GRAPHICS.COM .......................... 15

graphing utility ....................... 5

hard copy ............................. 35

how to exit editor .................... 39

how to program ......................... 7

how to use dancad3d .................... 5

insert a line ......................... 19

INSTALL.BAT ............................ 4

installation ........................... 6

installing the plotter ................ 35

label ................................. 11

looping example ........................ 7

looping filename ...................... 23

looping with goto ..................... 17

memory requirements .................... 5

menu .................................. 11

message on screen ..................... 16

mirror image .......................... 17

monochrome ............................. 6

mouse .................................. 5

name command, in loop. ................. 7

name command, use of. .................. 8

notice ................................. 3

number of elements .................... 33

number of lines ....................... 33

offset elements ....................... 24

outline of main menu .................. 33

parameters ............................ 11

pause program ......................... 31

pencil tests ........................... 5

pixel file, load ...................... 22

pixel file, save ...................... 29

plotter colors ......................... 5

plotter data files ..................... 4

plotter output ........................ 41

plotter printout ...................... 25

polygons .............................. 25

print screen .......................... 15

printer ................................ 5

programming commands .................. 11

quit .................................. 18

ram disk ............................... 5

read command file ..................... 39

[45]

README.BAT ............................. 4

remarks in program .................... 26

rotate data ........................... 26

round off data ........................ 13

run file of commands .................. 27

running demo files ..................... 6

running from DOS batch files ........... 5

save program .......................... 39

scale ................................. 32

screen dump ........................... 15

table of contents ...................... 2

trigger, camera ........................ 5

up-dates ............................... 3

user support ........................... 3

version changes ....................... 42

video graph mode ...................... 18

video text mode ....................... 31

write command file .................... 39

writing a program file ................ 33

zoom range ............................. 5

zoom scale ............................ 36

[46]