Cohort Version 1.0

Guy Gallo

January 2, 1986

1  Purpose

A Printer / Directory Utility for the IBM (and close

compatibles).

Cohort allows you to access and change several common

printer functions, such as print pitch, page length,

bold, italic, underline.

Cohort is completely configurable

Cohort includes a 66 line notepad.

Cohort includes several common DOS operations (Dir, Erase,

Rename, Copy, Print, Chkdsk).

Cohort can be run either of two ways:  as a standalone

program from the DOS level;  or as a Memory Resident

program.

2  Usage

2.1  Loading

Cohort can be loaded into memory and then invoked by a

single keystroke from within any application

program.

In order to make Cohort memory resident, type COHORT /L

at the DOS prompt.

The current HotKey (the key used to access Cohort from

within other programs) will be displayed.

Cohort will use approximately 67K of memory.  And this

memory can be reclaimed (ONLY FROM THE DOS LEVEL!)

by selecting "Unload Cohort" under the "Setup"

menu.

2.2  Configuration

2.2.1  HotKey

The current HotKey can be changed at any time by

selecting the "HotKey..." option under the

"Setup" menu.

This enables you to change the HotKey if there is

an incompatiblity between the default HotKey

(Alt-F10) and your application program.

This change can be made permanent by selecting

"Save settings" under the "Setup" menu.  The

next time you load Cohort, the new HotKey

will be in effect.

NOTE:  In order to "Save settings":  the program

MUST be named COHORT.COM, and it must be

available in the currently active directory.

2.2.2  Printer codes

The default printer for Cohort is the HP

ThinkJet.  As a result, several of the menu

options (such as Doublestrike and Italic) are

inapplicable.  And the codes for these

functions (visible via the "Edit control

codes" option of the "Setup" menu) are empty.

Each of the thirteen codes are easily changed.

Simply find the appropriate codes in your

printer manual, select "Edit control codes",

enter the codes USING THE ACTUAL CONTROL

CODES.  Be aware that the ASCII symbol for

the keys stroke will appear.  For instance if

you press <^O>, you will see <#>.  It is

important to realize that the Escape code

itself is entered, NOT by pressing the <Esc>

key, but by pressing <^[>.  What you will see

is: <#>.

Once you have entered the appropriate codes, you

can save them permanently by selecting "Save

settings" under the "Setup" menu.

NOTE:  In order to "Save settings":  the program

MUST be named COHORT.COM, and it must be

available in the currently active directory.

NOTE:  One control code will not appear on the

screen, although you CAN enter it:  this is

the NUL character.  It is <^@> and is entered

by holding down the ctrl key and

simultaneously selecting Shift-2 (or <@>).

This control code is part of the sequence for

"Inches per page".  What you see is only

<#C>.  The NUL, being nul, is invisible...

NOTE:  both Page Length and Inches per page take

the following form:

<escape sequence><user supplied number>

for instance:  Page Length is <#C#>.  If the code

for page length on your printer is not of

this form, leave me a message...

3  Credits

Cohort was written in Turbo Pascal V3.0B;  using

utilities by Nostradamus and Neil Rubenking;

The menuing system is adapted from Borland's Turbo

Editor ToolBox.  The typewriter is adapted from TSCRIPT

by Keven Menningen.  The memory residency code is by

Lane Ferris.

4  Payment

If you find this program useful and wish to make a

contribution, please send it to:

Guy Gallo

c/o The Harvard Club

27 West 44th Street

New York New York 10032

You can address any suggestions or bug-finds to that

address, or leave a message on any of the following

Electronic Bulletin Boards:

The Invention Factory - 1-212-431-1194

PCSI - 1-212-924-6598

TextBase - 1-212-826-8827

Gene Plantz Chicago IBBS.