/*

*

*

*			XMODEM.EXE

*			Version 3.1

*			----------

*

*

* This is a special implementation of the XMODEM protocol transfer. It runs

* "beneath" Crosstalk, and in invoked via the Crosstalk "RUN" command.

*

* CHANGES IN REVERSE ORDER:

* ------- -- ------- -----

*

* 7/10/85 - While preparing v3.0, we broke the CRC routines. They

*	     have been fixed. ANY v3.0 COPIES THAT ARE FLOATING

*	     AROUND HAVE BAD CRC CODE! DESTROY THEM!

*

*	     Moral: If it ain't broke, don't fix it.

*

* 6/29/85 - Added commline I/O error checking. Overrun, Framing, and

* v 3.0     Parity errors cause bad block. In short, we're no longer

*	     relying only on checksum or CRC. Don't ask me how you

*	     can get a Parity Error when you're running 8N1...

*

*	     Pulled initial purge of commline chars. While this is

*	     exceedingly safe, it is a pain in the neck 'cause it

*	     swallows the first NAK. Things go quicker this way.

*	     This also gives a greater chance of catching the other

*	     side's "C" CRC request and thus enabling CRC checking.

*

* 6/24/85 - Fixed RS232 support routines (assembler) to stay in

* v 2.1     IRQ loop until IIR shows no more interrupts pending.

*	     See rather lengthy explanation below.

*

* 3/03/85 - Initial version.

*

*

* HISTORY:

* --------

*

* Crosstalk STILL doesn't work right with XMODEM, especially sending files

* (Crosstalk's XX command). And neither the send nor the receive (RX)

* commands recognize the CRC option, which would greatly improve reliability.

* So this stand-alone version of XMODEM was developed as an add-on Crosstalk

* command. It uses CRC if the other side can support it, else uses the

* usual 8-bit checksum.

*

*

* USE:

* ----

*

*    1) While online to the remote system via Crosstalk terminal mode,

*	 start the other XMODEM transfer, just as you would do in preparation

*	 for using Crosstalk's RX or XX commands.

*

*    2) Get to Crosstalk's COMMAND? prompt (via ESC or whatever key you've

*	 set up as the ATtention key).

*

*    3) Use the Crosstalk RUN command to run this XMODEM to actually do the

*	 file transfer:

*

*		RUn xmodem 1 s foo		(send "foo" using port 1)

*		RUn xmodem 1 r b:xyz		(receive "b:xyz" using port 1)

*		RUn xmodem 2 r xmodem.c 	(receive "xmodem.c" using port 2)

*

*    4) As the transfer proceeds, you will see status messages on your screen.

*

*    5) When the transfer is complete, you will be returned to Crosstalk.

*

* Things are made easier if you put the invariant part of the command into

* a function key within Crosstalk. Mine are set up as follows:

*

*	FK 1 "@RUn c:xmodem 1 s "		(F1 = send a file)

*	FK 2 "@RUn c:xmodem 1 r "		(F2 = receive a file)

*

* Then you just have to fill in the filename. From within Crosstalk all I

* have to do to receive "foo.exe", for instance, is press:

*

*	<F2>foo.exe<RETURN>

*

* which Crosstalk translates into a DOS command of C:XMODEM 1 R FOO.EXE

*

*

* RECOMMENDATIONS:

* ---------------

*

* When you use the RUN command within Crosstalk, DOS has to load

* TWO programs - COMMAND.COM and the one that you're RUNning. So

* I strongly recommend that both COMMAND.COM and XMODEM.EXE reside on

* ramdisk. Things really move when it's set up like that, and can be

* rather pokey (thanks to DOS' file-opening code) if not.

*

* Maybe someday Crosstalk will have a cleaner interface to external

* programs than this RUN thing. On my system I have XMODEM, KERMIT,

* COMPUSERVE and MNP protocols, all implemented via this RUN thing.

* And each time I drop into one of them, I have to wait the second

* or so that it takes DOS to load COMMAND.COM and the program itself

* from my ramdisk. Are you listening, Microstuf???

*

*

* CAVEATS:

* -------

*

* NOTE: Do not - repeat DO NOT - attempt to run this as a stand-alone transfer

* program (i.e. - outside of Crosstalk). It has some very touchy and difficult

* code that was specially developed just to allow "re-using" the serial port

* without actually opening it. This program wants to use the same baud rate and

* data structure that Crosstalk has already set up. So it doesn't set these.

* It just (CAREFULLY) massages the 8250 and 8259 so that while this program

* is running it has full input AND output interrupt-driven access to the port,

* without disturbing the environment (hardware/software) set up by Crosstalk.

*

* If you run this without first having "opened" the serial port (by running

* Crosstalk etc), then the 8250 will very likely go haywire.

*

* Just don't.

*

*

*				- Joan Riff,  3 March 1985

*

*

* 6/24/85 - Version 2.1 Notes:

* ===========================

*

* Bob Baumann found a very subtle but fatal feature of the Hayes

* 1200b internal modem. Unlike a standard 8250, the one on the

* 1200b DOES NOT present one and only one IIR value per IRQ. Instead,

* it presents the next-lower-priority IIR value AT THE VERY INSTANT

* THAT THE CAUSE OF THE FIRST IS CLEARED BY THE IRQ ROUTINE. And

* I do mean the very instant (like between instructions). We haven't

* pulled a 1200b apart to see what's tied to the IIR bits, but it

* looks like their 8250 is specially hooked to their modem, so

* that the thing can present up to 4 separate "interrupts" per one

* PC IRQ.

*

* I have heard from 1200b users who reported unspecified "problems"

* using this program under Crosstalk. No wonder! If the 1200b gets

* a parity error while receiving characters, for example,

* then its 8250 will do one IRQ for the LSR, AND THEN IMMEDIATELY

* PRESENT AN IIR FOR THE RECEIVED CHAR. Until that "received data

* available" state is cleared, the 8250 stays in the interrupted

* state.

*

* Standard 8250's (i.e. - all that I've seen except for Hayes') will

* issue a second IRQ for the second condition.

*

* There are many IIR combinations that cause a fatal lockup of the

* 8250 when used the way Hayes is using it. Thus neither XMODEM.EXE

* nor Crosstalk can get chars in or out of the 8250. Only solution

* was to exit Crosstalk and restart it (so that it would init the

* 8250).

*

* The RS232 interface code has been changed in release 2.1 to

* expect more than 1 IIR value per IRQ. This solves the problem.

* Why fight it? So much for clean, interrupt-driven code. Sigh...

*

*

*				- Joan Riff   6/24/85

*

*/

##