UMM.SYS v0.1 - An Upper Memory Manager for MS-DOS

UMM.SYS allows you to register available memory above the 640KB boundary

but below the 1MB start of extended memory.  Once this memory is registered

by UMM.SYS, it becomes available for use by MS-DOS applications, or by

MS-DOS itself.

Memory above the 640KB base memory boundary but below the High Memory Area

(HMA) and extended memory is called Upper Memory, and is grouped into one

or more Upper Memory Blocks (UMBs).  If your computer has any available

upper memory, UMM.SYS will allow you to use it.

UMM.SYS requires that an XMS driver, such as HIMEM.SYS, be installed before

it can configure itself.  Note that despite this requirement, UMM.SYS does

not use any XMS memory, including the HMA.  The XMS driver must support at

least version 2.0 of the Extended Memory Specification.

UMM.SYS will not find hidden or "shadow" RAM if it is not already available.

If your computer requires special setup to make upper memory available, then

that setup must be performed before UMM.SYS is installed.  For instance, if

your computer's motherboard uses the NEATsx chipset, and it has an AMI BIOS,

you may be able to use the Advanced Setup option to enable upper memory.

This software implements XMS functions 10h and 11h.  The Extended Memory

Specification describes how these functions may be used.  The specification

is available at several archive sites, and can also be ordered directly

from Microsoft.

Syntax:

DEVICE=[d:][path]UMM.SYS [nn@addr ...]

d:	specifies the drive where UMM.SYS is to be found.

path	specifies the directory where UMM.SYS is to be found.

nn@addr	specifies a section of memory to be allocated as a UMB.

nn must be a decimal number representing the size of the

memory block in 4KB pages.  addr must be a hexadecimal

number representing the starting paragraph of the memory

block.  The memory block must begin on a 4KB boundary.

Examples:

DEVICE=UMM.SYS 4@c800 16@d000

Load UMM.SYS, allocate 16KB at c8000, and 64KB at d0000.

4 4KB pages total 16KB.  16 4KB pages total 64KB.  The

paragraph address of c8000 is c800.  In general, you can

obtain the paragraph address by dropping the last zero.

DEVICE=C:\SYS\UMM.SYS

Load UMM.SYS from the SYS directory on drive C, but don't

allocate any UMBs.  There are reasons why you might want

to do this, but not many.

DEVICE=UMM.SYS 16@d000 16@e000

Load UMM.SYS, allocate 64KB at d0000, and 64KB at e0000.

This is not the same as having 128KB at d0000.  It is

usually better to have a single large block instead of

several small ones.  Note that adjacent blocks will be

merged into one by UMM.SYS whenever UMBs are released.

DEVICE=UMM.SYS 64@ff00

Load UMM.SYS and attempt to allocate 256KB at ff000.

Your computer will probably crash if you attempt this.

UMM.SYS does not verify that its arguments make sense,

it only checks to see if they are well-formed.

Error Messages:

Error installing UMM:  XMS driver not found

The XMS driver could not be located.  Make sure the

driver supports at least XMS 2.0, and that it has been

installed correctly.

Error installing UMM:  Invalid arguments

UMM.SYS was unable to interpret its arguments.  Make

sure that your DEVICE= line uses the correct syntax.