TIMESYNC (TM)

USER'S GUIDE

V2.1

Copyright 1992 SolaComp Software.  All Rights Reserved.

DISCLAIMER OF WARRANTY

THIS SOFTWARE AND ANY ACCOMPANYING MANUALS ARE SOLD "AS IS"

WITHOUT WARRANTY OF ANY KIND, EXPRESSED OR IMPLIED.  THIS

INCLUDES, BUT IS NOT LIMITED TO, ANY IMPLIED WARRANTY OF

MERCHANTABILITY OR SUITABILITY AND ACCURACY FOR A PARTICULAR

TASK.  THE USER ACCEPTS ALL RESPONSIBILITY FOR THE USE AND

CONSEQUENCES ARISING FROM SUCH USE.  UNDER NO CIRCUMSTANCES SHALL

THE SELLER BE LIABLE FOR ANY DAMAGES OR COSTS, DIRECT OR

INDIRECT, RESULTING FROM USE OF THIS SOFTWARE.

NOTE

This program sets the computer's clocks by placing a telephone

call to one of two automated time services.  These calls will be

long distance for the majority of users, but will be less than a

minute in length.  These time services drop the connection

automatically within a minute of the connection being made.

INTRODUCTION

TimeSync sets the DOS and hardware real time clock with the

current date and time.  This information is obtained from a

telephone call to the United States Naval Observatory or the

National Institute of Standards and Technology using a modem

attached to the computer.

The United States Naval Observatory (USNO) is the source of

official time for the United States through its Master Clock.

This time is derived from the average of a number of highly

accurate timepieces based on "atomic" processes such as the

frequency of radiation emitted from cesium under specified

conditions or from a hydrogen maser under specified conditions.

The USNO is responsible for determination and distribution of

precise astronomical and timing data for use in activities such

as navigation, global positioning, and scientific research.  The

United States Naval Observatory is located in Washington, D.C.

(telephone area code 202).

The National Institute of Standards and Technology, formerly

the National Bureau of Standards, provides scientific and

technological services including time and frequency standards.

These include radio stations WWV, WWVB, and WWVH, and the

Automated Computer Telephone Time Service (ACTS).  To achieve

maximum accuracy, the NIST-ACTS system interactively changes the

timing of its signal based on the round trip time delay between

itself and a caller.  If everything works perfectly, the accuracy

of the time signal should be within 2 milliseconds of the

"correct" time.  Even if things are not perfect, an accuracy of

10 milliseconds can be achieved by many users.  The National

Institute of Standards and Technology Time and Frequency Division

is located in Boulder, Colorado (telephone area code 303).

Several factors limit the accuracy and resolution with which

the time is set within a PC compatible computer.  The time signal

is received over a telephone and through a modem.  Such things as

the transmission speed, the propagation delay between the

computer providing the time signal and the receiving computer,

and the delays internal to the modem all create some degree of

error from the "correct" time.  These errors (the NIST uses a

nominal value of 45 milliseconds) are balanced against the

resolution of the clocks within the computer.  The DOS system

time is based on 55 millisecond clock 'ticks' provided by a

hardware timer.  The real time clock inside an AT compatible, or

on an add-on card in a PC/XT usually only provides a resolution

of one second.

REQUIREMENTS

This program requires an IBM PC/XT/AT or compatible computer

with a 1200 baud modem.  A mouse is optional.

REGISTRATION

If you find this package useful, a registration fee of $7.50

would be greatly appreciated.  Send check or money order to:

SolaComp Software

P.O. Box 4464

Lutherville, MD 21094

Inquiries may be directed to the above address or to User ID

70732,3556 on the CompuServe Information Service.

INSTALLATION

The following files are included in this package:

TIMESYNC.EXE - The time setting utility

TIMESYNC.DOC - This file

TIMESYNC.HLP - The on-line help file for TIMESYNC

TIMESYNC.ICO - An icon for use in Windows

TIMESYNC.PIF - A sample PIF file for Windows

Installation consists of setting up a directory and copying

the files into it, or simply copying all files into an existing

directory.

OPERATION

TimeSync operates in either interactive or batch mode.

The batch mode is accessed using command line options.  The

command syntax is:

TIMESYNC /B [ NIST | USNO ]

The "/B" option will select batch operation which simply

uses the current configuration file to make the time call.  If

"NIST" or "USNO" is entered on the command line, that time signal

will be used;  otherwise, the default time signal saved in the

configuration file will be used.

Interactive mode is the default.  Simply enter `TIMESYNC'

from a DOS prompt.  The program is menu driven.  The menu choices

may be selected either using a mouse or pressing the [ALT] key

and the highlighted letter (use the space for the triple bar

symbol).  The [TAB] key is used to move among fields on a form.

Where appropriate, the [ENTER] key is used to accept the current

values and end entry on a form.  The "Cancel" selection will

terminate entry on a form without saving any changes which may

have been made.

The top level menu bar contains four selections: � (triple

bar), Settings, Call, and Quit.

The Info menu (triple bar symbol) provides access to

selections dealing with system information.  The About selection

displays information about the program.

The Settings submenu allows system specific information to

be entered.  The selections available are Default service, Time

Zone, Com Port, Modem Strings, Save Configuration, and Reset

Configuration.

The Default service selects either the NIST-ACTS or the USNO

Master Clock as the default time service.  This is used if no

parameters are given when executing TimeSync in batch mode.

Time Zone leads to a submenu which allows the user to

specify the time offset from the standard time zone they are

located and UTC (Universal Coordinated Time), also known as GMT.

For instance, Eastern Standard Time is 5 hours behind UTC.  If

the "Use Daylight Savings Time" selection is checked, the time

will be adjusted to daylight savings during the proper time of

the year.  Areas that do not use Daylight Savings Time incude

Arizona, Hawaii, and the easternmost part of Indiana.  The USNO

Master Clock signal does not include information about daylight

savings time, so if the time service selected is the USNO, the

program will assume that daylight savings time is in effect

between the first Sunday in April (2:00 AM local time) until the

last Sunday in October (2:00 AM local time).

The Com Port choice displays a submenu allowing the user to

specify the com port to which the modem is attached.

The Modem Strings selection will allow the user to edit the

modem command and status messages.  While the default values

should work for most "AT Command" type modems, the documentation

provided with the modem should be referenced in setting these

messages.  Information regarding these commands may also be found

in various communications packages.  Some are quite robust and

support for a wide range of modems.  One useful addition to the

initialization string is the command "M0" which turns off the

modem's speaker.  This may be added before the "^M" at the end of

the initialization string.  The "^" (caret) character will cause

the next character to be interpreted as a control character (e.g.

^M is a carriage return).  The "~" (tilde) will cause an 0.5

second pause when encountered (this is usually used when

switching the modem into command mode).

The XT Clock Set Command allows the user to specify the DOS

command used to set the real-time clock on a PC or XT computer.

On AT class machines the real time clock is built in, and may be

directly accessed.  In XT machines, the real time clock is often

on an add-on card, and each has its own method of setting the

time.  The input to this selection should be the DOS command used

to set the clock (e.g. "\util\setclock").

The Save Configuration command saves the configuration in a

file in the same directory as the program.

The Reset Configuration command restores the configuration

to the defaults built into the program.

The Call menu item initiates a time call.  The selections

are the two time services which may be used (NIST and USNO).

The final choice is Quit.  This command terminates the

program.  If the configuration file has not been written and any

items were modified, the user will be prompted to save the

configuration file.