NOS/BPQ Link - NODEDRV4.COM

The 'Packet Driver' I wrote to allow NOS to communicate with the switch

has been modified to use the BPQ Host mode interface. One copy of the

packet driver is needed for each radio port to be used by NOS, plus one

for the loopback port if you want NOS to be able to talk to the switch.

I dont have a lot of experience of using NOS, but I suggest that if you

are using TCP/IP over NETROM that you use the lookback port, and let the

switch handle all the radio links. Only if you are using IP in datagram

or ax.25 virtual circuit mode will you need to give NOS direct access to

the radio ports.

Please note that you need at least version 4.03 of BPQCODE.EXE

The following is an extract from my system. You will need to tailor

it to your own requirements.

If using Desqview, NODEDRV4 must be loaded in the same window as NOS -

if you put it under desqview the system will probably crash. It takes

three parameters, the software interrupt number used by NOS to communicate

with it (in the range 60-7f hex), the BPQ Host port to use to talk to the

switch (1 - 63), and the number of the radio port to be accessed.

Please note that NODEDRV4 reads BPQCFG.BIN to find what interrupt to use

to talk to the switch. BPQCFG.BIN must therefore be in the current directory

when NODEDRV4 is loaded.

STARTNOS.BAT

nodedrv4 0x61 33 1 		I am using BPQHost ports 33-35, as I use

nodedrv4 0x62 34 2		the first 32 for a PacketCluster Node.

nodedrv4 0x63 35 3		I have given NOS access to 2 radio ports,

nos				and a loopback port to the switch.

This is the 'attach' bit of AUTOEXEC.NET. I can't see where the parameter

following the interface name is used, but the code suggests it is the

maximum length of the transmit queue. The next param is the MTU.

#

attach packet 61 ax1 5 256		Radio Port 1

attach packet 62 ax2 5 256		Radio Port 2

attach packet 63 node 5 236		Loopback link to the switch

#

This is a new facility, and there is very little local IP activity for

me to try it with. Any feedback would be most welcome, both on problems

and suggestions on how best to set up the system.

73,

John Wiseman, G8BPQ

76, Haywood Road,

Mapperley,

Nottingham.

NG3 6AE

6th June, 1990

Revised 7 March 91.

#################################################################################################################