Printer def is an asc code for printer codes. Use a text editor to modify

for your own printer. The program already has CHR$(27) where needed. Use

the numeric code only, not the alphabet code. All 18 codes must conform

to your printer. If your printer does not require the second part of a

printer code, enter a numeric null code where the second part is asked.

If a null code scrambles the code, enter the same code as first part.

LINE 1  = type of printer, name/model  (this is printed on screen)

LINE 2  = double strike

LINE 3  = release double strike

LINE 4  = EXPANDED PRINT

LINE 5  = 17 CPI

LINE 6  = RELEASE 17 CPI

LINE 7  = EMPHASIZE

LINE 8  = RELEASE EXPANDED

LINE 9  = RELEASE EMPHASIZE

LINE 10  = RELEASE COMPRESS 17 CPI

LINE 11 = set small print line spacing

LINE 12 = second part small print line spacing (10 for 144, 15 for 216)

LINE 13 = superscript

LINE 14 = second part, superscript

LINE 15 = RE-INITIALIZE PRINTER

LINE 16 = release superscript

LINE 17 = 144 OR 216  (smallest micro line feed, use 144 if possible)

LINE 18 = definite 1st line width (24 for 144, 36 for 216)

Most printers have the ability of n/216 line spacing. That is 13.5= 1/16th

If your printer only has n/144, that is 9=1/16th

The above is for 144. Whatever you use, all must equal 1" (216) or (144)

The program prints 1 large line, and 10 small lines, then skips

for next label.

As of 6-10-90 , the program supplied from original source is made for STAR

Gemini standard codes. That printer's smalliest line spacing it can do is

144. That is 1/144. Try it on your printer, it may work for you. If not, then

you will have to submit to the author your printer codes so he can modify and

make a special program for you. The common divider of 144 and 216 is 72, if

this helps. This program uses 10 10/144 for micro lines and 24 for header

line. It then computes 1 1/144 for each printed micro line, adds these to-

gether for the necessary next label space skip. A facility is provided for

micro adjustment at every 10 labels. If the program is printing the labels

correctly and you are satisfied with it's results, then press 'D', the program

will print out the entire labels.dat file without pause.