WORLD3.BGL

Things I think I know.

by Dan Samuel [70110,434]

------------

FS5 information...

File is located in SCENERY subdirectory and contains NAVAID information (and

maybe even more stuff than that)

note: all lists of data shown in hex.

--------------------------------

VOR INDEX

Beginning at offset 0x80 in the file is the VOR INDEX table

This table consists of a series of type "01" records (5 bytes each) where the

first record points to the VOR info for all VORs at freq 108.2

Each following entry is the offset to VORs at higher freqs, in .05 steps.

0x000080:  01 LL MM HH 00  ; HHMMLL is offset to VORs at 108.2

0x000085:  01 LL MM HH 00  ; HHMMLL is offset to VORs at 108.25

etc.

0x00044A:  01 LL MM HH 00  ; HHMMLL is offset to VORs at 117.90

There are 137 entries in the index (up to freq 117.90)

The HHMMLL offsets are relative to the start of the index, so add 0x80 to

find the absolute offset into the file for the relevant VOR table.

If HHMMLL is 000000, then no VORs exist for that freq, and no VOR table

exists, either.

-----------------------

VOR TABLE  (one for each frequency)

The VOR Index points to a series of VOR tables.  Each VOR table has all

the VORs (and ILS) which are on the frequency assigned to that table (see

VOR INDEX).

A VOR table consists of a series of records.  The following record types

are known:

type	length	use

00h	  1	NO-OP or terminator, I'm not sure.  Used chaotically

04h	  44	VOR record

05h	  54	ILS record

15h	  9	not really sure.

Each VOR table begins with a varying number of type 15h records which I

assume are SDL commands to aid in determining which VORs are worth looking

at.  They are always 9 bytes long (possibly really they are 8 bytes long and

followed by a 00h record)  I suspect they provide offsets into the following

04h records along with some coarse lat/lng info, for quick filtering.

After the 15h records, come the 04h, anf 05h records with the occasional

00h record between them.  The 00h record divides the 04/05 records into

groups of 3 or 4 with no particular pattern seen yet.

The 04/05 records define actual navaids.

The next VOR table follows right after the last 04/05 record of the previous

table, so I use the first 15h after 04/05 as my clue that the table is done.

There is probably a more intelligent way to do this.

-----------------------------

The 04h record

This record defines a VOR.  It has the following format:

04 50 xx xx xx N1 N2 N3 E1 E2 E3 xx xx xx xx [ascii text padded with 20]

for a total length of 44 bytes (29 bytes of ascii).  It may be followed by

a 00h record, but not always.  29 characters of ascii are shown, with the

first 3 or 4 being the station ID.

N1 N2 N3 are the lattitude

E1 E2 E3 is the longitude (described later)

------------------------------

The 05h record

This record defines an ILS.  It has the following format:

05 50 xx xx xx N1 N2 N3 E1 E2 E3 xx xx xx xx [29 ascii/20] n1 n2 n3 e1 e2 e3 xx xx xx xx

for a total length of 55 bytes. (29 of ascii).	The difference between this

and an 04h record is the additional 10 bytes of data following the ascii

info.

n1 n2 n3 and e1 e2 e3 are a second lattitude longitude which I imagine is

maybe the 'starting point' of the ILS system (which implies those xx's at the

end are altidues).  I mean I suspect the ILS 'beam' is a line drawn between

the two points defined.

===============================

ADBs

ADBs are stored in a separate are within the file.  They probably have

another index, but since their freq information is fully contained within

their data record, I stopped looking for their index.  If anything, they

are grouped by geographical placedment.

However, in the 5.0 release of FS5, the ADB table starts at offset

0x00010779 of the file WORLD3.BGL  It is simply a list of type 04h records

with a slightly different shape.  It may have some 15h records in the list,

but I have forgotten...

------------------------

ADB type 04h record

This record defines an ADB and has the following format

04 LL HH xD N1 N2 N3 E1 E2 E3 xx xx	[29 characters of ascii]

Note that it is 2 bytes shorter than the VOR 04h record.

Nx and Ex are still the lattitude and longitude in the same format.

The VOR freq is  HHLL.D

for example:

04 23 45 05 ... is freq  4523.5

04 12 34 00 ... is freq  3412.0

---------------------------

Lattitude and longitude.

Each of these is expressed as a 24 bit number (3 bytes of data) in

2s complement fashion.	The lattitude is positive for North and negative

for south.  The longitude is positive for east and negative for west.

For example, if:  N1 N2 N3  =  01 23 45

You would use the 24 bit value:  0x452301

What I do is sign extend it to 32 bits and use a signed 32 bit long integer.

for longitude, the value  0x007fffff is just shy of 180 degrees

for lattitude, the value  0x004c4e99 is just shy of 90 degrees

That number is SO crazy, I assume it is wrong.

Here is some ugly code which I hacked out to do the conversions.

char *GetLattitude(char *buff, long lat)

{

double temp;

int degrees;

double minutes;

char suffix = 'N';

// show latitude

if ( lat < 0 )

{

suffix = 'S';

lat = -lat;

}

temp = (double) lat;

temp /= 0x004c4e99;

temp *= 90.0;

degrees = (int) temp;

minutes = temp - (double) degrees;

minutes *= 60.0;

if ( minutes < 9.99 )

sprintf(buff, "%3d  %2.2f%c", degrees, minutes, suffix);

else

sprintf(buff, "%3d %2.2f%c", degrees, minutes, suffix);

return buff;

}

char *GetLongitude(char *buff, long lng)

{

double temp;

int degrees;

double minutes;

char suffix = 'E';

// show longitude

if ( lng < 0 )

{

suffix = 'W';

lng = -lng;

}

temp = (double) lng;

temp /= 0x00800000;

temp *= 180.0;

degrees = (int) temp;

minutes = temp - (double) degrees;

minutes *= 60.0;

if ( minutes < 9.9 )

sprintf(buff, "%3d  %2.2f%c", degrees, minutes, suffix);

else

sprintf(buff, "%3d %2.2f%c", degrees, minutes, suffix);

return buff;

}