---------------------------------------------

INT 10 - VIDEO - SET INDIVIDUAL DAC REGISTER (EGA, VGA/MCGA)

AX = 1010h

BX = register number

CH = new value for green (0-63)

CL = new value for blue (0-63)

DH = new value for red (0-63)

SeeAlso: AX=1012h,1015h

---------------------------------------------

INT 10 - VIDEO - SET BLOCK OF DAC REGISTERS (EGA, VGA/MCGA)

AX = 1012h

BX = starting color register

CX = number of registers to set

ES:DX -> table of 3*CX bytes where each 3 byte group represents one

byte each of red, green and blue (0-63)

SeeAlso: AX=1010h,1017h

---------------------------------------------

INT 10 - VIDEO - SELECT VIDEO DAC COLOR PAGE (VGA)

AX = 1013h

BL = 00h Select paging mode

BH = 00h Select 4 blocks of 64

BH = 01h Select 16 blocks of 16

BL = 01h Select Page

BH = page number (00h to 03h) or (00h to 0Fh)

Note:	not valid in mode 13h

SeeAlso: AX=101Ah

---------------------------------------------

INT 10 - VIDEO - READ INDIVIDUAL DAC REGISTER (EGA, VGA/MCGA)

AX = 1015h

BL = palette register number

Return: DH = red value

CH = green value

CL = blue value

SeeAlso: AX=1010h,1017h

---------------------------------------------

INT 10 - VIDEO - READ BLOCK OF DAC REGISTERS (EGA, VGA/MCGA)

AX = 1017h

BX = starting palette register

CX = number of palette registers to read

ES:DX -> buffer (3 * CX bytes in size) (see also AX=1012h)

Return: CX number of red, green and blue triples in buffer

SeeAlso: AX=1012h,1015h