�������������������������������������������������������������������ķ

�                                                                   � �

�                                                                   � �

�                           A M I s e t u p                         � �

�                                                                   � �

� Documentation                                                     � �

�                                                                   � �

� Copyright (c) Robert Muchsel 1992-1994                            � �

� All rights reserved.                                              � �

�������������������������������������������������������������������ͼ �

����������������������������������������������������������������������

AMIsetup is an external setup program for 386/486 PCs with modern AMI

BIOS (AMI Hi-Flex BIOS).

If you ever considered your built-in setup as incomprehensible,

boring, inconvenient or incomplete, you've been waiting for

AMIsetup.

AMIsetup can save your configuration on disk and restore it. You

can even change setup options missing in your BIOS! AMIsetup cracks

your password and prints your setup as a list. You can install IDE

hard disks automatically! Add comprehensive on-line help, Soundblaster

support and an on-line reference that explains many setup options.

Of course you can protect AMIsetup itself with a password.

For diagnostic purposes, AMIsetup can on many machines measure the

DMA frequency (avoid burnout of the DMA chip due to overload) and

display the CMOS error flags. You can compare the CMOS real time

clock and DOS time, change setup registers manually (experts only)

and display the BIOS ID. Switch AMIsetup to high resolution video

mode (EGA/VGA) and use the mouse...

And once you are finished, AMIsetup can reboot your computer.

Professionals will like the batch mode of AMIsetup. Configure

identical computers in blitz speed!

Important

���������

��������������������������������������������������������������������

��������������������������������������������������������������������Ŀ

�                                                                    �

�                         USE AT YOUR OWN RISK!                      �

�  This program is based on undocumented data structures.            �

�  Improper use, program errors or bad luck can cause hardware,      �

�  software and stored data of the computer be irreversibly damaged! �

�  The user is advised always to make adequate Back Ups of all       �

�  valuable files or Data. Because he can have no control over how   �

�  this program is used                                              �

�          THE AUTHOR ACCEPTS NO LIABILITY FOR ANY DAMAGE!           �

�                                                                    �

��������������������������������������������������������������������;

��������������������������������������������������������������������

No part of this documentation may be reproduced, transmitted,

transcribed, stored in any retrieval system, or translated into any

other language or computer language in whole or in part, in any form

or by any means, whether it be electronic, mechanical, magnetic,

optical, manual or otherwise, without prior written consent of the

author, Robert Muchsel.

The author disclaims all warranties as to this software, whether

express or implied, including without limitation any implied warranties

of merchantability, fitness for a particular purpose, functionality,

data integrity or protection, in so far as permitted by applicable

legislation.

All trademarks are property of their respective owners and appear for

identification purposes only.

Shareware

���������

�������������������������������������������������������������������Ŀ

�  Writing this program has taken lots of work. If you continue     �

�  using it after a reasonable trial period of 30 days, you have    �

�  to pay a small fee to the author.                                �

�                                                                   �

�  SHAREWARE is NOT FREE SOFTWARE. You can, however, test shareware �

�  before purchasing it.                                            �

�                                                                   �

�  For detailed information, see appendix A.                        �

���������������������������������������������������������������������

You are encouraged to copy and pass on the unregistered shareware

version of this program, if

- the program is distributed in its original form

(complete, unmodified, uninstalled)

- the distribution fee does not exceed the equivalent of DM 15.-.

It is forbidden to distribute the program bundled with books,

magazines, etc. (including "bookware").

The information contained in the on-line reference for registered users

must not be copied, published or separated from the program. The author

has taken every care to ensure the technical correctness of the

information, but cannot accept any liability in case of any error.

If you have comments or find an error, please send them to the author!

Contents

��������

1   Caution!

2   Starting the program

3   Menus in detail

3.1 � (System) menu

3.2 BIOS menu

3.3 CMOS menu

3.4 Setup

1. Standard setup

2. Extended/chipset setup

3. Change register

4. Password

5. Load ROM setup defaults, load ROM power-on defaults

6. Print current setup

7. Analyze ID string

8. Save changes and quit

9. Exit without saving

3.5 Options dialogue

3.6 Help menu

4   Key shortcuts

Appendix

A   Registration, limits of the shareware version,

addresses, upgrades, about the author

B   AMIsetup and OS/2 2.x, AMI BIOS and OS/2, Windows, Windows NT

C   Known problems

1. Is AMIsetup compatible with your BIOS?

2. Frequent errors with compatible BIOS versions

3. Stopgap: -OVERRIDE

4. Additional trouble shooting options

D   Glossary

E   If the machine won't boot...

F   Switching to the English user interface

G   Soundblaster

H   EISA

I   Batch mode

J   Obtaining new or bug fixed program revisions

K   Tested BIOS revisions

What's new

Changes are described in a separate file, AMISETUP.NEW.

1  Caution!

�����������

If you are not familiar with setup options or if you don't know

what they do, please consider whether changing the setup options

is really necessary.

�������������������������������������������������������������������Ŀ

�  On many PCs, the DMA clock speed can be changed.                 �

�  This may work at first, but later the DMA chip may be destroyed, �

�  thus making the motherboard useless!                             �

�                                                                   �

�  Another critical point is the password option. Since you cannot  �

�  erase CMOS RAM on some main boards (see appendix E), it may      �

�  happen that because of - forgetfulness, BIOS errors, different   �

�  keyboard layout, etc. - you are irreversibly locked out of your  �

�  computer.                                                        �

���������������������������������������������������������������������

Before making ANY change, ask yourself:

- is it necessary?

- what are the consequences?

AMIsetup saves your configuration into a file after installation

and whenever you invoke the program. However, this works only if

you don't use AMIsetup on a write protected diskette.

�������������������������������������������������������������������Ŀ

�  N E V E R   C H A N G E   A   R U N N I N G   S Y S T E M  !!!   �

���������������������������������������������������������������������

WHEN DID YOU DO YOUR LAST COMPLETE BACKUP???

2  Starting the program

�����������������������

First of all, please read the manual and the file "AMISETUP.NEW".

The program needs write access to the current directory. Diskettes

must not be write protected if file operations are to be used.

Normally, you start AMIsetup without additional command-line options.

There are exceptions though (see appendix C). You should read

appendix B if you want to use AMIsetup under OS/2.

If you are using an LCD screen, invoke AMIsetup by "AMISETUP -MONO".

Adaptec SCSI Controllers: Be sure to use the ASPI4DOS driver to avoid

possible problems with the memory management unit.

If using AMIsetup for the first time, you have to choose the desired

language:

������������� Welcome / Willkommen ! �����������Ŀ

�                                                �

�  This is a bilingual version of AMIsetup.      �

�  For English help texts and user interface,    �

�  press [E].                                    �

�                                                �

�  Dies ist eine zweisprachige Version von       �

�  AMIsetup. Dr�cken Sie [D] f�r deutsche Hilfe- �

�  texte und Benutzeroberfl�che.                 �

�                                                �

�  [E] English version     [D] deutsche Version  �

�                                                �

����������������������������������������AMIsetup��

Should you decide on the German version, please continue reading in

the file "AMISETUP.DOK".

Next you must to accept the warning message with a "Y":

������������������ C A U T I O N ���������������Ŀ

�                                                �

�              USE AT YOUR OWN RISK!             �

�                                                �

�  This program is based on undocumented data    �

�  structures. Hard- and software and stored     �

�  data could be damaged irrevocably!            �

�                                                �

� THE AUTHOR WILL NOT BE LIABLE FOR ANY DAMAGES! �

�                                                �

� [Y] accept and start program   [N] back to DOS �

�                                                �

����������������������������������������AMIsetup��

If you have set a password in the "Options" dialogue (3.5), you will

be prompted to enter it.

��������������� Password �������������ͻ

�                                      �

� AMIsetup is password protected.      �

� Please enter the current password.   �

�                                      �

�  Password    ****                    �

�                                      �

�                                      �

�          OK   �      Help  �         �

�        ��������     ��������         �

��������������������������������������ͼ

Next, a dialogue box containing program revision and registration

number (or "UNREGISTERED") appears. If you haven't yet paid for

AMIsetup and a registration number appears anyway, you've got an

(illegal) pirated copy. Please contact the author.

��[�]���������� Information �������������ͻ

�                                         �

�             AMIsetup v2.60              �

�         REGISTERED (#.........)         �

�                                         �

�  Copyright (c) 1992-94 Robert Muchsel   �

�           All rights reserved           �

�                                         �

�                                         �

�           Heap memory:  84080           �

�               ISA System                �

�                                         �

�            OK    �     Help   �         �

�          ���������    ���������         �

�����������������������������������������ͼ

� You can also view this entire file from AMIsetup ("Help|View

� AMISETUP.DOC").

Quick start

�����������

The most frequent error stems from use of AMIsetup with a memory

manager (386MAX, QEMM-386, EMM386) loaded - EMM386 is installed

automatically by the DOS 5/6 and Windows setup.

Please note para 3.2 "BIOS|Write image to file" and appendix C.

3  Menus in detail

������������������

Using the mouse or pressing <F10>, you get to the main menu bar.

There, you have the following pull down menus at your disposal:

�  BIOS  CMOS  Setup  Options  Help

���������������������������������������

Pressing <F1> gets you help. The on-line help contains nearly all of

the manual.

Hint for mouse users: The right button is equivalent to <Esc>.

3.1 � (System) menu

�������������������

�  BIOS  CMOS  Setup  Options  Help

�������������������������Ŀ �����������������

� About         Shift-F10 � �����������������

� Evaluate DMA clock      � �����������������

� Test coprocessor status � �����������������

�������������������������Ĵ �����������������

� Video mode        Alt-V � �����������������

�������������������������Ĵ �����������������

� Reboot                  � �����������������

� Exit              Alt-X � �����������������

��������������������������� �����������������

����������������������������������������������

����������������������������������������������

About

Shows a dialogue box containing program revision and registration

number (if any).

Evaluate DMA clock

Tries to determine the current DMA clock speed (this operation is

very hardware dependent and it is possible it won't work - "Error"

is displayed then).

The measured clock speed is displayed in

green  (up to 4.25 MHz): Everything OK

yellow (up to 5.10 MHz): Caution, possibly decrease DMA clock speed

red    (from  5.10 MHz): The clock is too fast and can cause damage!

Conditioned by "DMA waits" it is possible that too low a clock is

shown (especially on fast i486 systems). I.e.: If AMIsetup detects a

"red" DMA clock, then it is truly time to decrease it - the DMA chip

can be destroyed by overclocking!

On many systems, DMA clock is coupled with bus clock (e.g. ETEQ,

OPTI, SIS, UMC).

You can compute the bus clock using the following formula:

Bus Clock = 2 * DMA Clock

Depending on the DMA waitstates, the factor can be 2.75

(1 Waitstate) or 3 (2 Waitstates) instead of 2.

It is neither possible to measure the bus clock nor the number of

DMA waitstates directly.

The routine doesn't work on some faster machines (i.e. Overdrive

(tm), 66DX2) (it displays garbage values then).

This function is disabled under OS/2.

Test coprocessor status

This function performs a coprocessor hardware check and compares the

result with the BIOS coprocessor presence bit, which is set during

system boot.

If both results correspond (i.e. "Coprocessor present" and "Status

bit set" or "No coprocessor present" and "Status bit not set"),

everything is OK.

If both results do not correspond ("*ERROR*" is displayed in

addition), the BIOS most probably doesn't sense the coprocessor.

If you have problems with applications which support a math

coprocessor and the above error is shown, you should first look into

"Setup|Extended Setup" and see if you can enable "Numeric

Coprocessor" or "Numeric Coprocessor Test".

If no such setting is available, you can create a small "SETNPU"

program and add it to your AUTOEXEC.BAT.

Please copy all characters exactly as shown:

C:\> DEBUG SETNPU.COM

The specified file name expression was not found.

-a

....:0100 mov ax,40

....:0103 mov ds,ax

....:0105 or byte ptr [10],2

....:010A mov ax,4c00

....:010D int 21

....:010F                     [Press Enter]

-rcx

CX 0000

:0f

-w

Writing 000F bytes

-q

C:\>

Video mode

Toggles the display mode between 25 lines and 43/50 lines and requires

an EGA/VGA graphics adapter.

AMIsetup automatically stores the last used video mode.

Reboot

Executes a cold boot. The keyboard controller is programmed to trigger

a hardware reset.

The "Reboot" function can be turned off using the "Options" dialogue.

CAUTION! If you use a hard disk cache or disk doubler, this function

could possibly cause data loss.

AMIsetup first tries to flush the SmartDrive cache (if it is

installed, you can disable this feature using the command line

argument "-NOSM"), waits 2 seconds and then resets the computer

(OS/2: only the program is closed).

Exit

Exits the program.

3.2 BIOS menu

�������������

�  BIOS  CMOS  Setup  Options  Help

��� ���������������������Ŀ ������������

��� � Test version        � ������������

��� ���������������������Ĵ ������������

��� � Write image to file � ������������

��� ����������������������� ������������

����������������������������������������

Test version

Verifies whether the BIOS and AMIsetup are compatible. This check

is executed automatically whenever you start "Setup". If this test

fails ("*ERROR*") and you use a memory manager (e.g. QEMM-386,

EMM386, 386MAX), you may have to create a BIOS image (see below), or

reboot from a plain vanilla DOS diskette. Please read appendix C for

more information about compatible BIOS versions.

Write image to file

Using this function, you can create a "BIOS image" file. This file

is necessary if you use a memory manager like QEMM-386, EMM386 or

386MAX. Otherwise you don't need an image file. "BIOS|Test version"

tells you whether you need an image file.

To create a "BIOS image" file, do the following:

1. Reboot the PC WITHOUT memory manager and WITHOUT shadow RAM (if

possible). It's best to boot from a floppy disk.

Example: To disable the DOS 6 memory manager, type

"MEMMAKER /UNDO"

2. Start AMIsetup.

3. Turn off "Use BIOS image" in the "Options" dialogue.

4. Select "BIOS|Test version" and verify that everything checks out

OK.

5. Select and execute "BIOS|Write image to file" (you can change the

file name in the "Options" dialogue, if need be. Default is

"BIOS.DAT").

6. Finally don't forget to turn on "Use BIOS image" in the "Options"

dialogue to use the newly created BIOS image.

7. Now you can use both AMIsetup and your memory manager.

3.3 CMOS menu

�������������

S  CMOS  Setup  Options  Help

� ������������������������������Ŀ ���

� � Test checksums               � ���

� ������������������������������Ĵ ���

� � Print                        � ���

� ������������������������������Ĵ ���

� � Write image to file          � ���

� � Restore from image           � ���

� � Write EISA NVRAM to file     � ���

� � Restore EISA NVRAM from file � ���

� ������������������������������Ĵ ���

� � Reset CMOS RAM               � ���

� ������������������������������Ĵ ���

� � Information                  � ���

� � Time compare                 � ���

� �������������������������������� ���

��������������������������������������

Test checksums

Verifies the CMOS RAM checksums. There are two areas - the first is

in all AT computers, the second only in PCs with an AMI BIOS. If

one of these two tests fails, the program declines to run "Setup".

If you have written your Setup to a file before this error occurred,

you can correct the problem through "Restore from image".

AMIsetup saves your configuration automatically

- when you first use the program into the file SAVECMOS.SAV

- whenever you invoke the program into the file CMOSBKUP.SAV.

Print

Prints the CMOS RAM contents and is used for diagnostic and

reference purposes only. To get a readable, plain English print,

use "Setup|Print current setup".

������������������������������������������

���[�]��������� Print CMOS ������������ͻ�

��                                      ��

�� �Print to file                       ��

��   PRN                                ��

��                                      ��

��                                      ��

��  [  OK  ]    [Cancel ]   [ Help  ]   ��

��                                      ��

���������������������������������������ͼ�

������������������������������������������

If you enter a name different from "PRN", the output will be

redirected to a file, e.g. "CMOS.TXT". Of course you can also use

device names (e.g. "LPT2")

If your printer cannot print graphic characters, you must turn off

"Extended character set" in the "Options" dialogue.

Data is written in the following format:

Reg � Current value

�������������������

00 � 13  00010011

01 � 23  00100011

...    ...

7E � 00  00000000

7F � 00  00000000

Opposite to the register number (hexadecimal), the current value is

printed in hexadecimal and binary format.

If you don't want a form feed at the end, turn it off in the

"Options" dialogue.

Write image to file

Copies contents of CMOS RAM to a file. You can change the default

directory in the "Options" dialogue.

Restore from image

Reads this file back to CMOS RAM and restores the saved state. Time

and date are not set, however!

Write EISA NVRAM to file

Restore EISA NVRAM from file

On some EISA computers, you can also save and restore the EISA

NVRAM (parts of the configuration data are stored in the EISA NVRAM

on these machines).

See appendix H for more information.

Reset CMOS RAM

Caution: Reset CMOS erases the complete CMOS RAM contents,

including checksums, date/time, hard disk parameters, etc.

Please make a paper backup of your configuration before

resetting the CMOS RAM!

Information

Shows the date of the RTC, the alarm time and information about

errors at system boot.

����� Information about CMOS RAM contents ����ͻ

�                                              �

�  Date �������������������Ŀ                  �

�  � 03/19/1993             �                  �

�  ��������������������������                  �

�                                              �

�  Alarm ������������������Ŀ                  �

�  � 00:00:00           off �       OK    �    �

�  ��������������������������    ����������    �

�                                              �

�  Status �����������������Ŀ     Print   �    �

�  � Battery:             � �    ����������    �

�  � Power:               � �                  �

�  � Checksum:            � �      Help   �    �

�  � Configuration:       � �    ����������    �

�  � Memory size:         � �                  �

�  � Hard disk C:         � �                  �

�  � Time:                � �                  �

�  ��������������������������                  �

����������������������������������������������ͼ

Date

The current system date of the real time clock (RTC).

Alarm

The RTC supports an "alarm clock" mode. You need an external

program (TSR) that processes the RTC message and then activates.

Status

Shows possible errors from the last system boot. A "�" means

"no error found".

- Battery: CMOS battery is discharged and CMOS contents were invalid

- Power: RTC power supply failed

- Checksum: CMOS checksums were invalid

- Configuration: Self test results and CMOS configuration were

different

- Memory size: Memory size determined by self test and memory size

stored in CMOS were different

- Hard disk C: Hard disk C: (if available) couldn't be initialized

and couldn't be booted from

- Time: Time was invalid

Errors often disappear after a reboot (e.g. Checksum).

Time compare

Compares the current DOS time and the RTC time and is used for

reference purposes only. If the difference is more than approx. 2

seconds, you should check hard and software.

3.4 Setup

���������

�  BIOS  CMOS  Setup  Options  Help

������������������������������������ Setup ����������������������������

�

�05/05/91(C)1990 American Megatrends Inc., All Rights Reserved

�ROM BIOS (C)1990 American Megatrends Inc.,

�

�

�                      �������������������������������Ŀ

�                      �        Standard setup         �

�                      �    Extended/chipset setup     �

�                      �        Change register        �

�                      �           Password            �

� DATE:                �    Load ROM setup defaults    �      WARNING:

� 05/05/91             �  Load ROM power-on defaults   �      Improper

�                      �������������������������������Ĵ      Setup may

�                      �      Print current setup      �      severe pr

�                      �       Analyze ID string       �

�                      �������������������������������Ĵ

�                      �     Save changes and quit     �

�                      �      Exit without saving      �

�                      ���������������������������������

�

� 30-0201-ABCDEF-00101111-050591-OPWB

�����������������������������������������������������������������������

If the CMOS RAM checksums and the internal BIOS copyright strings are

valid, the above Setup screen is displayed.

If you are a registered user (see appendix A), you can begin with this

screen automatically at program start. Simply set "Options|Auto start

setup" to on.

AMIsetup searches for two specific copyright strings at several

locations. One of these is displayed at the top. At the left is

the BIOS date, at the bottom the BIOS ID string (as displayed at

system boot).

3.4.1 Standard setup

��������������������

You can change the following settings (to change date and time,

please use the DOS commands DATE and TIME).

��[�]��������������������� Standard setup ������������������������ͻ

�                                                                  �

� Base memory:       640K                                          �

� Extended memory: 16384K           Auto detect hard disks �       �

�                                   ������������������������       �

� Drive A         Drive B           HDD 0 �         HDD 1 �        �

�                                   �������         �������        �

�  ( ) None        ( ) None          Type:    47    NONE / SCSI    �

�  ( )  360K 5�"   ( )  360K 5�"     Cyl:   1024                   �

�  ( )  1.2M 5�"   (x)  1.2M 5�"     Heads:   64                   �

�  ( )  720K 3�"   ( )  720K 3�"     S/Trk:   26                   �

�  (x) 1.44M 3�"   ( ) 1.44M 3�"     Size:  832M                   �

�  ( ) 2.88M 3�"   ( ) 2.88M 3�"                                   �

�                                                                  �

� Display adapter      Options                                     �

�  ( ) Not installed    [X] Keyboard installed                     �

�  ( ) Color 40x25                                                 �

�  ( ) Color 80x25                                                 �

�  ( ) Monochrome                                                  �

�  (x) VGA/PGA/EGA                  OK   �    Cancel �     Help  � �

�                                 ��������    ��������    �������� �

������������������������������������������������������������������ͼ

At top, the memory size as determined by the BIOS is displayed.

Drive A

Drive B

Allows you to change the type of installed floppy disk drives.

2.88 MB disk drives:

Using AMIsetup, you can configure the drive type "2.88 MB" on

any computer. To run a ED drive, you need a diskette controller

capable of the 1 Mbps data rate (e.g. AHA1542CF).

If your BIOS does not support 2.88 MB drives, AMIsetup displays

a warning message (BIOS versions earlier than 06/12/91).

Lack of BIOS support can cause the following problems:

- Problems booting from diskette (especially 720 KB diskettes)

- The built-in ROM "Standard setup" displays a line containing

garbage characters (this does not adversely affect other setup

functions).

- You'll need software support for your floppy drive. OS/2 2.1

automatically detects 2.88 MB drives. DOS 5.0 or later requires

a device "DRIVER.SYS" (please consult your DOS manual). The

author is currently developing a flexible BIOS enhancement which

must be installed in CONFIG.SYS, supporting 360 KB to 2.88 MB

floppy drives on any port, IRQ or DMA channel.

- Dependent on your hardware, it might be better instead to tell

the BIOS the drive was 1.44 MB.

You can disable the warning message (displayed if your BIOS does

not support 2.88 MB drives) using the command line parameter

"-288".

Display adapter

With this you set the type of the installed graphics adapter.

( ) Not installed

( ) Color 40x25

( ) Color 80x25   = CGA

( ) Monochrome    = MDA, Hercules

(x) VGA/PGA/EGA

Often you have to change a jumper on the main board besides this

setting.

Options - Keyboard installed

If the keyboard should be checked at system boot, check this box.

Should the keyboard not be checked or if there is no keyboard

installed (e.g. LAN server), leave it empty.

Auto detect hard disks

You don't need to enter the parameters of your hard disks manually

if you have IDE type drives installed (maybe this works with ESDI

drives, too). AMIsetup can detect the correct cylinder/head/sector

values automatically (non-OS/2 systems only).

��[�]����������������� Confirm ���������������������ͻ

�                                                    �

�               Install the following                �

�              hard drives into setup?               �

�                                                    �

�  HDD 0: Conner Peripherals 170MB, CP30174E         �

�  HDD 1: Not installed / SCSI                       �

�                                                    �

�                                                    �

�           Yes   �      No    �     Help   �        �

�         ���������    ���������    ���������        �

����������������������������������������������������ͼ

If no hard disks are found, it's most likely you have SCSI type

drives (or old MFM or RLL drives).

In the case of MFM/RLL type drives, you must enter the disk

parameters manually; this also holds true for some SCSI

controllers.

HDD 0

HDD 1

With this you can set the type of installed MFM, RLL or IDE

hard drives. Using a SCSI controller, you mustn't define a hard disk

here - please read your controller and hard disk manual to prevent

momentous wrong settings!

The current parameters are displayed below the push-button - type

(number) of the disk, cylinders, heads, sectors per track and the

calculated size.

Note: The disk size is displayed in real Megabytes, 2^20 bytes.

Many disk vendors calculate in 10^6 bytes (this number is larger!).

If you push the button, a list of predefined disk types appears:

��[�]��������� Hard disk 0 parameters ������������ͻ

� Type� Cyls. �Heads� Precomp� LZone �S/Trk�  Size �

��������������������������������������������������ĺ

�  32 �  1024 �  15 �   NONE �  1024 �  17 �  128M >

�  33 �  1024 �   5 �   1024 �  1024 �  17 �   43M �

�  34 �   816 �  15 �   NONE �   816 �  32 �  191M �

�  35 �  1024 �   9 �   NONE �  1024 �  17 �   77M �

�  36 �  1024 �   8 �    512 �  1024 �  17 �   68M �

�  37 �   615 �   8 �    128 �   615 �  17 �   41M �

�  38 �   745 �   4 �    512 �   745 �  28 �   41M �

�  39 �   987 �   7 �    987 �   987 �  17 �   57M �

�  40 �   820 �   6 �    820 �   820 �  17 �   41M �

�  41 �   977 �   5 �    977 �   977 �  17 �   41M �

�  42 �   981 �   5 �    981 �   981 �  17 �   41M �

�  43 �   755 �  16 �   NONE �   755 �  17 �  100M �

�  44 �   887 �  13 �   NONE �   887 �  34 �  191M �

�  45 �   968 �  10 �   NONE �   968 �  34 �  161M �

�  46 �   751 �   8 �      0 �   751 �  17 �   50M �

�� 47 � USER DEFINED�        �       �     �      �<

��������������������������������������������������ͼ

The columns contain type (number) of the disk, number of cylinders,

heads, the precompensation cylinder (see glossary), landing zone,

number of sectors per track and the calculated size (rounded) in

megabytes. Please compare these items with the data sheet of your

hard disk.

It's likely you CANNOT find the specifications of the hard disk to

be installed in this list. In this case, select type 47 (USER

DEFINED).

The following dialogue appears:

��[�]�������  Change type 47 parameters  �����������ͻ

�                                                    �

�                                                    �

�     615 �Cylinders                                 �

�                                                    �

�       4  Heads                                     �

�                                                    �

�     300  Precompensation (65535 for NONE)          �

�                                     [  Save   ]    �

�     615  Landing zone                              �

�                                     [ Cancel  ]    �

�      17  Sectors/track                             �

�                                     [  Help   ]    �

�                                                    �

����������������������������������������������������ͼ

Here you can set the parameters yourself. Note that the value of

precompensation has a special meaning: 65535 means DISABLED for the

whole hard disk. 0 means ENABLED for the whole disk.

Modern drives ignore precompensation and landing zone. For reasons

of software compatibility, set precompensation to "NONE" and landing

zone to the highest available cylinder+1.

3.4.2 Extended/chipset setup

����������������������������

A list of options and their current settings is displayed:

��[�]����������������������� Extended setup ���������������������������

�            Option            �Register�Bits � Current setting

�����������������������������������������������������������������������

��  Typematic Rate Programming � 11  10000000 � 10000000  Enabled     �

�   Typematic Rate Delay (msec)� 11  01100000 � 00000000  250

�   Typematic Rate (Chars/Sec) � 11  00011111 � 00000011  21.8

� � Mouse Support Option       � 13  10000000 � 10000000   <1>

�   Above 1 MB Memory Test     � 13  01000000 � 00000000  Disabled

�   Memory Test Tick Sound     � 13  00100000 � 00000000  Disabled

�   Memory Parity Error Check  � 13  00010000 � 00010000  Enabled

�   Hit <ESC> Message Display  � 13  00001000 � 00000000  Disabled

�   Hard Disk Type 47 Data Area� 13  00000100 � 00000000  0:300

�   Wait For <F1> If Any Error � 13  00000010 � 00000010  Enabled

�   System Boot Up Num Lock    � 13  00000001 � 00000001  On

� � Numeric Processor          � 14  00000010 � 00000010  Present

�   Weitek Processor           � 2D  10000000 � 00000000  Absent

�   Floppy Drive Seek At Boot  � 2D  01000000 � 00000000  Disabled

�   System Boot Up Sequence    � 2D  00100000 � 00000000  C:, A:

�   System Boot Up Speed       � 50  00000100 � 00000000  High

�   Cache Memory               � 2D  00001000 � 00001000  Enabled

� � Internal Cache Memory      � 2D  00000100 � 00000100   <1>

�����������������������������������������������������������������������

The four columns have the following meaning:

Option

The name of the setting to be changed, peeked from the BIOS.

The signs "�" and "�" denote options that cannot be changed from

the BIOS and are marked as "disabled" (see explanation under

"Options|Display ...").

If you change settings of options marked in this way, be especially

careful. You can disable the display of these options selectively

in the "Options" dialogue.

By the way, blame the BIOS manufacturer for reversed letters in this

row, not AMIsetup.

Register

Hexadecimal number of the CMOS register that stores the setting.

This information may help experienced users to change settings

by hand using "Change register". If a "+" sign is displayed behind

the register number, possibly more than one registers are affected.

Bits

Bit mask for the setting. Only Bits marked with a "1" are used

for the specified setting (see also Register).

Current setting

Bit mask of the setting stored in the program copy of CMOS RAM

and the corresponding text in clear. If plain language is not

available, a decimal value in acute brackets is shown. E.g.: <2>

denotes the third possible setting, since <0> is valid, too.

Now you can select an option with the mouse or the enter key, as

appropriate. A new window is displayed (see below).

Instead of pressing the enter key, <Ctrl><RightArrow>, <Ctrl><Enter>

or <Ctrl><LeftArrow> increase or decrease the current setting

respectively by one (for the hurried user).

List:                                       Radio buttons:

��[�]�������� Typematic Rate (Chars/Sec)    ��[�]���� Typematic Rat

�                                           �

�   �Options                 Bits           �   �Options

�     30.0             <     00000011       �    �(x) Disabled�

�     26.7             �    �00011111�      �     ( ) Enabled

�     24.0             �                    �

�    �21.8            ��              [     �

�     20.0             �                    �

�     18.5             �              [     �

�     17.1             �                    �

�     16.0             �              [     �

�     15.0             >                    �

�                                     [     �

�                                           �

����������������������������������������    �����������������������

Marked below "Options" or "Setting" is the current setting. To change

the setting, scroll the list or press the appropriate button.

The "Bits" field shows you which bit mask corresponds to the selected

setting. Listed below the bit field are once again the bits that can

be affected by the option.

"Save" copies the changed setting into the program copy of CMOS RAM

(use "Save changes and quit" to make the change permanent), "Cancel"

cancels the operation.

Caution: Some settings are "Reserved" or "RESERVED". Do consider

whether you really want to set an option to "Reserved". For safety

reasons, these changes have to be confirmed.

"BIOS help": Sometimes, the BIOS ROM has an own (English) help text

to the respective option. You can view this text using "BIOS help",

e.g.:

��[�]��������� BIOS help �������������ͻ

�The base address changes in           �

�steps of 16k and 512k                 �

��������������������������������������ͼ

"Reference": Registered users can look up explanations to the setup

options.

Options that are controlled entirely by the BIOS (these usually change

more than eight bits and are marked with a "+") display a window

containing the buttons "� < > �"; you can also use (<Ctrl>)<RightArrow>

and (<Ctrl>)<LeftArrow>. The buttons "� �" are intended for fast

leafing and don't work on all machines. On some machines, a window

appears after pressing "� < > �". Here you can enter data - this input

window is controlled by the BIOS, so mouse input and function keys are

disabled.

Setting controlled by BIOS:

��[�]�������� Non-Cacheable Block-0 Base �����������ͻ

�                                                    �

�                                                    �

�                                                    �

�                                      [BIOS help]   �

�   Setting                                          �

�   _0 KB_______________________       [Reference]   �

�                                                    �

�    [ � ]  [ < ]  [ > ]  [ � ]        [  Save   ]   �

�                                                    �

�                                      [ Cancel  ]   �

�                                                    �

�                                      [  Help   ]   �

�                                                    �

����������������������������������������������������ͼ

Note that some options affect each other!

3.4.3 Change register

���������������������

You may not be able to set the CMOS the way you want from the

"Extended/chipset setup" menu. In this case, you can make the change

manually.

Caution: Use this function only if you are absolutely, positively sure

about what you're doing!

��[�]����� Change register ��������ͻ

�                                   �

�  Enter register number:           �

�   __   Range 10 to 7F (hex)       �

�                                   �

�                                   �

� [ Change ]  [ Cancel ]  [ Help ]  �

�                                   �

�����������������������������������ͼ

��[�]��� Change register 12 �������ͻ

�  Current value: 00                �

�                                   �

�  Enter new value:                 �

�   __   Range 00 to FF (hex)       �

�                                   �

� [  Set   ]  [ Cancel ]  [ Help ]  �

�                                   �

�����������������������������������ͼ

First, you have to enter the register number in hexadecimal and then

the new value.

3.4.4 Password

��������������

This function enables you to display the current CMOS password and to

change it.

This password can be called for at system boot or when entering the

ROM setup.

��[�]���������� Password �������������ͻ

�                                      �

� Current password is "AMISET".        �

�                                      �

�                                      �

�                                      �

�  [Change]    [Cancel]    [ Help ]    �

�                                      �

��������������������������������������ͼ

��[�]������ Change password ����������ͻ

�                                      �

�  New password: FIDO__                �

�                                      �

�                                      �

�                                      �

�  [  OK  ]    [Cancel]    [ Help ]    �

�                                      �

��������������������������������������ͼ

Caution! If you use a foreign language keyboard (i.e. non-USA),

note that no foreign keyboard support is loaded at boot time.

Example: On German keyboards, "Z" and "Y" are reversed.

Often you can enable the US-American keyboard by pressing

<Ctrl><Alt><F1> and then test which keys are which. Use <Ctrl><Alt><F2>

to re-enable the foreign keyboard.

On some machines the password function has to be activated by the

"Password" option in "Extended/chipset setup" (3.4.2).

CAUTION! If you set the "Password" option to "Always", you cannot

boot your machine if you forget the password!

Hint: Try the setting "Setup" first.

3.4.5 Load ROM setup defaults, load ROM power-on defaults

���������������������������������������������������������

There are two tables of predefined CMOS settings in the ROM. One of

those, the "power-on defaults" is used to eliminate all possible

hardware problems; thus the processor is switched to low speed, the

cache is turned off, etc. Using "power-on defaults", the machine should

boot in any case.

Using this function, you can load a predefined configuration or simply

view it for reference purposes, since all settings will be written to

the program copy of CMOS RAM first and only be saved if you use "Save

changes and quit".

3.4.6 Print current setup

�������������������������

With this function, you can print the current settings of the program

copy of CMOS RAM to the printer or to a file.

��[�]�������� Print current setup �����������ͻ

�                                             �

� �Print to file                              �

�   PRN______________________________         �

�                                             �

�  Format                                     �

�   (x) Standard (current/possible)    Margin �

�   ( ) Alternate (current bin&hex)     79_   �

�                                             �

�                                             �

�  [  OK  ]    [Cancel]    [ Help ]           �

�                                             �

���������������������������������������������ͼ

First the settings of "Standard setup" will be printed; then, a list:

Standard

The list is formatted as follows:

Option            � Current  � Possible settings

��������������������������������������������������������������

Typematic Rate Programming   � Enabled  � Disabled

Typematic Rate Delay (msec)  � 250      � 500 750 1000

Typematic Rate (Chars/Sec)   � 24.0     � 30.0 26.7 21.8 [...]

� Mouse Support Option       � <1>      �

Above 1 MB Memory Test       � Disabled � Enabled

...

"Margin" (61-254) is the maximum width of the list. "Possible

settings" will be truncated to "[...]" if exceeding the maximum

width.

Alternate

The list looks as the list displayed in "Extended/chipset setup".

"Margin" is ignored.

Option            �Register�Bits � Current setting

����������������������������������������������������������������

Typematic Rate Programming   � 13  10000000 � 10000000  Enabled

Typematic Rate Delay (msec)  � 13  01100000 � 00000000  250

Typematic Rate (Chars/Sec)   � 13  00011100 � 00010000  15

� Mouse Support Option       � 11  10000000 � 10000000  Enabled

...

Note: If your printer cannot print extended characters, disable them

in the "Options" dialogue.

Hint: If you disable "�" and "�" in the "Options" dialogue, you can

create a list of those settings that can be changed from the ROM

setup. This list you can distribute e.g. to your customers as a

reference.

3.4.7 Analyze ID string

�����������������������

This function explains the meaning of the BIOS ID string displayed

in the lower left corner.

Note: The information determined by AMIsetup is not always 100 per cent

correct, since not all manufacturers adhere to the standard (this is

especially true for "Required processor").

The ID string displayed at boot time shows the keyboard controller

revision level in addition to the information displayed by AMIsetup.

Example: 30-0201-ABCDEF-00101111-050591-OPWB-KF, where F is the

revision level.

���������������� Analyze BIOS ID string ��������������ͻ

�                                                      �

�  30-0201-ABCDEF-00101111-050591-OPWB                 �

�  23H-0-0000-00-00-0000-00-00-000                     �

�  23H-1-0000-00-00-0000-00-00-00-2                    �

�                                                      �

�  Required processor:   386                OK    �    �

�  BIOS size:            64 KB           ����������    �

�  Version:              2.01                          �

�  Reference number:     ABCDEF           Print   �    �

�  BIOS date:            05/05/91        ����������    �

�  Chipset ID:           OPWB                          �

�  Clock switching pin:  23                Help   �    �

�  Cache control pin:    23              ����������    �

�  BIOS modified flag:   02                            �

�                                                      �

������������������������������������������������������ͼ

Use "Print" to print the information or to write it into a file.

3.4.8 Save changes and quit

���������������������������

Not until you use this function will changes be made permanent. "Save

changes and quit" writes all CMOS settings from the program copy of

CMOS RAM into the real CMOS RAM and exits setup.

If "Options|Enable reboot" is enabled, you will be asked whether you

want to reboot your computer (cf. "Reboot").

3.4.9 Exit without saving

�������������������������

Quits setup without changing the CMOS RAM.

3.5 Options dialogue

��������������������

��[�]���������������������� Options ��������������������������ͻ

�                                                              �

�  Setup                         Display                       �

�   [ ] Use BIOS image            [X] � Options                �

�   [X] Auto start setup          [ ] � Options                �

�   [ ] Print form feed                                        �

�   [X] Extended character set    [ ] Soundblaster             �

�   [ ] Enable reboot command                                  �

�   [X] Beep on error                                          �

�   [ ] Reverse mouse buttons      Password  �                 �

�   [ ] German language           ������������                 �

�                                                              �

�  BIOS image file               CMOS and EISA image files     �

�   C:\SETUP\BIOS.DAT             C:\SETUP\                    �

�                                                              �

�                                                              �

�    Save  �    Cancel �     Help  �                           �

�   ��������    ��������    ��������                           �

��������������������������������������������������������������ͼ

Customize the program to your own needs using this dialogue.

Use BIOS image

Usage of an image file instead of peeking directly into the ROM. See

"BIOS|Write image to file" for more information.

Auto start setup

Instead of selecting "Setup" manually each time using the menu,

"Setup" can also be started automatically at program start

(registered copies only).

Print form feed

Do a form feed after printing.

Extended character set

This box must be left blank if your printer doesn't print the extended

character set (code page 437), e.g. characters like "�" or "�".

Enable reboot command

If this box is blank, the "Reboot" function in the "�" menu is

disabled and you will not be asked whether you want to reboot

after saving your setup.

Beep on error

If this option is checked, the program beeps if you enter incorrect

values in (hexa-) decimal input boxes.

Reverse mouse buttons

You can reverse right and left mouse buttons (for left-handers or

southpaws).

German language

You can disable the English user interface and work with the German

one instead.

Display

... �

... �

There are two types of disabled options in the ROM BIOS.

1. Some options are actually there, but marked as "inadmissible".

This can be e.g. redundant options. AMIsetup displays a "�"

in front of the option. Mostly the "�" options work perfectly.

2. Some options are marked as "inadmissible" AND there are no valid

known settings. AMIsetup then displays a "�". Often "�" options

don't work.

You can enable and disable both types. If neither "�" nor "�" are

enabled, then AMIsetup shows the same set of options as the ROM

setup.

Soundblaster

See appendix G.

BIOS image file

CMOS and EISA image files

You can change file name/directory of these files.

Password

You can define a password that will be needed at program start.

Password protection reduces misuse of the AMIsetup.

��[�]������ Change password ����������ͻ

�                                      �

�  New password:  ****                 �

�                                      �

�  Confirm:       ****                 �

�                                      �

�                                      �

�                                      �

�     OK   �    Cancel �     Help  �   �

�   ��������    ��������    ��������   �

��������������������������������������ͼ

For security reasons, you have to enter the same password in both

lines.

If you enter an empty password (simply press Enter), the password

protection will be disabled.

Changes of the password are written into the file AMISETUP.EXE,

which must not be write protected for this operation.

3.6 Help menu

�������������

�  BIOS  CMOS  Setup  Options  Help

������������������������������� �������������������Ŀ ����������

������������������������������� � Contents          � ����������

������������������������������� � View AMISETUP.DOC � ����������

������������������������������� � Print order form  � ����������

������������������������������� �������������������Ĵ ����������

������������������������������� � On-line reference � ����������

������������������������������� ��������������������� ����������

����������������������������������������������������������������

Contents

Gives an overview about on-line help.

View AMISETUP.DOC

You can view AMISETUP.DOC without leaving AMIsetup. It is necessary

that AMISETUP.DOC be installed in the same directory as AMISETUP.EXE.

You can leave the AMISETUP.DOC window in the background while

working with AMIsetup.

Print order form

To order AMIsetup, you can print an order form to the printer or

to a file.

On-line reference

Registered users (read appendix A how to become a registered user)

can read explanations of approx. 100 of the most widespread setup

options.

Select "On-line reference", and the index is displayed.

��[�]�������������������������� On-line reference ������������������

�

�            � Index

�            �������

�

�    General advice

�    ��������������

�

�  Caution!

�  Beep codes

�

�

�    Tuning tips

�    �����������

�

�

�    Setup options

�    �������������

�

�  256KB�Memory�Relocation

�  256K/384K�Memory�Relocation

�  7CLK2�CAS�Pulse�Width

��#������������������������������������������������������������������

You can mark one of the bold terms using <Tab> or the mouse and get

a closer explanation:

��[�]�������������������������� On-line reference ������������������

�

�       Fast Gate A20 Option  [Disabled] [Enabled]

�       Gate A20 Emulation Option

�

� Activates a faster method for enabling and disabling address line

� needed for accessing extended memory.

�

� The address line A20 is normally toggled by the keyboard controlle

� comparably slow 8042 processor.

�

� For reasons of compatibility with PC/XT systems (8086/8088), addre

� A20 is disabled under DOS in most cases. PCs with the 8088 had onl

� address lines up to A19. When on the 8088 PC address FFFF:XXXX (se

� first 64KB was actually addressed.

�

� In order to access memory above 1MB (extended memory) on 286 or ne

� address line A20 must be toggled. This also holds for switching be

� line A20, allowing the first 63 KB of extended memory to be access

� mode (and used as "high memory").

�

� Control by the keyboard controller can be replaced by a faster tec

� option became famous as "OS/2 optimization".

�

� Modern software uses other methods of switching (triple fault or 3

��#�����������������������������������������������������������������

Use the <Tab> key to switch to the next highlighted topic (the

cursor keys are for scrolling only).

4 Key shortcuts

���������������

<F1>         Help

<Alt><F3>    Close active window

<F5>         Zoom active window

<Ctrl><F5>   Move active window (Cursor keys)

Change window size (<Shift>+Cursor keys)

<F9>         Start Setup

<F10>        Activate main menu

<Shift><F10> Show program information

<Alt><X>     Exit Setup or program

or <Alt><F4>

NB:  <#>            = <Shift>

<Return>, <#�> = <Enter>

<#>            = <Tab>

A P P E N D I X

���������������

A  Registration

���������������

If you want to continue using AMIsetup after 30 days, you have to

register.

The shareware version is limited in the following ways:

� The on-line reference is disabled

� Starting the setup automatically doesn't work

See REGISTER.FRM for the registration fee valid for your

country (DM 40.- to DM 55.- depending on the country), REGISTER.GER

for Germany, REGISTER.JPN for Japan.

Commercial users in the European Union ONLY: Please provide your

VAT registration number and save 15% VAT!

Inquire for more than 2 licenses!

Send DM Eurocheques (other cheques: add DM 10.-), cash or postal

money order to the following address:

Robert Muchsel

Hegaublick 2

D-78465 Konstanz

Germany

FAX +49-7533-3151

�������������������������������������������������������������������Ŀ

� Please use the form "REGISTER.FRM" included with this program.    �

� You can print it using "Help|Print order form" directly from      �

� AMIsetup.                                                         �

���������������������������������������������������������������������

�������������������������������������������������������������������Ŀ

�  Email:                                                           �

�            FidoNet 2:246/8100.11                                  �

�            mccs BBS, Singen/Germany, +49-7731-69523               �

���������������������������������������������������������������������

AMIsetup is shipped on 3.5" disks. If you really need a 5.25" disk,

please indicate <5.25"> in BIG letters in the upper right corner of

the order form.

Upgrades

��������

If the user has a modem, upgrades are free. Other registered users

only have to pay the fee of their shareware vendor.

Upgrades will not be free if

- there are too many illegal pirated copies

- there are major enhancements

Read README.!!! on your registration diskette for more information.

The Author

����������

...studies Computer Science at the ETH (Swiss Federal Institute of

Technology) in Zurich/Switzerland (this is the explanation of the

"Additional contributions" field on the order form).

B  AMIsetup and OS/2 2.x, Windows, Windows NT

���������������������������������������������

OS/2 Version 2.1

����������������

The program will run in a DOS session of OS/2 2.1. DMA clock speed

detection and "Auto detect hard disks" will be disabled, however.

OS/2 Version 2.0

����������������

The program will not run in a DOS session of OS/2 2.0, not even in a

DOS image (VDM). Write accesses to the CMOS RAM are redirected to a

buffer by OS/2, which is discarded when exiting the virtual machine

(source: "Redbooks, Vol. 2: DOS and Windows-Environment, Virtual CMOS

Device Driver"; IBM Document Number GG24-3731-00).

An OS/2 version of AMIsetup is impossible, since AMIsetup directly

executes code of the real mode BIOS.

If you create a DOS boot disk and copy AMIsetup as well as a disk

editor there, then you've got a sort of "emergency disk".

AMI BIOS and OS/2

�����������������

There are NO general problems with OS/2 2.x and the BIOS versions

supported by AMIsetup. OS/2 runs with BIOS versions PRIOR to 05/05/91

and with keyboard controller revision -K8.

However, problems might arise e.g. when using faulty memory chips,

too low refresh rates, wrongly installed cache memory or wrong

bus clock. Because of the protected mode architecture, OS/2 is much

more sensitive to errors than DOS software, which often ignores

or doesn't even notice them.

Windows

�������

Running AMIsetup under Windows is not recommended.

The icon file, AMISETUP.ICO, contains an OS/2 icon which cannot

be displayed under Windows.

Windows NT

����������

The Windows NT DOS emulator is not compatible with AMIsetup.

C  Known problems

�����������������

1. Is AMIsetup compatible with your BIOS?

For technical reasons, AMIsetup only works with the newer generation

of the AMI BIOS, the so-called "Hi-Flex BIOS".

On one hand, the BIOS must be manufactured by "American Megatrends

Inc." ("AMI") (this you can see when powering on your computer).

Example of power-on screen:

���������������������������������������������������������������Ŀ

� 05/05/91(C)1990 American Megatrends Inc., All Rights Reserved �

�                                                               �

� (C)1990 American Megatrends Inc.,                             �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

� 30-0101-ABCDEF-00011001-050591-FORX                           �

�����������������������������������������������������������������

On the other hand, a compatible BIOS has a colored ROM setup

(or built-in setup) and normally three levels of setup ("Standard",

"Advanced" and "Advanced Chipset"), "Password" and "Hard Disk

Utility".

��������������������������������������������������������������ͻ

�          BIOS SETUP PROGRAM-AMI BIOS SETUP UTILITIES         �

�    (C) 1990 American Megatrends Inc., All Rights Reserved    �

��������������������������������������������������������������͹

�                                                              �

�                     STANDARD CMOS SETUP                      �

�                     ADVANCED CMOS SETUP                      �

�                   ADVANCED CHIPSET SETUP                     �

�            AUTO CONFIGURATION WITH BIOS DEFAULTS             �

�          AUTO CONFIGURATION WITH POWER-ON DEFAULTS           �

�                       CHANGE PASSWORD                        �

�                      HARD DISK UTILITY                       �

�                    WRITE TO CMOS AND EXIT                    �

�                DO NOT WRITE TO CMOS AND EXIT                 �

�                                                              �

��������������������������������������������������������������͹

�                                                              �

�  Standard CMOS Setup for Changing Time, Date, Hard Disk, etc.�

�                                                              �

�����͵ ESC:Exit |-|-:Sel F2/F3:Color F10:Save & Exit ��������ͼ

Custom-made BIOS versions are NOT compatible with AMIsetup, even

though they are from AMI. These have a so-called "XCMOS Setup" (and

they are black & white).

������������Ŀ

2. Frequent errors with � compatible � BIOS versions

��������������

�����������������������������������������Ŀ

� Checking 34 .. 3F:              *ERROR* �

�������������������������������������������

Possible causes:

- A program has overwritten your CMOS. Restart your PC and follow

the instructions of the ROM BIOS.

�����������������������������������������Ŀ

� Checking "AAAAMMMMIIII":        *ERROR* �

�������������������������������������������

�����������������������������������������Ŀ

� Checking "American Megatrends": *ERROR* �

�������������������������������������������

�����������������������������������������Ŀ

� CX overflow - BIOS not compatible!      �

�������������������������������������������

Possible causes:

- You are using a memory manager that overwrites parts of your

BIOS. Create a BIOS image following instructions in 3.2.

- You are using an image file that is faulty (it was created while

a memory manager was loaded?).

Repeat the procedure EXACTLY following instructions (part 3.2)

- Your version of AMIsetup is obsolete

�����������������������������������������Ŀ

� Setup incomplete - may not work!        �

�������������������������������������������

Your BIOS seems to be hacked. There could be some options that

AMIsetup doesn't display. If your image file is OK (see above),

your BIOS may have been patched by the manufacturer.

���������������������������������������������������Ŀ

� The BIOS register table contains invalid entries! �

� Register: XXh, CMOS RAM = ... Bytes               �

�����������������������������������������������������

Your ROM setup defines values for CMOS registers that don't

exist. AMIsetup ignores all missing registers, so pressing "OK"

is perfectly OK.

All entries where AMIsetup reports a register >= XXh will contain

garbage values, changes made to these registers will be ignored.

This error is caused either by an EISA BIOS (additional setup

information is stored in a separate EISA NVRAM, see appendix H)

or a BIOS that doesn't fit to your hardware. This error also

occurs when running AMIsetup under OS/2 2.0 (not 2.1).

If you have Headland's SHASTA chip set, you can modify your

computer (you'll need a new clock chip and a BIOS patch). Other

than this, there is no way to circumvent the problem.

3. Stopgap: -OVERRIDE

As a last resort you can start the program with the command-line

option "-OVERRIDE". This option disables all error messages;

faulty operation or a locked-up computer may  result, though.

4. Additional trouble shooting options

While loading, AMIsetup displays the current initalisation phase.

If the program hangs, you might try the following phase dependent

switches: "-NOPCI" (on ISA/EISA systems) or "-PCI" (on PCI

systems), if the PCI check fails. If the EISA detection hangs,

edit EISA.CFG. If the program fails in any other phase, please

contact the author.

If the program hangs when you start the "Extended setup",

you may want to try the parameter "-NOROMJUMPS".

There are two more parameters built into the program,

"-EOBIOS:xxxx" and "-IMAGE:xxxx". Please use these two parameters

only if the author tells you to do so.

D  Glossary

�����������

Some frequently used abbreviations and terms (explanations for setup

options only on-line and only for registered users):

AMI

American Megatrends Inc., a BIOS manufacturer

BIOS

Basic routines contained in a nonvolatile memory (ROM) of the PC

which e.g. drive the hard disk, but are also needed for system boot

and system setup.

BIOS Checksum

The AMI BIOS calculates a 16 bit checksum (addition w/ overflow) over

the BIOS ROM. If this checksum is 0, it is assumed that the BIOS

is OK.

CMOS RAM

This chip can be read from and written to; unlike normal memory the

contents are buffered by a battery or an accumulator after turning

off the PC. CMOS RAM contains data like type of installed floppy

disk drives, the time, etc.

CMOS RAM Checksum

A checksum is calculated for parts of the CMOS and stored in CMOS.

If checksum and CMOS RAM contents differ, either the battery is

defect or the CMOS RAM contents are invalid. In any case you must

reenter the data, possibly replace the battery or accumulator,

too.

EISA

See appendix H.

Extended Character Set, Graphic characters

Characters like ��� or ��. Some printers cannot print these.

NVRAM

See appendix H.

Precompensation

Precompensation is used with some hard disks to compensate for the

higher recording density in the inner tracks during write.

Normally, precompensation and reduced write current begin

approximately at half the cylinder count. The precompensation

logic shifts certain bit patterns by +/- 12-15ns.

Precompensation only affects writing, not reading of data.

Many hard drives work perfectly with or without precompensation,

newer hard disks ignore the setting (you should turn it off then).

ROM

Read Only Memory - memory that can only be read from and not

written to.

E  If the machine won't boot...

�������������������������������

If the setup has been altered so that the PC doesn't even display

"Press <DEL> for Setup", some models have the following possibilities:

1. On newer models, you can press <Ins> while powering on. All

settings will be reset to standard values.

2. On many machines there is a jumper on the main board which you can

use to erase CMOS RAM. Sometimes this takes several hours.

3. Often there is the possibility of connecting an external battery

pack. To install it, you have to remove a jumper residing on a

4 pinned connector. This jumper does the same as (2).

4. Only when encountering password errors and only with BIOS from

12/12/91 on: Pin 32 of the keyboard controller can be grounded

to erase the password.

Please note that after these steps all configuration data must be

reentered. Make certain that you set the correct hard disk parameters!

If you are not a hardware expert, you should contact your dealer first

before destroying your hardware by wrong handling!

F  Switching to the English user interface

������������������������������������������

If you have installed the German version and want to switch,

do the following:

Disable "German language" or "Deutsche Oberfl�che" in the "Options"

dialogue.

Select OK, exit and restart AMIsetup.

G  Soundblaster

���������������

To make the monotonous task of setting up computers a little more

fun, AMIsetup supports the Soundblaster audio card.

Soundblaster support must be explicitly enabled (AMIsetup is, after

all, a serious program).

- Turn on "Soundblaster" in the "Options" dialogue (default: OFF)

- The environment variable "BLASTER" must be set correctly, e.g.:

SET BLASTER=A220 I5 D1

The BLASTER variable has the following meaning:

A220 : Port address of Soundblaster card

I5   : IRQ (Interrupt) level

D1   : DMA channel

(all other settings, e.g. T4, will be ignored)

If the variable is NOT set, the default values A220 I7 D1 will

take effect.

Note: The program could crash if a wrong DMA channel is given!

- You must have the VOC files.

Included are:

CONFIRM.VOC  - Confirmation

COPYRIGH.VOC - About AMIsetup

ERROR.VOC    - Replaces the error beep

ERRORBOX.VOC - Error

EXIT.VOC     - Program exit

WARNING.VOC  - Warning

Not included:

HELP.VOC     - Help

AMIsetup has its own, built-in Soundblaster driver. This built-in

driver should work with all Soundblaster and compatible cards with

DSP level 1.05 and higher.

If the driver CTVDSK.DRV is included with your Soundblaster card,

AMIsetup can use it and adapt itself to the special features of your

card (e.g. Stereo sound).

AMIsetup looks for CTVDSK.DRV in the following directories:

1. in the current directory

2. in the directory of AMISETUP.EXE

3. in the subdirectory "DRV\" of the directory specified by the

environment variable "SOUND"

4. in the subdirectory "DRV\" of the directory specified by the

environment variable "CPSPRO4"

5. in the PATH

The built-in driver will only be activated if CTVDSK.DRV cannot be

found.

You can delete single VOC files and the corresponding action will be

"quiet".

You can replace these files with files from your own sound collection.

The PLAY.EXE program (included with AMIsetup) can also play files

from the command line ("PLAY Name"). Please send your own, better

collection of VOC files to the author!

AMIsetup can play files any size you want; if disk activity is too

heavy, you can increase the replay buffer which defaults to 4*4 KB.

Set the DOS enviroment variable SBBUF to the desired value.

Example: SET SBBUF=16 -> replay buffer will be set to 4*16 KB.

All sounds are turned off immediately when the corresponding

action is aborted/finished.

If the program hangs when you activate Soundblaster support, you can

force the program to use its built-in driver. Type "SET SOUND=" at the

DOS command prompt.

Soundblaster support might fail on Media Vision sound cards. The

reason of this incompatibility is unknown.

AMIsetup Soundblaster support needs a lot of memory. If this is an

issue, please consider disabling Soundblaster support.

H  EISA

�������

EISA is an "Extended" and faster bus architecture found in better

machines. AMIsetup tells you whether you have a standard "ISA" or

EISA system.

On some EISA computers, parts of the setup data are stored in the

EISA NVRAM (Non Volatile RAM). On these machines, AMIsetup needs to

access the NVRAM. On others, AMIsetup doesn't need to know about EISA

NVRAM and simply ignores it.

Since EISA NVRAM access is chipset dependent, AMIsetup uses EISA.CFG

(see EISA.CFG for explanation of the syntax).

If AMIsetup detects an EISA system and EISA.CFG contains valid data,

"CMOS�Write EISA NVRAM to file" and "CMOS�Restore EISA NVRAM from

file" are automatically enabled. Setup registers 40h to 7F are mapped

into the EISA NVRAM.

However, it is possible that AMIsetup doesn't know about YOUR

particular EISA computer and displays "The BIOS register table

contains invalid entries!". Please mail a copy of your BIOS.DAT,

CMOS.SAV and EISANVR.SAV (if available). EISA.CFG can then be extended

to include your computer.

I   Batch mode

��������������

You can use AMIsetup in batch mode. This is especially useful if you

- want to configure several identical machines (you don't have to use

time consuming ECU, CMOS setup any more!)

- want to document CMOS settings and/or save CMOS/EISA NVRAM

automatically

The batch mode of AMIsetup understands the following commands:

-SAVE:CMOS:FILENAME          Save CMOS RAM into FILENAME.

-SAVE:EISA:FILENAME          Save EISA NVRAM into FILENAME.

-RESTORE:CMOS:FILENAME       Restore CMOS RAM from FILENAME.

-RESTORE:EISA:FILENAME       Restore EISA NVRAM from FILENAME.

-PRINT:DEVICE                Print setup to DEVICE.

-PRINT:ALT:DEVICE            Print setup (alternate style) to

DEVICE.

Examples:

You want to save CMOS & EISA NVRAM in a file:

AMISETUP -SAVE:CMOS:CMOS.BIN

AMISETUP -SAVE:EISA:EISA.BIN

And now configure several more machines with the same settings:

AMISETUP -RESTORE:CMOS:CMOS.BIN

AMISETUP -RESTORE:EISA:EISA.BIN

Finally, print setup to a parallel printer:

AMISETUP -PRINT:LPT1

J   Obtaining new or bug fixed program revisions

������������������������������������������������

There are lots of different BIOS/chipset combinations out there.

If the program doesn't work with your machine, please do the

following:

1. Read the manual, especially appendix C (you did, didn't you?).

2. Use the newest version of the program only. You can get it from

your shareware vendor or, if you have a modem, from the mccs BBS

in Singen/Germany.

mccs BBS Singen/Germany (Sysop Martin Grass, Fido 2:246/8100-8104)

Line 1: +49-7731-65833

Line 2: +49-7731-69523

Line 3: +49-7731-69524

Line 4: +49-7731-69525

Line 5: +49-7731-60195 (Multi line: Lines 1-5)

ISDN:   +49-7731-984026

FidoNet users: Request the file using the "magic" name AMISETUP.

Others: Please log in as "GAST", password "GAST" and download

AMISETUP directly from the main menu.

You can also get the AMIsetup shareware from the Internet.

SimTel ftp server and its mirrors (e.g. nic.switch.ch) carry the

newest version in the directory pub/msdos/sysutl. There might also

be a Compuserve-Internet gateway.

3. ONLY if the NEWEST version doesn't work and you are sure it should

work (appendix C!):

Send a BIOS image and a CMOS RAM image produced by AMIsetup to

the author.

Please follow the instructions in the manual; ensure that NO

memory manager is loaded.

Thank you.

K   Tested BIOS revisions

�������������������������

Among others, the program has been successfully tested on the

following chip set/BIOS combinations:

Chip set   � BIOS date  � ID string

�����������������������������������

ACC        � 12/12/91   � ACC2046

� 06/06/92   � ACC2046

�            �

Acer Logics� 07/07/91   � ACER1217

� 07/07/91   � ACER1409

� 12/12/91   � ACER1419

� 08/08/93   � ALI1429

�            �

ACT        � 07/07/91   � ACT

�            �

Am. Megatr.� 12/12/91   � AMI/S50

� 12/12/91   � AMI/S61

�            �

Contaq     � 07/07/91   � CTQ486

� 12/12/91   � CTQ 386

� 12/12/91   � CTQ386&486

� 12/12/91   � CTQAUTO

� 06/06/92   � CTQ596

� 11/11/92   � CTQ596

� 11/11/92   � CTQ596-A

� 11/11/92   � CTQ-596A

� 08/08/93   � CTQ596

�            �

Chips&Tech.� 02/02/91   � PEAKDM

� 05/05/91   � PEAKDM_B

� 05/05/91   � NEAT286

� 05/05/91   � C&T386

� 07/07/91   � PEAKDM

� 07/07/91   � NEAT386

� 07/07/91   � NEAT3SX

� 12/12/91   � PEAKDM_B

�            �

Headland   � 07/07/91   � HT12

(LSI)      � 12/12/91   � SUMMIT_C

� 11/11/92   � SHASTA    (may contain no Extended

�            �            CMOS RAM)

�            �

Eteq       � 02/02/91   � ET/386H

� 02/02/91   � ET/486H

� 05/05/91   � ET/486H

� 05/05/91   � ET/36H

� 07/07/91   � ETEQ486C

� 07/07/91   � ET/486H

� 07/07/91   � ETEQ/1C

� 12/12/91   � BENG486

� 12/12/91   � BENG386

�            �

Forex      � 05/05/91   � FORX

� 07/07/91   � FORX

� 12/12/91   � FORX

� 06/06/92   � FORX

� 04/04/93   � 4X521

�            �

Hint       � 07/07/91   � CAESAR

�            �

Intel      � 05/05/91   � 80486

� 05/05/91   � ITOPDX

� 07/07/91   � 80486

� 07/07/91   � ITOPDX

� 07/07/91   � ITOPSX

� 08/08/93   � SATURN

� 04/06/94   � PCI82430

�            �

Morse      � 05/05/91   � KP386DX

�            �

Opti       � 04/15/91   � OPWB

� 05/05/91   � OPWB

� 05/05/91   � 386WB

� 05/05/91   � OP386WB

� 06/06/91   � OPWBSX

� 06/06/91   � OPWB493

� 07/07/91   � OPWBSX

� 07/07/91   � OPWB3/B

� 07/07/91   � OP3DX/PI

� 07/07/91   � OPTIEISA

� 07/07/91   � OPBC

� 07/07/91   � OPWB

� 08/31/91   � OPWBDS

� 12/12/91   � OPEISA

� 12/12/91   � OPTIDXBB

� 12/12/91   � OPSXPI

� 12/12/91   � OPTI8290

� 12/12/91   � 0495AUTO

� 06/06/92   � OPT495SX

� 06/06/92   � OPWB4SXB

� 06/06/92   � OPT4 498

� 06/06/92   � OPDXBB-B

� 06/06/92   � OPTIEISA

� 06/06/92   � OPTIAUTO

� 06/06/92   � OPWBB/B

� 11/11/92   � OPWB4SXB

� 11/11/92   � OP495SLC

� 11/11/92   � OP495SX

� 08/08/93   � OP495SLC

�            �

PC Chip    � 07/07/91   � PC CHIP

� 12/12/91   � PC CHIP

�            �

PICO       � 06/06/92   � PICO168

�            �

SIS        � 02/02/91   � SISC

� 03/15/91   � SISA

� 03/15/91   � SISB

� 03/15/91   � SISC

� 05/05/91   � SIS-486

� 07/07/91   � SISAUTO

� 07/07/91   � SISD

� 07/07/91   � SIS486

� 12/12/91   � SIS-EISA

� 12/12/91   � SIS386D

� 06/06/92   � SISAUTO

� 06/06/92   � SIS3486

� 06/06/92   � SIS486

� 06/06/92   � SISEISA

� 06/06/92   � SIS-EISA

� 11/11/92   � SIS3486

� 11/11/92   � SIS461

� 12/12/92   � SIS-EISA

� 08/08/93   � SIS461

�            �

Symphony   � 03/15/91   � SYMP

� 05/05/91   � SYM-486

� 05/05/91   � SYMP

� 05/05/91   � SYM_386B

� 07/07/91   � SYM486

� 10/25/91   � SYMPH_1B

� 10/25/91   � SYM_386B

� 12/12/91   � SYMP

� 01/20/92   � SYM486B

� 11/11/92   � SYMP-461

�            �

Texas Instr� 12/12/91   � TIEISA/S

�            �

UMC        � 03/15/91   � UMCWB

� 05/05/91   � UMCWB

� 07/07/91   � UMC480A

� 07/07/91   � UMCWB

� 07/07/91   � U391B

� 07/07/91   � UCF/1

� 12/12/91   � UMCBAUTO

� 12/12/91   � UMCAUTO

� 06/06/92   � UMC480

� 06/06/92   � UMC4807

� 11/11/92   � UMC480

� 11/11/92   � UMC480B

� 11/11/92   � UMC480F

� 04/04/93   � UMC491

� 04/04/93   � UMC491C

� 08/08/93   � UMC480B

� 08/08/93   � UMC491F

�            �

UNI        � 12/12/91   � UNI480

� 12/12/91   � UNICHIP

� 06/06/92   � UNI-4800

� 11/11/92   � U4800VLX

�            �

USA        � 12/12/91   � USER

� 11/11/92   � USA-9204

�            �

VIA        � 07/07/91   � VIAFLEX

� 06/06/92   � VIAEISA   (requires "OFF" in EISA.CFG)

�            �

VLSI       � 05/05/91   � TOPCATSX

� 07/07/91   � SCMPSX

�            �

07/25/1994