XSUM			User's Manual			XSUM

NAME

crc, mdx - generate checksums of a file or files

SYNOPSIS

crc [options | filespec] ...

mdx [options | filespec] ...

VERSION

crc version 1.07

mdx version 1.05

DESCRIPTION

crc and mdx compute the checksum of a file or a set of files.

crc computes the 32-bit Cyclical Redundancy Check (CRC-32) and

mdx computes the 128-bit Message Digest (MD4 or MD5).

Running either one with no parameters will give a quick

summary of function and options.

OUTPUT

The output of both programs is written to standard output

(stdout), and can be redirected to a file.  The title and

any error messages will be sent to standard error (stderr).

The basic output of crc is formatted as:

e0c8a642  filename

and the basic output of mdx is formatted as:

e0c8a642e0c8a642e0c8a642e0c8a642  MDx  filename

where x is replaced by 5 or 4 to indicate MD5 or MD4.

In addition, each program has an extended output mode where extra

information is given about each file processed.  This information

includes the file size, the file time (with seconds) and the file

date (with century).

The extended output of crc is formatted as:

e0c8a642  1,234,567,890  23:36:18  01-Jan-1980  filename

and the extended output of mdx is formatted as:

e0c8a642e0c8a642e0c8a642e0c8a642  MDx  filename

1,457,664  08:45:58  05-Feb-1995

1

XSUM			User's Manual			XSUM

Be advised that a file which has its seconds set to 62 or its

year set ahead a century (e.g., 2095 instead of 1995) may be

infected with a virus.  This is not necessarily the case, but

there are some viruses which mark infected files in this way.

OPTIONS

filespec	The name of a file or a pattern which matches

several files.  The drive and path may be

specified, and there is no limit to the number

of files matched.  Also, as many filenames or

patterns as will fit on the command line may

be given.

The following options apply only to mdx:

-4	Use the MD4 algorithm.

-5	Use the MD5 algorithm (default).

-n	Don't use the faster 386 code.

-nn	Don't check for the presence of a 386 processor.

The -4 and -5 options may be used more than once on the command

line.  Whichever was last given affects which algorithm is used,

so MD4 and MD5 message digests can be computed on the same run.

For example,

mdx -4 *.exe *.com *.sys -5 *.txt *.doc *.man

would calculate the MD4 digests of all the programs in the

current directory, and the MD5 digests of all the text.  Or,

mdx foo.bar -4 foo.bar

would calculate both numbers for the file foo.bar.

The -n and -nn options allow you to run the standard MD5 code

instead of the faster 386-specific code.  The only reason you

would want to do this is to make a speed comparison.  Normally

mdx uses an enhanced 32-bit version of the MD5 translate

function, if it detects a 386 or better processor.  Just in case

this causes a problem (which it shouldn't), you can force mdx to

use the slower (but compatible) code with the -n switch, or even

suppress the test for the 386 entirely with the -nn switch.

2

XSUM			User's Manual			XSUM

The following options apply to both mdx and crc:

-h	show the help screen.

-e	display extended file information.

The -h switch shows the same usage summary that is given if

the programs are run with no parameters.

The -e switch selects the extended output format which

includes the file size, time, and date.

ERRORS

All errors are of the form prg: message where prg is either

crc or mdx.

prg: error opening file name

The file name could not be opened for reading.  It

may be locked by another application.

prg: error reading file name

An error occurred while reading file name.  The

disk may be damaged.

prg: no files found to process

None of the filespecs given on the command line

matched any files.

prg: program has been modified

On startup, the program checks itself to see if it

has been modified.  This provides some protection

against viruses however it will not detect stealth

viruses.

If the self-check fails, the program displays this

message and exits.

prg: integrity cannot be verified

If the self-check cannot be performed then this message

is displayed, and the program continues.  This could

happen if the self-checking information is tampered

with, or if the program is run under DOS 2.x.

3

XSUM			User's Manual			XSUM

Note that the self check code is smart enough not to be bothered

by the addition of validation data, which certain anti-virus

programs foolishly append to the executable file, nor is it

bothered if the compressed executables are expanded.

AUTHOR

crc and mdx were written by David R. Conrad, who can be reached

in the following ways, in order of preference:

Internet	ab411@detroit.freenet.org

Mail		432 Alter Rd.

Detroit, MI 48215-3105 USA

Users who have sent email to David_Conrad@mts.cc.wayne.edu will

please note that that address is (temporarily) disabled, and

any mail sent to it may not be read in a timely manner, if at

all.  Please use the Greater Detroit FreeNet address instead.

ACKNOWLEDGEMENTS

These programs would not exist without the code of Gary S. Brown

which I obtained by way of Rahul Dhesi, and of course the work

of Ron Rivest, both in developing MD4 and MD5 and in writing

the reference implementations thereof.  I would also like to

thank Phil Karn.  Without his md5trans code mdx would run much

slower.  The 386 detection is based on the p87test program by

Terje Mathisen.  My special thanks to Vesselin Bontchev who made

several excellent suggestions which have helped with the

development of mdx.

LEGALITIES

crc and mdx are Copyright 1993-1995 David R. Conrad.

All rights reserved.

The CRC-32 implementation used was extracted from the source

of brik by Rahul Dhesi, and is from the zmodem source code,

and is Copyright 1986 Gary S. Brown.  Used with permission.

MD4 is the RSA Data Security, Inc. MD4 Message-Digest Algorithm,

Copyright 1990 RSA Data Security, Inc.  Used with permission.

MD5 is the RSA Data Security, Inc. MD5 Message-Digest Algorithm,

Copyright 1991 RSA Data Security, Inc.  Used with permission.

MD5trans is a public domain implementation of the MD5 translate

function written by Phil Karn.  Used with permission.

4

XSUM			User's Manual			XSUM

David R. Conrad hereby disclaims all warranties relating to

this software, whether express or implied, including without

limitation any implied warranties of merchantability or fitness

for a particular purpose.  I will not be liable for any special,

incidental, consequential, indirect or similar damages due to

loss of data or any other reason.  In no case can Rahul Dhesi,

Gary S. Brown, Phil Karn, or RSA Data Security, Inc. be held

liable in any way for damages resulting from the use of this

software or otherwise related to this software.

crc and mdx are freeware, meaning that they are copyrighted

but can be used and distributed free of charge, as long as they

are distributed in an unmodified form.

5