Technical Note : FAST SERIAL PORTS & 16550 UARTS

Author : Michael Spalter

Email : segcom@cix.compulink.co.uk

Origin : United Kingdom, EC

Last Modified : 6th June 1994

Status : (c) 1994 Michael Spalter

* PLEASE UPLOAD THIS FILE TO BULLETIN BOARD AND OTHER HOSTS *

The filename should be FAST550.DOC or FAST550.ZIP etc.

NOTE:  This  document  may  be distributed freely providing that all

credits remain intact and that it is not modified in any  way.  This

document is provided for information only and no support or warranty

is offered by the author or publisher.

NOTE : SEE THE END OF THIS DOCUMENT FOR 'UART' VENDOR DETAILS

---------------

C O N T E N T S

---------------

0) INTRODUCTION

1) What is a UART ?

2) Why the sudden craze for high speed cards ?

3) Why aren't normal PC ports good enough ?

a) Internal PC Card Modems

4) How would I know if I need a faster port ?

5) Okay, I want one ! What should I buy ?

6) Is there just one type of 16550 UART ?

7) Replacing a soldered 16450/8250 UART

8) Windows and buffered UARTS

9) Serial Port configuration

10) Notebook PCs

11) Supplier information

12) About 'Cix'

INTRODUCTION

------------

Fast  serial  cards  have  become  very  popular  lately  due to the

proliferation of high speed (V32/V32bis) modems. Whilst the computer

press have been keen to cover modems in general, little coverage has

been given to  serial  cards  (they're  just  not  sexy  enough  for

mainstream  articles).  This  means that, whilst many people want or

need fast serial cards, there is much  confusion  about  them.  This

document attempts to answer the most common questions asked.

1) What is a UART ?

----------------

Throughout this document, we will refer to something called a 'UART'

(pronounced 'you-art').  As this is a very important  component,  it

is important to understand what it is.

A  UART  is  an IC (chip!) which takes parallel data from the PC and

outputs it to the serial port as a timed serial stream of data.   It

is  the  main component of the serial card; all other components are

there to support the UART.  Here is a simple functional diagram:

PC's BUS Slot   __________           Serial port

1 ------------> |        |

1 ------------> |  UART  |

0 ------------> |        |

1 ------------> |        | ---------> 11010001

0 ------------> |        |

0 ------------> |        |

0 ------------> |        |

1 ------------> |________|

The diagram shows conversion of data from a  parallel  to  a  serial

stream. Of course, serial ports can receive data as well as transmit

it, so the UART also converts serial data back into parallel data.

2) Why the sudden craze for fast serial ports ?

--------------------------------------------

Now that high speed modems (V32, V32bis) have  become  very  common,

many PC serial ports are having problems running fast enough to keep

up  with the modems which can be driven at speeds of up to 57600 bps

(bits per second).  A standard '8250 UART'  based  serial  port  (or

one using a multi i/o chip) cannot run at that speed due to the load

placed on a typical working PC. The '16450' UART is slightly better.

3) Why aren't normal PC ports good enough ?

----------------------------------------

Standard serial ports were designed  to  drive  mice,  plotters  and

modems.  At the time, modems ran at speeds of up to 4800 bps, and we

thought that was fast - anyone who has been using  modems  for  more

than  5  years  will  remember how lucky we thought we were !

Problems with serial ports arise for two reasons:

a) The 8250/16450 or equivalent UARTS are simply not fast enough  to

run  at  the  high  speeds  required  by  fast modems. Whilst the

BIOS/UART  will  allow  you  to select these speeds, they may not

reliably work at these speeds. The chips are rated for high speed

use, but that assumes a simple working enviroment (not a PC!).

b) PC's are busy things, with lots of interrupts to service. If  the

port is running  at  high  speed,  the  PC  may  miss  interrupts

generated by incoming  data  because  it  is  'busy  doing  other

things'  -  this  then means that the UART has nowhere to put the

incoming  byte  and  so  it  gets  overwritten  by  the next byte

received. This usually causes CRC errors.  The faster  you  drive

the port, the more CRC  errors  you  will  get.  This  is  almost

always a problem if you multitask with MS-Windows or Desqview, as

the  rapid  task switching keeps the processor very busy. So, for

instance, if you need to run your serial port  at  38400  bps  or

higher under MS-Windows then it is very likely that you will need

a buffered serial port.

The '16550' UART gets around both of these problems:

a) It is designed to run at  high  speed;  a  16550  chip  will  run

happily  at 57600 bps or faster which is fast enough for even the

fastest modems (for the moment anyway, but  V.Fast  modems,  just

around  the corner could require speeds up to 115200 bps. A 16550

is rated to run at this speed, but no-one  has  really  done  any

experiments yet to prove this).

b) The 16550 UART incorporates a 16 byte buffer;  if  the  PC  isn't

ready to received the current byte then the UART can retain it in

the  buffer  until  the  PC is ready for it.  16 bytes provides a

comfortatble time margin to ensure that none of the received data

is lost.  Many communications packages will empty the FIFO buffer

at every interrupt possible, rather than only taking one byte per

interrupt.   The  buffer  on  the  UART is a 'FIFO' buffer.  This

simply  means  that the first byte in will be the first byte out.

The alternative to this would be a LIFO buffer, but that would be

useless for a UART as all the data would become scrambled !

Analogy

-------

Imagine  a  mail  man  in the US collecting mail in a street. Assume

that each mail box can only hold ONE letter and  that  people  raise

their  'flag'  on their mail box to indicate that they have a letter

for collection - the flag is equivalent to a PC's  'interrupt'.  The

mail man sees the flag and runs along to collect the letter.

If  there  is  a  certain level of mail being collected, then it all

works well - the  mailman  has  enough  time  to  collect  all  mail

(equivalent to servicing all interrupts).

However,  if  someone  puts a letter in their box and the mailman is

too busy to collect it before that same person has another letter to

mail, then there is a problem: As the mail box  can  only  hold  one

letter, they would have to destroy the first letter.

This  is  how  a normal serial port works. A buffered serial port is

simply a bigger mailbox, which can hold up to 16 'letters' at  once,

so  that  when the mailman comes, he can collect up to 16 letters at

once.

( NOTE : The author of this document is in England, where  we  don't

have  mailmen who collect post from our houses - we have postmen who

deliver post and red pillar boxes into which we deposit our  post  -

the  system  referred to above is the one used in the US, because it

wouldn't work with the UK system. Any non-US people unfamiliar  with

these  'flags'  should  refer  to  a 'Peanuts/Charlie Brown' cartoon

where they seem to feature  regularly.)

3a) Internal (PC Card) Modems

-------------------------

Some  people  have  been confused about differences between internal

(PC Card) and external (desktop) modems.  An  internal  modem  plugs

into  a  normal expansion slot inside a PC. Instead of plugging into

the PC's serial port, it has its own serial port  on  board  and  so

takes its data directly from the PC BUS.  BUT REMEMBER : An internal

modem  still  has a UART which, just like with a normal port, can be

an 8250, 16450 or 16550 (or circuitry which behaves like one).

An interal card modem can either have  a  physical  40-pin  UART  or

sometimes it is embedded within another controller so you may not be

able  to  spot  it by looking at the board. When buying a V32bis (or

faster) internal card modem you should insist that it has  either  a

true  16550  or  16550  'functionality'. You can test this using the

'16550.EXE' program or if you have MS-Windows, the MSD program  will

tell  you  which  type  of UART the modem has. This can of course be

used to check the UARTs of regular serial cards too.

External modems do not need to have buffered UARTS within the actual

modem - the buffered UART is only required within the PC (either  on

a  serial card or a modem card) because the flow control system used

across the PC BUS is inadequate for modems (the interrupt method)  -

once  the  data  is  in 'serial form' then flow control becomes much

more reliable.

4) How would I know if I needed a faster serial card ?

---------------------------------------------------

If you are multitasking using MS-Windows or Desqview, then you  will

almost  certainly need a fast buffered serial card. Typical symptoms

will be CRC errors or block retries reported when downloading files.

As you will probably be using error correction,  then  these  errors

are unlikely to be caused by anything else.

Another  symptom  of an inadequate serial port is that you're simply

not getting the performance you'd expect from your modem. This could

be caused by you not having your modem configured correctly,  so  in

the  first instance, check with your suppliers or colleagues/friends

for any special setup strings particular to your modem (perhaps  you

have  forgotten  to enable RTS/CTS handshaking or data compression).

If  you  have your modem set up correctly and your performance still

isn't what it ought to be, then a faster  serial  card  may  be  the

solution.  On  a  V32bis/V42bis modem, you should be able to achieve

download speeds of up to 3000 cps on ASCII  text,  or  1500  cps  on

compressed  (e.g.  ZIP)  files.   Remember that poor phone lines can

also affect modem performance.

Even if you are not using Desqview or MS-Windows, you may still  get

the  symptoms details above, especially if you are driving the modem

at 38400 bps or faster, in which case you would be wise to invest in

a faster serial card. Having said that, there are some people  using

high  speed  modems  whose  existing serial port may be adequate, in

which case there is no point investing in a faster one !

A  fast  serial  card  with  a 16550 UART can be checked for using a

number of commonly available utilities. If you have  MS-WINDOWS  3.1

installed,  then  from DOS, you can run the MSD program which you'll

find in your C:\WINDOWS directory. Under the option labelled 'Serial

Ports' you can view your existing ports and MSD will report the UART

type fitted (8250, 16450 or 16550).

5) Okay, I want one, what should I buy ?

-------------------------------------

The  important part about a fast serial card is its UART; this means

that you may be able to simply swap the UART of your existing serial

card, rather than having to fit a whole new card.  You can  only  do

this  if your current serial card uses a 40 pin UART (8250, 16450 or

equivalent). This needs to be  socketed  for  easy  removal,  unless

you're  particularly  experienced  with  a  soldering iron and fancy

desoldering the chip yourself - this is detailed in section (7).

Unfortunately,  most PC's nowadays seem to come with multi i/o cards

with one multi-purpose chip which performs serial, parallel and disk

operations; if you have a multi i/o card, it will normally be a PLCC

package in the centre of the board ( a flat 'surface  mounted'  chip

with pins on all four sides). This obviously cannot be replaced with

a   16550.   Some  PC's  have  the  i/o  circuitry  built  into  the

motherboard; it is unlikely that you can replace the UARTs there.

Thankfully, and more easily, you can simply buy  a  new  card  based

around a 16550 UART. This can either replace or add to your existing

serial  ports and your supplier will advise you on the best type.

NOTE : This document relates exclusively to  buffered  serial  cards

based  around  the 16550 and equivalent chips. There is another kind

of high speed serial card which uses an on  board  processor;  these

are  generally  considerably  more  expensive and although there are

some circumstances where they might be of use, for the vast majority

of users, a 16550 based board should be adequate for  speeds  up  to

57600  bps.  A well known 'smart' serial board is the 'ESP' which is

made  by  modem  manufacturer 'Hayes'.

6) Is there just one type of 16550 UART ?

--------------------------------------

No! As with all  semiconductors, many manufacturers have  jumped  on

the 16550  bandwaggon. This is good inasmuch that there is a choice,

but bad in that some manufacturers have got it  wrong  -  some  have

FIFO  buffers which don't work and some aren't  recognised correctly

by applications (this can sometimes be worked around but ideally get

one which you know will work).

The 16550 chip 'by which others are  gauged'  is  made  by  National

Semiconductor, and the original part no.  was  NS16550AFN.  However,

due  to  the increasing popularity of the chip, NS decided to change

the   name.   The   latest   version  is  now  the  PC16550DN. There

were earlier  version  of  the  original  16550  which  didn't  work

correctly,  but  it  is  highly unlikely that any of these are still

around (it was the NS16550N - i.e. not the 'AN' or 'AFN').

Other manufacturers have now brought out their own  100%  compatible

versions of the NS UART, so you may find that your supplier stocks a

different  version,  some of which work well, others don't.  As long

as he guarantees 100% compatibility it should work as well as the NS

original.   Many  'clones'  are  made  in  what's  known  as  'CMOS'

versions.  They  are  functionally  identical (CMOS chips users less

power) so they should work fine too. A CMOS UART  will  normally  be

called  a  '16C550'  or similar.  Being the original, the NS type is

the most expensive 16550 UART, so you will probably  save  money  by

buying a clone, but take note of the above guidelines.

The  FIFO  buffer  on  a  16550 chip can be enabled and disabled. By

default it will be DISABLED. Many comms programs will  automatically

detect  and  enable the FIFO buffer, but if yours doesn't, there are

several programs to enable them,  which  you  can  include  in  your

AUTOEXEC.BAT  file  or another batch file. Ask your supplier. If you

are running under MS-Windows then see section 8.

There are several programs around to test 16550 chips, once they are

installed  so you'll soon know if your new chip/card is okay. If you

can't get hold of one, ask around on BBS's  or  ask  your  supplier.

MSD.EXE  is  supplied  with Windows 3.1 and will detect/report 16550

UARTS, but there are a couple of problems with MSD; see Section 8.

7) Replacing a soldered 8250 or 16450 UART

---------------------------------------

If your existing UART is a soldered 16450 or 8250 type  (40 pin DIP)

and you are competant with a soldering iron, then you  may  want  to

remove  the  chip yourself and replace it with a 40 pin socket (into

which you plug your new 16550).  It is not  too  complicated  -  the

most important thing is to work slowly and carefully.  Here are some

useful tips from Bob Kellock:

>>>> ALWAYS OBSERVE STATIC PRECAUTIONS WHEN HANDLING CHIPS <<<<

Get  hold  of a scrap double sided printed circuit board to practice

on; it's no use practicing on a single sided board as they're easy !

>>>>>> DO NOT ATTEMPT TO REMOVE THE CHIP WHOLE ! <<<<<<

Get hold of some fine nosed wire cutters and chop off  each  of  the

legs of the UART as close to the body as possible (therefore leaving

as much of the leg on the board as possible). Discard the main body.

Then,  remove  each  of  the legs in turn using a soldering iron and

tweezers. (Editors note: Bob says "real men use their fingers",  but

I'd  still recommend tweezers). Don't 'wiggle' the iron about on the

surface of the board as you will probably damage the tracks/pads  by

doing so. The time between applying the iron and pulling the leg  is

fairly critical - just keep practicing.

Make  sure  you  pull  the  leg out square from the board or you may

damage the tracks/pads on the component side.

You may find that it helps to tin the iron with new solder  now  and

then,  but don't use too much otherwise you may end up with a pubble

of solder, which will be a read problem.

Get hold of some solder wick or a solder sucker to remove the solder

from the vias (holes in the board where the legs came from).   As  a

last  resort  you  can  use  stranded wire and Fluxite but it's very

tedious. You may find that some holes just don't want to  clean  out

- resolder them with fresh solder and try again.

Check that the holes really are clean - they can be deceptive.  Hold

the board up to the light and check that you  can  see through  each

clearly.  Clean  off  at  least the component side of the board with

PCB cleaner or meths; use it in a well ventilated room using a stiff

brush - an old toothbrush is ideal.

Carefully  inspect the component side of the board - preferably with

a magnifying glass.  You can get minute  traces  of  solder  between

pads  which  completely  messed  it  up.  Remove any odd bits with a

scalpel. Check that there is plating  through  the  holes  -  it  is

possible  to  pull  it  out with the leg but it's not too terrible.

Insert  the  socket  in  the  board  with  the  notch  in  the right

direction. This is when you may find out that one (or more!) of  the

holes isn't cleaned out properly. Do it again.

Solder  socket  in position, clean, inspect and rectify as above. If

you lost any of the through hole platings then solder those pins  on

the top side of the board too.

The board is now ready to take your new 16550 chip. Remember to take

static  precautions  (earth  yourself and don't rub your slippers on

the nylon carpet). Ensure that all pins on the 16550  are  straight;

if  they  are  not,  carefully  press the chip on a table at a right

angle to straighten them. Pins tend of break if they  are  bent  too

much, so be careful.

Place  one  side of the chip just into the socket; do not push it in

yet. Then, place the other side's pins just into the  socket.  Check

that all pins are in the right holes and that they are all straight.

Slowly  press  the  chip into the socket using even pressure, making

sure no pins snag on their way in. Once it's in. Give it a firm push

against the table to make sure everything  is  home  tightly.  Check

that  the  chip  is  inserted properly, the right way round and that

none of the pins have curled up under the body instead of  being  in

its correct hole.

8) MS-WINDOWS and high speed UARTS

-------------------------------

Microsoft Windows is a prime cause  of  lost  interrupts.   This  is

mostly  due  to  the fact that MS-Windows is processor intensive; as

such, with high speed modems, it is even more likely that data  will

come  in  quicker  than  the PC can service the interrupts.  This is

made worse by the fact the Windows Multi-tasks.

A  properly  configured  16550 UART  should  solve  these  problems;

MS-Windows  is  16550 aware, though you may need to make a couple of

modification to your SYSTEM.INI file to enable the 16550,  otherwise

it will run in '16450 compatible mode' and you'll get no benefit.

Once you have installed your 16550 UART  or  16550  equipped  serial

card,  the  Microsoft  MSD program can be used to check it. MSD is a

DOS program which you'll find in your WINDOWS directory; it must  be

run  from  the  DOS prompt, not from within Windows itself. Remember

that if you installed your serial card to use  a  non-standard  IRQ,

you set it up with the 'Ports' option within Windows Control Panel.

If you look at your serial ports, it will show the type of UART each

is fitted with. If you have selected non-standard IRQs or you have a

COM4  but  no  COM3, MSD gets confused, so if it doesn't display the

expected results, don't panic !

Next, you'll need to edit your SYSTEM.INI file. You can do this from

DOS using any suitable text editor or from  within  Windows  Program

Manager,  select  'FILE'  then  'RUN'  and  enter  SYSTEM.INI as the

filename.  Windows' text editor will load your SYSTEM.INI file.

Find the section  within  SYSTEM.INI  labelled  '[386Enhanced]'  and

check for a line which reads:

COMxFIFO=1

where  'n'  will be the COM port with your 16550 fitted; chances are

that line won't exist, in which case you should add it. If there are

other lines beggining with COMx... then it makes sense  to  add  the

new  line  near them. For instance, if your 16550 UART was fitted to

COM3, you would add the line:

COM3FIFO=1

There  are  no  other options other than '1' here as it is a boolean

setting (either False=0 or True=1).

When using Windows, there is  no  need  to  enable  the  16550  UART

buffers  manually with any programs such as 16550.EXE. In fact, with

Windows 3.1, if you  enable  the

UART's  FIFO  buffers (using a 16550.EXE or whatever) before running

Windows, then it won't recognise the COM  port  any  more.

A company call CyberSoft, in Australia have produced  a  replacement

comms  driver  for  MS-Windows  3.1  which claims to improve Windows

comms performance and make best use of the 16550. The driver may  be

used  for  free  in non-commercial enviroments. Contact Cybersoft on

email at : 100033.1723@compuserve.com.  Also, Delrina,  the  company

who  make  the  'Winfax'  software  have their own replacement comms

driver called WFXDRV.DRV (or similar) which also claims to  improved

Windows  comms  performance.  WFXDRV is available from many bulletin

boards as well as Cix and Compuserve.

DOS Applications Under Windows

------------------------------

A  slight  bug with Windows 3.1 is that if you enable your 16550's

FIFO buffers  before  running  Windows,  then  the  port  will  be

unavailable,  so  make  sure  you  don't  enable  the FIFOs before

you run Windows !  Any DOS comms program should be  able  to  take

advantage  of  the 16550, but it depends on the program so results

may vary widely from user to user.

A  company called Cherry Hill Software have produced a replacement

comm driver for Windows 3.1 called CHCOMB.386  which  will  enable

DOS  programs running under windows to recognise 16550's properly.

It may not be necessary,  so  try  your  comms  with  the  regular

Windows  driver  first.  It  is added to your WIN.INI file.  It is

almost freeware - the author expects either a US$10 donation or  a

tacky  postcard  for his collection.  They can reached by email at

76050.2204@compuserve.com.

The following lines should also be amended in SYSTEM.INI:

device=*combuff      (delete this line - its in your WIN.INI)

device=chcomb.386

[386Enh]

COM3FIFO=1

COM3Buffer=10000

COMBoostTime=4        (normally 2)

COMIrqSharing=true

Experimentation  may  be  required  and doubtless there are people

more experienced than me on Windows and 16550's  so  if  you  have

something  to  add,  please  email me and I'll amend this document

appropriately. Also, practical experience  from  other  users  may

help  your  a lot, so always ask around to see if someone had (and

has) solved your problem already.

9) Serial Port Configuration

-------------------------

This section is relevant to all serial  cards,  whether  they  are

16550 based or the normal 8250/16450 types.

There  are  only  two  IRQ's  officially allocated for serial ports,

namely IRQs 3 and 4. In theory, this means that  you  can  therefore

only  have  a  maximum  of two serial ports. You can have more (i.e.

COM3  and  COM4)  by  sharing  the  interrupts  (although  different

addresses),  but  this  causes problems for modems as you cannot use

the ports simultaneously when sharing interrupts.

Some  serial cards allow you to select non-standard IRQ's, typically

any of 2,3,4,5,7 or 9.  You can only use non-standard IRQs  if  your

software  allows  you  to and also if that IRQ setting doesn't clash

with another i/o card already installed. IRQs 5 and 7 are  allocated

for  the  parallel  ports,  but  often  not  used,  so  they  may be

available.  Higher IRQ's are only available on 16-bit cards  as  the

IRQ hardware lines are on the second part of the BUS.

Typically, if you are adding a third serial port to your PC, and you

can't use non-standard IRQs, set your  new  card  as  COM3/IRQ4  and

don't  use  your  existing COM1 as it shares interrupts - as long as

there's  nothing  connected  to  a  port,  it   shouldn't   generate

interrupts.  If you need to use your existing COM1, you can put your

new  card  in  as  COM4/IRQ3, but then you can't use your COM2 as it

shares its  interrupt  with  COM4.   Always  experiment  though;  an

interrupt   clash   will  at  worst  prevent  your  machine  booting

(temporarily!).  If all else fails,  disable  your  existing  COM1/2

(there should be some jumpers on the i/o card) and put your new card

in as COM1/2.

Windows  Users : If you fit your third port as COM4, the MSD program

will report it as COM3 due to the way DOS stores the  addresses  for

COM  ports.  When MSD reports 'COM3' what it actually means is 'your

third serial port'. Other programs may report similarly.

10) Notebook PCs

----------------

Users  of  Notebook  PCs often want to upgrade their serial ports to

buffers types.  The most common answer is that you  can't;  to  save

space,  notebooks  use  multi-function chips, so the chip which does

the serial i/o will have  many  other  function  too  and  so  isn't

replaceable.

Worse  still,  notebooks can't take normal PC expansion cards so you

can add additional ports, unless your notebook has a docking station

option which can take cards. Also, many Notebook PCs have  dedicated

modems  which  fit  into  propritary slots; again the UARTs on these

modems cannot be changed, though any sensible V32bis modem  designer

would have included 16550 functionalty in his modem design.

The  only solution to a notebook PC without buffered serial ports is

to fit a PCMCIA modem if you have a PCMCIA slot. As  well  as  being

very  neat  and  compact,  many  V32bis  PCMCIA modems include 16550

functionality - to check the Microsoft  MSD  utility  or  any  other

16550 checker should confirm the presence of it.

11) Supplier information

--------------------

SEG Communications, based in London, England, stock a range of 16550

UARTs  and 16550 based serial cards at very competitive prices.  SEG

also sell a complete range of modems from all leading  manufacturers

at highly competive prices. SEG specialises in modems, so you can be

assured of excellent service.

For full details and pricing information contact:

SEG Communications

137 Hale Lane

Edgware

Middlesex HA8 9QP

England, U.K.

Tel: (081) 959 3377

Fax: (081) 959 2137

Email: segcom@cix.compulink.co.uk

From outside the UK, Fax : + 44 81 959 2137

Popular Model : SC5500 Serial Card; Single port card.

COM 1,2,3,4 Selectable IRQ 3,4,5,7,9

Credit Card orders are welcome. Trade enquiries welcome.

12) About Cix

-------------

Why are we mentioning Cix here ? Because it is my 'Home' system (the

one  I use all day) and I am happy to recommend it to everyone; they

are not paying for this mention, it's a genuine endorsement :-)

Cix  (pronounced  'Kicks') is a conferencing based host system based

in London, England.  It is similar is style to  Compuserve  although

considerably  less expensive than other similar services and as it's

UK based it has a much more friendly 'local' feel than Compuserve.

Services offered include Email, Internet access, local conferencing,

Usenet conferencing, Fax sending, gigabytes  of  file  downloads  as

well  as the collective wealth of the 1000's of members in 1000's of

conferences, each one dedicated to a particular subject.

Details of Cix are available from them on (081) 390 8446 (voice)  or

their  data  number  on  (081)  399  5252.  Data  speeds from 300 to

28800bps are supported with MNP5/V42bis. You can  also  telnet  into

Cix at their address : Telnet cix.compuserve.co.uk

------------------ End of Document ----------------

(c) 1993/4 Michael Spalter, SEG Communications

This  document may only be distributed un-modified.  Reproduction as

part  of  any  commercial  transaction  or  distribution   via   any

non-electronic  medium is prohibited in part of full without written

permission  from  the  author.  This  file  may  not  be compiled or

included within any other document  or  help  system  without  prior

written permission from the author.

----- end of file -----