���������������������������������������������Ŀ

�             RADIOSIM v 3.01                 ÿ

�                                             ��

� Put a radio-electric software into your PC  ��

�                                             ��

����������������������������������������������ٳ

�����������������������������������������������

�

�

�

�

This file includes five sections:

1)  Installation

2)  Documentation

3)  On-line notice and User's guide updates

4)  History of RADIOSIM versions

5)  Cost, copyright

������������������Ŀ

� 1) Installation  �

��������������������

The RADIOSIM files need about 1 Mb disk space.

Two cases must be marked out:

1.1 You received a diskette directly from us (then you own a four pages

documentation). The diskette is of 3 1/2  720 Kb type. Files are

compressed into an unique archive file INSTALL.EXE.

You are encouraged to make copies of the diskette (DOS instructions

COPY, or DISKCOPY) and of its accompanying documentation, for safety

and for your friends.

Insert the diskette in drive A: and just type:

A:INSTALL  C:\RADIOSIM  and the ENTER key. A repertory named RADIOSIM

will be created in your hard disk C:, and the program will be

installed in it.

If needed, the A: floppy drive name can be replaced by B:, and the

hard disk name C: can be replaced by D:, E:, F:, G:, H: or I:

You can also give a repertory name different from RADIOSIM.

Run the program typing RS, then the ENTER key, from the RADIOSIM

repertory.

1.2 The software comes from a BBS as Compuserve, or from a shareware

vendor. Then, follow the instructions given to you and, before running

RADIOSIM the first time, execute REORDER.BAT to rearrange the files

(type REORDER, then the ENTER key). A sub-repertory named STOCK will

be created under the current repertory (named for instance, RADIOSIM),

and several software files will be moved in it.

REORDER.BAT needs to be run only one time.

Run the program typing RS, then the ENTER key, from the RADIOSIM

repertory.

Installation on a diskette

--------------------------

If your computer does not include a hard disk, RADIOSIM can be run

directly from a floppy disk, with some performance degradations (the

diskette capacity must be at least 1.2 Mb or 1.44 Mb).

�������������������Ŀ

� 2) Documentation  �

���������������������

Normally, the base version of RADIOSIM comes with a four pages

documentation. If you do not have it at your's disposal, the following

lines reproduce the text only, without images.

RADIOSIM computes the performances of digital/analog radio-electric links

in presence of linear or non-linear distortions and interferences.

The following list gives some examples of problems that RADIOSIM can

resolve and illustrate with a graph: those graphs cannot be carried by an

ASCII file as this one. They are displayed in a four pages leaflet, freely

available on request.

- Multicarrier amplification, mixed type (carriers modulated with

different bit rates): spectral density at TWT input and output,

error rate

- Path of a 140 Mbit/s, 4PSK modulated in the Fresnel plan: intersymbol

-free or not-free channel, 4096 symbols

- 64 QAM eye diagram. Intersymbol distortion by non-optimal filtering

- Amplitude/frequency, Phase/frequency, Group delay/frequency

characteristics of a Tchebychev filter, B3dB = 40 MHz, 5 poles, 0.5 dB

ripple. Four cells group delay equalizer, optimized with MiniMax

- Bit error rate versus C/N (16QAM, 40 Mbit/s, "SAT" TWT with 5 dB input

backoff)

- Group delay of a 25 m waveguide feeder (F = 6GHz, 20 dB return loss at

both ends)

- Differential gain at 3.58 MHz due to a Butterworth filter: TV signal,

frequency modulation, 4 DC steps

- 128QAM constellation, after adaptive equalization of a selective fading

(4096 symbols, 140 Mbit/s)

- Non-linear amplification of four unmodulated carriers, with different

levels. Predistorter optimized (Minimax) to improve the C/I of the 3

low level carriers

RADIOSIM is a powerful tool for engineers, to aid specification settlement

and equipment design. It also concerns electrical engineering teachers.

Its tutorial constitutes a comprehensive introduction to digital

modulations.

Technical domains are: transmission or broadcasting by satellite,

transmission by terrestrial radio links, broadcasting in metric or

decametric waves, satellite or terrestrial links between mobiles, etc.

RADIOSIM is easier to use than any comparable package on the market: It

works on any PC running DOS or WINDOWS, even a portable one. It is not

necessary to be an ace in mathematics to use it. Assuming a 80386 or 80486

PC, the calculation speed of RADIOSIM can be compared with the speed

obtained by existing softwares running on a work station, and its cost is

one order of magnitude lower.

The results of computation: tables or graphs, can easily be imported in

any word processor to be included in technical reports.

All source files are available, allowing a customer to develop its own

specific modules, and affording security to professional users.

Main characteristics:

--------------------

Needed hardware: any IBM PC/XT/AT or compatible computer running DOS 3.0

or higher.

A math coprocessor is useful to speed computation. Any CGA / EGA / VGA

/HERCULES / ATT screen type is supported. The program (full version) draws

profit from the whole computer (extended/expanded) available memory.

The user is offered a library (list) of modules among which he selects

some ones to build a canal (sequence of modules).

Main available modules:

----------------------

Digital modulator               Coherent demodulator

Differential demodulator        Bit error rate

Gaussian noise                  Raised cosine filter

Butterworth filter *            Tchebychev filter *

Anti Tchebychev filter *        Gaussian filter

Linear / quadratic variation of amplitude and GD

Transversal filter              User's filter *

Auto-adaptative equalizer       Non-linear amplifier

Limiter                         Interferences

Butler matrix                   Frequency modulator

Frequency demodulator           Pre/deemphasis network

Differential gain and phase     IRF

Graphics                        Spectral density

Saving the signal               Iterative computation

Multicarrier amplification      MiniMax optimization

Group delay equalizer           Waveguide or coaxial feeder

Predistorter                    Analyse of a RLC network

* either low-pass, band-pass, high-pass or band-stop filters

Supported digital modulations:

-----------------------------

nPSK (n= 2,4,8,etc.),

mQAM (m = 4, 16, 32, 64, 128, 512, etc.), [TCM]

offset QAM (f.i. OQPSK),

contant envelope nPM (f.i. MSK, GMSK),

k-2kPSK (f.i. �/4 DQPSK),

FSK with quadratic or differential demodulation.

Also, analog modulations AM, FM, PM, SSB.

Functions

---------

- On line contextual/navigating Help (120 screens), full page scrolling

menus,

- For every module among 44, an entry screen allows to edit parameter

values, with associated Help and fault correction,

- Utility softwares to write auxiliary files: filters entered point per

point, non-linearity characteristics, templates, etc.,

Graphics

--------

- Graphical presentation of results, on screen or printer:

- characteristics of filter(s) or signal, at any point of the transmission

chain, in time domain or in frequency domain,

- polar or rectangular coordinates, eye diagram, linear or logarithmic

scale,

- digital signal constellation, or signal path in Fresnel plan,

- curves of BER versus C/N,

- horizontal and vertical graduations are set automatically or manually.

When displaying spectral power density, graduations give directly the

difference between power in a specified bandwidth and a reference power.

User's guide contents (350 pages):

---------------------

Introduction. Installation. Examples (50 detailed examples of resolved

problems). Specific files. Entry screens. Software principle. Description

of modules. Description of procedures (for users developing additional

modules). Utility programs. Recalls: Fresnel plan, Fourier transform,

Digital modulations.

Compatibility with version 2.0

------------------------------

The menu of three modules of the last version 2.0 have been modified:

"Loop", Frequency modulator" and "Filter defined point per point".

If you wrote .CAN files using those modules, with a different name from

names of the pre-recorded modules, proceed as follows:

- print those .CAN files, using the "PRINTCAN.EXE" utility,

- run RS.EXE v.3.0, load these files at screen #1 then, in the screen #3

corresponding to one of the three above modules, enter the data in the

new way.

How to use the software

-----------------------

The program is run typing: RS, then the ENTER key (the active repertory

must be C:\RADIOSIM). You can also run RADIOSIM from the File Manager of

Windows.

Just after the software is run (screen #0, then screen #1), the user is

proposed a list of modules (screen #2) among them he chooses, from the

keyboard, the modules he wants to be included in his canal. A canal is a

sequence of modules. A module generally represents an electric circuit

(quadripole).

The canal being built, the user gives a numerical value to every parameter

of every module (screen #3), reads Help screens, runs the program, watches

results on graphs and tables, and runs again the program after modifying

some parameter values.

A "Loop" module allows to write a table giving one or several results, as

a function of one, two or three parameters, incremented in fixed steps.

Without stopping the program, the user can read the on-line contextual

Notice, save the canal and a comment on disk in order to avoid a long

entry at a further session, modify the presentation of graphics (margins,

graduations, titles) and print them on paper or save them in a file for

further importation in a word processor (Using WINDOWS, or the DOSSHELL of

DOS 5 or DOS 6, you can switch instantaneously from any screen of RADIOSIM

to any screen of your word processor).

The software recognizes by itself the type of screen, and also the

presence or not of an arithmetic co-processor.

The utility program used to write user-specific files, needed by some

modules:

- Filter defined point per point,

- Non-linear quadripole (amplitude/amplitude and phase/amplitude

characteristics),

- Specification templates (filters, spectral density, rise time, etc.),

- Interferers, coefficients of a transversal filter, etc.

is not included in the base version, but some examples of those ASCII

files are saved in the STOCK sub-repertory: Some pre-recorded canals use

them, you can watch them, and even modify them (be cautious) using a text

editor.

The computing time of a typical canal (DUMMY.CAN) with N = 256 samples,

including a digital modem, transmit and receive raised cosine filters, a

non-linear amplifier and two "Graphics" modules, varies from 2 to 400

seconds, depending on the type of PC.

The computing time is almost proportional to the number of samples N. N is

limited to 8192 in the RADIOSIM base version.

Note: If your PC microprocessor is a 80386 or 80486, and if an arithmetic

co-processor is present, an expanded memory (EMS) driver present in

CONFIG.SYS (f.i. EMM386.EXE) usually increases computation time twofold:

deactivate it, if possible.

Recommendations

---------------

The software principle can be approximately understood without written

documentation, using Help pages (keys "h" or H" or F1) and on-line notice

(keys "n" or N"). You are prompted to hit these keys as soon as the first

screen #0 is displayed.

From time to time, read a section of the on-line Notice. It is much easier

to read it on screen because its contextual and navigation features.

Have in mind the role of the four screens, #0, #1, #2 and #3.

Perform the computation of pre-registered canals proposed in screen #1 (a

list is displayed typing "999"), particularly DUMMY.CAN. Do not erase

them.

Try to understand,

- the canal structure; it is sometimes difficult, for instance in case of

using "Stock" modules to save the signal,

- the software principle: perhaps you will feel difficulties if you are no

more familiar with Fourier transform and Fresnel plan concepts, and with

digital modulations (a chapter deals with these subjects in the User's

guide).

The base version of RADIOSIM allows to build entirely new "canals".

Case of mis-functioning

-----------------------

If the computer goes back to DOS as soon the software is run, or during

computation:

- Verify that your computer is an IBM compatible PC, not a MacIntosh,

- File RS.EXE must be present:

� in the repertory from which the program is run (generally RADIOSIM:

the DOS prompt, before you type RS, must be C:\RADIOSIM> ),

� or in a repertory pointed by a PATH instruction,

- Files NOTICE.TXT, CONTENTS, RS.VAL, twelve files with a .HLP extension,

and eight files with a .DID extension must be present in the repertory

from which the program is run,

- The repertory from which the program is run must include a sub-repertory

named STOCK, in which you can found files with extension .CAN, .TWT,

.PER, .FIL, .TF, .GAB, .MCR and .ANA

This sub-repertory must include at least one file with each above

extension.

- If the software comes from a BBS or a shareware vendor, did you execute

REORDER.BAT ? (see the above section "Installation").

If the diskette has been damaged in postage transportation, ask for a new

free one.

If mis-functioning persists, try to make it reproducible, note the

conditions of occurrence, and write us.

���������������������������������������������Ŀ

� 3) On-line notice and User's guide updates  �

�����������������������������������������������

The following remarks are not yet included in the on-line notice and in

the User's guide of the present version:

- Module "Analyse of a RLC network"

---------------------------------

In the screen #3 corresponding to this module, after:

"Tune the ad-hoc sampling bandwidth "

replace: "using N and Ns"

by     : "using Ns and the symbol rate"

As a matter of fact, N must be defined in screen #1 only to set the

number of computed frequencies (not too high, in order to limit the

computation time). Ns and the symbol rate are entered in the screen #3

corresponding to module "Sequence generation and Digital modulation".

If you need a table of the transfer function, in addition to the graph

displayed by module "Graphics", insert a "Loop" module at the canal end,

increment the calculation frequency (7th parameter of module "Analyse of

a RLC network"), and specify as results those of this module.

The wanted table will be found in the "LOOP" file.

����������������������������������Ŀ

� 4) History of RADIOSIM versions  �

������������������������������������

This section describes the different RADIOSIM versions, giving more

details on the last ones. Only the versions which have been notably

commercialized are mentioned.

Version 1.03

������������

It was the first commercialized version, come out in December 1991,

only in French.

Version 2.0

�����������

Version 2.0 came out in February 1993, in French and in English.

It has been distributed in "shareware" under the 1.5 appellation.

It includes several new modules: direct simulation of a gaussian noise,

gaussian filter, limiter, elevation of signal to power K, modifications

of the frame.

A new utility RSXM.EXE allows to implement a very high number of samples,

only limited by the computer memory size.

RADIOSIM version 2.0 is now used by many governmental and industrial

users in Western and Eastern Europe.

Version 3.0

�����������

Version 3.0 came out in January 1995.

The new characteristics of version 3.0 are as follows. Most of them have

been suggested by users of version 2.0.

- the "Graphics" module is able to display the group delay/frequency

characteristic of a filter or cascaded filters,

- extension of Butterworth, Tchebychev and anti-Tchebychev low-pass and

band-pass filtering functions to high-pass and stop-band

characteristics,

- the number of coefficients in the "Auto-adaptive equalizer" module is

no more limited,

- FSK using frequency demodulation can also be simulated,

- simulation of impairments of an analog modem (slope in % per MHz, notch

in % per MHz�),

- the "Loop" module can perform up to three nested loops.

- a second Comment screen is available, common to all RADIOSIM sessions,

acting as a diary,

- the User's guide includes a theoretical chapter recalling the main

properties of digital modulations,

- the contextual on-line Notice includes "navigation" tools.

New modules:

-----------

- "All-pass network" is a group-delay equalizer (one pole LC cell),

- "MiniMax" finds the parameter's values of one or several modules

maximizing or minimizing the results of other specified modules,

- "Predistorter" linearizes a power amplifier, using a predistortion

method,

- "Feeder" simulates echos in long lines,

- "Multicarrier amplification" computes intermodulation products appearing

when a large number of carriers share a same non-linear amplifier,

- "Signal or Filter results" records any signal or filter characteristic

along the canal, to be processed by "Loop" or "Minimax" modules,

- "Analyse of a RLC network" computes the transfer function of a RLC

network between any pair of ports,

- "Filter synthesis" computes the LC component values of a classical

filter,

- "Return to DOS" returns temporarily to DOS,

- two utilities allow to establish the thermal budget of a satellite link

or a terrestrial radio link, respectively. These utilities are linked to

RADIOSIM by the "degradation" concept,

- tutorial pre-recorded canals are intended to beginners, to help them

both to use the software and to deal with digital modulations.

Version 3.01

������������

Version 3.01 came out in May 1995.

The following bugs of version 3.0 have been corrected:

- Module "Feeder": error in a ratio 2 on the echo duration.

- Module "All-pass network": error on the phase of the central frequency

sample (the group delay was correct).

Other improvements:

- The line #26 of .CAN files (pre-recorded canals) includes now two

other parameters of screen #1, in addition to the number of samples N:

� ArrayFile$ (file/arrays choice)

� VF$ (visualization of the last filter or all filters).

When loading a .CAN file, the user is warned if those three parameters

are not the same in the file and in screen #1.

- The duration of audio warnings BipTime!, that can be entered in the

command line, is now saved in the RS.VAL file. At the next session,

hitting RS ENTER is enough to introduce again this duration.

- When moving from screen #1 to screen #2, the parameters of the first

module (particularly Ns and the symbol rate) are memorized, easying

to check the validity of entries made by the user in following modules.

- Module "Coherent demodulation": improvement of optimization in case of

using the "precise" method (procedure "Eye").

- Module "Interferences": the number of interferers is limited to 15, to

ease the entries.

- Module "Auto-adaptive equalizer":

� improvement of entries in the "hold" signature mode.

� more detailed description of entries in the on-line notice.

- Module "Spectral density" more checks of the validity of entries

(out of band frequencies, too large calculation bandwidth, etc.).

���������������������Ŀ

� 5) Cost, copyright  �

�����������������������

This software is the base version of RADIOSIM. It includes the main

executable file, exactly as it is, uncrippled. This base version is

distributed as "shareware" (try before buy), a low-cost but rather unusal

way to promote a professional software.

100 days are enough to know what is the scope of RADIOSIM, and how it

works. After 100 days, and although the program will continue to operate

normally:

- if you are a student, or a hamradio, continue using this software

freely, and make copies to your friends.

- if you are a professional, please register the full version (acces to

the whole computer memory, utilities, written documentation, hot line

support): doing so, you will save time, an item much more expansive than

the paid cost. Also, registration will enable us to provide you with

updates, as they become available.

Details on add-on items coming with the full version:

� written documentation (350 pages User's guide, hard cover),

� the utility program used to ease writing user-specific files,

described above,

� two utility programs, allowing to establish the thermal budget of a

satellite link or a terrestrial radiolink. These utilities are linked

to RADIOSIM by the "degradation" concept.

� the comment files (with a .DID extension) corresponding to all

tutorial files. Only the !DIGITRA.CAN tutorial canal is supplied with

comment files in the base version.

� a more powerful version RSXM.EXE, drawing profit from the whole

computer (extended/expanded) available memory.

Although RS.EXE can solve the majority of problems encountered in

radio-communication engineering, it runs only in the 640 kb DOS

memory, so the number of samples cannot be higher than 8192.

With RSXM.EXE, up to 1,048,576 samples can be implemented.

A high performance PC (486/66 MHz or Pentium) is necessary to keep a

reasonably low computation time, the computation time being quasi-

proportional to the number of samples.

A high number of samples is mainly needed in three cases:

. when the filter's roll-off is very small, as in modern high spectrum

efficiency radiolinks. Then, the impulse response has long trails,

and a long pseudo-random sequence is needed to test the worst case

of a symbol sequence.

. for non-coherent demodulation schemes, in which the thermal receiver

noise is directly simulated, instead of using the erfc function.

. when several carriers with very different bit rates share a same

transmission medium.

� utility programs to print a canal structure, and to write a new

version of the on-line notice (for users developing new modules).

� hot line support, by telephone, fax or mail, during one year from the

date of registration. During this period, we shall answer any question

and fix serious bugs.

������������������������������������������������������������������������ͻ

�                 Item (full version)                            Cost    �

������������������������������������������������������������������������Ķ

� - RADIOSIM v 3.01 executable file, up to 8,192 samples,    �           �

� - Executable file, up to 1,048,576 samples in extended/    �           �

�   expanded memory,                                         �           �

� - Utilities: Entry of filters, non-linearities, templates, � 1,800 US$ �

�   etc. Link budget (satellite link, terrestrial radiolink).�           �

� - User's guide, tutorial, Hot line support.                �           �

�                                                                        �

� - Option: full set of sources, BASIC ASCII files.            1,700 US$ �

������������������������������������������������������������������������ͼ

RADIOSIM is available in two versions, French and English.

���������Ŀ

���������Ŀ     �              (R)

�ĳ         �o    �������������������

�   ���������Ŀ �  Association of

�   �         ���  Shareware

���ĳ    o    �    Professionals

�����ĳ    �    ���������������������

�����������    MEMBER

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to make sure that the shareware principle

works for you. If you are unable to resolve a shareware-related problem

with an ASP member by contacting the member directly, ASP may be able to

help. The ASP Ombudsman can help you resolve a dispute or problem with an

ASP member, but does not provide technical support for members' products.

Please write to the ASP Ombudsman at 545 Grover Rd., Muskegon, MI, 49442-

9427, USA, Fax. 616-788-2765, or send a Compuserve message via CIS MAIL to

ASP Ombudsman 70007,3536.

������������������������������������������������������������������������ͻ

�                            DISCLAIMER                                  �

� This program is provided AS IS without warranty, expressed or implied. �

�                                                                        �

� BREMENSON DISCLAIMS ALL WARRANTIES RELATING TO THIS SOFTWARE, WHETHER  �

� EXPRESS OR IMPLIED, INCLUDING, BUT NOT LIMITED TO ANY IMPLIED          �

� WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE,    �

� AND ALL SUCH WARRANTIES ARE SPECIFICALLY DISCLAIMED. NEITHER BREMENSON,�

� NOR ANYONE ELSE WHO HAS BEEN INVOLVED IN THE CREATION, PRODUCTION, OR  �

� DELIVERY OF THIS SOFTWARE SHALL BE LIABLE FOR ANY INDIRECT,            �

� CONSEQUENTIAL, OR INCIDENTAL DAMAGES ARISING OUT OF THE USE OF OR      �

� INABILITY TO USE SUCH SOFTWARE EVEN IF BREMENSON HAS BEEN ADVISED OF   �

� THE POSSIBILITY OF SUCH DAMAGES OR CLAIMS. IN NO EVENT SHALL           �

� BREMENSON'S LIABILITY FOR ANY DAMAGES EVER EXCEED THE PRICE PAID FOR   �

� THE LICENSE TO USE THE SOFTWARE, REGARDLESS OF THE FORM OF CLAIM.      �

� THE PERSON USING THE SOFTWARE BEARS ALL RISKS AS TO THE QUALITY        �

� AND PERFORMANCE OF THE SOFTWARE.                                       �

������������������������������������������������������������������������ͼ                                                                         *

About the author.

C.Bremenson is a graduate of the Ecole Polytechnique and the Ecole

Nationale Superieure des Telecommunications. He was employed during twenty

years at THOMSON CSF and ALCATEL as a technical manager of the Radio Link

and Earth Station R&D department. He is now a consultant, and a professor

at the Ecole Superieure d'Electricite.

Bremenson

60 rue d'Herblay

95150 TAVERNY

FRANCE

Tel.           33 1 39 32 91 55

Fax.           33 1 39 32 11 97

Compuserve     100337,622