VUUCP

�����

VUUCP.EXE is the UUCP network software provided with

Virtual Advanced software.  It allows for UUCP type network

operations with the world-wide Internet.  It might even be used

to connect to alternative and/or private UUCP type networks.

VUUCP is complete -- there is no other mailer or software

needed, and set-up couldn't be simpler.

Virtual Advanced features an extremely sophisticated

version of VUUCP.EXE which can gate email between the Internet

and the other networks, gate newsgroups between the Internet

and other networks, and can even toss (unpack) mail in

the background, so it doesn't tie up a serial port/modem.

With VUUCP, it's easy to gate email and messages:

Internet <--> VirtualNET-type

Internet <--> FIDOnet                        (messages only)

Internet <--> QWK type net                   (messages only)

VUUCP.EXE COMMAND LINE USAGE

����������������������������

Note that <ch> refers to Virtual Advanced Channel Number,

and that if the NETWORKID= is omitted, NETWORKID=9 is assumed.

VUUCP <ch> /U NETWORKID=9

Update any outgoing mail.  Automatically called by VME as needed.

VUUCP <ch> /R NETWORKID=9

Pull in data received by remote.

(Automatically run when data is received after a net call.)

VUUCP <ch> /D<full system name> NETWORKID=9

Dial into the specified system.

VUUCP <ch> /A NETWORKID=9

Used to handle an incoming UUCP call-in.  Use of this option

requires script programming.

VCONFIG SET-UP FOR VUUCP

������������������������

When setting up a UUCP based network using VCONFIG, you

will be given the following choices:

1. System Address:

This is where you enter your full Internet address.

(Example: mybbs.myhost.com)

2. Spool Directory:

This setting specifies the directory to be used by this network for

storage of outgoing mail, and temporary storage for incoming data.

(Example: C:\SPOOL)

Directories will be created beneath this directory automatically,

on an as-needed basis.

D. Edit Remote List

This choice lets you edit the list of systems you are connected to.

Many of you will only have one system in this list -- this one

system being your Internet Service Provider.  There are 5 settings

for each remote system you connect with:

1. System Address

The full Internet address of the remote system.

2. Phone Number

The data phone number, as it needs to be dialed, to call this system.

3. Wait-For #1

Send #1

4. Wait-For #2

Send #2

Typically this sequence is used to complete the "Login:" part

(Wait-For #1/Send #1) and "Password:" part (Wait-For #2/Send #2)

of the login process during an outgoing net call to a UNIX-based

host.

If you connect to a UNIX or Waffle-based UUCP site, you'll need the

"Wait-For" and "Send" fields to "login" to the remote site when you

place a net call, sort of like a regular user.

The difference is that UNIX will detect your account as being a

systems mail account, and will automatically start-up the UUCP

mail session as soon as you're logged in.

(Your account login ID and password are provided to you by your

Internet Service Provider, and are not related to your Internet

system address.  Provider's require logins/passwords to prevent

unauthorized use of your systems mail account, and to facilitate

billing.)

Example:

Wait-For #1: ogin:

Send #1:     mybbs

Wait-For #2: word:

Send #2:     mybbspw

Waits for the word "ogin:" (as in "Login:" or "login:") and then

sends "mybbs" plus a carriage return.  Then, waits for the word

"word:" (as in "Password:" or "password:") and then sends "mybbspw"

plus a carriage return.

NOTE: The above login provisions will be sufficient for the vast

majority of the users of this software.  If you need something

more complex to login to your Internet/UNIX host, an optional

login "macro" capability is provided.  See section ADVANCED

OPTIONS for more information.

5. Knows *Us* As:

When you call into an Internet site to pickup your mail, after

the login/password sequence, your system must identify itself

to the host.  Some systems require the full Internet address

(eg "mybbs.win.net").  Most systems, particularly UNIX ones,

want to see only the first part of our name up to the first

period, maximum 7 characters, eg "mybbs.win.net" -> "mybbs".

This field, therefore, controls how *your* system identifies

itself to this particular remote system during a net callout

to the remote system.

By default, VCONFIG will try to "guess" the proper data to put

into this field whenever you add a new remote system to the remote

system list.  VCONFIG should guess correctly about 90% of the time.

The important thing to remember is that different UUCP hosts are

going to have different requirements for logging into the system

and then for system identification during the actual network

identification handshake.

You should try the VCONFIG-supplied default for this field.

However, if you receive the message:

"You are unknown to me"

during a net-callout, this might be an indication that you need

to fiddle with this field.  (It could also be an indication that your

service provider hasn't set-up your UUCP mail account yet!)

The important thing to remember is that different UUCP hosts are

going to have different requirements for logging into the system

and then for system identification during the actual network

identification handshake.

If in doubt, contact your service provider; they should be able

to tell you what is proper to enter for this field.

SETTING UP EMAIL

����������������

There is nothing extra you need to do, beyond the initial

VUUCP set-up described above, to be able to send and receive

Internet Email.  There is nothing further you must do to be

able to GATE email to other networks, either.

Each user, though, must send an initial email to

"fire up" their Internet email handle.  In other words, the

first time someone sends mail through VUUCP, a unique

handle is generated for them so that return replies will

find them.  It is recommended that they send mail to:

mailid@your.address

where your.address is replaced by your actual Internet address.

This mail will be loop-ed back to your BBS without going out

over the Internet, but will accomplish the task of assigning

the user a unique handle.

It is recommended that you set up a VME auto-reply robot

with the name "mailid," and then your users will actually get back

the message you want to send them, with their newly-generated

Internet address showing in the "To:" line of the message.

SETTING UP INTERNET NEWSGROUPS

������������������������������

Newsgroups can be quickly and easily set-up using the

included VMB utility program.  Make sure you have ASCII file

NEWSGRP.9 installed in your DATA directory for VMB to function

properly.  NEWSGRP.9 is the file containinng the newsgroup

listings.

An alternative method to newsgroup set-up is to use

VCONFIG to manually set-up/edit each database entry.  In this

case, you would use the DATABASE CONFIGURATION portion of

VCONFIG to display/edit a DATABASE CONFIGURATION ENTRY. At

this point, selecting the field "(G) Networks Configuration"

would let you manually do what VMB does automatically.

A typical newsgroup name looks like:

alt.bbs

biz.comp.hardware

comp.sys.ibm-pc

To GATE an Internet newsgroup, you merely set-up its

DATABASE COFIGURATION ENTRY to show that the field

"(G) Networks Configuration" shows multiple networks --

Internet, and the other network(s) you want to GATE to/from.

For example, you might see:

Internet=alt.bbs

VirtualNET=100000

This means that this particular message base is a gateway

between Internet's alt.bbs newsgroup, and VirtualNET's

message sub number 100000.

SETTING UP INTERNET "MAIL-LISTS"

��������������������������������

This does not refer to a multi-mailing list.  This refers

to an Internet Mailing List - a type of message conference

which is hosted by a particular site.  The host has complete

control over distribution (who is allowed to join) and

approval of postings before they get re-distributed to the

rest of the list membership.

Although an Internet MAIL-LIST is like a message conference,

all messages incoming and outgoing are sent as emails.  VUUCP

handles the conversion for you in both cases.

Unlike newsgroups, MAIL-LISTs cannot be set-up with VMB.

You must use VCONFIG to manually set them up.  (See the section

previous SETTING UP INTERNET NEWSGROUPS for more info on manually

editing Database Network Configuration.)  A typical configuration

entry in this case might be:

VNEWS,virtual.win.net

Where VNEWS is the name of the MAIL-LIST, and virtual.win.net

is the host where new posts should be sent.  All MAIL-LIST posts

are converted to emails when outgoing; our example would send

the new posts (replies) to VNEWS@virtual.win.net, where the post

would be redistributed to other list members from there.

SETTING UP VUUCP FOR USE WITH VIRTUAL WINDOWS INTERNET ACCESS KIT

�����������������������������������������������������������������

When used with VWICK, VUUCP functions only to toss the

outgoing and incoming mail and news.  The UUCP transfer mechanism

is not used, and instead SMTP & POP3 are used to handle the email,

with NNTP handling newsgroups.

Configuration of VUUCP is the same as usual; you

will need to configure a dialing-entry for the Internet

site that you will use for your SMTP/POP3/NNTP services,

however, instead of entering a phone number, you will

enter in directory path information.

For more detailed instructions, please consult the VWICK

documentation.

ADVANCED OPTIONS

����������������

The features described hereafter are optional, and unneeded

in most cases.

*  Extended Login Macro Script

If you need more versatility to login to your Internet

Service Provider you must create a text file called MACRO.TXT

in the storage directory for that particular system.  For example,

suppose your Spool directory is set to C:\SPOOL, and your

host is myhost.myprovider.com.  The directory where you would

place MACRO.TXT is C:\SPOOL\MYHOST (a directory VUUCP

has automatically created for you already, assuming you've run

VUUCP at least once since configuration.)

The format of the MACRO.TXT file ASCII text with one commmand

per line.   Allowable commands are SEND, WAIT, and WAITFOR, and

they use the following syntax:

SEND <string to transmit>

Examples:

SEND ThisString

SEND Hello{

The SEND command automatically replaces the "{" character with

a carriage return.

WAIT <number of seconds>

Examples:

WAIT 2

WAIT 4

Waits the number of seconds specified.

WAITFOR <waitfor string>,<send string>

Examples:

WAITFOR prompt:,myname

WAITFOR ogin:,mysystem

With WAITFOR, the string to be sent will have a carriage return

automatically appended at the end.

When the last line of MACRO.TXT has been executed, VUUCP proceeds

to the built-in Waitfor/Send pairs unless you've disabled them.

*  VUUCP /R command line option

This function might be used to "import" standard Internet

Message Format mesages received via a different source/method

into the BBS.  In other words, you'd copy the files you wanted

to import into your SPOOL\TMP directory, and simply execute:

VUUCP 1 /R NETWORKID=9

VME 1 /TOSS

*   Using VUUCP to import messages received via Planet Connect

A special command line option has been added to make importing

Internet newsfeeds received via Planet Connect as simple as possible.

The Planet Connect software delivers to you a file with the

name PCUSENET.xxx where xxx is a sequence number.  Simply copy

the received PCUSENET.xxx file to your VUUCP SPOOL directory,

and from your main directory execute:

VUUCP 1 /PC NETWORKID=9

IMPORTANT: The PCUSENET.xxx file is a 2.04g ZIP format file,

and VUUCP will call upon PKUNZIP.EXE to unzip it for you

automatically.  Make sure you have an accessable 2.04g version

(or newer) PKUNZIP.EXE!

*   Troubleshooting

Still not able to make it work?  Try adding either of the

following lines to your DATA\NET.9 file.  This is an ASCII

file which contains your Internet main configuration information.

You can edit this file with MSDOS EDIT or similar text editor

or word processor in PLAIN TEXT mode.

First try:

debuglevel=2

And if that doesn't give enough information, try:

debuglevel=3

Both will adversely affect program performance, but you will

get more information in the VUUCP.LOG file for study in solving

your problem.  These lines should, of course, be removed

once you've solved the problem.