Personal Communications Library

For Windows

(PCL4W)

REFERENCE MANUAL

Version 1.3

July 8, 1996

This software is provided as-is.

There are no warranties, expressed or implied.

Copyright (C) 1996

All rights reserved

MarshallSoft Computing, Inc.

Post Office Box 4543

Huntsville AL 35815

Voice : 205-881-4630

FAX : 205|880|0925

BBS : 205-880-9748

email : info@marshallsoft.com

web : www.marshallsoft.com

_______

____|__     |                (R)

--+       |    +-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

--+--+   |   +---------------------

|___|___|    MEMBER

PCL4W Reference Manual                                            Page 1

T A B L E

O F

C O N T E N T S

Chapter                                          Page

Table of Contents..................................2

Introduction.......................................3

SioBaud.........................................4

SioBrkSig.......................................4

SioCTS..........................................5

SioDCD..........................................5

SioDone.........................................6

SioDSR..........................................6

SioDTR..........................................7

SioFIFO.........................................7

SioFlow.........................................8

SioGetc.........................................8

SioGetDiv.......................................9

SioGets.........................................9

SioInfo........................................10

SioIRQ.........................................10

SioLine........................................11

SioLoopBack....................................11

SioModem.......................................12

SioParms.......................................12

SioPorts.......................................13

SioPutc........................................13

SioPuts........................................14

SioRead........................................14

SioReset.......................................15

SioRI..........................................15

SioRTS.........................................16

SioRxBuf.......................................16

SioRxClear.....................................17

SioRxQue.......................................17

SioTxBuf.......................................18

SioTxClear.....................................18

SioTxFlush.....................................19

SioTxQue.......................................19

SioUART........................................20

SioUnGetc......................................20

Function Summary..................................21

Error Code Summary................................22

Code Examples.....................................22

PCL4W Reference Manual                                            Page 2

Introduction

This  manual  lists  all of the PCL4W functions in alphabetical order. Every

library function will return a value as follows:

1.  Negative values for error conditions. See last page of this  manual  for

a list of error values and their meanings.

2.  Non-negative values when returning data (eg: SioLine).

3.  Zero otherwise.

When debugging an application, be sure  to  test  all  return  values.   Use

SioError() (see file SIOERROR.C) to print the associated text for errors.

Example Code Segment

+---------------------------------------------------------------+

| int Code;                 /* MUST be 'int', not 'char' */     |

|                                                               |

| Code = SioFunction( );    /* any PCL4W function */            |

| if(Code<0)                                                    |

|   {/* error returned */                                       |

|    SioError(Code);        /* SioError prints error text */    |

|    SioDone(Port);         /* always call SioDone last */      |

|    exit(1);                                                   |

|   }                                                           |

| /* no errors */                                               |

| ...your application code...                                   |

|                                                               |

+---------------------------------------------------------------+

The  program  SIMPLE is provided as a straight forward and simple example of

the use of the PCL4W library - at least as straight forward as  any  Windows

programming can be.

Also look at the TERM program, which is distributed separately from PCL4W as

WTERM12.ZIP. Many PCL4W functions are used somewhere in TERM.  Other example

programs will be provided. Look for them on our User Support BBS.

Also note that there are two versions of the library. One  version  is  with

transmitter  interrupts  disabled  and  one  is  with transmitter interrupts

enabled.

Refer to the User's Manual (PCL4W.USR) for additional information.

PCL4W Reference Manual                                            Page 3

+-------------+------------------------------------------------------------+

|   SioBaud   |  Sets the baud rate of the selected port.                  |

+-------------+------------------------------------------------------------+

Syntax    int SioBaud(int Port,int BaudCode)

/* Port:      Port selected (COM1 - COM20) */

/* BaudCode:  Baud code */

Remarks    The  SioBaud  function sets the baud rate without resetting  the

port. It is used to change the baud rate after calling SioReset.

Code   Rate    Name             Code   Rate   Name

0     300    Baud300            5    9600   Baud9600

1     600    Baud600            6   19200   Baud19200

2    1200    Baud1200           7   38400   Baud38400

3    2400    Baud2400           8   57600   Baud57600

4    4800    Baud4800           9  115200   Baud115200

Returns     -4 : No such port. Expect 0 to MaxPort.

-11 : Bad baud rate code. See above code values.

See Also    SioReset

+-------------+------------------------------------------------------------+

|  SioBrkSig  |  Asserts, cancels, or detects BREAK signal.                |

+-------------+------------------------------------------------------------+

Syntax    int SioBrkSig(int Port,char Cmd)

/* Port:      Port selected (COM1 thru COM20) */

/* char Cmd:  ASSERT, CANCEL, or DETECT */

Remarks    The SioBrkSig function controls the BREAK bit in the line status

register. The legal commands are:

ASSERT_BREAK ('A') to assert BREAK

CANCEL_BREAK ('C') to cancel BREAK

DETECT_BREAK ('D') to detect BREAK

ASSERT_BREAK, CANCEL_BREAK,  and  DETECT_BREAK  are  defined  in

PCL4W.H. See TERM.C for an example of the use of SioBrkSig.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-6 : Illegal command. Expected 'A', 'C', or 'D'.

>0 : BREAK signal detected (DETECT command only)

See Also    N/A

PCL4W Reference Manual                                            Page 4

+-------------+------------------------------------------------------------+

|   SioCTS    |  Reads the Clear to Send (CTS) modem status bit.           |

+-------------+------------------------------------------------------------+

Syntax    int SioCTS(int Port)

/* int Port: Port selected (COM1 thru COM20) */

Remarks    The SioCTS function is used to read  the  Clear  to  Send  (CTS)

modem status bit.

The  CTS  line  is  used  by  some  error  correcting  modems to

implement hardware flow control.  CTS is dropped by the modem to

signal the computer not to send data  and is  raised  to  signal

the computer to continue.

Refer to the User's Manual for a discussion of flow control.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : CTS is clear.

>0 : CTS is set.

See Also    SioFlow, SioDSR, SioRI, SioDCD, and SioModem.

+-------------+------------------------------------------------------------+

|   SioDCD    |  Reads the Data Carrier Detect (DCD) modem staus bit.      |

+-------------+------------------------------------------------------------+

Syntax    int SioDCD(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The  SioDCD  function is  used to read the Data Carrier Detect

(DCD) modem status bit. Also see SioModem.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : DCD is clear.

>0 : DCD is set.

See Also    SioDSR, SioCTS, SioRI, and SioModem.

PCL4W Reference Manual                                            Page 5

+-------------+------------------------------------------------------------+

|   SioDone   |  Terminates further serial processing.                     |

+-------------+------------------------------------------------------------+

Syntax    int SioDone(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The  SioDone  function  terminates  further  serial  processing.

SioDone  MUST  be called before exiting your application so that

interrupts can be restored to their original state.  Failure  to

do  this  can crash the operating system.  If you forget to call

SioDone before exiting, be sure to re-boot your  computer.   You

can  call SioDone even if SioReset has not been called, so it is

good  practice  to  always  call  SioDone  before  exiting  your

application.

Also  note  that  SioDone  dereferences the transmit and receive

buffers set up by SioRxQue and SioTxQue.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioReset.

+-------------+------------------------------------------------------------+

|    SioDSR   |  Reads the Data Set Ready (DSR) modem status bit.          |

+-------------+------------------------------------------------------------+

Syntax    int SioDSR(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The SioDSR function is used to read the  Data  Set  Ready  (DSR)

modem status bit.  The DSR line is normally  used  by  a  serial

device  (such  as  a  modem)  to  signal the computer that it is

ready.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : DSR is clear.

>0 : DSR is set

See Also    SioCTS, SioRI, SioDCD, and SioModem

PCL4W Reference Manual                                            Page 6

+-------------+------------------------------------------------------------+

|    SioDTR   |  Set, clear, or read Data Terminal Ready (DTR) status bit. |

+-------------+------------------------------------------------------------+

Syntax    int SioDTR(int Port,char Cmd)

/* Port: Port selected (COM1 thru COM20) */

/* Cmd:  DTR command (SET, CLEAR, or READ) */

Remarks    The SioDTR function controls the Data Terminal Ready (DTR) bit

in the modem control register.  DTR should always  be  set  when

communicating with a modem. Commands (defined in PCL4W.H) are:

SET_LINE ('S')  to set DTR (ON)

CLEAR_LINE ('C')  to clear DTR (OFF)

READ_LINE ('R')  to read DTR

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-5 : Not one of 'S', 'C', or 'R'.

0 : DTR is OFF (READ_LINE Command).

>0 : DTR is ON (READ_LINE Command).

See Also    SioRTS.

+-------------+------------------------------------------------------------+

|   SioFIFO   |  Sets the FIFO trigger level (16550 UART only).            |

+-------------+------------------------------------------------------------+

Syntax    int SioFIFO(int Port,int LevelCode)

/* Port:      Port selected (COM1 thru COM20) */

/* LevelCode: FIFO level code (see below) */

Remarks    The SioFIFO function  is  used to enable both transmit & receive

FIFOs for 16550 UARTS, and to set the  trigger  level  at  which

receive interrupts are generated.

For  example, if the FIFO level is set to 8, then the 16550 UART

will not generate an interrupt until 8 bytes have been received.

This reduces the  number  of  interrupts  generated  and  allows

faster   processing   with   slower  machines  or  when  running

simultaneous ports.

In order to take advantage of the TX FIFO, use the PCL4W library

with TX interrupts enabled.

In order to test if your port is a 16550 UART, call SioFIFO with

a LevelCode of other than FIFO_OFF. SioFIFO can  be  called  for

the 8250 and 16450 UART without ill effect.

Code  PCL4W.H Name  Trigger Level

-1      FIFO_OFF     Disable FIFO

0      LEVEL_1      1 byte

1      LEVEL_4      4 bytes

2      LEVEL_8      8 bytes

3      LEVEL_14     14 bytes

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

>0 : FIFO level set.

0 : FIFO level not set (not #16550).

PCL4W Reference Manual                                            Page 7

+------------+-------------------------------------------------------------+

|   SioFlow  |  Sets hardware (RTS/CTS) flow control.                      |

+------------+-------------------------------------------------------------+

Syntax    int SioFlow(int Port,int Tics)

/* Port: Port selected (COM1 thru COM20) */

/* Tics: # tics before TX timeout */

Remarks    The SioFlow function is used to enable or disable hardware  flow

control.  Hardware flow control uses RTS and CTS to control data

flow  between  the  modem and the computer.  Refer to the User's

Manual  for  a  discussion  of flow control.

'Tics'  is  the  number of timer tics before SioPutc will return

timeout on  transmission  because  CTS  is  down.  Refer  to  the

User's  Manual for a discussion of flow control. To disable flow

control, call SioFlow with Tics = -1.

In order for hardware flow control to work correctly, your modem

must also be configured to work with hardware flow control,  and

your  computer  to  modem  cable  must  have  RTS  and CTS wired

straight through. If you enable hardware flow  control,  do  not

modify  the  RTS  line  (by  calling  SioRTS)  unless  you  know

exactly what you are doing.

Flow control is disabled after resetting a port.  To  explicitly

disable hardware flow control, call SioFlow with Flag=FALSE.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

=0 : Flow control disabled.

>0 : Flow control enabled.

See Also    SioPutc

+------------+-------------------------------------------------------------+

|   SioGetc  |  Reads the next character from the serial line.             |

+------------+-------------------------------------------------------------+

Syntax    int SioGetc(int Port, int Tics)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The SioGetc function reads a byte from the selected serial port.

The  function  will  wait for the number of system tics given by

the 'Tics' argument before returning 'timed out'. There are 18.2

tics per second. If is best  to  limit  'Tics'  since  no  other

Windows task can run while waiting for input.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-1 : If timed out.

>0 : Character read.

See Also    SioGets, SioUnGetc and SioPutc.

PCL4W Reference Manual                                            Page 8

+---------------+----------------------------------------------------------+

|   SioGetDiv   |  Reads the baud rate divisor.                            |

+---------------+----------------------------------------------------------+

Syntax    int SioGetDiv(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks     The  SioGetDiv  function reads the baud rate divisor. The baud

rate can then be determined by the divisor:

Baud   Divisor      Baud  Divisor      Baud  Divisor

300    0180        4800   0018       38400   0003

1200    0060        9600   000C       57600   0002

2400    0030       19200   0006      115200   0001

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>0 : Baud rate divisor.

See Also    SioParms.

+------------+----------------------------------------------------------------+

|   SioGets  |  Receive a buffer from the serial line.                        |

+------------+----------------------------------------------------------------+

Syntax  int SioGets(int Port,char *Buffer,int Length)

/* Port: Port selected (COM1 thru COM20) */

/* Buffer: Character buffer */

/* Length: Length of Buffer (# of requested bytes) */

Remarks  The SioGets function reads bytes from the selected serial port.  This

function will read either the number of requested bytes or read fewer

if  no character is immediately available. SioGets cannot return more

characters than are in the receive buffer at the time it  is  called.

The number of characters actually read is returned.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-1 : If timed out.

>0 : Number of characters read into Buffer.

See Also  SioUnGetc and SioGetc.

PCL4W Reference Manual                                            Page 9

+-------------+---------------------------------------------------------------+

|   SioInfo   |  Returns PCL4C library information.                           |

+-------------+---------------------------------------------------------------+

Syntax  int SioInfo(char Cmd)

/* Cmd: Command (see below) */

Remarks  The SioInfo function returns a word value depending on the  character

argument   in   the  table  below.  This  function  is  usefull   for

understanding  what  the  hardware  is  doing.  Note that the command

argument is case sensitive.

Info Codes

'V' : Version [as hex byte XY means version X.Y]

'I' : TRUE if library compiled with TX interrupts enabled.

'P' : TRUE if compiled for protected mode.

'T' : Total (over all ports) transmitter interrupts.

'R' : Total (over all ports) receiver interrupts.

'd' : Total (over all ports) times RTS dropped.

'r' : Total (over all ports) times RTS raised.

'c' : Total (over all ports) times CTS drop detected.

Returns   -1 : Function argument not recognized.

>=0 : Value as per above table.

+-------------+------------------------------------------------------------+

|    SioIRQ   |  Assigns an IRQ line to a port.                            |

+-------------+------------------------------------------------------------+

Syntax    int SioIRQ(int Port,intIRQcode)

/* Port:    Port selected (COM1 thru COM20) */

/* IRQcode: IRQ number [IRQ2..IRQ15] */

Remarks    The SioIRQ function assigns an IRQ line to  a  port.   That  is,

SioIRQ  maps  an  IRQ  to a port.  SioIRQ (like SioUART) must be

called before calling SioReset. Unless you have  a  non-standard

(COM1  &  COM3  use IRQ4 while COM2 & COM4 use IRQ3) serial port

configuration  (or  don't  want  to  run  more  than   2   ports

concurrently),  you  will not need to call SioIRQ.  Be EXTREMELY

careful with SioIRQ as it can lock  your  machine  up  if  given

incorrect information.

In  particular,  remember  that your port hardware must generate

the interrupt that you have specified.  You should refer to your

serial  board  hardware  manual  for  specfic  instructions   in

configuring  your  hardware.  Be  sure to call SioPorts first to

setup DigiBoard ports if you have  them.   Refer  to  the  PCL4W

Users Manual for additional information.

Returns    -4 : No such port. Expect 0 to MaxPort.

-15 : Port already enabled.  SioReset has already been called.

|17 : No such IRQ.

-18 : ISR limit (maximum of 4 PC IRQs) exceeded.

0 : Otherwise

See Also    SioUART and SioPorts.

PCL4W Reference Manual                                            Page 10

+-------------+------------------------------------------------------------+

|   SioLine   |  Reads the line status register.                           |

+-------------+------------------------------------------------------------+

Syntax    int SioLine(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The  SioLine  function  reads  the  line  status  register.  The

individual bit masks are as follows:

0x40  = Transmitter empty.

0x20  = Transmitter buffer empty.

0x10  = Break detected.

0x08  = Framming error.

0x04  = Parity error.

0x02  = Overrun error.

0x01  = Data ready.

The above are documented in the file PCL4W.H.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>0 : Line status (rightmost byte of word).

See Also    SioModem.

+-------------+------------------------------------------------------------+

| SioLoopBack |  Does a UART loopback test.                                |

+-------------+------------------------------------------------------------+

Syntax    int SioLoopBack(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    SioLoopBack makes use of the built in loopback  test  capability

of the INS8250 family UART.   Normally  SioLoopBack  will  never

need  to  be called unless you suspect that your UART is bad. Be

sure to reset a UART after running a loopback test on it.

Returns      0 : Loopback test is successfull.

-2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

-12 : Loopback test fails.

PCL4W Reference Manual                                            Page 11

+-------------+------------------------------------------------------------+

|   SioModem  |  Reads the modem status register.                          |

+-------------+------------------------------------------------------------+

Syntax    int SioModem(int Port, int Mask)

/* Port: Port selected (COM1 thru COM20) */

/* Mask: Modem function mask */

Remarks    The  SioModem  function  reads  the  modem  register.    The bit

definitions for the function mask are as follows:

Bit Name  Function               Bit  Name      Function

7  DCD   Data Carrier Detect     3   DeltaDCD  DCD has changed

6  RI    Ring Indicator          2   DeltaRI   RI has changed

5  DSR   Data Set Ready          1   DeltaDSR  DSR has changed

4  CTS   Clear To Send           0   DeltaCTS  CTS has changed

Bits  4  through  7  represent  the  absolute  state  of   their

respective  RS-232  inputs.  Bits 0 through 3 represent a change

in the state of their respective RS-232 inputs since last  read.

Once UART register 3 is read, the Delta bits are cleared.  Thus,

it is best not to depend on the Delta bits.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>0 : Modem status (rightmost byte of word).

See Also   SioCTS, SioDCD, SioDSR and SioRI.

+-------------+------------------------------------------------------------+

|  SioParms   |  Sets parity, stop bits, and word length.                  |

+-------------+------------------------------------------------------------+

Syntax     int  SioParms(int Port,int Parity,int StopBits, int DataBits)

/* Port:     Port selected (COM1 - COM20) */

/* Parity:   Parity code [0,1,2] */

/* StopBits: Stop bits code [0,1] */

/* DataBits: Word length code [0,1,2,3] */

Remarks    The SioParms function sets  the  parity,  stop  bits,  and  word

length.   If  the  default parity (none), stop bits (1), or word

length (8) is not  acceptable,  then  they  can  be  changed  by

calling SioParms.  SioParms can be called either before or after

calling SioReset. See file PCL4W.H. (8 = default)

Value   Description    PCL4W.H Name

ParityCode:    *0      no parity      NoParity

1      odd parity     OddParity

3      even parity    EvenParity

StopBitsCode:  *0      1 stop bit     OneStopBit

1      2 stop bits    TwoStopBits

WordLengthCode: 0      5 data bits    WordLength5

1      6 data bits    WordLength6

2      7 data bits    WordLength7

*3      8 data bits    WordLength8

Returns     -4 : No such port. Expect 0 to MaxPort.

-7 : Bad parity code selected. Expecting  0 to 2.

|8 : Bad stop bits code. Expecting  0 or 1.

-9 : Bad word length code. Expecting  0 to 3.

See Also    SioReset.

PCL4W Reference Manual                                            Page 12

+-------------+------------------------------------------------------------+

|  SioPorts   |  Sets number of PC & Digiboard / BOCA Board ports.         |

+-------------+------------------------------------------------------------+

Syntax     int  SioPorts(int NbrPorts,int FirstPort,int StatusReg,

int Code)

/* NbrPorts:  Total number of ports */

/* FirstPort: First DigiBoard port */

/* StatusReg: DigiBoard Status Register */

/* Code:      PC_PORTS, DIGIBOARD, or BOCABOARD */

Remarks    The SioPorts function must be called  before  ANY  other  serial

functions.   The  purpose of the SioPorts function is to set the

total number of ports, the first DigiBoard (or BOCA board)  port

and the DigiBoard (or BOCA board) status register address.

Once SioPorts is called, all COM ports starting with "FirstPort"

will be treated as DigiBoard (or BOCA board) ports.  The default

setup is 4 standard PC ports and no DigiBoard or  BOCA  ports  [

SioPorts(4,4,0,PC_PORTS) ].

Refer  to  ther  PCL4W users maniual for more information on the

DigiBoard and BOCA board.

Returns    -4 : No such port. Expect 0 to 9.

0 : No error (sets MaxPort to NumberPorts-1).

See Also    SioUART, SioIRQ, and Code Examples.

+-------------+------------------------------------------------------------+

|   SioPutc   |  Transmit a character over a serial line.                  |

+-------------+------------------------------------------------------------+

Syntax    int SioPutc(int Port,char Ch)

/* Port: Port selected (COM1 thru COM20) */

/* Ch:   Character to send */

Remarks    The SioPutc function transmits one character  over the  selected

serial line.

If  flow  control has been enabled, then SioPutc may return a -1

(time out) if the  number  of  tics  specified  in  the  SioFlow

function was exceeded waiting for the modem to raise CTS.

Refer to the User's Manual for a discussion of flow control.

If  transmitter  interrupts  are  enabled  (there  are  separate

versions of the library  for  transmitter  interrupts  enabled),

then  the  byte  is  placed  in  the  transmit  buffer, awaiting

transmission by the PCL4W interrupt service routine.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-1 : Timed out waiting for CTS (flow control enabled)

See Also    SioPuts, SioGetc and SioFlow.

PCL4W Reference Manual                                            Page 13

+-------------+---------------------------------------------------------------+

|   SioPuts   |  Transmit a buffer over a serial line.                        |

+-------------+---------------------------------------------------------------+

Syntax  int SioPuts(int Port,char *Buffer,int Length)

/* Pot: Port selected (COM1 thru COM20) */

/* Buffer: Character buffer */

/* Length: Length of Buffer */

Remarks  The  SioPuts  function  transmits  each  buffer  character  over  the

selected  serial line.  The number of characters actually transmitted

is returned. This function can take a long time for large buffers  if

transmitter interrupts are not enabled!

If  flow  control  has  been enabled, then SioPuts may transmit fewer

characters than requested if the number  of  tics  specified  in  the

SioFlow  function  was  exceeded  waiting for the modem to raise CTS.

Refer to the User's Manual for a discussion of flow control.

If transmitter interrupts are enabled, SioPuts  will  transmit  fewer

than  the number of characters requested if the transmit buffer would

overflow. The actual number of characters transmitted is returned.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>=0: Number of characters actually transmitted.

See Also  SioPutc, SioFlow, and SioTxFlush.

+-------------+------------------------------------------------------------+

|   SioRead   |  Reads any UART register.                                  |

+-------------+------------------------------------------------------------+

Syntax    int SioRead(int Port,int Reg)

/* Port: Port selected (COM1 thru COM20) */

/* Reg:  UART register (0 to 7) */

Remarks    The SioReset function directly reads the contents of any of  the

7  UART  registers.   This  function  is  useful  when debugging

application  programs  and  as  a  method  for  verifying   UART

contents.   Refer  to the PCL4W Users Manual for a discussion of

the 7 UART registers.

The line status register (register 5)  can  also  be  read  with

SioLine while the modem status register (register 6) can also be

read  with  SioModem.  Refer  to  the  PCL4W User's Manual for a

discussion of the UART registers.

Returns     -3 : No buffer available. Call SioRxBuf first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioLine and SioModem.

PCL4W Reference Manual                                            Page 14

+-------------+------------------------------------------------------------+

|  SioReset   |  Initialize a serial port for processing.                  |

+-------------+------------------------------------------------------------+

Syntax    int SioReset(int Port,int BaudCode)

/* Port:     Port selected (COM1 thru COM20) */

/* BaudCode: Baud code or -1 */

Remarks    The SioReset function  initializes  the  selected  serial  port.

SioReset should be called after calling SioParm and SioRxBuf but

before  making  any  other  calls  to  PCL4W.  SioReset uses the

parity, stop bits, and  word  length  value  previously  set  if

SioParm  was  called, otherwise the default values (see SioParm)

are used.

Recall that COM1 and COM3 share the same  interrupt  vector  and

therefore  cannot  operate  simultaneously. Similiarly, COM2 and

COM4 cannot operate simultaneously. Any other combination of two

ports can be used.

By specifing NORESET (-1) for the baud rate code, the port  will

NOT  be  reset.   This is used to "take over" a port from a host

communications program that allows a  "DOS  gateway".   External

protocols  can  be implemented this way.  See SioBaud for a list

of the baud rate codes, or see "PCL4W.H".

Returns     -3 : No buffer available. Call SioRxBuf first.

-4 : No such port. Expect 0 to MaxPort.

-11 : Bad baud rate code selected. Expecting  0 to 9.

|13 : UART undefined.  SioUART(Port,0) was called previously.

|14 : Bad or missing UART.  You may not have hardware present.

|15 : Port already enabled.  SioReset has already been called.

-16 : Interrupt already in use.

See Also    SioBaud, SioParms, SioRxBuf, SioDone, and SioUART.

+-------------+------------------------------------------------------------+

|    SioRI    |  Reads the Ring Indicator (RI) modem status bit.           |

+-------------+------------------------------------------------------------+

Syntax    int SioRI(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The SioRI function is used to read the Ring  Indicator

(RI) modem status bit. Also see SioModem.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : RI is clear.

>0 : RI is set (RING has occurred).

See Also    SioDSR, SioCTS, SioDCD, and SioModem.

PCL4W Reference Manual                                            Page 15

+-------------+------------------------------------------------------------+

|    SioRTS   |  Sets, clears, or reads the Request to Send (RTS) line.    |

+-------------+------------------------------------------------------------+

Syntax    int SioRTS(Port, char Cmd)

/* Port: Port selected (COM1 thru COM20) */

/* Cmd:  RTS command (SET, CLEAR, or READ) */

Remarks    The SioRTS function controls the Request to Send (RTS bit in the

modem control register.

The  RTS  line  is  used  by  some  error  correcting  modems to

implement hardware flow control.  RTS is dropped by the computer

to signal the modem not to send data, and is  raised  to  signal

the modem to continue. RTS should be set when communicating with

a modem unless Flow Control is being used.

Refer  to  the  User's  Manual for a discussion of flow control.

Commands (defined in PCL4W.H) are:

SET_LINE ('S')   - set RTS (ON)

CLEAR_LINE ('C') | clear RTS (OFF)

READ_LINE ('R')  - read RTS

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-5 : Command is not one of 'S', 'C', or 'R'.

0 : RTS is OFF (READ_LINE Command).

>0 : RTS is ON  (READ_LINE Command).

See Also    SioFlow and SioDTR.

+------------+-------------------------------------------------------------+

|  SioRxBuf  |  Sets up receive buffers.                                   |

+------------+-------------------------------------------------------------+

Syntax    int SioRxBuf(int Port,int Selector,SizeCode)

/* Port:     Port selected (COM1 thru COM20) */

/* Selector: Receive buffer selector */

/* SizeCode: Buffer size code */

Remarks     The  SioRxBuf  function passes the address selector and size of

the receive buffer to  PCL4W.   Recall  that  PCL4W  requires  a

receive buffer for each port in simultaneous operation since the

receive  function is interrupt driven.  It must be called before

any incoming characters can be received. SioRxBuf must be called

before SioReset.  Buffer size codes are listed in "PCL4W.H".

Size Code       Buffer Size    PCL4W.H Name

4            128 bytes        Size128

5            256 bytes        Size256

6            512 bytes        Size512

7           1024 bytes        Size1024 or Size1K

8           2048 bytes        Size2048 or Size2K

9           4096 bytes        Size4096 or Size4K

10           8192 bytes        Size8192 or Size8K

11          16384 bytes        Size16384 or Size16K

12          32768 bytes        Size32768 or Size32K

Returns    -4 : No such port. Expect 0 to MaxPort.

-10 : Bad buffer size code. Expecting 0 to 11.

See Also    SioReset and Code Examples.

PCL4W Reference Manual                                            Page 16

+------------+-------------------------------------------------------------+

| SioRxClear |  Flushes (clears) the receive buffer.                       |

+------------+-------------------------------------------------------------+

Syntax    int SioRxClear(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The SioRxClear  function  will  delete  any  characters  in  the

receive  buffer  for  the  specified port.  After execution, the

receive buffer will be empty.  Call SioRxBuf after  resetting  a

port in order to delete any spurious characters.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioRxQue.

+-------------+------------------------------------------------------------+

|  SioRxQue   |  Returns the number of bytes in the receive queue.         |

+-------------+------------------------------------------------------------+

Syntax    int SioRxQue(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The  SioRxQue  function  will return the number of characters in

the receive queue at the time of the call. It  can  be  used  to

implement XON/XOFF flow control.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also     SioRxClear.

PCL4W Reference Manual                                            Page 17

+------------+-------------------------------------------------------------+

|  SioTxBuf  |  Sets up transmitter buffer.                                |

+------------+-------------------------------------------------------------+

Syntax    int SioTxBuf(int Port,int Selector,int SizeCode)

/* Port:     Port selected (COM1 thru COM20) */

/* Selector: Receive buffer selector */

/* SizeCode: Buffer size code */

Remarks     The  SioTxBuf  function passes the address selector and size of

the transmit buffer to PCL4W, provided that you will link with a

PCL4W  library  with  transmitter  interrupts  enabled.    PCL4W

requires  a  transmit  buffer  for  each  port  in  simultaneous

operation if you are using  the  version  of  the  library  with

transmitter interrupts enabled (PCL4W2.LIB).  SioTxBuf() must be

called before SioReset.

Size Code       Buffer Size    PCL4W.H Name

4            128 bytes        Size128

5            256 bytes        Size256

6            512 bytes        Size512

7           1024 bytes        Size1024 or Size1K

8           2048 bytes        Size2048 or Size2K

9           4096 bytes        Size4096 or Size4K

10           8192 bytes        Size8192 or Size8K

11          16384 bytes        Size16384 or Size16K

12          32768 bytes        Size32768 or Size32K

This function is not used unless the transmitter interrupts  are

enabled  (PCL4W2.LIB).  Refer  to  the  PCL4W  Users  Manual for

information on transmitter (TX) interrupts.

Returns     -4 : No such port. Expect 0 to MaxPort.

-10 : Bad buffer size code. Expecting 0 to 11.

See Also    SioRxBuf, SioReset and Code Examples.

+------------+-------------------------------------------------------------+

| SioTxClear |  Flushes (clears) the transmitter buffer.                   |

+------------+-------------------------------------------------------------+

Syntax    int SioTxClear(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The SioTxClear  function  will  delete  any  characters  in  the

transmit  buffer  for the specified port, provided that you will

link with a PCL4W library with transmitter  interrupts  enabled.

After execution, the transmit buffer will be empty.

Once  this  function is called, any character  in  the  transmit

buffer  (put  there  by  SioPutc) will be lost and therefore not

transmitted. This function is not used  unless  the  transmitter

interrupts  are  enabled  Refer  to  the  PCL4W Users Manual for

information on transmitter (TX) interrupts.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioTxQue.

PCL4W Reference Manual                                            Page 18

+------------+----------------------------------------------------------------+

| SioTxFlush |  Flushes the transmitter buffer.                               |

+------------+----------------------------------------------------------------+

Syntax  int SioTxFlush(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks  The SioTxFlush function will re-enable transmitter interrupts if they

had been disabled by CTS dropping. This has the effect of forcing all

data out of the transmitter queue unless stopped again by  CTS  going

down.

Once the transmitter interrupt has been disabled (by CTS going down),

then the only way to restart it is by calling one of the  serial  I/O

functions  (SioPutc,  SioPuts,  SioGetc,  or  SioGets),  SioTxQue, or

SioTxFlush.

The primary purpose of SioTxFlush is to provide an explicit means  of

restarting  transmitter  interrupts.

Returns  -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also  SioTxQue.

+------------+-------------------------------------------------------------+

|  SioTxQue  |  Returns the number of bytes in the transmit queue.         |

+------------+-------------------------------------------------------------+

Syntax    int SioTxQue(int Port)

/* Port: Port selected (COM1 thru COM20) */

Remarks    The SioTxQue function will return the number  of  characters  in

the  transmit  queue  at the time of the call, provided that you

will link with  a  PCL4W  library  with  transmitter  interrupts

enabled.

This function is not used unless the transmitter interrupts  are

enabled.  Refer  to  the  PCL4W  Users Manual for information on

transmitter (TX) interrupts.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also     SioTxClear.

PCL4W Reference Manual                                            Page 19

+------------+-------------------------------------------------------------+

|  SioUART   |  Sets the UART base address.                                |

+------------+-------------------------------------------------------------+

Syntax    int SioUART(int Port,int Address)

/* Port:    Port selected (COM1 thru COM20) */

/* Address: UART address */

Remarks    The  SioUART  function  sets  the  UART  base  address  for  the

specified port.  SioUART must be called before SioReset in order

to have effect.  Be extremely sure that you know  what  you  are

doing!   Note   that  PCL4W  uses  the  standard  PC/XT/AT  port

addresses,  interrupt  request  lines,  and  interrupt   service

vectors.   Therefore,   this   function   is   only  needed  for

non-standard ports.

Returns    >0 : The previous base address for this port.

-4 : No such port.  Expect 0 to MaxPort.

-15 : Port already enabled.  SioReset has already been called.

See Also    SioPorts, SioIRQ, and SioReset.

+------------+-----+-------------------------------------------------------+

| SioUnGetc  |  "Un|gets" the last character read with SioGetc().          |

+------------+-----+-------------------------------------------------------+

Syntax    int SioUnGetc(int Port,char Ch)

/* Port: Port selected (COM1 thru COM20) */

/* Ch:   Character to unget */

Remarks    The SioUnGetc function returns  ("pushes")  the  character  back

into  the serial input buffer.  The character pushed will be the

next character returned by SioGetc.  Only one character  can  be

pushed back. This function works just like the "ungetc" function

in the C language.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioReset.

PCL4W Reference Manual                                            Page 20

Function Summary

+-------------+----------+----------+----------+----------+

| Function    |   Arg1   |   Arg2   |   Arg3   |   Arg4   |

+-------------+----------+----------+----------+----------+

| SioBaud     | Port     | BaudCode |          |          |

| SioBrkSig   | Port     | Cmd      |          |          |

| SioCTS      | Port     |          |          |          |

| SioDCD      | Port     |          |          |          |

| SioDone     | Port     |          |          |          |

| SioDSR      | Port     |          |          |          |

| SioDTR      | Port     | Cmd      |          |          |

| SioFIFO     | Port     | FIFOcode |          |          |

| SioFlow     | Port     | Tics     |          |          |

| SioGetc     | Port     | Tics     |          |          |

| SioGetDiv   | Port     |          |          |          |

| SioGets     | Port     | Buffer   | Length   |          |

| SioInfo     | Cmd      |          |          |          |

| SioIRQ      | Port     | IRQcode  | ISRcode  |          |

| SioLine     | Port     |          |          |          |

| SioLoopBack | Port     |          |          |          |

| SioModem    | Port     | Mask     |          |          |

| SioParms    | Port     | Parity   | StopBits |WordLength|

| SioPorts    | NbrPorts | FirstDBP |StatusReg | Code     |

| SioPutc     | Port     | Ch       |          |          |

| SioPuts     | Port     | Buffer   |  Length  |          |

| SioRead     | Port     | Register |          |          |

| SioReset    | Port     | BaudCode |          |          |

| SioRI       | Port     |          |          |          |

| SioRTS      | Port     | Cmd      |          |          |

| SioRxBuf    | Port     | Buffer   | SizeCode |          |

| SioRxClear  | Port     |          |          |          |

| SioRxQue    | Port     |          |          |          |

| SioTxBuf    | Port     | Buffer   | SizeCode |          |

| SioTxClear  | Port     |          |          |          |

| SioTxFlush  | Port     |          |          |          |

| SioTxQue    | Port     |          |          |          |

| SioUART     | Port     | Address  |          |          |

| SioUnGetc   | Port     | Ch       |          |          |

+-------------+----------+----------+----------+----------+

PCL4W Reference Manual                                            Page 21

Error Code Summary

+------+--------------------------------------------------------+

| Code |  Description                                           |

+------+--------------------------------------------------------+

|   0  |  No error.                                             |

|  -1  |  Timeout waiting for I/O.                              |

|  |2  |  Port not enabled. Call SioReset first.                |

|  |3  |  No buffer available. Call SioRxBuf first.             |

|  |4  |  No such port. Expect 0 to MaxPort. (COM1 = 0)         |

|  |5  |  Expected 'S', 'C', or 'R' as 2nd argument.            |

|  |6  |  Expected 'A', 'C', or 'D' as 2nd argument.            |

|  |7  |  Bad parity code specified. Expected 0 to 7.           |

|  |8  |  Bad stop bits code specified. Expected 0 or 1.        |

|  -9  |  Bad wordlength code specified. Expect 0 to MaxPort.   |

| -10  |  Bad buffer size code specified. Expected 0 to 11.     |

| |11  |  Bad baud rate code. Expected 0 to 9.                  |

| |12  |  Loopback test fails.                                  |

| |13  |  UART undefined.                                       |

| |14  |  Missing or bad UART. (no UART hardware ?)             |

| |15  |  Port already enabled.                                 |

| |16  |  ISR (interrupt) already in use.                       |

| |17  |  No such IRQ. (Should be 2 to 15)                      |

| |18  |  ISR limit (maximum of 4 PC IRQs) exceeded.            |

+-+----+--------------------------------------------------------+

Code Examples

SioRxBuf() Example

dwValue = GlobalDosAlloc(1024));

if(dwValue)

{/* get selector & lock */

RxSelector = LOWORD(dwValue);

GlobalPageLock(RxSelector);

/* pass selector to PCL4W */

SioRxBuf(Port,RxSelector,Size1024);

}

DigiBoard Example

/* use 0x140 for odd IRQs & 0x141 for even IRQs */

SioPorts(8,COM1,0x140,DIGIBOARD);

for(i=COM1;i<=COM20;i++)

{/* set DigiBoard UART addresses */

SioUART(i,0x100+8*i);

/* set DigiBoard for IRQ5 */

SioIRQ(i,IRQ5);

}

BOCA Board Example

/* use base port + 7 */

SioPorts(16,COM1,0x100+7,BOCABOARD);

for(i=COM1;i<=COM16;i++)

{/* set BOCA Board UART addresses */

SioUART(i,0x100+8*i);

/* set DigiBoard for IRQ15 */

SioIRQ(i,IRQ15);

}

PCL4W Reference Manual                                            Page 22