*** PCLVBW 1.1 Release Notes ***

(1) Always choose LEVEL_8 rather than LEVEL_14 in calling SioFIFO. Windows

is slow, and LEVEL_14 only allows 2 additional characters before an overun.

(2) Be sure to copy PCLVBW.DLL to the directory in which you have installed

Visual Basic so it can be used by the interpreter. Also copy MIO.DLL.

(3) The module SIOERROR.BAS will print the text associated with an error code

returned from any of the PCLVBW functions.

(4) If you will be using the standard PC ports COM1 to COM4, they should be

disabled from Windows's control becuase Windows "virtualizes" all ports it

controls. For example, to disable Windows control of COM1 and COM3 (which

share IRQ3), add

COM1Base=0

COM1Irq=-1

COM3Base=0

COM3Irq=-1

to the [386Enh] section of SYSTEM.INI in the Windows directory.

(5) We will be releasing a version of our VB communications library that

uses the standard Windows API, rather than talking to the port hardware

directly as is done in PCLVBW. It will be named the "Windows Standard

Communications" (WSC) library, and will not require that the SYSTEM.INI

file be modified. Look for WSC4VB10.ZIP to be released July 15th, 1996.

#