*** Making a loopback adapter ***

A loopback adapter can easily be made from a female-female adapter. Orient

the female-female adapter such that the widest part is facing up. Connect

pins 2 and 3 together with a short piece of wire as shown below.

Female DB 25                        Female DB 9

*****************************************      *****************

* 13 12 11 10  9  8  7  6  5  4  3==2  1  *    *  5  4  3==2  1  *

*   25 24 23 22 21 20 19 18 17 16 15 14 *      *   9  8  7  6  *

***************************************        ***************

DB 25   DB 9    Pin Definition

=====   ====    ==============

7       5      Signal Ground (SG)

2       3      Transmit Data (TD)

3       2      Receive Data (RD)

20       4      Data Terminal Ready (DTR)

6       6      Data Set Ready (DSR)

4       7      Request to Send (RTS)

5       8      Clear to Send (CTS)

8       1      Carrier Detect (CD)

Note: The pin positions are reversed for male connectors.

#