Mouse Driver for Windows 3.0

A.  Driver Files

There are 2 drivers for Mouse Systems Mice and Windows 3.0:

3MSCWIN3.DRV - driver for 3-button protocol mouse on bus card.

2MSCWIN3.DRV - driver for 2-button protocol mouse (or PC Trackball

on COM ports or bus card.

B. Installation

To install the above drivers,

1.  change directory to your "Windows" directory and type

SETUP <ENTER>

2.  use UP cursor key to select "Mouse:" on the 1st SETUP screen

and press <ENTER>

3.  use DOWN cursor key to select "Other" on the 2nd SETUP screen

and press <ENTER>

4.  SETUP will now display the default drive and directory where

Windows mouse driver(s) can be found.  If you have copied the

drivers to your hard disk, type the drive and directory where

you copied the drivers and press <ENTER>.  If the driver files

are on a floppy disk, insert it to driver A: and press <ENTER>.

5.  SETUP displays the description of the 2 drivers:

Mouse Systems 2 Button Mouse, BUS VERSION, Windows 2.x

Mouse Systems 3 Button Mouse on Bus Card, Windows 2.x

Mouse Systems 2 Button Mouse, BUS VERSION, Windows 3.0

Mouse Systems 3 Button Mouse on Bus Card, Windows 3.0

use the cursor keys to select one of the Windows 3.0 drivers

and press <ENTER>.

6.  select "Complete Changes:" and press <ENTER>.

SETUP will copy the driver file and modify your SYSTEM.INI file

automatically.

C.  Note

At this time, Microsoft does not provide support for mice on

IRQ's above 7.  These drivers work only on IRQs 2, 3, 4, 5, 7.