Enter choice !1

#: 19150 S1/General [Win]

03-Jul-90  21:44:31

Sb: #Win3.0 disk problem

Fm: MS SysOp Runnoe Connally 76701,144

To: All

The Microsoft Windows development team, working closely with several hardware

and software vendors, has identified two rare scenarios that might result in

loss or corruption of data. The scenarios involve using third-party

disk-partitioning software with Microsoft's SMARTDrive disk caching utility. We

have shipped several hundred thousand copies of Windows 3.0 and have only 12

customers with verified problems.

Microsoft Windows 3.0 itself works fine on these systems.  The conflict arises

between the disk caching utility included with Windows called "SmartDrive" and

the listed third-party disk- partitioning software.  Windows 3.0 is fully

functional without the optional disk caching utility.

SYSTEMS THAT MIGHT BE AT RISK

-----------------------------

Scenario 1

----------

Your system IS at risk if:

*  You are using THIRD-PARTY disk positioning software, such

as those listed below AND

*  A hard disk on your system has more than 1024 cylinders, AND

*  You are using Microsoft Windows' SMARTDrive disk caching utility

This scenario can only occur if you are running third-party disk partitioning

software, because the DOS Fdisk partitioning software does not support disks

that have more than 1024 cylinders.

Scenario 2

----------

Your system MIGHT be at risk if:

*  You are using THIRD-PARTY disk positioning software, such

as those listed below AND

*  Your system contains a hard disk whose type is not directly

supported by the system's ROM BIOS AND

*  You are using Microsoft Windows' SMARTDrive AND

*  You are using DOS version 3.30 or higher

The risk probability is greater if your system was customized with

off-the-shelf parts. Name brand computers ship with ROM BIOS versions that

support the hard drives installed by the manufacturer.

-------------------- *** Continued on the next message ***

There are 3 Replies.

Press <CR> for next or type CHOICES !

#: 19153 S1/General [Win]

03-Jul-90  21:52:31

Sb: #19150-#Win3.0 disk problem

Fm: MS SysOp Runnoe Connally 76701,144

To: MS SysOp Runnoe Connally 76701,144 (X)

*** Continued from previous message ***

DETERMINING THE RISK

--------------------

Below is a list of known disk partitioning systems which when used in

conjunction with SMARTDrive pose a risk.

Each software package uses a device= line in the CONFIG.SYS file in your root

directory. The device= entry tells your system that the software exists.

Open your CONFIG.SYS file with any text editor and see if one of the following

device= lines exists:

SOFTWARE:     Disk Manager from Ontrack Computer Systems

CONFIG.SYS    device=DMDRVR.BIN

SOFTWARE:     SpeedStor from Storage Dimensions

CONFIG.SYS:   device=SSTOR.SYS or device=HARDRIVE.SYS

SOFTWARE:     InnerSpace from Priam Systems

CONFIG.SYS:   device=EDVR.SYS

SOFTWARE:     Vfeatures Deluxe from Golden Bow Systems

CONFIG.SYS:   device=FIXT_DRV.SYS

NOTE: At the end of this communication you'll find a procedure for using Disk

Manager to diagnose the risk. Use that procedure if you have Disk Manager by

Ontrack.

REMOVING THE RISK

-----------------

If you've identified your system at risk, you can either remove SMARTDrive or

repartition your hard disk using the DOS Fdisk utility instead of third part

disk partitioning software.

To remove SMARTDrive from your system:

1. Open your CONFIG.SYS file with any text editor.

2. Delete the following line:

device=smartdrv.sys

3. Save the changes to CONFIG.SYS.

4. Press CTRL+ALT+DEL to reboot your system (or turn off your

system and then turn it back on).

Microsoft is working on making SMARTDrive aware of these disk partitioning

packages, and will provide a free update to affected users in the near future.

If you have additional questions, contact Microsoft Product Support at (206)

454-2030.

*** Continued on the next message ***

There is 1 Reply.

Press <CR> for next or type CHOICES !

#: 19155 S1/General [Win]

03-Jul-90  22:00:51

Sb: #19153-Win3.0 disk problem

Fm: MS SysOp Runnoe Connally 76701,144

To: MS SysOp Runnoe Connally 76701,144 (X)

*** Continued from the previous message ***

IF YOU USE DISK MANAGER

-----------------------

The following procedure applies to Disk Manager users only. Use it

to determine if your system is at risk.

To determine if you have a hard drive with more than 1024 cylinders:

1. At the DOS prompt, invoke Disk Manager in manual mode by typing

"DM /M".

2. Type "C": to display the disk configuration.

3. If any of the disks displayed have an Actual Cylinder Count over

1024, you are at risk when using SMARTDrive.

To determine if your hard disk is not supported in the system BIOS:

1. At the DOS prompt, type "VER" to display the version of DOS you

are running.  If it is earlier than version 3.30, you are not at

risk. Otherwise continue.

2. At the DOS prompt, invoke Disk Manager in manual mode by typing

"DM /M".

3. Type "C" to display the disk configuration.

4. If the display says Parameters Are Standard, you are not at risk.

Otherwise, if it says Parameters Are Replaced, continue.

5. For each disk in your system, compare the Setup Parameters to the

Actual Parameters displayed. If all of the disks displayed have a

Setup Head Count which matches the Actual Head Count, and a Setup

Sector Count which matches the Actual Sector Count, then you are

not at risk. Otherwise, that drive is referred to as "non-

homogeneous", and you will need to continue.

6. From the Disk Manager main menu, type "S" to select the non-

homogeneous drive.

7. Type "P" to see the partition table for this drive.

8. If the drive contains a partition with the Type "DOS", AND one of

more Type "Write-Read" or "Read-Only" partitions, then you are at

risk.

9. Repeat steps 6 through 8 for each non-homogeneous drive in your

system.

Press <CR> for next or type CHOICES !