Version 0.0f

INSTALLATION OF SUMMAGRAPHICS' WINDOWS 3.0 TABLET DRIVER

----------------------------------------------------------

There are two ways to install the Summagraphics MM format tablet

driver, tablet.drv:

1. during the installation of Windows 3.0

2. after the installation of Windows 3.0

If Windows 3.0 has already been installed on your machine, go to

Step 1, below.

If Windows 3.0 has NOT been installed, follow the installation

procedure in your Windows 3.0 release material.  When you reach the

section where you're asked to choose a pointing device, choose "Other,"

and hit RETURN.	 Now, go to Step 5, below.

1. Go to the Windows 3.0 directory, e.g.,

cd c:\windows

2. Run the Windows 3.0 setup program, ie,

setup

3. In the System Information section, move the highlight to the

Mouse: line.   Hit RETURN.

4. In the Mouse selection section, move the highlight to "Other."

Hit RETURN.

5. Insert the Summagraphics Tablet Driver disk.  Hit RETURN.

6. ("Summagraphics Tablet Driver/Mouse Emulator" should now be

highlighted.)  Hit RETURN.

7. When the System Information section is shown, "accept the

above configuration" by hitting RETURN.

8.  Modify the Windows initialization file, win.ini, in the \Windows

directory, as follows:

a. Put the Summagraphics Windows 3.0 Driver disk in Drive A

b. Type:  TYPE A:\SUMMA.INI >> \WINDOWS\WIN.INI

9.  Run Windows 3.0 ("win") to test the tablet driver installation.

Note that the driver defaults to :

Absolute mode

Four-button cursor with

Button 1 = Left mouse button

Button 2 = Double click of left mouse button

Full scale on a 12X12 SummaSketch tablet

10.  If the driver defaults are unsatisfactory, read the following section,

"Modifying Default Tablet Driver Variables."

11.  If the above installation fails, read the section, "What to do if the

tablet driver installation fails."

---------------------------------------------------------------------------

Modifying Default Tablet Driver Variables

--------------------------------------------

The driver reads its setup variables from win.ini in the \Windows directory.

To change these variables, edit the following sections in win.ini and modify

to suit your situation:

; Setup variables for pointing device

[Tablet Mouse Emulation]

; 1 for absolute mode, 0 for relative mode

AbsoluteMode=1

; Fraction that we multiply raw tablet deltas by in relative mode;

;  ignored in absolute mode.  If the cursor moves too fast for your tastes,

;   try 1/5 or 2/5.

Sensitivity=1/5

; Coordinates of mouse area in tablet units:

;  origin in top left corner, x increases rightward, y downward

;    Use 0    0   4499  2999 for a  9-by-6  tablet

;    Use 0    0   5850  5850 for a 12-by-12 tablet

;    Use 0    0   8999  5999 for a 18-by-12 tablet

;        left top right	bottom

MouseBox=0    0   5850  5850

; 0 for no mouse button function

; 1 for left click

; 2 for right click

; 3 for left double click

; 4 for right double click

; btn #:  1 2 3 4 etc. (up to 16)

ButtonMap=1 3 2 4

; hardware setup

[SummaSketch Tablet]

; 0 = stylus

; 1 = 3-button

; 2 = 4-button

Cursor=2

---------------------------------------------------------------------------

**** After modifying win.ini, restart Windows. ****

---------------------------------------------------------------------------

What to do if the tablet driver installation fails:

-----------------------------------------------------

If the Windows 3.0 Setup program failed to properly install the tablet

driver, try the following:

1. copy tablet.drv to the Windows 3.0 system subdirectory, e.g.,

copy  a:\tablet.drv  \windows\system

2. edit the system.ini file in the Windows 3.0 directory so that the

[boot] section contains the line:

mouse.drv=tablet.drv

The above steps are sufficient to install the driver. Optionally,

you can inform the Windows 3.0 Setup Program of tablet.drv by adding

the following line to the [pointing.device] section of setup.inf

in the Windows 3.0 system directory:

tablet   = 1:tablet.drv, "Summagraphics Tablet",         x:*vmd