The ERASECHK program will measure the effectiveness of bulk

erasure of a floppy disk. If you have sensitive data, it would be

wise to test a floppy after using an eraser to make sure no data

remains on the disk.

The program is written in Turbo Pascal (version 3.0) and runs

under IBM PC-DOS, but it should also work under MS-DOS on IBM-

PC clones and will probably run under Turbo 2.0. Currently the B:

drive is hardcoded into the program as the drive the erasechk will

be performed on.

ERASECHK attempts to read each sector on a disk and summarizes

the results. A normal 360k-byte disk has 720 sectors (40 tracks x  2

heads x 9 sectors/track), each of which will be readable (return

code 0). A blank disk will have 720 "address mark not found" errors

(return code 2). Partially erased disks will have mixed results:

some sectors will be missing, others will have CRC (cyclic redundancy

check) errors (return code 16), and some will be unaffected.#############