Version 1.10 		   EGA�RAM		  1 June 1988

------------------------------------------------------------------

EGA�RAM is yet another entry in my amazingly popular series of EGA

enhancement programs (I like them, anyway).	This particular program

was inspired by the recent announcements by Compaq, Quarterdeck and

others of so-called "shadow RAM" products.  Basically, the idea is to

copy the BIOS code from slow ROM to fast RAM.  Since most EGA cards

are designed to fit the least common denominator, i.e. 8-bit PC and

XT slots, one can achieve remarkable improvements in performance on

(16-bit) AT and (32-bit) 386 machines.  [It may be that performance

improves a little on plain-jane PCs, too, simply because the RAM is

slightly faster than ROM; I haven't tested this].

HOW TO USE IT:

--------------

EGA�RAM is written as a device driver and as such is loaded at boot

time by a statement in your CONFIG.SYS file:

device = EGA2RAM.SYS     (assuming it's in your

root directory)

EGA�RAM should be loaded BEFORE any other drivers which mess with

the video routines, in particular ANSI.SYS and any Microsoft-compat-

ible mouse drivers.	If in doubt, just load EGA�RAM first; it will

create no compatibility problems that are not already present with

the EGA BIOS.  EGA�RAM typically uses 16k to 32k of RAM, depending

on your particular EGA card (like all device drivers, it is RAM-

resident):  IBM's EGA requires only 16k, but most clone EGA's will

require a full 32k.	(Please read the disclaimer below before using

EGA�RAM...)

HOW FAST IT IS:

---------------

I tested EGA�RAM on a 9 MHz IBM AT with IBM's EGA card (it had 256k

on it, but that shouldn't matter any).  I used PC Magazine's bench-

mark series (version 4.01), with the following results:

TEST	  REGULAR BIOS	 EGA�RAM BIOS	% IMPROVEMENT IN SPEED

----	  ------------	 ------------	----------------------

No scrolling:    7.53 sec	   3.92 sec		  92%

Scrolling:	   10.97 sec	   7.27 sec		  51%

Direct write:    7.90 sec	   7.89 sec		  .1%

Line pattern:   36.5  sec	  19.21 sec		  90%

All times given are the average of three trials, and the last test

was timed using a stop watch (so is accurate to only +/- .05 sec).

Note that the direct write to display memory, since it bypasses the

page 2

BIOS completely, shows no improvement.  However, the other tests

show a remarkable 50 to 100% improvement!!!	On a 32-bit 386 machine

the results are even more striking -- up to 330% on a 16 MHz Toshiba

T5100:

TEST	  REGULAR BIOS	 EGA�RAM BIOS	% IMPROVEMENT IN SPEED

----	  ------------	 ------------	----------------------

No scrolling:    3.85 sec	   1.16 sec		 232%

Scrolling:	    7.52 sec	   4.28 sec		  76%

Direct write:    8.19 sec	   7.36 sec		  11%

Line pattern:   73.55 sec	  16.95 sec		 334%  (!)

(Of course, the Toshiba comes with an option called "FastROM" which

makes EGA�RAM redundant, but if it *didn't* ....)

I have (obviously) installed EGA�RAM on my system, and the speedup is

quite noticeable in ordinary DOS tasks such as directory listings

and so forth.  Programs such as Microsoft WORD and Borland's SIDEKICK

will not benefit much since they write directly to memory, but "well-

behaved" programs which use the BIOS interface for video things will

seem livelier.

TECHNICAL DETAILS, ETC.

-----------------------

First of all, EGA�RAM does NOT contain a copy of the EGA BIOS; it

simply copies what is already in your machine (so I'm not violating

any copyrights or anything).  Just copying the BIOS to RAM doesn't

get you anything, though; it is necessary to tell the machine to use

the RAM copy instead of the one in ROM.  This is where things begin

to get a bit complicated.  Clearly we must fix up the INT 10h video

interrupt; as it happens, the EGA also uses several others, namely

interrupts 1Fh, 42h, 43h, and 44h.  Of these latter four, however,

only 1Fh and 43h actually point to the EGA BIOS, so there are really

only three interrupts to re-vector.	In addition, there is a pointer

in low memory (address 0:04A8h) to a table in the BIOS which itself

points to some other tables in BIOS.  The latter tables initially

contain only the parameter settings for each EGA video mode, but they

may be altered by the user to also include custom font and palette

information.  This is how programs such as KCSETPAL (Kent Cedola),

SPECTRUM and EGAITAL (PC Magazine), and WORD (Microsoft) work.  So

actually there are only two addresses to worry about besides the

interrupts, the one at 0:04A8h and the first entry in the table that

is pointed at by 0:04A8h.  (Confused yet?  Good.)

The conclusions in the previous paragraph are only valid if one fur-

ther thing holds, namely that no other programs have fooled around

with the video interrupts/addresses before EGA�RAM.	This stipulation

obvious but not trivial:  clearly TSR programs ("pop-ups") such as

SIDEKICK, SUPERKEY, and my own EGA�LQ are likely culprits and should

be loaded after EGA�RAM; but what about device drivers which do the

same thing?	ANSI.SYS is a good candidate; but surprisingly enough,

your mouse driver might be just as guilty.  In particular, Microsoft's

page 3

MOUSE.SYS and any Microsoft-compatible drivers (e.g. Mouse Systems'

MSMOUSE.SYS) also hook into interrupt 10h, for reasons I won't go into

here.  The upshot of all this is that EGA�RAM cannot simply be loaded

as an ordinary TSR -- it has to be loaded *before* any other video-

stealing routines, which is why it is written as a device driver.

In version 1.00 I included the additional stipulation that the EGA

BIOS be IBM's; since then I have had the chance to test EGA�RAM on

a couple of clone EGA cards and as a result, this requirement has

been dropped.  My thanks to Leonard Linde and Chett Salzman for noting

a bug involving the patch I had suggested.  The current version of

the program no longer checks for IBM's signature; it does, however,

check for the BIOS length-indicator byte at address C000:2 and copies

the appropriate amount of code to RAM, which is why it should work

with just about any EGA card out there, and possibly some VGA cards,

too.

Dyed-in-the-wool hackers may wish to check their interrupt table in

low memory to make sure there are no extra references to the C000h

segment where the EGA BIOS resides.	Even if some are found, your

system should still work (with some speedup); it will just go a bit

faster if all the interrupts point to the RAM BIOS, instead of having

some point to RAM and some to ROM.  Changing the program to accomodate

the additional vectors is simple:  add the appropriate interrupt to

IntrptTable (around line 105), and lengthen the loop at FixInterrupts

(around line 180) accordingly.  Of course, you need an assembler to

do this (I used Microsoft's MASM 5.1 for the current version), but

if you're really a dyed-in-the-wool code cowboy, you probably have

several.  I would appreciate hearing about any such findings.

DISCLAIMER

----------

EGA�RAM is provided as is; the user accepts any liability, etc., and

all that stuff.  In particular, I make no guarantee that the program

will work as stated, nor that it will not harm your machine.

I'm sorry that such a statement is necessary, but these are litigious

times, and I want to make sure my hiney is covered.

At one time I believed that it was impossible for software to damage

hardware, but jolly old IBM has created just such a possibility in

allowing monochrome monitors to be connected to color video cards; the

signals for color modes are not compatible with those for monochrome

modes, and one's monitor may be damaged in this way.  Thus there *is*

a possibility (almost inconceivably remote, but still there) that

EGA�RAM will inadvertantly interact with some strange BIOS code and

switch your card to the wrong video mode, thus frying your monitor.

Of course, this is a possibility with ANY program, particularly in

this day and age of large quantities of RAM-resident software....

Anyway, if you should happen to hear a high-pitched squeal from your

monitor or anything else unusual, TURN IT OFF IMMEDIATELY!!!  [I note

page 4

in passing that I've accidentally done this several times while working

on other programs, with no apparent ill effects.  Of course, I turned

the thing off pretty damn quick, too....]

There have also been recent reports of a Trojan horse program out in

California somewhere, which diddled with the 6845 CRT controller in

such a manner that it caused the attached monitors to catch fire (!).

I have included the source code to EGA�RAM, so you can see for your-

self that it doesn't do anything intentionally bad.

-----------------------

This program is free to all; the only thing I ask is that it not be

sold or used for any other commercial purpose, and that any modifications

that might be made are *specifically noted* in some form of documentation,

along with the name of whomever made the change.  I wouldn't mind a small

donation (say, $5) if you find EGA�RAM useful, but there is absolutely no

obligation in this regard, so technically this is not a shareware product.

Questions and comments are not only accepted but actively encouraged,

especially if you find an EGA card for which EGA�RAM doesn't work, or a

VGA card for which it does.	Feel free to drop me a line at:

Greg Roelofs

TAU Software Development

5433 South Cornell Avenue, Suite 3N

Chicago, IL	60615

(Note my company's snappy new name -- a bit more respectable than the

old one, eh?).  Or, leave me a note on either the Chicago Computer

Society BBS (312-942-0706) or Loren Jones's Chicago Board (312-352-1035).