README.DOC for AutoCAD Release 11 c1(386)

February 27, 1991

CONTENTS

General notes  .......................................   2

Plot spooling  .......................................   2

Networking notes  ....................................   4

International character (8-bit font) support  ........   5

Manual corrections

AutoCAD Reference Manual  ........................   9

AutoCAD 386 Installation and Performance Guide  ..  14

AutoLISP Programmer's Reference  .................  22

ADS Programmer's Reference  ......................  22

AutoCAD Tutorial  ................................  24

Also see the separate Advanced Modeling Extension documentation

update, README.AME

The AME option is not available for the SHOW version of AutoCAD.

See the printed "R11 c1 Update Detail Document" included with the

AutoCAD R11 c1 package for a list identifying the anomalies

addressed in this release.

AC11386RD-2                                                 Page  1 of 24

General notes

-------------

- Release 11 adds the following device support to AutoCAD 386:

- Summagraphics Microgrid digitizer

- Interrupt-driven choice for Summagraphics Microgrid and MM series

digitizers (offering greatly enhanced digitizer performance)

- IBM 8514/A display (integrated driver)

- COMPAQ Portable III plasma display (integrated driver)

- JDL printer plotter model 750

- Canon Laser Beam printer models LBP-8 and LBP-8II

- Enhanced CalComp plotter driver (now sends "beginning of plot" and

"end of plot" records).  This driver should now work with CalComp's

electrostatic plotters.

- AutoCAD 386 now writes prompts and progress messages to the screen much

faster than previous releases.  Plot to file and Hide are faster as a

result of this improvement.

- A utility program is provided to allow large programs to be run using

AutoCAD 386's Shell.  SHROOM.COM and its documentation file SHROOM.DOC

are supplied in the SAMPLE directory.

Plot spooling

-------------

- The ACADPLCMD and ACADPPCMD environment variable hooks for Plot Spooling

have been enabled for all platforms except the DEC VAXstation.

When plotting to a file named "AUTOSPOOL", AutoCAD generates a unique

file name in the configured plot file directory.  Then, if the ACADPLCMD

(for Plot) or ACADPPCMD (for Prplot) environment variable exists, it

composes an operating system command using the string defined by that

variable (substituting the plot file name for the first occurrence of

"%s" in the string), and submits the command to the operating system.

The spooler program (supplied by the operating system or a third-party

application vendor) is typically responsible for the following actions,

although it may do other things as well.

-  plotting the file as a background task.  A simple case would use

the DOS PRINT utility, but this only works for ASCII plot files.

(PRINT must be loaded as a TSR before running AutoCAD, and must be

directed to the parallel port to which the plotter is attached.)

AC11386RD-2                                                 Page  2 of 24

PRINT doesn't support XON/XOFF protocol, so serial ports can't be

used unless the plotter is wired for hardware handshaking.)

-  deleting the file upon completion to make way for more plot files.

Those are the typical requirements, although the actual functionality is

not monitored by AutoCAD in any way.

To use a plot spooler with AutoCAD:

A.  Configure AutoCAD to use either an internal driver or a P386 ADI

driver that uses AutoCAD's plotter I/O support (see the supporting

documents for your driver).  The spooling mechanism is activated

only if the plot is written to a file, so if you want to use

spooling by default, configure the plotter for plot to file.

(Real-mode ADI drivers have their own method for writing plots to a

file, and cannot use AutoCAD's plot spooling facility.  See page 53

of the AutoCAD 386 Installation and Performance Guide.)

B.  The configured plot spooler directory must exist, and both AutoCAD

and the spooler program need read/write access to it.  When

configuring, include the trailing directory delimiter character for

the system, such as "\" for DOS.

C.  Configure the default plot file name as "AUTOSPOOL", or enter

"AUTOSPOOL" as the file at plot time.

D.  Set the environment variable "ACADPLCMD" to a DOS command string

that invokes the plot spooler, with "%s" where the plot file name

should get filled in.

E.  If the operation of the plot spooler command depends on a TSR having

been installed, the TSR must be installed BEFORE you start AutoCAD.

Loading a TSR via execution of ACADPLCMD at plot time (or via the

Shell command) is likely to be fatal to both the TSR and AutoCAD, if

the TSR trashes any of the DOS Extender's interrupt vectors.

For example, if plots are to be processed by a SPOOLPL program on DOS,

the definition of ACADPLCMD might be:

SET ACADPLCMD=SPOOLPL %s

assuming that a plot spooler program named SPOOLPL is present on the

search path and accepts the plot file name as a parameter.

The mechanism is the same for the Prplot command, except the ACADPPCMD

environment variable is used.

NOTE:  Placing the definition in AUTOEXEC.BAT is advisable if you intend

to use plot spooling frequently.  However, in DOS batch (.BAT)

files, "%" is used to reference parameters.  If you set ACADPLCMD

or ACADPPCMD using a .BAT file, remember to use two "%" signs in

a row, as in:

SET ACADPLCMD=PRINT %%s

AC11386RD-2                                                 Page  3 of 24

Networking notes

----------------

AutoCAD Release 11 is designed to operate on a wide variety of networks.

Although several popular networks have been tested, no particular vendors

have been designated as "supported".  A few problem areas have been

identified:

- Directory permissions for the 386|VMM swap file location must allow file

creation and read/write operations to occur.  If the swap file cannot be

created, 386|VMM prints a message stating "Error creating swap file" and

AutoCAD exits.

- If you plan to execute AutoCAD while your current directory is one that

resides on a remote node, performance will be substantially better if

AutoCAD 386 is configured to place its VMM|386 swap file on a local disk

drive using the "-swapdir" switch documented on pages 76 and 170 of the

AutoCAD 386 Installation and Performance Guide (I&PG).

Furthermore, placing the swap directory on local storage is a workaround

for an apparent problem on Novell Netware 386, where running with a

current directory residing on a remote node often causes AutoCAD to exit

immediately after being called from the DOS command prompt.

- If you plan to run AutoCAD frequently from a drive mounted on a remote

node, you should also consider setting the "-nopgexp" switch to reduce

network traffic when paging program code.  This switch is documented on

pages 43 and 169 of the AutoCAD 386 I&PG.

- AutoCAD's file locking, and AutoCAD itself, will not run properly on

Novell Netware 286 if read-only directories are used.  There is no known

fix at this time.

- Novell NetWare 386 3.1 allows programs to open read-only files in

read-write mode if you have "READ ONLY COMPATIBILITY = On" in your

SHELL.CFG file, but reports an error when you try to write to the file.

In general, we recommend that you do not turn on this mode.  However, if

this mode is required by another software application that you use, you

will need to obtain the "FIXOPEN" NLM patch available from Novell, or

AutoCAD will not function properly when editing read-only files.

- SunOS 4.0 lacks the DOS file sharing support required for proper

operation of AutoCAD on PC-NFS networks.  Operation of AutoCAD in this

environment can result in bad data transfers.  You can work around the

problem by using the /SHARE option on the NET USE command when mounting

any network drives used to store drawing files.  (See your PC-NFS manual

for further information on the /SHARE option.)  SunOS 4.0.1 and later

releases fix the problem.

- On 3Com networks, library/support files such as ACAD.DWG, ACAD.MNX, and

TXT.SHX must be read-only in order to be shared by multiple users.  This

is also true of any other files to be shared on 3Com networks.  In

particular, the ACAD.PWD login file shared by users must be read-only.

However, the copy of ACAD.PWD in the server's temporary files directory

must be read-write-createable, as this file keeps track of the number of

users currently using AutoCAD.

AC11386RD-2                                                 Page  4 of 24

- AutoCAD holds the ACAD.MNX compiled menu file and any running or paused

script file open while you're in the drawing editor.  Thus, if you've

loaded the DOS SHARE utility and you use Shell to run a DOS command such

as COPY or DEL on any of these open files, you'll receive a Sharing

violation and be given an opportunity to abort the command.

International character (8-bit font) support

--------------------------------------------

The International versions of AutoCAD 386 Release 11 support 8-bit ASCII

text characters for the benefit of languages that require them (provided

that the host system, the supplied shape files, and the display drivers

support them as well).  8-bit ASCII support means that 8-bit ASCII text

characters can be displayed in AUI and on the text screen, as well as in

Text, Attribute, and Dimension entity strings.  There are no commands

specific to using 8-bit ASCII text; when an International version is

installed and configured with the proper display driver, more characters

become available.

The domestic (North American) version of AutoCAD 386 continues to support

only 7-bit fonts for AUI and text-screen characters.  (AUI dialogue boxes

use character number 128 as the "check" mark.)  If a text character with a

value greater than 128 is output to the display driver (in text mode or

through the AUI menus, dialog boxes, command line, etc), it is replaced

with a "?" character.  However, entity text strings can contain 8-bit

characters (shape numbers 32 through 255).

The following is a fairly complete technical description of AutoCAD's

8-bit ASCII support.  Drawings created with 8-bit and 7-bit versions of

AutoCAD are compatible with both versions of AutoCAD, with exceptions

noted in a later section.

I.  The features of the 8-bit ASCII (International) version of AutoCAD are

listed here.  Behavior of the 7-bit version for the same circumstances

is also listed when appropriate.

1. Built-in display drivers

In the 7-bit version of AutoCAD, built-in display drivers treat

character code 128 as the AUI "check" mark, and display "?" when

sent ASCII codes greater than 128.

In the 8-bit version of AutoCAD, built-in display drivers treat

character code 255 as the AUI "check" mark, and can display ASCII

codes greater than 127 for AUI text (such as dialogue boxes), the

command line, menus, and the text screen.  The text "glyphs" are

obtained from the active DOS code page, as described in DOS

reference manuals.

NOTE: The Swedish version of AutoCAD is 8-bit, but may use yet

another value for the check mark, as both 128 and 255 are

used for ordinary characters by some Swedish platforms.

AC11386RD-2                                                 Page  5 of 24

2.  ADI display drivers

ADI 4.1 protected- and real-mode display drivers now perform

additional communication with AutoCAD to determine:

- whether they are talking to a 7-bit or 8-bit version of AutoCAD,

and:

- If an 8-bit version of AutoCAD is in use, which character code is

used for the AUI "check" mark.

ADI 4.1 display drivers also tell AutoCAD whether they support

8-bit characters.  The 7-bit version of AutoCAD assumes that the

driver operates in 7-bit mode.  8-bit versions of AutoCAD can work

with either 7-bit or 8-bit drivers, but a 7-bit driver forces an

8-bit AutoCAD to operate in 7-bit mode.  All display drivers

written for ADI 4.0 and earlier operate in 7-bit mode.  See the ADI

4.1 Driver Development Kit manual for more information.

3.  Special codes for dimensioning symbols

The 8-bit and 7-bit versions of AutoCAD use different integer codes

for the characters representing the three special dimensioning

symbols.  The table below shows the names of these symbols, their

generic text control sequences, and their version-dependent integer

values.

Symbol            Control     7-Bit     8-Bit

Name              Sequence    Value     Value

---------------   --------    -----     -----

Degrees             %%d        127       248

Plus-Minus          %%p        128       241

Circle Diameter     %%c        129       237

Although it is possible to use the integer values for these symbols

directly in a text control sequence (as in %%127), or to enter them

using the ALT key (as in ALT + 1-2-7), these approaches don't

account for the differing values in the 7-bit and 8-bit versions of

AutoCAD.  Therefore, we recommend against using them if you want

your special symbols to be compatible with both 7-bit and 8-bit

versions of AutoCAD.

4.  AutoCAD dimension strings

In previous versions of AutoCAD, the dimensioning commands used the

control sequences "%%127", "%%128" and "%%129" to generate the above

special symbols.  In both the 7-bit and 8-bit versions of Release

11, these commands now use the "%%d", "%%p" and "%%c" sequences.

Thus, drawings created in one version can be loaded and displayed

without error in the other version.

AC11386RD-2                                                 Page  6 of 24

5.  Automatic conversion of Dimension entity strings in older drawings

When a pre-Release-11 drawing is loaded into Release 11's drawing

editor (from the Main Menu, or via an Insert, Xref, or Dxfin

command), its Dimensions may have text strings containing "%%127",

"%%128", and "%%129" control sequences for the three special

dimensioning symbols.  If any are found, they are translated

automatically to "%%d", "%%p", and "%%c", respectively.  Once you

End, Save, or Wblock the drawing, the new control sequences will be

permanently stored.  This conversion is not performed on Release 11

drawings.

6.  Automatic symbol mapping

When an 8-bit AutoCAD loads a font file and determines that it has

no entry for code 248, 241, or 237 (but that it does have an entry

for the corresponding 127, 128, or 129 code), the 127-129 shape

definitions are copied into their corresponding 8-bit codes.  In

this manner, existing drawings and 7-bit font files can be used with

the 8-bit version of AutoCAD without difficulty.

7.  Differing alphabetic character sets for different languages

Different sets of ASCII characters make up the alphabet in different

languages.  However, to ensure that drawings will be compatible

across different language versions of AutoCAD, symbol table (e.g.,

layer, linetype, text style) entries must use a common subset of the

available characters.  Currently, this subset is the upper-case

Roman letters "A" through "Z", the digits "0" through "9", and the

characters "$", "_" and "-".

8.  Command option keyword abbreviations

The 8-bit ASCII versions of AutoCAD support abbreviations for

command option keywords that begin after the first letter of the

keyword, when the allowed abbreviation is represented by the first

contiguous string of capitalized alphabetic characters shown in the

prompt.

For example, the keyword "nanTUCKet" could be specified by the

string "TUCK".

The 7-bit ASCII version of AutoCAD continues to require

abbreviations of command option keywords to begin at the start of

the keyword.  (Certain keywords, such as "eXit", are exceptions to

this rule.  In the 8-bit ASCII versions, this is typical.)

II.  Where 8-bit ASCII characters are defined

1.  The DOS Code Page facility is used to change the "glyphs" of all

ASCII characters in text mode (and for AUI text on most display

drivers).  Instructions for the use of Code Pages can be found in

the DOS Reference Manual.

AC11386RD-2                                                 Page  7 of 24

2.  The AutoCAD font files contain symbol definitions for the characters

in Text, Attribute, and Dimension entity text strings.  Entries for

ASCII character codes 32 through 255 have always been supported, but

many of the font files shipped with AutoCAD have only their first

129 characters defined.  There are exceptions, usually denoted by an

"8" at the end of their font name, such as ISO8.SHP/SHX.  So, if you

intend to use a full 8-bit ASCII character set, you may have to

extend some of the font files yourself, or obtain 8-bit font files

from other sources.  Defining the 8-bit characters is done in the

same manner as the existing 7-bit characters, and they can be added

to existing font files with no harm done.

III.  Compatibility notes and exceptions

1.  A drawing created in an 8-bit AutoCAD can be edited with a 7-bit

AutoCAD.  However, if the drawing has the character code 128 in

entity text strings, AUI dialogue boxes that display the text (such

as for Ddatte or Ddedit) will display a "check" mark in place of the

desired character.  (The AUI "check" mark is two character cells

wide, so it will obscure the following character as well.) This is

harmless, but looks ugly.  Other ASCII codes greater than 128 are

replaced by "?" marks in AUI text strings; this is also harmless.

2.  KANJI (Japanese) AutoCAD uses 7-bit codes, but certain codes are

interpreted as the first byte of a two-byte (16-bit) KANJI code.

Therefore, KANJI AutoCAD drawings will not display correctly on

8-bit ASCII AutoCAD, because all characters will be interpreted as

8-bit ASCII characters.

Conversely, 8-bit ASCII strings can be confused for KANJI 16-bit

characters in a 7-bit AutoCAD, although this is far less likely,

since the code must be defined in a BIGFONT shape file, etc.

3.  Drawings that reference font files containing definitions for

character codes 248, 241, and 237, when loaded into 8-bit AutoCAD,

will use those codes for the degree, plus/minus, and diameter

symbols when the AutoCAD dimensioning commands or other commands

enter the "%%d", "%%p", and "%%c" control sequences into drawing

entity text strings.

4.  The text control sequences "%%127", "%%128", and "%%129" are

converted to "%%d", "%%p", and "%%c" if found within Dimension

entities in a pre-Release-11 drawing.  Only Dimension entities are

converted, however.  Therefore, if such a drawing is loaded into an

8-bit AutoCAD, any non-Dimension entities containing these control

sequences may no longer display the desired symbols.  There should

be no problem in the 7-bit version of AutoCAD, however.

5.  At this writing, it is not clear that all languages can be

restricted to use of upper-case Roman letters for symbol table

entries, and thus be compatible with each other.

AC11386RD-2                                                 Page  8 of 24

AutoCAD Reference Manual corrections

------------------------------------

Chapter 2

- "Entity Selection".  In the description of the "Previous" option on page

62, the last sentence in the Note should read:

AutoCAD remembers the space (model or paper) in which each

selection-set is obtained; the "previous" selection-set is also

cleared if you switch spaces and attempt to use it in the other

space.

Chapter 4

- "Text Command", pages 129-134.  The description of the new text

alignment modes is incorrect, and should be changed as follows:

Page 129.  The last paragraph should read:

You can align text using any combination of

top/middle/baseline/bottom and left/center/right alignment modes.

Baseline refers to the line along which the bases of the capital

letters lie.  If you imagine a box surrounding the text, then

letters with descenders (such as p, g, and y) dip below the

baseline to the bottom of the box, and capital letters reach the

top of the box.  The middle position is halfway between the

baseline and the top.  The following figure shows the text

alignment positions.

Page 130.  The "middle" indicator in the figure should be halfway

between the baseline and the top.

Page 134, "Text M - Fully Centered Text".  The Note at the top of the

page should read:

The difference between this option and MCenter is that rather than

using the halfway point between the baseline and the top, Middle

uses the midpoint of an imaginary box surrounding the text string.

Therefore, its effect differs depending on whether the text string

has descenders or any characters that reach the top.

Page 134.  In Table 4-2, the notation "(same as M)" should be removed

from the MC entry.

Chapter 5

- "Entity Selection".  In the third paragraph on page 146, it should be

noted that UCSes whose extrusion directions are directly opposite each

other are not considered "parallel".

AC11386RD-2                                                 Page  9 of 24

- "Filleting Two Straight Lines", page 172.  The first paragraph's second

sentence is incorrect and should read:

It then trims the lines (retaining the "picked" segments if they

cross) and creates a fillet arc with the current fillet radius.

- "Undo - Notes and Interactions", page 201.  The following should be added:

Undo does not undo changes made to the CVPORT system variable by the

SETVAR command (or the equivalent AutoLISP/ADS functions).

Chapter 7

- "Color Numbers", page 260.  At the end of the fourth paragraph, "Mslide

command" should be replaced by "Vslide command".

- "Entity Handles", page 262.  The following should be appended to the

first paragraph:

Entity handles are often, but not always, assigned sequentially.

- "Which Operations Change Handles?".  The "Wblock *" command now retains

entity handles, so the last paragraph on page 263 should begin:

With the exception of "Wblock *", all forms ...

- "Ddlmodes Command".  Append the following to the first paragraph on page

283:

(Hatch blocks, Dimensions, and Blocks whose entities use color "by

block" won't adapt to transparent color changes until the next

regen, although no alert will appear.)

Chapter 8

- "Axis Command", page 291.  In addition to the restrictions noted in the

second paragraph, the Axis command works only when TILEMODE is on.

- "Object Snap Modes", page 307.  Add the following to the description of

"Nearest" mode:

Use of this mode with Circles and Arcs requires that their extrusion

direction be parallel to the Z axis of the current UCS.

AC11386RD-2                                                 Page 10 of 24

Chapter 9

- "Wblock Command".  The "*" form of the Wblock command now retains entity

handles, as well as all named views, User Coordinate Systems, and

viewport configurations.  The first paragraph on page 331 should now

read:

The entire drawing is written.  This is similar to the Save command

("Save Command - Updating Without Exit" on page 68), except that

unreferenced symbols (Block Definitions, layers, linetypes, text

styles, and dimension styles) are not written.  AutoCAD writes model

space entities to model space in the new drawing, and paper space

entities to paper space.

and the third paragraph on page 331 should now read:

With the exception of the "*" form of the command, Wblock sets the

output file's HANDLES system variable to 0 (disabled) and does not

write entity handles to the output file.

- "Using an Xref's Dependent Symbols", page 347.  In the second paragraph,

replace "though any changes" with "but if the VISRETAIN system variable

is set to 0 (the default) any changes".  Following the second paragraph,

add:

When VISRETAIN is set to 1, the current drawing's On/Off and

Freeze/Thaw settings for Xref-dependent layers take precedence.

They are saved with the drawing and are preserved during Xref reload

operations.

Color and linetype settings for Xref-dependent layers are not

affected by VISRETAIN, but are always reset upon reload of the Xref.

Chapter 10

- "Diameter Dimensioning", page 375.  In the paragraph beginning "The

third style ...", the last two sentences are incorrect and should be

deleted.

Chapter 11

- "Variables and Expressions".  Note #2 at the bottom of page 414 is

incorrect.  TEXTEVAL does not affect the Dtext command.

Chapter 13

- New "Processing entity: nnn" messages may appear during plotting if many

entities have been processed without generating plot vectors.  The new

message is simply meant to let you know that something's happening.

AC11386RD-2                                                 Page 11 of 24

Appendix A

- "Standard Prototype Drawing", pages 446-447.  The object selection

pickbox size is shown as being stored in both the prototype drawing and

AutoCAD's configuration file.  It is actually stored in the

configuration file only.

- "Modify Pull-down Menu", page 451.  The "Mirror" description incorrectly

states that Auto selection mode is used.

- "Options Pull-down Menu", page 453.  The "Donut Diameters" description

should read:

Invokes the Donut command, allowing you to set the inside and

outside diameters.

- "System Variables", pages 467-475.  The following new system variables

should be added to Table A-1.

DIASTAT    (integer, not saved)

Dialogue box exit status.  If 0, the most recent dialogue box

was exited via "CANCEL".  If 1, exit was via "OK".  (read-only)

PLATFORM   (string, not saved, read-only)

Indicates which version of AutoCAD is in use.  This is a string

such as one of the following:

DOS                   Sun4/SPARCstation

386 DOS Extender      DECstation

Apple Macintosh       Apollo Domain

Sun 386i              XENIX 386

Sun 3                 OS/2

VISRETAIN  (integer, saved with drawing)

If set to 0 (the default value), On/Off and Freeze/Thaw settings

for Xref-dependent layers are reset to their state in the

referenced drawing upon each reload.  When set to 1, the current

drawing's settings are preserved.

Appendix B

- "Extended and Expanded Memory Usage", page 481.  This section applies

only to the 640K DOS version of AutoCAD.  Use of expanded memory with

AutoCAD 386 requires a memory manager that conforms to the VCPI

interface standard.  See the AutoCAD 386 I&PG for further information.

- "Menu Bar and Pull-down Menus".  Two changes should be made to page 489.

The following should be appended to the second paragraph:

If the titles won't all fit in the menu bar, AutoCAD truncates

characters one by one from the longest title (checking each time to

see which is now longest), until they all fit.  This can result in

some silly-looking titles, so you should attempt to fit everything

in 80 columns if possible.

AC11386RD-2                                                 Page 12 of 24

and the last paragraph should be changed to read:

Each pull-down menu ordinarily appears directly beneath its menu bar

title.  However, the rightmost menus shift to the left to

accommodate long menu items.  The maximum number of items in a

pull-down menu is governed by the display device, and may be as low

as 21; be sure to consider this when designing custom menus.

- "Extended big-font", page 512.  The first two lines of the bigfont

format description should be replaced by:

*0, 5, font name

character height, character depth, modes, character width, 0

- "Command Aliases and Abbreviations", page 523.  The following note

should be added at the bottom of the page:

Aliases cannot be used in a command script.  Use of aliases in menu

files is not recommended.

Appendix C

- "HEADER Section", page 536.  $VISRETAIN should be added to the list of

DXF header variables (type 70; 1 = retain Xref-dependent visibility

settings, 0 = don't retain).

- "DIMSTYLE", page 537.  Delete the phrase "some or all of".  Add to the

listing: 146 (dimtfac), and 147 (dimgap).  Also, dimclrd, dimclre, and

dimclrt should be groups 176, 177, and 178 (not 67, 68, and 69).

- "LAYER", page 538.  Replace Table C-3 (LAYER group 70 codes) with:

Flag bit value               Meaning

--------------   --------------------------------------------------

0          Layer is On and Thawed.

1          Layer is Frozen.

2          If set, layer is frozen by default in new Viewports.

4          Unused

8          Unused

16          if set, layer is externally dependent.

32          if this bit and bit 16 are both set, this externally

dependent layer has been successfully resolved.

Xref-dependent layers are output during Dxfout.  For such layers, the

associated linetype name in the DXF file is always CONTINUOUS.

- "VIEW", page 538.  Add to the VIEW group code listing: If the first bit

(1) is set in the 70 group flags, this is a paper space view.

- "ENTITES Section", page 539.  Entities in the BLOCKS section can now

have handles, so the exception in the first paragraph should be deleted.

- "Block", page 541.  Group 3 (Block name) should be removed from the list.

- "Attdef" and "Attrib", pages 542-543.  Groups 72, 74 are the horizontal

and vertical text justification modes, respectively.  Groups 11, 21, 31

(allgnment point) appear if group 72 or 74 is present and nonzero.

AC11386RD-2                                                 Page 13 of 24

- "Viewport", pages 545-547.  Group 68 value -1 should read "On, but fully

off-screen...".  In Table C-13, the first 1070 group should be described

as an integer whose value is 16 for Release 11.  Also, the "1003..."

entry near the end of the table should read:

The names of layers frozen in this Viewport.  This list may include

Xref-dependent layers.  Any number of 1003 groups may appear here.

- "Organization of Extended Entity Data", page 554.  The "World space

displacement" and "World direction" descriptions should read as follows:

World space       (1012, 1022, 1032).  Also a 3D point that is scaled,

displacement      rotated, and mirrored along with the parent (but not

moved or stretched).

World direction   (1013, 1023, 1033).  Also a 3D point that is rotated

and mirrored along with the parent (but not moved,

scaled, or stretched).

- "DXB File Format".  Byte code 255 should be removed from Table C-15 on

page 560.  It is valid for Binary DXF, but not for DXB.

Appendix D

- "Command Differences".  Add the following to Table D-3, pages 571-572:

Dim      The Radius and Diameter subcommands now issue an additional

prompt in some situations.  This may cause failure of some

old scripts and AutoLISP programs that use these commands.

Dxfin    All non-baseline/left justified Text, Attrib, and Attdef

entities read from the DXF file are now rejustified.

Fillet   When filleting two Lines that cross, AutoCAD now retains

the "picked" line segments.  In previous versions, the

shorter ends were trimmed, no matter what points you used

to select the Lines.

Osnap    Running Osnap modes are now ignored during entity selection.

Also add the following to the "Wblock" entry on page 572:

The "*" form of the Wblock command now retains all entity

handles, named views, UCSes, and viewport configurations.

- "System Variable Differences".  Add the following to Table D-4, page 573:

CECOLOR  In Release 10, this string contained "BYLAYER", "BYBLOCK",

a color number (e.g., "15"), or a standard color number and

name (as in "1 (red)").  The last form has been dropped, so

applications no longer need to cope with its embedded

blank.  The standard colors now appear simply as "1" - "7".

OSMODE   Running Osnap modes are now ignored during entity selection.

AC11386RD-2                                                 Page 14 of 24

AutoCAD 386 Installation and Performance Guide corrections

----------------------------------------------------------

- "Network Compatibility," bulleted item on page 2. Add this paragraph:

The International version of AutoCAD does not support server

authorization, which allows a single copy of AutoCAD to be shared

over a network by concurrent users. Instead, networking is

accomplished by installing and configuring a single-user copy on

each network node. The International version supports all other

networking-related features such as file locking and plot spooling.

- "AutoCAD 386 and Memory Managers", page 3.  Add to this section:

"HIMEM.SYS does not conform to the VCPI specification, and is not

compatible with AutoCAD 386."

- "Essential Hardware and Software".  In the second paragraph on page 8,

"8.7 megabytes" should be changed to "9.5 megabytes".  Also, it should be

noted that additional free disk space is needed by AutoCAD 386's virtual

memory system, especially when editing large drawings.

- "Solving the 80386 Chip Problem", The following replaces the entire

section on pages 8 - 9:

The "B Step" 80386 chips used in early 16-MHz, 20-MHz, and some 25-

MHz machines have bugs that appear only when executing 80387 (math

coprocessor) instructions while paging is enabled. This bug is in

the 80386 chip itself, not in the math chip. Please consult your

dealer to find out if you have this or any other problem associated

with the 80386/80387 chip set.

Technical background: These bugs are known as the Intel 80386

Erratum #17 and Erratum #21. The problems can occur while you use

AutoCAD and may result in the system locking. If you suspect you

have one of these problems, please consult your dealer. Your dealer

should check the 80386 CPU chip itself for one of the following

markings.

16-MHz chips affected: A80386-16    or    A80386-16

S40343             S40344

20-MHz chips affected: A80386-20

S40362

25-MHz chips affected: A80386-25

SX050

The only known solution is to replace a bad 80386 CPU chip with a

good one. Contact the hardware manufacturer of your computer for a

BUG-FREE 80386 CPU chip.

- "Supported Peripherals", pages 9 - 11. The following peripherals are

either not supported or are obsolete:

- Under "Digitizers", Houston Instrument tablets, HIPAD DT11AA and

True Grid 8000 Series, (described on page 112) are not supported.

AC11386RD-2                                                 Page 15 of 24

- Under "Plotters", the Alpha Merics Alphaplot (described on page 130)

is not supported. In the Houston Instrument DMP Series (described on

page 136), DMP-7, DMP-8, and DMP-29 are not supported.

- Under "Printer Plotters", the Okidata Printer Plotters, 84 Step 2

and 93 (described on pages 156 - 157), are obsolete. This means that

they will not be supported in future releases.

- "1.2 Megabyte Release Format" and "720K Release Format", pages 13-18.

The acad.pwd login file is not supplied on the release disks; it is

created during initial configuration.  Also, the ADS disk for AutoCAD

386 does not include a DDE directory.

- "Installation Steps", The second paragraph on page 24 (a bullet

concerning acadcfg.old) is incorrect and should be deleted.

- "Server Setup Before Installation", page 25. In the first paragraph,

the location of the temporary files is incorrect. The second sentence

should read: "By default, AutoCAD writes these temporary files in the

root directory of the drive containing the active AutoCAD executable,

acad.exe."

- "Reconfiguring AutoCAD", page 27.  Add the following:

Never swap ACAD.CFG configuration files (via a Shell or Files

command) while AutoCAD is running; doing so may cause unpredictable

behavior of AutoCAD.  In some cases AutoCAD will prematurely reach

the limit of concurrent users and can even fail to run at all.

- "Files Not Subject to File Locking", page 37. This section should

include files output through AME commands.

- "Configure for Authorized Maximum Only". This is a new section

that would follow "Server Authorization Code" on page 38:

You should not configure AutoCAD for fewer users than the

authorized maximum of your network license. For example, you

have installed AutoCAD with a license for ten users but only

six users are currently on the network. Configure AutoCAD for

your authorized maximum -- 10 in this example:

Enter the maximum number of users for this package <1>: 10

The configuration routine accepts any number of users without

checking. However, the server authorization code is derived

from this number and the serial number. If the number of users

is different from your network license, AutoCAD displays an

error message when you try to open a drawing. Be sure to enter

the correct number of users to avoid reconfiguring.

- "Network Configuration Steps", page 38. These are changes and

additions to individual steps:

- Page 39, step 5, at end of step: "If server authorization is

already configured, you should accept the default established

during server configuration. Continue to accept defaults to

the end of configuration."

AC11386RD-2                                                 Page 16 of 24

- Page 39, step 7, after prompt at end of step: "The path

specification you enter must indicate the same physical file

for all nodes."

- Page 39, step 8. Replace the last paragraph (on page 40) with

this text: "If the password is lost or forgotten, you can

either delete the acad.pwd file in the executable directory

and reconfigure Server Authorization or reinstall AutoCAD."

- "Changing the Login Name", page 41. First paragraph, first sentence,

should read: "The login name is stored in the acad.pwd file."

Additional paragraphs:

The login name is case-sensitive: this means that "william"

and "William" are different login names. The login name must

be at least one character in length and begin with a nonblank

character; spaces are permitted between characters.

The login name identifies owners of locked files. Each login

name must be unique during concurrent AutoCAD sessions. In

other words, avoid having two people log in as "maryb" at the

same time; this defeats the purpose of clear identification

of locked files.

- "The Acad.pwd File", page 42.  The two paragraphs in this section are

incorrect and should be replaced by the following:

Acad.pwd is the "login file" that contains encrypted authorization

data essential to running AutoCAD on both stand-alone and networked

machines.  AutoCAD creates the acad.pwd file during the server

authorization phase of the configuration process.  If the file

becomes corrupted, AutoCAD forces a reconfiguration and rebuilds it.

- "The Acad.pwd File on 3Com". This is a new section that would

appear on page 42.

You might want to keep a backup copy of the original acad.pwd

on your hard disk to use for future reconfiguration. For

example, copy it to another filename such as acadpwd.saf. If

acad.pwd becomes inoperable, follow these steps:

1. Delete the acad.cfg file (the configuration file) from the

network node's directory specified by the ACADCFG environment

variable.

2. Link to the read-write directory containing the executables

("execadmn", for example). Delete acad.pwd from that

directory.

3. Delete any copy of acad.pwd from the directory containing

server temporary files.

4. Copy the uncorrupted acad.pwd (acadpwd.saf in this example) to

the read-write "execadmn" sharename directory. Be sure to

rename acadpwd.saf to acad.pwd.

5. Start AutoCAD and follow the configuration procedure. This

creates a new acad.cfg and updates acad.pwd.

6. After you have completed and saved your configuration, link

the sharename "exec" for read-only access by concurrent

multiple users to the AutoCAD executables.

AC11386RD-2                                                 Page 17 of 24

- "The Ame.pwd File on 3Com". This is a new section that would

appear on page 42.

Follow these steps when you load the Advanced Modeling

Extension (AME) for the first time:

1. Change the permissions on the main AutoCAD directory to READ-

WRITE.

2. Follow the directions in "Loading AME the First Time" on page

35. This creates the ame.pwd file in the AutoCAD directory.

3. After ame.pwd is created, return the permissions of the

AutoCAD directory to READ-ONLY.

If ame.pwd becomes corrupted, repeat these steps to recreate

the ame.pwd file.

- "File Locking on Single-User Node", page 42.

- Add to the Important note (second paragraph): "With this kind

of network configuration, we STRONGLY recommend that you

enable file locking."

- Add this sentence: "When file locking is off, AutoCAD still

honors locks but does not create them."

The Ame.pwd File

----------------

This is a new section that would appear on page 42 of the existing IPG.

Like acad.pwd, ame.pwd is a "login file" that contains

encrypted authorization data essential to running the

Advanced Modeling Extension (AME) on both stand-alone and

networked machines. AutoCAD creates the ame.pwd file when you

load AME the first time. If the file becomes corrupted,

AutoCAD forces a reconfiguration and recreates it.

The ame.pwd file stores the authorization number and number

of users. It is functionally identical to acad.pwd.

- "Plot Spooler Directory", page 53. When you enter the filename

AUTOSPOOL during configuration, use all capital letters: "AUTOSPOOL".

- "AutoLISP Feature", page 55. Add this note: "AutoLISP must be enabled

to run the Advanced Modeling Extension (AME)."

- "Calculating RAM Requirements for AutoCAD 386".  This is a new

section that would appear on page 57.

To run AutoCAD 386 Release 11, your computer must be equipped with

at least 2 megabytes of physical memory (RAM), of which at least one

megabyte must be extended memory reserved for AutoCAD 386. The

AutoCAD 386 executable, acad.exe, uses most of the extended memory

requirement.

AC11386RD-2                                                 Page 18 of 24

When all available RAM is used, AutoCAD 386 begins to page unused

executable code and data to disk. The installation of more RAM will

improve performance by reducing the need for AutoCAD to utilize

virtual memory (page to disk). Most customers find that 4 megabytes

is the minimum needed for satisfactory performance in design and

production work. We recommend adding additional RAM as the single

most effective means of "tuning" your system for improved

performance.

Virtual memory is the sum of available physical RAM and the free

disk space on the partition containing the swap file. The total

amount of virtual memory required by AutoCAD 386 Release 11 cannot

be calculated precisely because it is dependent on the following:

the size and type of drawing being edited; the size of the

additional application programs (such as ADS and AutoLISP programs)

that are loaded and executed.

As a rule of thumb, the virtual memory used by AutoCAD 386 will be

at least three times the size of the drawing file being edited plus

the size of any executable code loaded by the 386|DOS-Extender. For

example, a one-megabyte (MB) drawing will require at least 4.9

megabytes of memory:

3 x 1MB    +    1.8MB    +  0.1MB   =        4.9MB

(3 x .dwg file) (AutoCAD 386) (AutoLISP) (minimum virtual memory)

These figures show the amount of virtual memory needed to enter the

AutoCAD drawing editor. Memory requirements can grow with

subsequent editing operations up to one or two megabytes. Disk

requirements can increase if the drawing file contains External

References, since these are stored in the drawing file in a compact

form and expand during an editing session.

If you are using AME, you need an additional 1.1 megabytes. ADI

drivers used in conjunction with AutoCAD 386 may also increase the

total memory required, since these are actually executable programs

competing for memory with AutoCAD.

- "The AutoCAD Shell Command".  The following should be added after the

fourth paragraph on page 63:

A utility program called SHROOM.COM is provided to allow very large

programs to be run via AutoCAD 386's Shell.  [ For details, see the

separate file SHROOM.DOC.  SHROOM.COM and SHROOM.DOC are supplied in

the SAMPLE directory. ]  Before running SHROOM.COM, load any TSR

programs (such as DOS PRINT) that you plan to use from the Shell

command.

AC11386RD-2                                                 Page 19 of 24

- "What To Do If AutoCAD Crashes", page 82.  This replaces the original

section on page 82, including the subsection "Network Node Crash".

A "crash" is any abnormal termination of AutoCAD. The

possible reasons include the following: power failure;

program error; rebooting your computer before you exit

AutoCAD.

After a crash, a few files are left on disk. They include the

swap file and its associated swap reserve file. By default,

AutoCAD places these files in the root directory of the drive

where you started AutoCAD. See "Swapdefdisk Switch" on page 76 for

further details.

The swap file has an 8-character name with no extension; the reserve

file has the same name with a .SWR extension. Use the DOS DIR

command to list these files. This is a representative example:

0F2B265F       397313    9-9-90   3:44p

0F2B265F.SWR   100000    9-9-90   3:43p

Other temporary files may be left in the drawing directory or the

temporary file directory, if one is configured. These have

extensions like .AC$ (for temporary files) and .ACK (the

lock file for .AC$), and possibly .ACL (the lockfile for .ACK).

Ordinarily AutoCAD erases the swap files when you exit from the Main

menu and deletes temporary files when they are no longer needed. If

the time stamp is not recent, you can safely delete them. If disk

space is not an issue, or if the files may still be in use by other

active sessions, you can also leave them in place. In general it's a

good idea to delete any old swap or temporary files you find, but

only after verifying that they are not part of any active editing

session.

Network caution: Before deleting these or any other temporary files

on a network, be sure all other users are logged off. Otherwise you

might delete files that belong to a session someone is currently

working on.

- Network Node Crash

After a crash occurs on a network node, you must execute AutoCAD

again using the same configuration file (acad.cfg) you used before

the crash. Usually this means running it from the same network node.

AutoCAD can then safely detect the crash and correctly update its

list of concurrent users. By the time you see the Main menu, the

acad.pwd file has been cleared, and you can continue editing or exit

AutoCAD.

If the configuration file is accidentally deleted, you need to do

the following: delete the acad.pwd file; reconfigure server

authorization. See "Revising Server Authorization" on page 40.

- "Special Instructions for the IBM CGA", page 89. The second prompt

should read: "C> dscga -v79"

AC11386RD-2                                                 Page 20 of 24

- "IBM Enhanced Graphics Display", page 92.  Append the following to the

first paragraph:

EGA cards with less than 256K video memory are not supported.  A

"semi" dual-screen mode occurs if you are using two video displays,

and the EGA is not the current display.  In "semi" dual-screen mode,

AutoCAD uses the EGA to display the drawing, the status line, the

menus, the dialogue boxes, and the command prompts, and uses the

other display for the text screen.  (In true dual-screen mode, the

other display is also used for the command prompts.)

- "Color Numbering Scheme for 1024 x 768 16-Color Mode", page 98. The

next-to-last paragraph should read as follows:

To each hue number, adding 0, 1, 2, or 3 will have no effect on

brightness. Adding 4, 5, 6, or 7 will make the color a dark hue.

Adding 8 or 9 to the number will make the color display as dark

gray. For example, colors 10 to 13 are bright red, colors 14 to 17

are dark red, and colors 18 and 19 are dark gray.

- "Configuration" for ADI Interface for Plotter Drivers, page 127.

The paragraph and prompt for plotter steps should read as follows:

These parameters are real numbers, used to compute the

plotter size in plotter steps. The calculated step count

cannot exceed 65535 (or 32767 for DXB files). If the

specifications exceed the limit, the following message

appears (after you specify the steps per inch):

** Error: Plot size multiplied by steps per inch cannot

exceed 65535. Specifications resulted in ?????.

- "Houston Instrument DMP Series", page 136.  The number of pens supported

on the DMP-62MP plotter has been increased from 6 to 8.

- "PostScript Laser Printers", page 138.  Replace the first paragraph with

the following:

AutoCAD 386 supports PostScript laser printers and plotters using

either a Centronics-type parallel I/O port or an RS-232C serial I/O

port.  If a serial port is used, set the printer for 9600 baud, even

parity, 7 data bits, 1 stop bit, and XON/XOFF protocol (some have a

choice of XON/XOFF or DTR).  See your hardware documentation for

specific installation instructions.

- "PostScript Laser Printers", page 138. Add this note to the end

of the section:

Note: PostScript now supports a variety of sizes. To obtain

correct plots on 8.5-by-11-inch paper, enter the following

during configuration for a PostScript printer:

Enter the Size or Width,Height (in Inches) <MAX>: 8.0,10.5

No spaces are allowed. These measurements are the reverse of

those given for size A on the configuration menu.

AC11386RD-2                                                 Page 21 of 24

- "Hewlett-Packard PaintJet", page 150. At end of section, add this

to the second bulleted note: "For B-size paper, you must use the

paper tray."

- "Noswfgrow1st Switch", page 170.  The following should be added:

The longer you remain in AutoCAD, the larger the swap file will grow

(up to a point).  If you often spend hours or days without exiting

AutoCAD, we suggest that you use the "-noswfgrow1st" switch so

you'll be less likely to run out of disk space during an editing

session.

- "Swap File Size", page 170.  In the second paragraph, "-maxswfize"

should be "-maxswfsize".

AutoLISP Programmer's Reference corrections

-------------------------------------------

- "(fix)", page 42.  If the result is outside the range of integers for

this implementation of AutoLISP, (fix) returns as a real.  For instance,

if the implementation uses 16-bit integers,

(fix 32767.5)   would return  32767

but

(fix 32768.5)   would return  32768.0

- "(*error*)", page 76.  The following should be added:

Your *error* function should NOT include any calls to (command).

- "Error Codes".  The description of ERRNO code 52 on page 169 is

incorrect.  It should be "Entity selection: null response".

ADS Programmer's Reference corrections

--------------------------------------

- "Common Characteristics of ADS Library Functions", page 8.  Add the

following to the end of this section:

ADS user functions and data type definitions begin with the prefix

"ads_".  Another prefix, "adsi_", is reserved for internal use.

- "Retrieving Extended Entity Data".  At the bottom of page 78, "static

struc resbuf" should be "static struct resbuf" (add missing "t").

- "Entity Data Functions".  In the example on page 71, the first argument to

ads_buildlist() should be RTDXF0 rather than 0.

- "ads_cmd", page 89.  In the second paragraph, RTSTRING should be RTSTR.

- "ads_cmd" and "ads_command", pages 89-90.  The following details should

be added to the descriptions:

AC11386RD-2                                                 Page 22 of 24

- ADS programs can pause for user input by using ads_cmd() or

ads_command() to pass a string containing a single backslash.  Since

the particular control string may change in the future, we suggest

that a preprocessor macro be used for this purpose.  For example:

#define  PAUSE  "\\"             /* a single backslash */

ads_command(RTSTR, "Zoom", RTSTR, PAUSE, RTNONE);

(In C, as in AutoLISP, two backslashes must be used to produce one

backslash in a string constant.)

- Certain AutoCAD commands (such as Trim, Extend, and Fillet) require

a "pick" point to be supplied along with the entity to be operated

upon.  In order to supply such entity/point data pairs, an ADS

program must use the sequence:

RTLB, RTENAME, <entity>, RTPOINT, <point>, RTLE

or

RTLB, RTENAME, <entity>, RT3DPOINT, <point>, RTLE

with ads_command().  A similar chain of result buffers can also be built

using ads_buildlist()) and passed to ads_cmd().  The 2D or 3D "pick"

point must follow the entity name, and the pair must be surrounded by

RTLB/RTLE.  For example, the following trims a line to a circle.

ads_point p1

ads_name first, last;

ads_command(RTSTR, "Circle", RTSTR, "5,5", RTSTR, "2", 0);

ads_command(RTSTR, "Line", RTSTR, "1,5", RTSTR, "8,5",

RTSTR, "", 0);

ads_entnext(NULL, first);               /* Get circle */

ads_entlast(last);                      /* Get line */

p1[X] = 2.0; p1[Y] = 5.0; p1[Z] = 0.0;  /* Set pick point */

ads_command(RTSTR, "Trim", RTENAME, first, RTSTR, "",

RTLB, RTENAME, last, RT3DPOINT, p1, RTLE,

RTSTR, "", 0);

- "The DOS Extender (AutoCAD 386) Environment", page 171.  The supported

version of MetaWare High C 386 is 1.62 (for 32-bit systems).  The Watcom

C 386 compiler is not yet supported.  A Watcom environment will be

provided in a future update.

- "Avoiding Trouble", page 163.  The following new topic should be added:

File Access Modes

For compatibility with the way AutoCAD and AutoLISP open files,

ADS programs should set the appropriate network file access modes

whenever opening files with the fopen() library function.

In MetaWare High C 386, this is done by #including <system.cf> and

setting a global variable just before calling the fopen() library

function.  If you're opening a file for writing or appending, use:

Sharing_mode = Deny_read_write;

AC11386RD-2                                                 Page 23 of 24

to prevent others from reading or writing the file while your

application is accessing it.  To open a file for reading:

Sharing_mode = Deny_write;

allowing others to read the file, but not write to it.  When

fopen() returns, reset Sharing_mode via:

Sharing_mode = Compatibility;

- "Symbolic Codes for On-Line Program Errors".  The description of error

code OL_ENTSELNULL (value 52) on page 185 is incorrect.  It should be

"Entity selection: null response".

AutoCAD Tutorial corrections

----------------------------

- "Creating Another Dimension Style", page 54.  In the example, "dimclre"

should be "dimclrd" to change the dimension line color.

- "Plotting to Scale in Model Space Viewports", page 166.  At the bottom

of the page, the ".25xp" example should read "1mm=4mm" rather than

"1mm=14mm".

- "Editing Viewports", page 170.  The third paragraph should begin:

Since we no longer need the 2-foot line ...

- "Standardizing the Height of Dimension Text in Model Space", page 179.

The arithmetic should read:

.18   (size in model space drawing units)

X  1/180   (XP zoom scale factor)

--------

=   .001   (size in plot units)

In the next large paragraph, "setting DIMSCALE to 96" should read

"setting DIMSCALE to 180".

- "Adding the Border and View Labels".  On page 195, in the paragraph

below Figure 6-46, "8-by-10-inch border" should be "32-by-21-inch

border".

AC11386RD-2                                                 Page 24 of 24

README File for AutoCAD AME 1.02

March 15, 1991

CAUTION: Dangling edges in AME 1.0 Solids

-------------------------------------------

The SOLCHAM command in AME 1.0 and 1.01 does not work properly for

straight edges of solidifed polylines or swept solids whose adjoining

faces meet at an angle of less than 90 degrees. It leaves the edge

behind as a dangling edge on the chamfered solid.

This bug have been fixed in AME 1.02.

Solids created by AME 1.0 that have dangling edges can be repaired

with the following procedure:

1. XLOAD AME 1.02 and load the affected .DWG file.

2. Do SOLPURGE(Bfile). (This has to be done as the first command)

The SOLPURGE Bfile option on the selected solid(s) will delete the

old boundary-file of the solid(s) and force a recomputation of the

boundary-file for the solid(s) during any subsequent AME command.

3. Save the drawing.

This will automatically eliminate the dangling edges.

AME Executables and AME.TXT

---------------------------

The ame and amelite executable files (ame.exp and amelite.exp for DOS386),

and the ame.txt file must reside in the same directory or AME will not be

able to locate the ame.txt file. The AutoCAD Install program will always

install them together.

International User Authorization (DOS386)

-----------------------------------------

When AME is used in conjunction with an International version of AutoCAD

Release 11, there is no initial prompt for the number of users, only for AME

Authorization Code.

AME Reference Manual corrections

--------------------------------

p.46, in Step 5:

The layer name "0-ph-2" should be "ph-2".

p.51, all occurrences (3) of "SOLAREAV" should read "SOLAREAU".

p.57, last line on the page should read:

"... current UCS except when extruding or solidifying -- then

the height is along the positive Z axis of the entities'

coordinate system (ECS)."

p.65, fourth paragraph, first sentence and

p.66, last paragraph, first sentence

list "3D Poly" as one of the entities that can be extruded

or revolved.  "3D Poly" should not be listed.

p.67, second paragraph, second sentence and

p.68, last body paragraph, third sentence - Should read

"...Solext command.", not "...Extrude command.".

p.76, under the heading "Revolution" there should be no prompt

"Specify taper angle of extrusion <current>:"

p.88, After the first paragraph the naming conventions for the layers

should read:

"PV-viewport handle        Visible profile lines"

"PH-viewport handle        Hidden profile lines"

The second paragraph contains an explanation of the SOLPROF

layer naming convention which is no longer valid.  Read

section on SOLPROF below for description of new convention.

p.94, first sentence under "SOLAXcol" should read:

"This variable sets the color of the MCS icon used in the Solmove

and Solchp commands."

p.95, first prompt - Should read:

"Hatch angle <0>:"

"Enter a valid hatch angle."

p.105, second paragraph, second sentence - Should read :

"See chapter 4,...",not "See chapter 5,...".

p.118, third paragraph, third sentence

"readme.doc" should read "readme.ame".

SOLPROF layer naming convention has changed

-------------------------------------------

SOLPROF previously created layer names that included the layer

of the solid and the current viewport id.  The layer of a solid

can be changed and the id of a viewport is not guaranteed for the

life of a drawing.

The Handle of the vport will remain constant.  The layer names

created by SOLPROF will now contain the letters "PV" or "PH"

followed by a "-" followed by the Handle of the vport (i.e. if

the Handle of the current vport is 4B the hidden entities would

be placed on a layer named "PH-4B").  The use of the viewport

Handle will assure that profile lines remain in the correct

viewport and that SOLPROF will update the information on the

SOLREV and SOLEXT reject 3D Polyline entities

---------------------------------------------

3D Polyline entities are not accepted as input to SOLREV or SOLEXT.

SOLREV and SOLEXT expect 2D Polylines and Circles.

SOLPROF command

---------------

The AME SOLPROF command ignores any non-solid entities that are selected.

No attempt is made to solidify such entities or inform the user that non-solid

entities were selected.

SOLPROF in perspective view

---------------------------

The SOLPROF command will not give correct results in perspective view.

Its algorithm is designed for parallel projections only.

Caution on accuracy of DXFOUT

-----------------------------

The default DXFOUT accuracy of 6 places is not sufficient for AME.

In order to reliably recover AME solids via DXFIN it is recommended

that 16 decimal places of accuracy are used for a DXFOUT of AME solids.

Caution on use of DXFOUT/DXFIN

------------------------------

Since all AME solids are internally generated blocks, using the ENTITIES option

of the AutoCAD DXFOUT command on solids will create DXF files that are pretty

much useless, as they will not have the solid BLOCK definition included.

Also, using DXFIN in a drawing with existing entities will not bring in

any solid BLOCKs in the DXF file and the resulting drawing will be incomplete.

Caution on AME Solids and AutoCAD Blocks

----------------------------------------

Caution should be exercised when inserting AME solids into AutoCAD blocks. AME

solids can be placed within blocks and the blocks can then be used as any other

AutoCAD block is used. However, the AME solid loses its identity as a solid

while it is within the block. It regains its identity when the block is

exploded, provided a SOLPURGE ERASED command has not been issued while the

solid was in the block. If a SOLPURGE ERASED command is issued while an AME

solid is within a block, the solid will lose its identity permanently. When the

block containing the AME solid is exploded, the solid will no longer update. If

you place AME solids within an AutoCAD block, and expect to explode the block

later and use the AME solids that were in the block, the SOLPURGE ERASED com-

mand should not be issued while solids are within the block. If you place AME

solids within a block, and do not expect to explode the block, then the

SOLPURGE ERASED command will operate as documented.

AutoCAD OOPS command and AME Solids

-----------------------------------

The AutoCAD OOPS command is used to unerase the last erased entity. If you

erase an AME solid, you can resurrect it using the OOPS command, provided you

have not entered the SOLPURGE ERASED command since the solid was erased. If you

entered the SOLPURGE ERASED command since the solid was erased, the OOPS com-

mand will unerase the AME solid (provided it was the last entity erased), but

if the solid is a complex (booleaned) solid, it cannot be updated because the

SOLPURGE ERASED command has removed its children. In this situation, you can

retrieve the solid by using the UNDO command. Undo back to the ERASE command

that erased the solid and it will again be a valid solid.

AME Solids/Dimensioning/Osnap

-----------------------------

Page 110 of the AME manual states that when dimensioning solids you cannot

"snap" to features within a block (solid). You can use Osnap options to

select points within a block, but you cannot select lines, arcs, or circles

within a block for dimensioning purposes.

Single edges as multiple pieces

-------------------------------

Some circular, elliptical, cone-cone and cylinder-cylinder edges might be bro-

ken into pieces even though the pieces appear to be continuous. This can occur

when extruding or revolving an arc segment of a polyline. The arc edge may be

broken into multiple edges.

Commands that will show the multiple pieces are SOLFILL, SOLCHAM, SOLFEAT, and

SOLLIST EDGE.

SOLCHP and LAYERS

-----------------

If you copy a primitive using the SOLCHP INSTANCE command, the instanced

primitive is created on the current layer. However, the DELETE and REPLACE

options of the SOLCHP command let you retain a primitive as a top level

solid on the layer on which it was originally created. You do this by answer-

ing "Yes" to the "Retain detached primitive? <N>: " prompt.

Restrictions of SOLFILL/SOLCHAM

-------------------------------

Only straight and circular edges can be filleted and chamfered.

For straight edges, the adjacent faces should be planar. For circular

edges, the adjacent faces can be planar, circular cylindrical, or circluar

conical.

For example, edges which have elliptical cones or elliptical cylinders

as adjacent faces cannot be chamfered or filletted.  This case may result

when trying to fillet/chamfer tapered extrusions of polylines consisting

of arcs.  An appropriate message is printed when the user tries to

fillet/chamfer such an edge.

Limits on SOLFILL radius and SOLCHAM distances

----------------------------------------------

When filleting or chamfering a solid's circular edge, it is possible to enter

a fillet radius or chamfer distance for which a fillet or chamfer cannot be

constructed. The SOLFILL and SOLCHAM commands will detect these cases and

prompt you for new values.

Specifically, two cases can lead to fillet or chamfer values being rejected.

First, if the radius/distance is so large that the fillet/chamfer extends

beyond the center of the circular edge, it results in a self-intersecting

solid which is not allowed. Second, if the circular edge has planes at the

end points which make an angle of more than 90 degrees to the edge, the fil-

let/chamfer may not intersect the end-plane for large fillet radius or cham-

fer distances and will be rejected.  If this angle is close to 180.0 degrees,

acceptable fillet radius and chamfer distances are very small. This will give

rise to fillet/chamfer primitives which have null wireframes, which may cause

confusion. In these circumstances, it's better to recreate the solid so that

the edges are tangential (angle is exactly 180.0 degrees) or the angle is

significantly away from 180.0 degrees.

After creating a fillet or chamfer, you can change its radius or distances

using SOLCHP SIZE command.

AME Command Synchronization

---------------------------

During the course of executing AME commands, the AME executable invokes

various AutoCAD commands via the ads_command function.  Occasionally, the

AutoCAD command may fail to execute properly and the system will abort to

the command prompt.  When this occurs, AME becomes unsynchronized and the

system will fail to recognize the next AME command that you try to execute.

In this event, re-invoke the AME command a second, or even a third time.

This will force the system to re-synchronize.  AME commands will then be

accepted.

It should be noted that exactly repeating the command that failed will cause

this situation to re-occur. One example of this may be seen when SOLUCS is used

with certain OSNAP modes set.  This condition results in duplicate points being

passed to the AutoCAD UCS command causing it to fail.

AutoCAD AME Applications Program Interface

------------------------------------------

Unique Solid-ids

----------------

Functions affected: ap_union(), ap_int() and ap_diff()

Solid-id's passed to the above mentioned functions should be unique (no solid-

id should be repeated). If a solid used in a boolean operation needs to be re-

used in another boolean operation, you must use ap_dupsol() to duplicate the

solid before re-using it.

Invalid sequence of events:

ap_box(1.0, 1.0, 1.0, &sol[0]);

ap_cone(1.0, 2.0, 2.0, &sol[1]);

ap_sphere(1.0, &sol[2]);

sol[3] = 0L;

ap_diff(sol[1], sol[1], &sol_diff); /* NOT VALID, Cannot use the same

solid-id twice in a boolean

operation */

ap_diff(sol[1], sol[2], &sol_diff);

ap_union(sol, &sol_union);      /* This is NOT VALID as sol[1] and

sol[2] are already used in ap_diff() */

Valid sequence:

ap_box(1.0, 1.0, 1.0, &sol[0]);

ap_dupsol( sol[0], &solnew[0] );

ap_cone(1.0, 2.0, 2.0, &sol[1]);

ap_dupsol( sol[1], &solnew[1] );

ap_sphere(1.0, &solnew[2]);

solnew[3] = 0L; sol[2] = 0L;

ap_diff(sol[1], sol[2], &sol_diff);

ap_union(solnew, &sol_union); /* VALID, solnew solids not yet used */

ads_command() and ap_name2sol()

------------------------------

If ads_command() is used to manipulate AME solids, you must use

ap_name2sol() to update the solid before calling any other API function.

Invalid sequence:

ap_box(1.0, 1.0, 1.0, &sol);

ap_postsol(sol, AP_POSTWIRE);

ap_sol2name(sol, name);

ads_command(RTSTR, "MOVE", RTENAME, name, RTSTR, "",

RT3DPOINT, p1, RT3DPOINT, p2, NULL);

ap_q_solinfo(sol, &info);  /* Wrong! Solid "sol" has been moved

but not updated. */

Valid sequence:

ap_box(1.0, 1.0, 1.0, &sol);

ap_postsol(sol, AP_POSTWIRE);

ap_sol2name(sol, name);

ads_command(RTSTR, "MOVE", RTENAME, name, RTSTR, "",

RT3DPOINT, p1, RT3DPOINT, p2, NULL);

ap_name2sol(name, &sol);   /* Update solid */

ap_q_solinfo(sol, &info);  /* Valid. Solid "sol" has been updated */

ads_entdel() Cautions

---------------------

ap_sol2name() returns the entity representing the solid in AutoCAD.

Deletion of this entity using ads_entdel() potentially deletes the

solid from the AutoCAD database.

ap_evalpmesh() and ap_evalwire() return the representations of the

solid which are used by AME. You must NEVER use ads_entdel() on

entity names returned by these functions.