07/19/89

REMOTE CONTROL  V2.0  Copyright 1989 by Charles Eglinton

Distributed by ELECTRONIC TECHNOLOGIES,

3985 S.Rochester Road, Suite H

Rochester, MI 48063

Compuserve ID:70337,1654

A/R BBS: (313)680-9154

A donation of $30.00 is requested for the continued use of REMCTRL

THE REGISTERED COPIES OF REMOTE CONTROL HAVE AN UNLIMITED CONNECT TIME.

THE DEMO VERSION ALLOWS ONLY FIVE MINUTES OF CONNECT TIME.

REMOTE CONTROL is a memory resident program which allows you to toggle

your computer's phone answering ability remotely.  REMOTE CONTROL

makes it possible for others to have normal access to your facsimile,

answering machine or voice line and for you to have private access to

your computer on the same phone line.

REMOTE CONTROL watches the incoming phone line and does one of the

following:

* IF MORE THAN TWO RINGS ARE DETECTED the computer will ignore the call

allowing the fax, answering machine or human on the same line to receive

the call.

* IF YOU RING ONCE THEN CALL BACK the computer will start your

communication software and allow the computer to answer.

* IF THERE IS A FALSE ALARM, that is, if someone rings the phone once by

chance and your computer doesn't get a call from another computer within a

specified time then it will reset itself automatically.

SPECIAL NOTE TO KIDS: Another phone device not mentioned is your mom.  You

can use REMOTE CONTROL to leave your computer on the line all the time.

Only you know the secret!  Your mom will still be able to get phone calls

from the neighbors and you will be able to get access to your home computer

from school. Cool!

SPECIAL NOTE TO COMPUTER LITERATES WITH COMPUTER ILLITERATE SPOUSES: Damn, you

worked all weekend on those reports, the boss needs them in an hour, and

you left them on your P.C. at home.  You can call home and you struggle for

twenty minutes with instructions to your spouse about how to load Crosstalk

XI in host mode or you could have used REMOTE CONTROL.

SPECIAL NOTE TO SINGLES: Damn, you worked all weekend on those reports, the

boss needs them in an hour, and you left them on your P.C. at home.  You

can call home and you struggle for twenty minutes with instructions to your

answering machine about how to load Crosstalk XI in host mode or you could

have used REMOTE CONTROL.

SPECIAL NOTE: Did I mention security? How can those hacker guys get access

to your P.C. if they don't know it's there?  REMOTE CONTROL keeps your

computer transparent to anyone who doesn't know how to activate REMOTE

CONTROL.

REMOTE CONTROL  V2.0  Copyright 1989 by Charles Eglinton

PAGE 2

ABOUT THIS SHAREWARE:

The demo version is crippled in the following way:  Only the first line

of the keyboard macro will be executed.  The second line will always boot

the system regardless of what is in the script.  A registration request

screen appears everytime REMCTRL is started which requires a keypress to

enter the program.  Also, the demonstration version of REMCTRL will only

allow a five minute phone call and then will reboot the computer.

Therefore, once the demo version of REMCTRL has be started you will only

be able to call in once from the road and only for a five minute phone

call.  When you register REMCTRL all of these restrictions are removed.

INSTALLING REMOTE CONTROL:

Although you can run REMCTRL from your fixed disk, a floppy disk copy

makes a lot of sense.  When you want to start REMCTRL you can insert the

bootable floppy disk version of REMCTRL and just restart the computer.

If there should happen to be a power outage or if the computer should

restart for any reason then REMCTRL will automatically restart.  When you

are not on the road simply remove the REMCTRL diskette for normal access

to your computer.

MAKING A BOOTABLE REMCTRL DISKETTE:

1) Make a DOS bootable diskette using the "FORMAT A:/S" command.

2) Copy REMCTRL.EXE to the bootable diskette.

3) Copy the CONFIG.SYS file from your fixed disk to the bootable diskette.

4) Make an AUTOEXEC.BAT file on the diskette which contains the following:

* The path information from your fixed disk

* The REMOTE CONTROL command line including the following REMOTE CONTROL

parameters: REMCTRL /C /T /SCRIPTNAME /ASCII

C is number of the RS-232 port where the modem is located (1 or 2)

T is the maximum seconds allowed for a call back before computer resets

SCRIPTNAME is the name of the script file that you would like to

execute to start the chosen communications program.

ASCII- is an ASCII character that is used by RemCtrl as a pace

character for execution of your stript.  If your script file halts

suddenly without completing then try adding one of the following to

the end of your command line: "/255" or "/254" or "/253".

* A typical AUTOEXEC.BAT file on the boot disk might look like this:

PATH C:\;C:\DOS;       'To set up the paths

REMCTRL/1/30/PROCOMM/254   'To use REMCTRL to start PROCOMM on COM1 and

and 30 seconds for a valid data phone call.

Use ASCII 254 as a pace character.

REMOTE CONTROL  V2.0  Copyright 1989 by Charles Eglinton PAGE 3

OPERATING REMOTE CONTROL:

1) Set your communications software to answer on the second ring.

2) Set your facsimile or answering machine to answer on the third or fourth

ring.

3) Put the REMOTE CONTROL diskette in the A: drive and restart your computer.

4) Leave both your computer and your facsimile (or answering machine) powered

up and attached to the same telephone line.

5) TO GET ACCESS TO YOUR FACSIMILE OR ANSWERING MACHINE: Just dial in as you

normally would.  If there is more than one ring REMOTE CONTROL will tell the

computer to ignore the call allowing the other device to answer.

6) TO GET ACCESS TO THE COMPUTER: Call once, hang up and wait 10 seconds for

REMOTE CONTROL to start your communications program.  If you do not call

back within the time you have specified on the command line then REMOTE

CONTROL will automatically restore facsimile, answering machine or voice

service.

7) RETURNING YOUR PHONE TO NORMAL OPERATIONS: Some communications programs

will allow REMCTRL to reset automatically, and some will not. Refer to

the summary on the following page for specific macro information.

REMOTE CONTROL  V2.0  Copyright 1989 by Charles Eglinton

PAGE 4

THE KEYBOARD MACROS:

A macro is a listing of keystrokes which are to be sent to the computer

as if they were typed at the keyboard.  REMCTRL uses the first line of

the macro script file to start your communication program and the second

line of the macro script file is used to exit the same communication

program. Normal keys are typed in the script in the sequence required to

start the communication program.  Special keys like the {ENTER} key are

typed in the script surrounded by curly brackets (as it is shown in this

text).  Keystrokes which require you to hold down the {ALT} or {CTRL}

keys should be preceded by the words {ALT} or {CTRL}, in the script file.

Function keys ({F1}, {F2}, etc..) should also be surrounded by curly

brackets when typed in the macro script.  One additional command, {BOOT},

can be used on the second line of the macro script to reboot the computer

after a REMCTRL communication session. Refer to the script files which

have been included for examples of working macro script files.

ABOUT POPULAR COMMUNICATIONS PROGRAMS:

Many communication programs such as PROCOMM and QMODEM have a host mode

which allows automated remote connection for file transfers but they do

not default to host mode on startup.  Generally, a parameter must be

passed through the comm program command line to start a script which

contains host mode instructions.  REMCTRL script files should use the

extension ".SCR".  Some sample script files for popular communication

programs have been included with this copy of REMCTRL.

PROCOMM: This is the program that the author uses.  Procomm doesn't take up

much disk space and is fairly intuitive. REMCTRL works exactly as it is

supposed to with PROCOMM.  The first line of the PROCOMM.SCR keyboard

macro starts up procomm in host mode and the macro on the second line

leaves host mode and returns to a dos prompt without rebooting the

system.  This is ideal for a SYSOP that is running multi-node bbs since

the REMCTRL macros for PROCOMM will not reboot between calls or interfere

with the other nodes.  Type the files PROCOMM.SCR and PROCOMM.BAT to see

the keyboard macros.

CROSSTALK: This program also works perfectly with REMCTRL.  A configuration

file named "!HOST.XTK" has been included with REMCTRL.  You should first

copy "!HOST.XTK" to the subdirectory where the Crosstalk files reside.

Type the files XTALK.SCR and XTALK.BAT to see the macros.

MIRROR: Mirror is a Crosstalk clone and should work using the same macros

and described for Crosstalk.  A slight change may be required in the

scripts for program and directory name differences.

PCAnywhere: This program also works well with REMCTRL with one exception.

REMCTRL doesn't reset after a session when the carrier is lost.  To

remedy this, the remote user can type "ACANCEL" at a dos prompt to reset

the host computer (or execute the REBOOT.EXE program also included with

this archive).  Once the host has been reset REMCTRL will reset also.

PCAnywhere will reset automatically in the case of "false alarms".  Type

the files ANYWHERE.SCR and ANYWHERE.BAT to see the macros.

REMOTE CONTROL  V2.0  Copyright 1989 by Charles Eglinton

PAGE 5

QMODEM SST: The REMCTRL macros do not work once Qmodem has started.

Therefore you cannot use the macro on the second line to exit QMODEM to

reset REMOTE CONTROL.  Instead, use {BOOT} on the second line to reset

the computer, resetting REMOTE CONTROL.  Also, QMODEM Host takes a bit

longer to activate then other host programs so be sure to allow enough

time for the host mode to become active before calling.  A host script

named "HOST.HST" has been included and must be in the QMODEM script

subdirectory for REMCTRL to work with QMODEM. Check QMODEM.SCR and

QMODEM.BAT for sample macro files.

QMODEM 4.0: QM4.0 Should work the same as QMODEM SST, unfortunately the

authors have made changes in the script language.  If anyone would like

to convert the existing QMODEM SST script to the QMODEM 4.0 format and

send me a copy I would appreciate it.

CARBON COPY: The memory resident nature of CARBON COPY causes conflicts

with REMCTRL which is also a TSR.  The startup script works fine but

REMCTRL is knocked out immediately upon activation of CARBON COPY. This

means that REMCTRL will not reset the computer if the call is a false

al

and it will not reset the computer  not reseREM

3) Copy tan one

Thereset autocomputer  notte fript wopute.SCRrire.rher2MODe command line including the following REMOTE CONTROL

Val

and it will and only for a five minute phone

call.  When k to the bootable diskette.

4) Make an AUTOEXEl.  WSome communications p REBOnT.X/xed disk

* The REMOTE CONTROL command line including ywherm your fixed her dh REM the time you have specified on the command linekPY: The memory resident ncopy

makes a lot of sense.  When you want to start RCTRL and just restart the DI up p 4

THE KEYB

QMODEM SST: The REMCr shou your fixed her dh REM the time you have specified on the comDour st restartCopy REMnfers but they do

nanocked out immediatelhost mode on startup.  Generally, a parameter must be

passeuthe second line to r< the same commun an ASCII character that is used by RemCtrl as a pace

cGenerao

nanocked ouNTROL

Vand the secondn    SCof CAR#�,Iire.rMCTRL (1 or 2)

T is the maximum seconds allowed for a call IM(1 or 2)

T is OT},

can be used on the secon�MOTE CONTROL

�eset

the host computer (or execute the REBOOT.EXE pro){BOOT},

can kaCTRL.  You 1 like to

execute to start the chosen communicationnecuro script filrrounded byE proONTRO2 pCRIPTNAME /ASCII

C is number  filrrounded byE proONTO NORMAL OPERATIONS: Some co�hen try addik 1 like to

execute to start the chosen communicationnecurDl back befo name OL

the nameIle thvion program.  Special key).  Once the host has been reset RMUN  'To set up thAcally in the case of "false alarms    exec       Rut ion programomputeimipace.BAT to see the macros.

REMOTE CONTROL  V:\DOS;       'To e halts

suddIle thvion prograre.rMCTRL (1 or 2)

sudI.d byE proONTRO2 pCRIPTNAME /ASCII

C is number  filrroo sta" or all allow.XTK" tocter that i�t the computer

after a REMCTRL communication session. ReI     'To e .SCR".  Some sample script files for p����������������������������������������������������for programto stalCII 254 as a pace character.

REMOTE CONTROL  V2.0  Copyrightfor a va keys sho to the sacluding ywherm your fixed her dh REM the time you have specwing theNSWERING as a pace cha RCTRionROL  V989 byose7 on startup.  Generally, a parameter must be

ote connection for file transfers      et autom procomm in host mode and the macro o�e computer

�kallow REMCTRL to reseOMPUTER: iROL  V989 byose7 on startup.: iROL  V989  is number comm in host mode and the macr�in

rinnd line includiOL.  Also, tion session. Re#r facsimile (or answering machtion programs such as PROCOMM and QMODEM have a host mode

ephone line.

5) TO GET ACCESS TO YOUR FACSIMILE OR ANSWERING kERING as  as a pace convert  you

normf CAR#�,Iire.rMCTRL (1 or 2)

T is the maximum seconds a,

can CONTROL  V2Iand linekkkkkk on the �the other device to answer.

6) TO GET ACCESS TO THE COMPUTER: convert  you7 should work usinahostut surroundeications programs

C programs

will allow REMCTRL to reset automaticum ihe keybowill autotartuartup.  Geneddik 1 lied heeseOMP) TO G9d some will not. Refer to

the summary on the following paosen coNTR serv fixe script  G9  c�������s

will alIire.rceiseOMuarM tltscN COPYwaarmst autotartpt feitEM.SCR riWERINGse macr�in

s

I pace charne the �teV:\DOSt cha TO G9d rt the ex isrl

am,

cG9d ROLNTROode

nd  {F2 saLICENSEdI.d bha TO G9d CCESS iuter

aft the boon the �tust rtartupCCESwhen toe

uiatelhrMCc

o bTHEl  pro  worksine of en HomavhostOL.rtupCCld usen the �thCCESS iutr 2)

wing for sae wipogramcludi riWthe auynamed "HOSTsettMPUTER: convy umi  cG  makeamedROLomm ite {ALT be requtr 2)

rroun TO  seng RCPC

s

ated  make the       Tr 2)

rrrequtacations pst restart,THE C the

Tr 2)

rrand line to stt mode instrucaK" to the s scr

No  make the isifySS iuter

aftudiOL.on theayith MAL he oHE