A

Configurable

Disk Parking Utility

for

IBM PC clones

Copyright 1991 Dave Dunfield

All rights reserved.

Configurable Park Utility                                        Page: 1

1. INTRODUCTION

I recently put together an XT computer out of spare  parts  I  had

lying around, and ended up with a rather weird disk configuration.  I

had this old 12 Meg MEMOREX disk drive sitting on  the  shelf  for  a

number of years, which had 6 heads and  240  cylinders.  The  closest

drive table entry on the XT controller was 6 heads and 400 cylinders.

I was able to use the drive by doing a partial format of only  the

first 240 cylinders, and establishing a primary DOS partition  (using

FDISK) which was only 240  cylinders  in  size.  Thus,  although  the

system thought it was a 400 cylinder drive, it never had  any  reason

to attempt access to the cylinders which were not physically present.

All of the above worked perfectly,  until  I  tried  to  park  the

drive.... BUZZZZ... (Insert the sound  of  a  head  actuator  banging

against the drive stop about 160 times).

Since I had run into similar situations  (particularily  on  XT's)

where I didn't always have a park utility which put  the  heads  away

where I wanted, I decided to solve the problem once and for all,  and

sat down and wrote the PARK.COM program described  on  the  following

pages.

The source code to PARK is provided in the file PARK.C, and may be

re-compiled using my MICRO-C compiler (See enclosed CATALOG file). It

MUST be complied in TINY model, which allows it to easily  write  out

an updated copy of itself (See '-s' option).

The program may be freely used and distributed, provided  that  my

copyright notices are not removed or altered.

Configurable Park Utility                                        Page: 2

2. PARK.COM

The PARK.COM program  supplied  in  this  archive  allows  you  to

specify the number of hard drives to park, and the cylinder number at

which to park each drive.

In its simplest form, the PARK command  consists  of  one  decimal

operand for each drive in the system, which is the cylinder number at

which to park the drive. If  you  specify  a  cylinder  number  of  0

(zero), the highest cylinder number for that  drive  as  reported  by

BIOS is used:

PARK 0          ; Park drive C at highest known cylinder

PARK 240        ; Park drive C at cylinder 240

PARK 240 0      ; Park drive C at 240, drive D at highest

PARK 240 400    ; Park drive C at 240, drive D at 400

If the BIOS does not acknowledge the existance of a drive, it will

be ignored. The last two examples above would be equivalent to  "PARK

240" on a single drive system.

PARK.COM supports a '-s' option, which causes it to  write  a  new

PARK.COM which defaults to the remaining options. Executing PARK with

no arguments will PARK at these default settings:

PARK -s 240 0   ; Save the settings

PARK            ; Park drive C at 240, drive D at highest

You may see the currently defined default  settings  by  executing

PARK with the '-s' option, and no other arguments:

PARK -s         ; Display the default settings

The PARK.COM program supplied in this archive is defaulted to  TWO

drives, both of which are  parked  at  the  highest  cylinder  number

indicated by BIOS. This is equivalent to:

PARK -s 0 0     ; Default to drive C+D, highest cylinder

PARK will display a brief help summary if you give it  an  operand

of '?' or '-?'.

PARK ?          ; Display usage info

Configurable Park Utility

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

2. PARK.COM                                                            2