AT-SLOW.DOC

There are no implied warranties or guarantees with this software.  To the best

of our knowledge it is free from defect.

Provided by PCGURU BBS

Tested on 9/08/88 by David R. Foley

PC GURU BBS  (617) 783-5154  24 Hours 2400bps

AT-SLOW

PC-AT High Performance Slowdown

By David Keil

AT-SLOW is a utility which when loaded will use the high

resolution timer found in a PC-AT or close clone to slow down

games and other programs which run to fast on PC-AT's.

This program is hardware dependent and requires a PC-AT or

close clone to operate. AT-SLOW uses the AT's real time clock

found at interrupt 70h to slow down the AT's speed.

Using the real time clock's interrupt has two advantages

over the usual use of the normal PC's timer.

1 - The high resolution timer occurs 1024 times a second (as

opposed to 18.2 with the normal timer). This provides a much

smoother slow down than with the normal timer. This also

provides a higher degree of performance reduction.

2 - Since this slow down uses interrupt 70h and only AT's use

this interrupt, programs don't know to re-vector it. So

programs that don't work with mormal slow down programs

will work with AT-SLOW.

Using AT-SLOW:

The command for using AT-SLOW is

at-slow [/delay][/d][/h]

Where:

/delay = Amount of performance reduction.

/d     = Disables AT-SLOW

/h     = Show brief help message

The range of values for delay is 1-1000. Values of 300-400

on 286 machines is usually adequate. Care must be used when using

values over 400 (over 800 on 386 machines) as the computer will

be slowed down to the point of uselessness where it appears to be

locked up or if slowed down enough will lock up. If this occurs

the only solution is to re-boot the computer.

Once installed AT-SLOW can be disabled by using the /d

option. This option will disable the timer interrupt and return

the computer to normal speed. AT-SLOW can be removed with TSR

management programs such as MARK & RELEASE however care must be

taken to disable AT-SLOW first or the interrupt may remain active

and this will cause about a 10% reduction in performance. The

program uses very little memory however and when disabled does

not effect performance so leaving it in memory should not be of

great concern.

To reactivate AT-SLOW after disabling it or to change the

delay value just re-run AT-SLOW with the new delay value.

If you find AT-SLOW useful:

This program is copyrighted and may only be distributed

in unmodified form and with this DOC file included. Absolutely no

money may be charged for distribution of this program. If you

were charged for the disk that contained this software and have

the name of the company that did so please contact me.

A lot of time went into the finding of information which

enabled the writing of AT-SLOW, the Program itself is not that

complex. So if you find this program useful (and I'm sure you

will) and would like to support the development of more PC-AT

(and other high performance computers) related software

(here it comes). Please send a contribution of $5.00 payable to:

Better Software Co.

10 W. Wilburn Ave.

Greenville, S.C. 29611

Home (803) 295-4971

BBS  (803) 269-7899 after 1st of July   (1200-2400)

after 1st of August (1200-9600)

EOF