REAME FILE FOR IDE386.ZIP

This README file of IDE386.ZIP accompanies IDE.DSK for NetWare v3.11.

The following files are included for this revision:

IDE      DSK    13284   1-09-92   1:24p

IDE386   DOC    (This file)

This driver supports most IDE drives with Netware v3.11.  Along with other

IDE drives, this driver specifically supports the IBM PS/2 models 35 and

40.

INSTALLATION INSTRUCTIONS:

To install this Disk Driver:

1)    Copy IDE.DSK to the boot-up directory of the server. (Same location as

your SERVER.EXE)

2)    After the system is brought up,  example: LOAD A:IDE.DSK

NOTES:

The IDE disk driver was written to work with ATA/IDE disk drives.  The

ISADISK driver should not be used with these disk drives.  This driver

is a self configuring driver and does not rely on the BIOS for disk

drive support.

The driver supports Read-after-write verify at the software level.

Using the VERIFY ON mode will cause writing to the disk to be a

little slower than normal.

The driver will also support up to eight ATA/IDE disk drives in a

file server.  This is possible by using addressable paddleboards.

These paddleboards allow the user to configure the base I/O address

and IRQ.  (Primary I/O address at 1F0h and IRQ 14, second address at

170h, third address at 1E8h, and fourth  address at 168h.)  IRQs 9,

10, 11, and 15 may be used with the second, third, and fourth base

I/O addresses.  It is not recommanded to use IRQ 12 because of

possible conflicts with the mouse port.

Not all paddleboards will work correctly in all file servers.  Refer

to your hardware manufacturer or system integrator for recommended

configurations.

LIMITATIONS:

* The IDE disk driver will not work with ESDI and the ST506/412 bus

controllers.  The ATA/IDE, ST506, and ESDI interfaces look similar

to the system software; however, they are handled differently by

the disk driver.

* If a disk drive is to have a DOS partion installed, it will need

to be installed BEFORE the NetWare partion is installed.

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************