CQ-VHF

version 2.10

Notice

Better Software Co. reserves the right to make improvements in

the  product described in this manual at any time and without

notice.

This manual is copyrighted. All rights are reserved. No portion

of this manual may be copied, photocopied, reproduced, translated

or reduced to any electronic medium or machine form without prior

consent in writing from Better Software Co.

Copyright 1992, Better Software Co.

CQ-VHF Users Guide              1

Table of Contents

Introduction  . . . . . . . . . . . . . . . . . . . . . . . .   3

Features  . . . . . . . . . . . . . . . . . . . . . . . . . .   4

Installation  . . . . . . . . . . . . . . . . . . . . . . . .   5

Program Startup . . . . . . . . . . . . . . . . . . . . . . .   7

The Screen  . . . . . . . . . . . . . . . . . . . . . . . . .   9

Logging Function  . . . . . . . . . . . . . . . . . . . . .    11

Editing Log Entries . . . . . . . . . . . . . . . . . . . .    14

Grid Map  . . . . . . . . . . . . . . . . . . . . . . . . .    15

Direction and Distance Calculator . . . . . . . . . . . . .    16

Scheduling  . . . . . . . . . . . . . . . . . . . . . . . .    17

QSOs for a Call . . . . . . . . . . . . . . . . . . . . . .    20

QSOs from a Grid  . . . . . . . . . . . . . . . . . . . . .    21

CW Keyboard . . . . . . . . . . . . . . . . . . . . . . . .    22

Exit to DOS . . . . . . . . . . . . . . . . . . . . . . . .    23

Change your Grid/State (Rovers Only)  . . . . . . . . . . .    24

Backup Logs . . . . . . . . . . . . . . . . . . . . . . . .    25

Load CW Memory Buffers  . . . . . . . . . . . . . . . . . .    27

DOS Shell . . . . . . . . . . . . . . . . . . . . . . . . .    28

RECOVER.EXE . . . . . . . . . . . . . . . . . . . . . . . .    29

SUBMIT.EXE  . . . . . . . . . . . . . . . . . . . . . . . .    30

Appendix A  . . . . . . . . . . . . . . . . . . . . . . . .    32

Appendix B  . . . . . . . . . . . . . . . . . . . . . . . .    33

Appendix C  . . . . . . . . . . . . . . . . . . . . . . . .    34

CQ-VHF Users Guide              2

Introduction

CQ-VHF is a software product of Better Software Company of

Greenville, South Carolina. The initial release of the software

is for contest logging during the ARRL VHF contests. Later

releases  will be for the ARRL UHF and CQWPX contests. This

software runs  on most IBM or compatible PC's.

The software was designed by VHF contesters with many years

experience  and several Top 10 finishes. Until now no contest

logging software  supported multiuser, multiop stations. CQ-VHF

is available in single user and multiuser versions.

Better Software wishes to express our sincere appreciation to

Danny Taylor, N4DT for his input as a beta tester.

CQ-VHF Users Guide              3

Features

Fast log entry even with XT class computer

Will operate from floppy disk based systems

Automatic duplicate checking

Rover duplicates allowed

Display of bands needed as each grid is worked

Grid map with worked grids highlighted

Band score and total score always current

Built-in CW keyboard

Calculate direction, distance and reciprocal heading

for any grid

Contact scheduling feature with alarm

Display QSOs logged for a given call

Display calls missed from a grid for Rover operators

Multiuser version available for multiop stations with

message sending system

Output of ASCII files for submission to ARRL

Printouts of contest logs and score sheets

Automatic backup to floppy drive

Context sensitive help messages

Automatic backup to printer

CQ-VHF Users Guide              4

Installation

The CQ-VHF disk is supplied with at least eight files.

BTRIEVE.EXE         This is the file I/O

system.

CQ-VHF.EXE          This is the main logging

program.

CQ-VHF.HLP          Help file for ROVER.EXE,

contents displayed when

you press the F1 key.

CQ-VHF.DOC          This is the user manual

file for CQ-VHF.

CQ-VHF.BAT          This is the batch file to

start CQ-VHF.

SUBMIT.EXE          This program creates the

submission report and

disk file.

SUBMIT.HLP          Help file for SUBMIT.EXE,

contents displayed when

you press the F1 key.

RECOVER.EXE         Reconstructs data files

from the filename.DT6

backup file.

READ.ME             This file if present

contains the latest news

about CQ-VHF.

CQ-VHF Users Guide              5

To install CQ-VHF on hard disk systems, follow these easy steps:

1)   Create a directory on your hard disk drive.

MD CQ-VHF  <<ENTER>>

2)   Move to the new directory.

CD CQ-VHF  <<ENTER>>

3)   Place the CQ-VHF diskette in the A:

drive and copy the files to the new directory.

COPY A:*.*/V  <<ENTER>>

To install CQ-VHF on floppy disk systems, you should set up the

following diskettes.

1)   A bootable diskette with the BTRIEVE.COM file.  This

diskette is used to boot and load the ISAM file system.

2)   A diskette with CQ-VHF.EXE, CQ-VHF.HLP and COMMAND.COM.

This is the diskette you use during the contest.

3)   A diskette with the other programs.  This diskette is

used for file recovery and submission log creation.

CQ-VHF Users Guide              6

Program Startup

Once you are in the CQ-VHF directory type

CQ-VHF  <<ENTER>>

If you are using a color monitor.

CQ-VHF/M  <<ENTER>>

If you are using a CGA card with a composite monitor.

CQ-VHF/P  <<ENTER>>

If you are using a Laptop with a plasma screen.

You can also change the COM port the CW keyboard feature will

use.  The default is COM1.  To change you add a /2 for COM2, /3

for COM3, /4 for COM4.

Example:  CQ-VHF/M/2  this means you have a monochrome monitor

and want to use COM2 for the CW keyboard feature.

This will bring up the program and display the opening credits

screen.  Pressing the <<ENTER>> key removes the credits screen

and begins a series of prompts you must answer.

ENTER CONTEST NAME:

Enter the name you wish to call the files for this contest.  A

suggestion would be to use the date the contest begins, ie.

061392 for the 1992 June VHF contest.  The name you chose will be

the name used for all data files.

ENTER HOURS FROM UTC:

Here you enter the time offset from your computer's clock to UTC.

UTC is used for all logging functions.  If, for example, your

computer is on EDT the offset would be 4 hours.

ENTER STATION GRID:

Here you enter the grid square you are operating from. This

information  will be used in calculating direction and distance

to other grids.  If you are a Rover station, enter the grid you

are starting the contest from.  Later as you move to a new grid

square you can change this entry.

ENTER STATE:

Enter the state you are operating from.  Rover stations should

enter the state they are starting the contest in.  This entry can

be changed as Rovers move to a new state.

CQ-VHF Users Guide              7

ENTER STATION CALL:

Here you enter the call used by your contest station.

The grid map will now display using the defaults of your station

grid and band A (50 mhz). The scoreboard will be updated with

information from the data base.

You are now in logging mode.

At any prompt within CQ-VHF you may press F1 and get a help

message about the prompt you are on.  Pressing <<ENTER>> will

remove the help message.

CQ-VHF Users Guide              8

The Screen

The screen has seven sections.

Across the top line is the UTC date and time, your grid, state

and call for your reference.  Rover stations should carefully

check this line after each move to make sure they have the

correct grid and state entered.

Just below the reference line and to the left is the scoreboard.

Then to the right of the scoreboard is the grid map.

Below these sections on the left is the logging section. To the

right of the logging section is the multi-use section. This

section is used for the multi-user message system, for the

direction and distance  calculator, and for setting up contact

schedules.

Below these two sections are the function key definitions.  The

definitions will change if you hold down the ALT or CTL keys.

The ALT+function keys are used for additional features of CQ-VHF.

The CTL+function keys are memory buffers for the CW sending

feature.

Below the function key line is a message line used by the program

to prompt the operator.

CQ-VHF Users Guide              9

EDITING KEYS

When entering data the following editing keys are available.

Ins         -  Toggles between typeover and insert

modes.  In type over new text

overlays old. In insert mode text

to the right of the cursor is push

to the right, characters pushed off

the screen are lost.

Del         -  Deletes characters beneath cursor

and moves text to right of cursor

one character to the left.

Back Space  -  Deletes character to left of cursor

and moves text one character to

left.

Left Arrow  -  Moves cursor one character to left.

Right Arrow -  Moves cursor one character to

right.

Up Arrow    -  Moves cursor to previous input

prompt.

Home        -  Moves cursor to beginning of input.

End         -  Moves cursor to end of input.

ESC         -  Aborts changes to input and

restores original data.

CQ-VHF Users Guide              10

Logging Function

To log a contact, you must supply four pieces of data; band,

mode, call and grid.

BAND:

Enter the letter associated with the band you are working. The

definition  of the letters are on the scoreboard. The default

band will the  band last worked. To accept the default press

<<ENTER>>.

MODE:

Enter the mode of this contact.  AM, FM, CW, SSB are the allowed

modes.  The default mode is the mode last used.  To accept the

default press <<ENTER>>.

CALL:

Enter the call of the station you are working.  The program will

check for a duplicate and give you a warning message is it finds

a dup.  After entering the call, the current date and time are

also displayed. If you want to change to a date & time other than

the current, you press the 'up arrow' key and move back to time

ant then date.

GRID:

Enter the grid of the station you are working.

A message will now appear on the last line of the screen asking

you to confirm your entry. Pressing <<ENTER>> will begin the

second dup check and logging process.  If you typed something

wrong, enter 'N' and you can correct the contact prior to

logging.

DUP CHECKING & WRITING CONTACTS

Rover dups are  allowed. If the contact is a dup, a message will

be displayed on  the last line of the screen and the entry will

not be logged.  The  dup message includes the date and time of

dup contact.

If the entry  passes the dup check, the program will display any

bands needed in  the grid you just worked. This might allow you

to work another multiplier on a different band. The grid map and

scoreboard are also updated.

CQ-VHF Users Guide              11

If at any point during the logging process you decide not to log

the contact you may press ALT+F9 to clear all log entries and

return you to the band entry.

LOGGING FUNCTION KEYS

At any point in logging mode you may press a function key and

move  to another operation.

F1   Brings up a help message about the prompt you are on.

F2   Brings up the log data editor.

F3   Allows you to move or change the displayed grid map.

F4   Takes you to the direction and distance calculator.

F5   Takes you to the contact scheduling feature.

F6   Allows you to display the QSOs for a particular call.

F7   Allows you to display the QSOs missed from another

grid.  This feature is for Rover operation.

F8   Allows you to change the CW sending speed and spacing.

F9   Puts you in CW send mode.

F10  Exit the program.

CQ-VHF Users Guide              12

LOGGING ALT+FUNCTION KEYS

To use these function keys, hold down the ATL key while pressing

the function key.

F1   Brings up a help message about the prompt you are on.

F2   Allows you to change your grid and state.  Used by

Rover operators after moving to a new grid or state.

F3   Toggle off and on the printer for log entries.

F4   Toggle off and on the floppy disk backup of log

entries.

F5   Load the CW memory buffers.

F6   Toggle the scoreboard between current grid only and

totals for all grids.

F7   Currently not used in this version of CQ-VHF.

F8   Currently not used in this version of CQ-VHF.

F9   Clear the current logging entry.

F10  Shell to DOS to execute another command.

CQ-VHF Users Guide              13

Editing Log Entries

The log data editor is obtained by pressing the F2 key while in

the logging function.  Once selected, the grid map section will

be overwritten by the last twelve logged contacts.

The display shows the Band, Mode, State, Grid, Call, Date and

Time of the stations  worked with the first logged contact

highlighted.

The highlight bar is moved by using the Up, Dn, Pg Up and Pg Dn

keys. The Up & Dn keys move the highlight bar one entry up and

down at a time. If the bar is at the top or bottom of the list of

contacts, pushing an arrow key will display the previous or next

page of log entries. The Pg Up & Pg Dn keys display the previous

or next page of log entries.

If the highlight bar is at the last contact logged or the end of

the  log file then pushing the arrow keys will cause a beep and

will not  move the highlight bar, pushing the Pg Up or Pg Dn keys

will move the highlight bar to the top or bottom of that page.

EDITING A LOG ENTRY

Once the contact is highlighted, you may press, F3 to delete  the

contact completely or press, F2 to edit the contact. In  both

cases the contact is removed from the log, and the scoreboard,

grid map, and bands needed files will be updated.

If you chose to edit the contact (F2) the contact will now be

displayed in the logging section. You may now accept items that

are  correct and change incorrect items. You will also be allowed

to edit  the date and time, however once you have accepted them

you must press  F9 in order to be allowed to edit them again.

This editor is for making changes that you find during and after

the  contest. If you entered the data editor by mistake, press

<<ESC>> to exit the editor and return to the logging function.

LOG EDITOR FUNCTION KEYS

F1  -     Brings up a help message about the prompt you

are on.

F2  -     Deletes the log entry and places a copy of it

into the logging section, then returns to the

logging section.

F3  -     Deletes the log entry, then returns to the

logging section.

CQ-VHF Users Guide              14

Grid Map

The grid map will show at a glance what squares for a particular

grid  and band, you need to work. As you work contacts the grid

maps are  updated. Any map can be displayed. The worked squares

are marked  in reverse video.

The grid map display change option is obtained by pressing the F3

key. You can now use the arrow keys to move around the map.  The

grid map can be scrolled to the desired position. Scrolling  has

two advantages over entering the grid and band.  First, it allows

you set the grid map across grid squares. This is very useful if

the grid you are operating from is at a grid's border.  Second,

it allows you to move the map around one square at a time

allowing you to get an idea of the area you are working (much

like  moving a magnifying glass over a map).

The map can be scrolled one row or column at a time by pushing

the four arrow keys. The band can also be changed by pressing

the PG-UP & PG-DN keys. Once the desired grid area is displayed

pushing <<ENTER>> sets it and returns you to the logging

function.

To change to a particular map, you press F2 and supply two pieces

of data, grid and band.

GRID:

Enter the two letter prefix of the new grid square to display or

press <<ENTER>> to leave unchanged.

BAND:

Enter the letter associated with the new band you want to display

or press <<ENTER>> to leave unchanged. The scoreboard has the

definitions of the letters.

If you selected the map change by mistake, press <<ESC>> to exit

without changing the displayed map.

The grid map also has a unique toggle feature to compare grid

squares.  First enter the grid to compare by pressing F3 and

entering a grid square.  Once entered you can now compare the

entered grid to your current grid by pressing F4.

CQ-VHF Users Guide              15

Direction and Distance Calculator

This function helps you zero in on rare grids when you hear the

grid  square but don't know which way to point the antenna. After

entering  the target grid the direction to and from the target is

shown in degrees and minutes and the distance in miles.

ENTERING TARGET

The direction and distance calculator is obtained by pressing the

F4 key from then logging function.  To calculate you supply one

piece of data.

ENTER TARGET GRID:

Enter the grid square to calculate or press <<ENTER>> to use the

default value.

CQ-VHF now displays the heading to the grid, the reciprocal

heading from the grid and the distance between the grids.

If you selected the direction and distance calculator by mistake,

press <<ESC>> to exit.

CQ-VHF Users Guide              16

Scheduling

This feature is used to schedule contacts or events in the future

and to be told about them when their scheduled time approaches.

The scheduling function is selected by pressing F5 from the

logging function.  An alarm will sound informing the operator of

the sched at 15 minutes, 10 minutes, 5 minutes, 1 minutes and at

the scheduled time.

SCHEDULING A CONTACT

To schedule a contact or event, you can supply up to five pieces

of  data; freq, call, date, time and grid.

FREQ:

Enter the frequency of the contact you are scheduling. The

frequency  is not validated or required. You can push <<ENTER>>

without entering  a frequency if you are scheduling an event

rather than a contact.

CALL:

Enter the call of the station you are scheduling.  The call sign

is not validated and may be any text message ten characters in

length such as GAS GEN or SHIFT CHG if you are scheduling an

event rather than a contact.

DATE:

Enter the date of the scheduled event. The current date is the

default.  Press <<ENTER>> to select the default.

TIME:

Enter the time of the scheduled event. The current time is the

default  and must be edited.

GRID:

Enter the grid of the station you are scheduling.  This entry is

not required and may be left blank. The prompt is validated  if

anything is entered.

A message will now appear on the last line of the screen asking

you  to confirm your entry. Pressing <<ENTER>> will store the

event.  If  you typed something wrong, enter 'N' and you can

correct the event  prior to saving.

CQ-VHF Users Guide              17

SCHEDULING FUNCTION KEYS

At any point in scheduling mode you may press a function key and

move  to another operation.

F1        Brings up a help message about the prompt you

are on.

F2        Brings up the schedule data editor.

<<ESC>>   Returns to the logging section.

CQ-VHF Users Guide              18

SCHEDULE DATA EDITOR

The schedule data editor is obtained by pressing the F2 key while

in the schedule function.  Once selected, the grid map section

will be overwritten by the last twelve schedules.

The display shows the Date, Time, Call and Frequency of the

schedules with the first highlighted.

The highlight bar is moved by using the Up, Dn, Pg Up and Pg  Dn

keys. The Up & Dn key move the highlight bar one entry

up and down at a time. If the bar is at the top or bottom of the

list  of events, pushing an arrow key will display the previous

or next  page of schedule entries. The Pg Up & Pg Dn keys display

the  previous or next page of schedule entries.

If the highlight bar is at the last schedule or the end of  the

schedule file then pushing the arrow keys will cause a beep and

will not move the highlight bar, pushing the Pg Up or Pg Dn keys

will move the highlight bar to the top or bottom of that  page.

EDITING A SCHEDULE ENTRY

Once the scheduled contact is highlighted, you may press, F3  to

delete the event completely or press, F2 to edit the event. In

both cases the event is removed from the schedule.

If you chose to edit the event (F2) the event will now be

displayed  in the scheduling section. You may now accept items

that are correct  and change incorrect items.

This editor is for making changes when, schedules change and

entry  errors are made. If you entered the data editor by

mistake, press <<ESC>> to exit the editor and return to the

scheduling function.

SCHEDULE EDITOR FUNCTION KEYS

F1        Brings up a help message about the prompt you

are on.

F2        Deletes the schedule event and places a copy

of it into the scheduling section, then

returns to the schedule function.

F3        Deletes the schedule event, then returns to

the schedule function.

<<ESC>>   Aborts editor and returns to schedule

function.

CQ-VHF Users Guide              19

QSOs for a Call

The QSOs for a call display will show at a glance all QSOs for a

given call. This option is useful in several ways.  First, use it

to check if you have  made all the contacts with a Rover before

he moves on.  Second, check if  the contact you are about to make

is a dup.  Third, check if you have  contacted a station on all

bands planned.

DISPLAYING THE QSOs

The display QSOs for a call function is obtained by pressing the

F6 key from the logging function.  The grid map section will be

cleared and you will prompted for one piece of data; the call.

ENTER CALL:

Enter the call of the station who's QSOs you want displayed. The

default  will be the one that is entered in the logging section.

Once the call is entered the grid map section will be overwritten

by up to twelve QSOs for the call entered.  If there are no QSOs

for the call entered you will be informed of this and returned to

the logging function.

The display shows the Call, Band, Grid, Date and  Time of the

QSOs  for the entered station.

If there are more than twelve QSOs for a call the rest of the

QSOs can  be displayed by using the Pg Up and Pg Dn keys. The

Pg Up & Pg Dn keys display the previous or next page of QSOs.

When finished looking at the QSOs or if you entered the QSO

display  by mistake, press <<ESC>> to exit and return to the

logging function.

HINT:  For quick dup checking in the logging function; enter the

band and mode you are working.  Now you can enter calls heard and

then press F6 to put you in the QSOs for a call function.  The

call you just entered is the default.  Press <<ENTER>> and CQ-VHF

will quickly tell you if you have worked.  The call will remain

in the logging function so you can work and log a "new one".

CQ-VHF Users Guide              20

QSOs from a Grid

The QSOs from a grid display will show at a glance all QSOs from

a given grid that have not been worked form the current grid.

This allows Rover operators to make sure they have worked all the

stations they can before moving on to a new grid.

DISPLAYING THE QSOs

The display QSOs from a grid function is obtained be pressing F7

from the logging function.  The grid map section will be cleared

and you will be prompted for one piece of data; the grid.

ENTER GRID:

Enter the grid you wish to compare to the current grid.  Once the

grid is entered the grid map section will be overwritten by up to

twelve QSOs for the grid entered.  These will be stations not

worked from the current grid.

If there are more than twelve QSO's to display, the rest can be

displayed by using the <<Pg Up>> and <<Pg Dn>> keys.

This function is exited by pressing the <<ESC>> key.

CQ-VHF Users Guide              21

CW Keyboard

CQ-VHF provides a full feature CW keyboard.  The CW keyboard

requires a simple hardware connection to your radio.  A schematic

diagram for the hardware is provided in Appendix A.  You can also

purchase a manufactured version of the hardware from Better

Software (see the order form).

The character speed and spacing for the CW is set by pressing F8

from the logging function.  You enter a four digit number

followed by <<ENTER>>.  The first two digits indicate the

character spacing and the second two digits indicate the speed.

For example, 0520 would mean 20 WPM characters with 5 WPM

spacing.

To manually send CW text, you press F9 from the logging function.

Once selected, any characters entered will be sent to the

selected COM port.  Pressing F1 while in manual CW mode will

toggle off and on the computer speaker for sidetone.  F2 while in

manual CW mode will toggle on and off a continuous tone.

Ten CW memory buffers are also provided and are explained later

in the users guide.

CQ-VHF Users Guide              22

Exit to DOS

To exit CQ-VHF and return to DOS, press F10 while in the logging

function.  You will be asked to confirm your entry and then

dropped out of the program to DOS.

CQ-VHF uses ISAM files which are constantly updated.  There will

be no lost data when your exit the program.

CQ-VHF Users Guide              23

Change your Grid/State (Rovers Only)

Rover stations need the ability to change their grid or state

when they move.  This is done by pressing ALT+F2 from the logging

function.  You may now enter the new grid square or state.  The

scoreboard can be set to show only the score from this new grid

or all totals for all grids.

CQ-VHF Users Guide              24

Backup Logs

There are three types of backup logs kept by CQ-VHF.

The first is a text file called filename.DT5 and is kept in the

same directory as the other data files used by CQ-VHF.

The second is a optional hard copy printout identical to the

filename.DT5 file.

The third is a copy of the filename.DT5 file optionally copied

and maintained after each log entry on the A: floppy drive.  This

copy is called filename.DT6.

The filename.DT5 file is a text file kept for backup purposes. If

the data base used by CQ-VHF becomes corrupt, it can be rebuilt

from this text file.

The records in filename.DT5 must follow this format:

Position 1 - 4      Grid operated from

Position 5          space

Position 6 - 7      State operated from

Position 8          Band

Position 9 - 10     spaces

Position 11         Mode

Position 13 - 16    Grid Square

Position 17 - 28    Call Worked

Position 29 - 36    UTC Date as MM/DD/YY

Position 38 - 42    UTC Time as HH:MM

Position 44 - 46    Record Code: ADD for new contacts

EDT for edits

DEL for deletes

HARD COPY

The optional hard copy printout is toggled on & off by pushing

the ATL+F3 key. When this function is on the keyword PRT will

appear on the top line of the screen and each contact logged will

be printed on the printer.

Keeping a hard copy printout of the log is recommended in case of

computer  problems during the contest. With the printout you

should be able  to continue with the contest in case your

computer stops working.  You can also use a text editor to enter

the log entries from the printout  into a file called

filename.DT5 and recover the data base used by CQ-VHF in case you

loose all data on the computer.

CQ-VHF Users Guide              25

BACKUP OF filename.DT6 TO FLOPPY DISK

The filename.DT6 file can be copied and updated on floppy by

pushing  then ALT+F4 key from the logging function. After pushing

the ALT+F4 key you will be asked to insert a backup disk in drive

A:. This disk should be a blank formatted disk.  If you selected

this option by mistake then push the letter 'A' to abort, else

push <<ENTER>> and the filename.DT6 file will be copied to the

floppy and you will be returned the to logging function. The

keyword DSK will also appear on the top line of the screen and

all contacts logged will also be written to the floppy disk.

If a disk error occurs while copying the log file to the floppy

you will be informed of the disk error and allowed to retry with

another disk or abort the process. If an error occurs writing to

the floppy  while logging contacts the error will be displayed

and the program will be aborted. The program may the be restarted

with no loss of data.

DO NOT REMOVE THE FLOPPY FROM THE DRIVE WHILE THE KEYWORD DSK  IS

DISPLAYED AT THE TOP OF THE SCREEN.

Multiple backups of the filename.DT6 file can be kept by pushing

ALT+F4 when the DSK keyword is displayed to close the log file,

then  changing the disk, and pushing the ALT+F4 key again to copy

the log file to another disk.

Backing up to multiple floppies is recommended in case of

computer  problems. The backup log can be used to reconstruct the

data base  after a computer crash or it can be printed to

continue contesting  after a computer failure.

CQ-VHF Users Guide              26

Load CW Memory Buffers

CQ-VHF offers the user ten definable CW memory buffers.  To load

a buffer, press ALT+F5 from the logging function.  You can now

enter up to 120 characters of text to send.  Once you have

entered your text your are asked to supply a description of the

buffer.  This description can be up to six characters in length.

You are then asked to indicated which function key to store this

buffer under.  The description you supplied will be visible on

the screen any time you press and hold the CTL key.

To send a CW memory buffer all you need to do is press and hold

the CTL key and then press the function key associated with the

buffer you want to send.  If you want to stop the transmission,

press <<ESC>>.  F1 will toggle on and off the speaker sidetone.

You may edit previously entered buffers by pressing F2 while in

the CW memory buffer function.

There are several keywords that can be used in the CW memory

buffers.  These keywords are replaced at runtime with their

equivalents.

:1   your call

:2   your current grid

:3   the call you are currently logging

:4   the grid you are currently logging

CQ-VHF Users Guide              27

DOS Shell

CQ-VHF allows you to "shell to DOS" to perform DOS commands.  The

DOS shell function is obtained by pressing ALT+F10.  You are

instantly taken to DOS.  You return to CQ-VHF by typing "EXIT"

<<ENTER>>.  You will then instantly pop back into CQ-VHF.

CQ-VHF Users Guide              28

RECOVER.EXE

The RECOVER program allows the user to recreate the contest data

files from the file filename.DT5 or filename.DT6. This could be

used after a computer problem that corrupted your other data

files. You could also restore from a backup copy of the

filename.DT5 or filename.DT6 file.

filename.DT5 is a text file (see the section on Backup Logs for

layout) that you can  edit or create, enabling you to enter data

from a previous contest,  from a printed backup or a hand written

log and then use the RECOVER  program to create the other data

files used by CQ-VHF.

USING THE RECOVER PROGRAM

To execute the RECOVER program change into the directory that

contains the CQ-VHF programs and the filename.DT5 or filename.DT6

file you want to recover.

Then type :

RECOVER  <<ENTER>> at the DOS prompt.

The program will request that you <<Press any key to begin>>.

Once you push a key the program will initialize the data files.

The program will make two passes on the data to create the other

data  files. At the end of the second pass you are asked to

<<Press any key  to end>>. The CQ-VHF data base has been

restored.

The RECOVER program also allows an additional feature. If you

wish  to exchange logs with other users of CQ-VHF, simply send

then the filename.DT5 file. They can then use RECOVER to create

the other data files.

CQ-VHF Users Guide              29

SUBMIT.EXE

The SUBMIT program creates both hardcopy and disk file logs for

submission to the contest committee.

USING THE SUBMIT PROGRAM

Change to the directory where the CQ-VHF programs and the  data

files you want to create the submission log for are located.

Then type:

SUBMIT <<ENTER>> at the DOS prompt.

Once the program is started you must supply several pieces of

data.

STATION:

Enter the call used by your contest station.

OPERATING FROM GRID:

Enter the grid you operated from during the contest

CITY:

Enter the nearest city or landmark to your contest operation.

CQ-VHF:

Enter the name of the contest you operated in for this log. (ex.

June 1992 ARRL VHF QSO Party)

START:

Enter the date the contest started.

END:

Enter the date the contest ended.

CATEGORY:

Enter the category you are entering (ex. Single Operator)

CQ-VHF Users Guide              30

MULTIOP CALLS:

If you operated a multiop station, enter the calls for the other

operators.  Leave this area blank otherwise.

At anytime while you are entering the above information you may

push  F2 key to save the information to a disk file. Pushing the

F3  key will retrieve this information.  Use this feature so you

only  have to enter this information once even though you may run

the SUBMIT program several times.

After entering the last piece of information you are asked to

confirm  that the Data is ok. Push <<ENTER>> to continue

processing or push 'N' to edit the information.

You are the asked if you want a printed copy of the log. Answer

'Y' or 'N' to the question. In either case the disk files

SUBMIT.LOG & SUMMARY.LOG are created.

Once the program begins processing it sorts the log entries and

processes  the grid multipliers. If printer output was selected

the program asks  if the printer is ready and at top of form.

Press <<ENTER>> when the  printer is ready.

The summary sheet should be signed and submitted to the contest

committee  in all cases. Even if submitting your log on disk you

need to attest  to the data.

CQ-VHF Users Guide              31

Appendix A

The hardware to attach you computer to your radio for CW

operation is detailed below.  The parts are all readily available

at Radio Shack or hamfests.  Should you desire to purchase a

manufactured version of the hardware please see the order form in

Appendix C.

CQ-VHF Users Guide              32

Appendix B

CQ-VHF is available in a demo version for those wishing to try

the software before purchasing.  If you obtained your copy of CQ-

VHF from a computer bulletin board it is the demo version.  This

version will give you a feel for how CQ-VHF works.

The demo version is supplied with a sample contest data base.  To

use the sample data enter 060891 for the contest name at program

startup.  Using this data you can see how the various functions

and displays perform.  This sample data is from a Rover station

and will have contact made from several different grid squares.

We may also create your own blank data base by supplying another

contest name at startup.  We will be allowed to enter only 25

contacts and the editing function has been removed.  This should

let you see how easy CQ-VHF is to use.

The RECOVER and SUBMIT programs described in the users guide are

not included in the demo version.

The latest version of CQ-VHF with full features and the RECOVER

and SUBMIT programs is available by completing the order form in

Appendix C.

CQ-VHF Users Guide              33

Appendix C

To order the complete and latest version of CQ-VHF please

complete the order form below.  Each copy of CQ-VHF will be

shipped with a complete, illustrated Users Guide.

Order Form

Send _______ copies of CQ-VHF software at $25.00 each.

Send _______ CW computer to radio interfaces at $25.00 each.

Include $3.00 shipping and handling with each order.

Name:  ___________________________________ Call:  ___________

Address:  ___________________________________________________

City/State/Zip:  ____________________________________________

Mail the completed order form along with payment to:

Better Software Company

10 Wilburn Avenue

Greenville, SC 29611

CQ-VHF Users Guide              34