* * * * *    P C - I N F O   * * * * *

VERSION - 5.10

10-14-92

PC-INFO is a utility program written for users of PC/MS DOS

to display various tables and control blocks found

in the operating system.  There are 19 separate

information selections available from the main menu

as follows:

1.  List the Contents of the BIOS Data Area

which is located at address 0040:0000.

2.  List the contents of CMOS for AT type

machines.

3.  List the options of the math coprocessor,

if present.

4.  List the number and sizes of the disk partitions

on the hard disk(s).

5.  List the physical characteristics and space

information of all disk(ette) drives.

6.  List the contents of the BIOS Disk Paramter

tables.

7.  List the device assigned device drivers in

your system.

8.  List the contents of the Environment area.

9.  List the amount and use of Expanded Memory

(EMS) in your system, if present.

10.  List the amount and use of Extended Memory

in your system, if present.

11.  List the number and usage of DOS's FCB's and

Files.

12.  List the amount of free conventional, expanded and

extended memory available in your system.

13.  List the usage of conventional memory in your

system.

14.  List the current setting for parallel, serial,

mouse and audio ports in your system.

15.  List the various ROM Signatures found in your

system.

16. List the system information.

17. List the Upper Memory Blocks (UMB's) in your

system.

18. List the interrupt vectors and assignments

in your system.

19. List the current video mode, video adapter

and VESA settings.

Requirements:  An IBM compatible PC/AT machine with at least

64K of available memory and a version of

DOS 2.0 or greater.  If a mouse is available,

the program will detect it and use it.  If you

wish to disable the mouse, use the "-M"

parameter option.

Starting Instructions:

(1)  To start  PC-INFO,

just enter "PC-INFO"

(2)  To list the various parameter options

of this program,

enter "PC-INFO -?"

NOTE!   If your color monitor does not behave properly,

use the -b parameter which will force the use

of monochrome colors.

LISTUTIL Messages:

.  NO Upper Memory Blocks (UMB) Found.

Explanation:  PC-INFO was not able to detect any

UMB's in your system.  Currently,

this utility will only detect UMB's

associated with Quarterdecks's QRAM

or QEMM.

.  ERROR: General Error Occurred When Reading Boot Record

on Drive  X

Explanation:  An error occurred while attempting to

read the boot record on drive "X".

This may occur if a diskette is not

formatted.

.  ERROR: Unknown Disk Format Encountered on Drive X

Explanation:  PC-INFO detected an non-standard

DOS formatted on the specified drive.

.  ERROR:  Not Ready Condition on Drive X

Explanation:  "When PC-INFO attempted to read

information on Drive X, the drive

was not ready.

.  ERROR: Failure Occurred When Reading Drive X

Explanation:  DOS returned an error condition when

PC-INFO attempted to read information

from drive X.

.  CMOS is Not Present on This Machine.

Explanation:  PC-INFO did not detect any CMOS on

the system.  Note:  PC's and XT's

do not have CMOS.

.  Incorrect DOS Version - Must be DOS 3.0 or Greater.

Explanation:  PC-INFO will not work on a DOS

version less than 3.0.

. ***** No Device Drivers Found *****

Explanation:  PC-INFO could not find any device

drivers.  This situation should not

occur unless you have a non-standard

version of DOS.

.  *** Extended Memory NOT Installed or NOT Working ***

Explanation:  PC-INFO could not find any XMS

memory manager on your system.  Check

your CONFIG.SYS file.

.  *** Expanded Memory NOT Installed or NOT Working ***

Explanation:  PC-INFO could not find any Expanded

Memory (EMS) on your system.  Check

your CONFIG.SYS file.