Connecting the Nintendo PowerGlove to a Standard Parallel Port

Written by Bernie Roehl, July 1992

(based on an article on page 288 of the July 1990 Byte)

What you're doing is building a cable that does two things: connects control

signals from your parallel port to the Nintendo PowerGlove, and provides

power (+5 volts and ground) to the PowerGlove.

The connector you're looking for has 7 pins; if you're looking at a 5-pin

connector, you've got the wrong cable.

I would strongly suggest getting one of the Curtis "Super Extendo

Cables", in order to avoid hacking away at the glove cable itself.

The wiring is as follows:

DB-25 Connector        Nintendo Cable

2                     2     (DATA CLOCK)

3                     3     (LATCH)

13                     4     (DATA)

18                     1     (GROUND)

In addition, you must supply +5 volts to the Nintendo Cable on pin 7

(remember to connect ground (pin 1) to the supply's +5 volts!).

You can get the +5 volts from the keyboard cable, or from an external supply.

So which pins are which on the Nintendo cable?  If you look end-on at the

connector you're building, as if you were the glove interface box, you will

see that the connector is not rectangular.  It has a bulge out at one corner;

the pin in the bulging corner is pin 1.  It's actually the first in a column

of four pins; these are pins 1, 2, 3 and 4.  Continue reading around counter-

clockwise and up the column of three pins; these are pins 5, 6 and 7.

(If you're not sure about any of this, go to your local library and get the

July 1990 issue of BYTE magazine; on page 288 you'll find all the info you

need, including a nice illustration and details about getting power from

your keyboard).

NOTE: I make absolutely NO WARRANTY whatsoever that any of the above actually

works as expected, and take NO RESPONSIBILITY for any damage to your computer,

your PowerGlove or yourself.

If you're not hardware-smart, you may want to contact a fellow named Jim

Brain -- he's selling little boxes that do it all for you.

Advantages of using his box:

- It handles all the overhead of talking to the glove, so your computer

doesn't have to.

- It deals with all the timing-related issues, so your computer doesn't

have to.

- It doesn't tie up your parallel port.

- It provides support for the Sega 3D glasses.

Disadvantages of using his box:

- It costs money (though not much).

You can contact Jim Brain through the sci.virtual-worlds newsgroup on

USENET; the email address I have for him looks like a student account,

so it will probably have expired by the time you read this.

Please don't contact me with questions about this; the above is the sum total

of all my knowledge of the PowerGlove interface, and virtually all of it came

from the Byte article.  If you've got questions, go to your library and look

up that issue.

PowerGlove is a registered trademark of Mattel, Inc U.S.A.

Nintendo is a registered trademark of Nintendo of America Inc.